| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0272 |
| --- | --- |

| Title: | Crosscheck of JVET-AC0064 (EE1-1.4: On adjustment of residual for NNLF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Renjie Chang | Email: | renjiechang@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the inference crosscheck results for JVET-AC0064 (JVET-AB-EE1-1.4), which adjusts the residual for NNLF’s output. In this report, three tests based on integer inference were crosschecked including test the proposed residual adjustment implemented on top of Filter Set #0, test the combination of chroma order adjustment (EE1-1.3) and residual adjustment (EE1-1.4) implemented on top of Filter Set #0 and test the combination of chroma order adjustment (EE1-1.3) and residual adjustment (EE1-1.4) implemented on top of Filter Set #1. Besides, there are two additional tests for testing chroma order adjustment (EE1-1.3) and residual adjustment (EE1-1.4) on top of Filter Set #1, respectively. It is reported that, the full crosscheck results for RA and existing partial results for LDB can exactly match.

Introduction

The JVET-AB-EE1-1.4 described in JVET-AC0064 [] study a chroma order adjustment method in NNLF, where NNVC-3.0 anchor (NnlfOption=0), NNVC-3.0 (NnlfOption=1) and NNVC-3.0 (NnlfOption=2) are used as anchors for comparison. In this crosscheck report, the performance of proposed method in [1] was tested compared to above three anchors.

Results

All the experiments are performed following the common test conditions in []. The inference framework is SADL with integer precision. The crosscheck results are shown in the following tables.

EE1-1.4.1

Table 1. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=1) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.01% | -2.18% | -0.35% | 100% | 100% |
| Class A2 | -0.06% | -0.78% | -0.12% | 100% | 100% |
| Class B | -0.02% | -0.96% | -0.22% | 100% | 101% |
| Class C | -0.01% | -0.68% | -0.30% | 99% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -1.10% | -0.25% | 100% | 101% |
| Class D | 0.03% | -0.50% | -0.35% | 99% | 101% |
| Class F | 0.01% | -0.08% | -0.14% | 99% | 102% |

Table 2. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=1) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.44% | -1.61% | -0.50% | 100% | 104% |
| Class C | -0.54% | -2.24% | -1.02% | 99% | 104% |
| Class E | -1.32% | 2.02% | 1.19% | 100% | 135% |
| Overall | -0.69% | -1.89% | -0.73% | 100% | 111% |
| Class D | -0.14% | -2.44% | -1.64% | 99% | 102% |
| Class F | -0.19% | -0.96% | -2.31% | 100% | 105% |

Table 3. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.27% | -15.04% | -19.19% | 158% | 60053% |
| Class A2 | -9.56% | -20.11% | -14.84% | 153% | 57337% |
| Class B | -8.61% | -19.84% | -20.96% | 155% | 59455% |
| Class C | -8.63% | -21.48% | -20.98% | 134% | 48107% |
| Class E |  |  |  |  |  |
| Overall | -8.94% | -19.37% | -19.39% | 149% | 55896% |
| Class D | -9.71% | -20.40% | -21.41% | 133% | 42544% |
| Class F | -3.78% | -12.24% | -11.21% | 188% | 23492% |

Table 4. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -8.19% | -19.61% | -20.76% | 150% | 56821% |
| Class C | -8.75% | -21.92% | -21.29% | 133% | 47411% |
| Class E | -10.11% | -13.84% | -15.61% | 229% | 66815% |
| Overall | -8.86% | -20.64% | -20.99% | 160% | 55705% |
| Class D | -9.74% | -20.85% | -21.95% | 130% | 41848% |
| Class F | -4.36% | -11.56% | -11.08% | 190% | 25382% |

EE1-1.4.2 subtest1

Table 5. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=1) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.08% | -4.12% | -1.14% | 106% | 100% |
| Class A2 | -0.06% | -1.61% | -0.51% | 106% | 100% |
| Class B | -0.04% | -1.89% | -0.82% | 105% | 101% |
| Class C | -0.03% | -1.31% | -1.08% | 104% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -2.13% | -0.89% | 105% | 101% |
| Class D | 0.03% | -0.53% | -2.01% | 104% | 102% |
| Class F | -0.05% | -0.67% | -0.99% | 107% | 112% |

Table 6. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=1) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.60% | -3.82% | -1.79% | 105% | 105% |
| Class C | -0.73% | -4.38% | -3.19% | 104% | 104% |
| Class E | -1.47% | 0.56% | -0.37% | 109% | 137% |
| Overall | -0.86% | -4.07% | -2.42% | 106% | 112% |
| Class D | -0.22% | -3.86% | -6.06% | 104% | 102% |
| Class F | -0.34% | -2.95% | -4.23% | 107% | 113% |

Table 7. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.18% | -16.66% | -19.80% | 167% | 60262% |
| Class A2 | -9.55% | -20.71% | -15.15% | 162% | 57492% |
| Class B | -8.62% | -20.57% | -21.40% | 163% | 59587% |
| Class C | -8.64% | -21.99% | -21.59% | 140% | 48391% |
| Class E |  |  |  |  |  |
| Overall | -8.93% | -20.20% | -19.88% | 157% | 56094% |
| Class D | -9.71% | -20.43% | -22.66% | 139% | 42945% |
| Class F | -3.84% | -12.73% | -11.95% | 202% | 25812% |

Table 8. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -8.34% | -21.44% | -21.75% | 158% | 57397% |
| Class C | -8.92% | -23.67% | -23.02% | 138% | 47596% |
| Class E | -10.25% | -15.08% | -16.84% | 250% | 67812% |
| Overall | -9.01% | -22.43% | -22.32% | 170% | 56219% |
| Class D | -9.82% | -22.08% | -25.50% | 137% | 41883% |
| Class F | -4.51% | -13.26% | -12.92% | 203% | 27223% |

EE1-1.4.2 subtest2

Table 9. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=2) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.03% | -1.66% | -1.09% | 103% | 101% |
| Class A2 | -0.01% | -0.86% | -0.28% | 103% | 100% |
| Class B | -0.03% | -1.18% | -0.73% | 104% | 103% |
| Class C | -0.06% | -0.84% | -1.11% | 103% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -1.12% | -0.81% | 103% | 103% |
| Class D | 0.10% | -0.72% | -0.76% | 102% | 102% |
| Class F | 0.02% | -0.41% | -1.06% | 104% | 106% |

Table 10. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=2) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.40% | -3.76% | -1.73% | 103% | 107% |
| Class C | -0.40% | -4.16% | -5.34% | 102% | 106% |
| Class E | -1.35% | -2.25% | -4.66% | 105% | 126% |
| Overall | -0.64% | -3.94% | -3.34% | 103% | 111% |
| Class D | -0.06% | -2.58% | -2.57% | 102% | 102% |
| Class F | -0.37% | -2.45% | -4.63% | 104% | 111% |

Table 11. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.91% | -17.16% | -19.60% | 197% | 39972% |
| Class A2 | -10.24% | -20.87% | -17.65% | 189% | 38328% |
| Class B | -9.07% | -23.85% | -22.21% | 193% | 39929% |
| Class C | -9.97% | -22.42% | -23.14% | 164% | 35640% |
| Class E |  |  |  |  |  |
| Overall | -9.52% | -21.54% | -21.03% | 185% | 38430% |
| Class D | -11.39% | -24.15% | -24.50% | 158% | 32353% |
| Class F | -5.00% | -12.11% | -11.53% | 252% | 16398% |

Table 12. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -8.20% | -19.11% | -15.24% | 186% | 38176% |
| Class C | -9.51% | -18.91% | -19.24% | 163% | 35192% |
| Class E | -10.04% | -18.94% | -21.10% | 322% | 37546% |
| Overall | -9.10% | -19.02% | -17.02% | 204% | 37000% |
| Class D | -10.88% | -21.51% | -20.71% | 156% | 30659% |
| Class F | -5.29% | -11.54% | -10.66% | 253% | 17778% |

Additional test1 chroma order adjustment on top of Filter set #1

Table 13. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=2) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.02% | -1.53% | -0.86% | 104% | 101% |
| Class A2 | 0.00% | -0.80% | -0.20% | 103% | 101% |
| Class B | 0.00% | -1.02% | -0.62% | 103% | 102% |
| Class C | -0.01% | -0.63% | -0.94% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.97% | -0.67% | 103% | 101% |
| Class D | 0.03% | -0.70% | -0.70% | 102% | 101% |
| Class F | 0.02% | -0.36% | -1.00% | 104% | 104% |

Table 14. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=2) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.05% | -2.73% | -3.99% | 102% | 101% |
| Class E | 0.08% | -1.72% | -2.24% | 105% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -1.90% | -1.92% | 102% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 15. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.90% | -17.05% | -19.41% | 198% | 39878% |
| Class A2 | -10.24% | -20.82% | -17.58% | 189% | 38509% |
| Class B | -9.05% | -23.75% | -22.13% | 193% | 39350% |
| Class C | -9.92% | -22.26% | -23.01% | 164% | 34599% |
| Class E |  |  |  |  |  |
| Overall | -9.49% | -21.43% | -20.91% | 185% | 37960% |
| Class D | -11.45% | -24.14% | -24.46% | 158% | 31860% |
| Class F | -5.00% | -12.07% | -11.48% | 252% | 16131% |

Table 16. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -9.19% | -17.65% | -18.04% | 163% | 33715% |
| Class E | -8.72% | -18.55% | -19.04% | 322% | 30149% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -10.82% | -20.99% | -20.19% | 155% | 30215% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Additional test1 residual adjustment on top of Filter set #1

Table 17. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=2) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.01% | -0.38% | -0.46% | 100% | 101% |
| Class A2 | -0.01% | -0.16% | -0.16% | 101% | 100% |
| Class B | -0.02% | -0.29% | -0.18% | 100% | 102% |
| Class C | -0.05% | -0.33% | -0.30% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.29% | -0.26% | 100% | 101% |
| Class D | 0.07% | -0.03% | -0.09% | 100% | 101% |
| Class F | -0.01% | -0.20% | -0.13% | 100% | 102% |

Table 18. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=2) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.37% | -1.58% | -1.96% | 100% | 104% |
| Class E | -1.30% | -0.67% | -0.40% | 100% | 125% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.11% | -1.22% | -0.49% | 100% | 101% |
| Class F | -0.39% | -1.16% | -1.20% | 100% | 106% |

Table 19. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.90% | -16.21% | -19.10% | 192% | 39748% |
| Class A2 | -10.25% | -20.37% | -17.56% | 184% | 38271% |
| Class B | -9.07% | -23.22% | -21.80% | 187% | 39273% |
| Class C | -9.96% | -22.02% | -22.52% | 161% | 35177% |
| Class E |  |  |  |  |  |
| Overall | -9.51% | -20.93% | -20.61% | 180% | 38031% |
| Class D | -11.41% | -23.67% | -24.01% | 154% | 31991% |
| Class F | -5.03% | -11.96% | -10.70% | 243% | 15828% |

Table 20. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -9.48% | -16.66% | -16.28% | 159% | 34804% |
| Class E | -10.00% | -17.67% | -17.68% | 309% | 37191% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -10.93% | -20.41% | -18.93% | 152% | 30319% |
| Class F | -5.31% | -10.37% | -7.50% | 243% | 16968% |

Reference

- Z. Dai, Y. Yu, H. Yu, D. Wang. EE1-1.4: On adjustment of residual for NNLF. JVET-AC0064, 29th Meeting, by teleconference, 11–20 January 2023

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AB2016, 28th Meeting, Mainz, DE, 20–28 October 2022.