| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0277-v2 |
| --- | --- |

| Title: | Cross-check of JVET-AC0144 (EE2 2.2, 2.4a and 2.4b: Affine DMVR) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Médéric Blestel Xiaomi Technology Cesson-Sévigné, France | Email: | medericb@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results of EE2 Tests 2.2, 2.4a and 2.4b that are reported in JVET-AC0144.

In Test 2.2, the affine DMVR process is altered such that bilateral matching cost is calculated per sub-block. The method is used to determine all the control-point motion vectors for the affine merge candidate.

Test 2.4.a is a combination of Test 2.1 and Test 2.2.

The Test 2.4b is a combination of Test 2.1, Test 2.2 and the step 1 of Test 2.3, without extensions to affine MMVD and adaptive BM modes.

The cross-checked source code is consistent with the description of the proposal.

# Introduction

Cross-check of the source code and simulation results have been conducted. Inspected source code follows the description of the proposal.

# Tests descriptions

In Test 2.1, the non-translation parameters of affine model for affine merge coded blocks are refined. Firstly, the top-left CPMV is fixed as base MV of the affine model and the non-translation parameters are searched by minimizing the cost of bilateral matching. Then CPMVs are recalculated with the refined non-translation parameter values. Secondly, the new top-right CPMV is fixed as the base MV and the non-translation parameters are searched again to find new best values, and CPMVs are recalculated again after search. At last, the new left-bottom CPMV is fixed as the base MV and the same search process is applied on the non-translation parameters once again to get the final refined model. For each search point in the search process, motion compensation is applied to the whole CU and the SAD between two predictors of the CU are used as the cost. To reduce the complexity, different number of search rounds and search directions are set for different stages with cost based early termination.

In Test 2.2, the affine DMVR process is altered such that bilateral matching cost is calculated per sub-block. Then, the subblock bilateral matching costs and refined subblock MVs are used to determine the overall best refined control-point motion vectors for the affine merge block. The proposed method can be summarized as the following:

- Perform integer bilateral matching for subblocks. Accumulate the subblock bilateral matching cost to determine the best integer MV offset.

- Perform half-pel bilateral matching search using the best integer MV offset as initial offset and output the best MV offset that minimizes the bilateral matching cost for the same subset of the subblocks of step 1.

- Perform linear regression using the refined subblock MVs from step 1 as input and output a set of control-point motion vectors.

- Compare the bilateral matching cost of the output of the steps 2 and 3 to select the one with the smallest cost.

In Test 2.3, CPMV refinement for affine DMVR is proposed, wherein a different MV offset may be applied to different control-point motion vectors. The method may be applied in merge candidate of affine merge mode and affine MMVD mode.

Given initial control-point motion vectors initCpMvLX[cpIdx] with cpIdx=0..numCpMv – 1, wherein the numCpMv is the number of CPMVs of the current affine coding block. The proposed method can be described in the following two steps:

- Step 1: For each control-point, perform bilateral matching for a block that centered by the control-points to derive the refined CPMVs bmRefinedCpMvLx[cpIdx], as illustrated in Fig. 1. Loop over combinations of initCpMvLX[cpIdx] and bmRefinedCpMvLx[cpIdx], derive the best set of CPMVs that minimize the bilateral matching cost of the current block.

- Step 2: Iteratively further refine the CPMVs to minimize the bilateral matching cost of the current block. In each iteration, one CPMV is refined while the others are fixed.

The affine DMVR is extended to be used in affine MMVD, adaptive BM. In Test 2.3.a, only the first step is applied, in Test 2.3.b all the steps are applied.

Test 2.4.a is a combination of Test 2.1 and Test 2.2.

Test 2.4.b is a combination of Test 2.1, Test 2.2 and the step 1 of Test 2.3, without extensions to affine MMVD and adaptive BM modes.

Test 2.4.c is a combination of Test 2.1, Test 2.2 and all steps of Test 2.3 and with extensions to affine MMVD and adaptive BM modes.

# Cross-check results

## EE2-2.2 cross-check results

The test result 2.2 against ECM-7.0 is shown in the following table. Cross-checked results match with proponent results regarding Class B, Class C, Class D, and Class F contents.

Encoder and decoder run-times are not accurate.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,09% | -0,10% | -0,08% | 93% |  |
| Class C | -0,02% | -0,09% | -0,03% | 96% | 96% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0,00% | 0,01% | -0,06% | 96% | 97% |
| Class F | 0,01% | -0,04% | 0,01% | 95% | 99% |
| Class TGM | -0,02% | -0,02% | 0,02% | 95% | 98% |

## EE2-2.4a cross-check results

The test result 2.4a against ECM-7.0 is shown in the following table. Cross-checked results match with proponent results regarding Class C and Class D contents.

Encoder and decoder run-times are not accurate.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0,02% | -0,10% | -0,09% | 98% | 93% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0,04% | -0,10% | -0,15% | 102% | 99% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

## EE2-2.4b cross-check results

The test result 2.4b against ECM-7.0 is shown in the following table. Cross-checked results match with proponent results regarding Class B, Class C and Class D sequences.

Encoder and decoder run-times are not accurate.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,13% | -0,01% | -0,13% | 98% | 104% |
| Class C | -0,04% | -0,07% | -0,08% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0,06% | -0,17% | -0,12% | 98% | 98% |
| Class F | 0,01% | -0,03% | -0,04% | 96% | 99% |
| Class TGM | 0,00% | 0,01% | 0,03% | 96% | 100% |

# Conclusion

Tests 2.2, 2.4a and 2.4b from JVET-AC0144 were cross-checked and the BD-rates perfectly match with the result provided by the proponent. Inspection of the code shows that the implementation is consistent with the proposition documented in the contribution.

# Reference

- H. Huang et al, “EE2 Test2.1, 2.2, 2.3, 2, 4: Affine DMVR”, JVET-AC0144, January 2023, online.

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0.

# Patent rights declaration(s)

Xiaomi does not have any current or pending patent rights relating to the technology described in this contribution.