| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0278-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AC0194 (EE1-1.1: More refinements on NN based in-loop filter with a single model) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhihuang Xie | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results for JVET-AC0194[1], which is also known as EE1-1.1. Three tests were studied in EE1-1.1. Both training and inference crosschecks were carried out for EE1-1.1.1 and EE1-1.1.2, and only inference crosscheck was conducted for EE1-1.1.3. All these three tests investigated model retraining for filter set #0 and NNVC-3.0[2] is used for testing. Cross-check results are summaried as below:

- EE1-1.1.1 (model fine-tuning for NNVC-3.0 filter set #0):

Performance of crosschecker’s retrained model vs. tester’s model:

RA: 0.01% Y, 0.44% U, 0.12% V;

AI: -0.01% Y, 0.08% U, 0.01% V.

- EE1-1.1.2 (train model by new training scripts):

Performance of crosschecker’s trained model vs. tester’s model:

RA: 0.03% Y, -0.16% U, -0.66% V;

AI: 0.01% Y, 0.37% U, -0.35% V.

- EE1-1.1.3 (Real-iterative training based on EE1-1.1.2):

Model provided by tester, and results match.

# Introduction

JVET-AC0194 reports the results of EE1-1.1 (JVET-AB0083)[3], which is related to NNVC-3.0 (filter set #0). In order to further improve the coding performance of NNVC-3.0 filter set #0, two training tasks were studied and three tests were conducted in the EE. The first training task was carried out for fine-tuning model using new compressed dataset (EncDbOpt was enabled in NNVC-2.0). The second training task used new scripts to train model. In a word, EE1-1.1 contains three sub-tests, which can be summarized as below:

  - EE1-1.1.1: model fine-tuning (using training scripts in NNVC-3.0).

  - EE1-1.1.2: training by new scripts (using training scripts in EE1-1.1).

  - EE1-1.1.3: iterative training based on EE1-1.1.2.

In EE1-1.1.3, tester conducted the real-iterative training based on EE1-1.1.2.

Compared with the training scripts in NNVC-3.0, the new training scripts of EE1-1.1.2 contains the following difference/aspects, which may contribute to the improvement of model performance:

- Different dataset range.

The new training scripts of EE1-1.1.2 use the whole DIV2K dataset, the first 20 sequences in TVD dataset, and class B, C and D in BVI-DVC dataset, for training.

In contrast, the training scripts in NNVC-3.0 use the whole DIV2K dataset, the whole TVD dataset, and only class B in BVI-DVC dataset, for training.

- Different loss function.

The new training scripts of EE1-1.1.2 use L1 firstly then L1+L2 in the later training stage.

In contrast, the training scripts in NNVC-3.0 always use L1.

# Cross-check status

Crosschecker of EE1-1.1 conducted two training tasks and three inference tests, which can be described briefly as below:

- Training and inference crosscheck of EE1-1.1.1.

Crosschecker retrained the initial model provided by tester using the training scripts in NNVC-3.0. Guided by convergence curve in Tensorboard, model at 118th epoch was selected as the final model for inference crosscheck.

Summary: Training crosscheck was done. Compared with the results provided by tester, the performance difference between crosschecker’s and tester’s is negligible.

- Training and inference crosscheck of EE1-1.1.2.

Crosschecker trained the model from scratch using the new training scripts, which can be fetched from Git-EE1-1.1[4]. Guided by convergence curve in Tensorboard, model at 130th epoch was selected as the final model for inference crosscheck.

Summary: Training crosscheck was done. Compared with the results provided by tester, the performance difference between crosschecker’s and tester’s is negligible.

- Inference crosscheck of EE1-1.1.3.

Crosschecker conducted the inference crosscheck using the model provided by tester.

Summary: Results match well.

The NNVC-3.0 software was used in these three tests. All models used in inference crosscheck were converted to SADL int16.

# Results

The simulations were carried out following the NNVC common test conditions[5], and NnlfOption = 1 was configured during testing.

## EE1-1.1.1 (model trained by crosscheker)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -9.84% | -16.99% | -18.64% | 177% | 67564% |
| Class A2 | -9.89% | -18.20% | -15.12% | 172% | 65037% |
| Class B | -8.55% | -19.53% | -18.80% | 172% | 64429% |
| Class C | -8.91% | -21.66% | -22.71% | 149% | 50904% |
| Class E |  |  |  |  |  |
| Overall | -9.17% | -19.32% | -19.07% | 167% | 61198% |
| Class D | -9.69% | -19.38% | -22.35% | 151% | 46230% |
| Class F | -4.13% | -12.57% | -11.99% | 215% | 25839% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6.22% | -13.85% | -15.79% | 206% | 43755% |
| Class A2 | -5.85% | -15.21% | -12.80% | 156% | 35251% |
| Class B | -5.99% | -15.19% | -16.89% | 147% | 33461% |
| Class C | -6.55% | -16.61% | -18.79% | 123% | 22643% |
| Class E | -9.00% | -15.17% | -16.45% | 156% | 38602% |
| Overall | -6.63% | -15.28% | -16.37% | 152% | 33142% |
| Class D | -6.52% | -14.71% | -18.20% | 117% | 19973% |
| Class F | -4.36% | -12.29% | -12.57% | 120% | 28748% |

## EE1-1.1.2 (model trained by crosschecker)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -9.64% | -17.56% | -20.86% | 179% | 67184% |
| Class A2 | -10.19% | -20.37% | -16.62% | 173% | 65107% |
| Class B | -9.21% | -21.75% | -22.01% | 172% | 64916% |
| Class C | -10.06% | -24.00% | -23.41% | 149% | 50505% |
| Class E |  |  |  |  |  |
| Overall | -9.72% | -21.24% | -21.08% | 167% | 61167% |
| Class D | -11.29% | -23.08% | -24.41% | 149% | 46837% |
| Class F | -4.69% | -14.23% | -12.79% | 215% | 28543% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -7.01% | -15.23% | -18.43% | 204% | 44046% |
| Class A2 | -6.51% | -16.95% | -14.45% | 155% | 36954% |
| Class B | -6.63% | -16.21% | -17.60% | 144% | 33386% |
| Class C | -7.76% | -17.22% | -19.65% | 120% | 22281% |
| Class E | -10.13% | -17.13% | -17.93% | 152% | 38318% |
| Overall | -7.51% | -16.55% | -17.73% | 150% | 33260% |
| Class D | -7.67% | -15.90% | -19.26% | 115% | 20002% |
| Class F | -4.70% | -12.96% | -12.72% | 118% | 27367% |

## EE1-1.1.3 (model provided by tester)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -10.24% | -17.67% | -20.18% | 178% | 62685% |
| Class A2 | -10.30% | -20.79% | -15.67% | 169% | 61028% |
| Class B | -9.49% | -21.85% | -22.41% | 172% | 63033% |
| Class C | -10.14% | -24.16% | -23.55% | 149% | 49729% |
| Class E |  |  |  |  |  |
| Overall | -9.98% | -21.42% | -20.92% | 166% | 58725% |
| Class D | -11.51% | -23.86% | -24.19% | 148% | 46206% |
| Class F | -4.79% | -14.78% | -14.04% | 203% | 28491% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -7.01% | -14.91% | -17.98% | 186% | 37748% |
| Class A2 | -6.63% | -16.95% | -14.44% | 142% | 31225% |
| Class B | -6.76% | -16.82% | -17.39% | 134% | 28913% |
| Class C | -7.88% | -16.87% | -19.30% | 113% | 19516% |
| Class E | -10.22% | -17.52% | -17.60% | 141% | 33521% |
| Overall | -7.61% | -16.65% | -17.45% | 139% | 28756% |
| Class D | -7.80% | -15.95% | -18.58% | 108% | 17558% |
| Class F | -4.83% | -12.82% | -13.29% | 111% | 23481% |

# Conclusion

Training and inference crosscheck of EE1-1.1.1 and EE1-1.1.2 were done successfully, and the performance difference between crosschecker’s and proponent’s is negligible for both two tests.

Inference crosscheck of EE1-1.1.3 was done successfully, and results match proponent’s

# Reference

[1] R. Chang, L. Wang, X. Xu, S. Liu. EE1-1.1: More refinements on NN based in-loop filter with a single model. JVET-AC0194. JVET 29th meeting, by teleconference, Jan 2023.

[2] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM.

[3] L. Wang, X. Xu, S. Liu, Z. Xie, Y. Yu, H. Yu, D. Wang. EE1-1.8: More refinements on NN based in-loop filter with a single model. JVET-AB0083. JVET 28th meeting, Mainz, Oct 2022.

[4] https://vcgit.hhi.fraunhofer.de/jvet-ab-ee1/VVCSoftware_VTM/-/tree/EE1-1.1.

[5] E. Alshina, R.-L. Liao, S. Liu, A. Segall	. Common Test Conditions and evaluation procedures for neural network-based video coding technology. JVET-AB2016. JVET 28th meeting, Mainz, Oct 2022.