| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0279-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AC0195 (EE1-1.2: encoder-only optimization for NN based in-loop filter with a single model) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhihuang Xie | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results for JVET-AC0195[1], which is also known as EE1-1.2. Two tests were studied in EE1-1.2. Both training and inference crosschecks were carried out for EE1-1.2.1, and only inference crosscheck was conducted for EE1-1.2.2. Both two tests investigated the encoder-side optimization for NNVC-3.0[2] filter set #0. Cross-check results are summaried as below:

- EE1-1.2.1 (base model from NNVC-3.0 filter set #0):

Performance of crosschecker’s retrained model vs. tester’s model:

RA: 0.02% Y, -0.04% U, -0.03% V;

AI: 0.01% Y, -0.04% U, -0.05% V.

- EE1-1.2.2 (base model from EE1-1.1.3):

Model provided by tester, and cross-check results match tester’s.

# Introduction

JVET-AC0195 reports the EE1-1.2 results, which is related to NNVC-3.0 (filter set #0). In order to further improve the coding performance of NNVC-3.0 filter set #0, an encoder-optimization method was studied in EE. In detail, EE1-1.2 trained two new simplified models based on the architecture of NNVC-3.0 filter set #0, then applied these two models to the RDO process for further enhancing the coding performance of NNVC-3.0 filter set #0. Those two new models were trained for I slices and B slices, respectively. EE1-1.2 contains two sub-tests, which can be summarized as below:

  - EE1-1.2.1: two RDO-process models + base model from NNVC-3.0 filter set #0.

  - EE1-1.2.2: two RDO-process models + base model from EE1-1.1.3.

# Cross-check status

Crosschecker of EE1-1.2 conducted one training crosscheck and two inference crosschecks, which can be described briefly as below:

- Training and inference crosscheck of EE1-1.2.1.

Crosschecker trained two new models from scratch using the training scripts provided by tester, and those scripts can also be fetched from Git-EE1-1.2[3]. Guided by convergence curve in Tensorboard, I-model at 640th epoch and B-model at 638th epoch were selected as the final models for inference crosscheck.

Summary: Training crosscheck was done. Models trained by crosschecker were used for inference crosscheck. Compared with the results provided by tester, the performance difference between crosschecker’s and tester’s is quite small.

- Inference crosscheck of EE1-1.2.2.

Crosschecker conducted the inference crosscheck using the models provided by tester. These models provided by tester include base model and two RDO-process models.

Summary: Results match well.

The software of EE1-1.2 does what proponents described. All models used in inference crosscheck were converted to SADL int16.

# Results

The simulations were carried out following the NNVC common test conditions[4], and NnlfOption = 1 was configured during testing.

## EE1-1.2.1 (models trained by crosscheker)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -9.38% | -13.27% | -18.94% | 179% | 65708% |
| Class A2 | -9.80% | -19.56% | -14.79% | 178% | 64113% |
| Class B | -8.83% | -19.13% | -20.78% | 181% | 63394% |
| Class C | -8.92% | -21.01% | -20.74% | 154% | 47852% |
| Class E |  |  |  |  |  |
| Overall | -9.15% | -18.55% | -19.21% | 173% | 59370% |
| Class D | -10.11% | -19.79% | -20.88% | 157% | 44633% |
| Class F | -3.97% | -12.26% | -11.22% | 222% | 25310% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6.44% | -14.25% | -17.32% | 246% | 44354% |
| Class A2 | -6.17% | -15.90% | -13.06% | 195% | 36610% |
| Class B | -6.36% | -15.64% | -17.52% | 186% | 33105% |
| Class C | -6.88% | -16.45% | -18.21% | 163% | 22211% |
| Class E | -9.35% | -15.38% | -16.40% | 194% | 38055% |
| Overall | -6.96% | -15.59% | -16.71% | 192% | 33107% |
| Class D | -6.84% | -15.06% | -18.38% | 155% | 19865% |
| Class F | -4.33% | -11.79% | -11.80% | 132% | 27978% |

## EE1-1.2.2 (models provided by tester)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -10.35% | -17.57% | -20.12% | 200% | 70824% |
| Class A2 | -10.59% | -20.82% | -15.73% | 198% | 69059% |
| Class B | -9.75% | -21.79% | -22.33% | 194% | 69387% |
| Class C | -10.52% | -24.22% | -23.62% | 166% | 54068% |
| Class E |  |  |  |  |  |
| Overall | -10.24% | -21.40% | -20.91% | 188% | 65127% |
| Class D | -11.84% | -23.63% | -24.09% | 166% | 50094% |
| Class F | -4.92% | -14.94% | -14.17% | 232% | 32593% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -7.30% | -14.99% | -18.09% | 245% | 43526% |
| Class A2 | -7.02% | -16.95% | -14.50% | 195% | 36620% |
| Class B | -7.18% | -17.00% | -17.62% | 186% | 33090% |
| Class C | -8.36% | -16.98% | -19.49% | 162% | 22295% |
| Class E | -10.74% | -17.71% | -17.86% | 193% | 38190% |
| Overall | -8.03% | -16.77% | -17.63% | 192% | 33048% |
| Class D | -8.22% | -16.09% | -18.55% | 156% | 19831% |
| Class F | -5.13% | -12.87% | -13.34% | 132% | 26813% |

# Conclusion

Training and inference crosscheck of EE1-1.2.1 were done successfully, and the performance difference between crosschecker’s and proponent’s is negligible.

Inference crosscheck of EE1-1.2.2 was done successfully, and results match proponent’s.

# Reference

[1] R. Chang, L. Wang, X. Xu, S. Liu. EE1-1.1: More refinements on NN based in-loop filter with a single model. JVET-AC0194. JVET 29th meeting, by teleconference, Jan 2023.

[2] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM.

[3] https://vcgit.hhi.fraunhofer.de/jvet-ab-ee1/VVCSoftware_VTM/-/tree/EE1-1.2.

[4] E. Alshina, R.-L. Liao, S. Liu, A. Segall	. Common Test Conditions and evaluation procedures for neural network-based video coding technology. JVET-AB2016. JVET 28th meeting, Mainz, Oct 2022.