| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0299-v1 |
| --- | --- |

| Title: | AHG9: On indicating processing order in the NNPFC SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tong Shao, Arjun Arora, Peng Yin, Sean McCarthy, Taoran Lu, Fangjun Pu, Walt Husak Hendry, Seung hwan Kim | Email: | sean.mccarthy@dolby.com dr.hendry@lge.com |
| Source: | Dolby Laboratories, Inc., LG Electronics Inc. |  |  |

_____________________________

# Abstract

At the 28th Meeting of JVET, it was agreed that there can be several NNPFA SEI messages present for the same picture, for example, when the post-processing filters are meant for different purposes. It is asserted that it would be beneficial to enable signaling of the preferred processing order, as determined by the encoder (i.e., the content producer), for different neural-network post-filters when multiple neural-network post-filters are present. This contribution proposes to add syntax elements to the NNPFC SEI message to enable signalling of preferred processing order.

# Introduction

At the 27th Meeting of JVET, JVET-AA0101 proposed syntax elements and corresponding semantics for the NNPFA SEI message to convey information on the preferred dependencies, if any, between neural-network post-filters when more than one neural-network post-filter is active for the current picture. It was discussed that it might be premature at that moment to think about concatenation of different neural networks. Subsequently, it was agreed at the 28th Meeting of JVET, that there can be several NNPFA SEI messages present for the same picture, for example, when the post-processing filters are meant for different purposes or filter different colour components

It is asserted that it would be beneficial to enable signaling of the preferred processing order, as determined by the encoder (i.e., the content producer), for different neural-network post-filters when multiple neural-network post-filters are present.

# Proposal

### 8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| … |  |
| nnpfc_processing_order_present_flag | u(1) |
| if( nnpfc_processing_order_present_flag ) |  |
| nnpfc_processing_order | u(16) |
| … |  |
| } |  |

    - Neural-network post-filter characteristics SEI message semantics

nnpfc_processing_order_present_flag equal to 1 specifies that the syntax element nnpfc_processing_order is present. nnpfc_processing_order_present_flag equal to 0 specifies that syntax element nnpfc_processing_order is not present.

nnpfc_processing_order indicates the preferred order of processing of the post-processing filter. When multiple NNPFC SEI messages are present, a value of nnpfc_processing_order in this NNPFC SEI message less than the value of nnpfc_processing_order in another NNPFC SEI message indicates the post-processing filter specified in this NNPFC SEI message, if activated by an NNPFA SEI message, should be processed before the post-processing filter specified in the other NNPFC SEI message, if activated by an NNPFA SEI message, and a value of nnpfc_processing_order in this NNPFC SEI message equal to the value of nnpfc_processing_order in another NNPFC SEI message indicates that there is no preferred order of processing between the post-processing filters specified in the NNPFC SEI messages. When nnpfc_processing_order is not present, the value of nnpfc_processing_order is either not specified or specified by external means.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).