| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0301-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-AC0083 (EE2-1.4 : Combination of directional planar prediction methods) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Woong Lim Sung-Chang Lim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-1836 woong.lim@etri.re.kr sclim@etri.re.kr |
| Source: | Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

This contribution reports a crosscheck result for JVET-AC0083 contribution which proposes combination of directional planar prediction methods with some improvements. The crosschecking is conducted on implementation on top of ECM-7.0. It is confirmed that the crosscheck results are matched with those provided by the proponent.

Introduction

In JVET-AB2024[1], EE2-Test1.4 is to test improved directional planar prediction described in JVET-AB0257[2] which is a combination of EE2-Test1.1, Test1.2 and Test1.3, related to directional planar prediction methods with some improvements.

Experimental results

The experiments were performed based on ECM-7.0 software [3] under JVET ECM common test conditions [4]. And, the experiments were conducted on the platform of Intel Xeon Gold 6256 (3.6GHz) and 256GB RAM with GCC 9.4.0 compiler under Ubuntu 20.04.4 LTS. The partial experimental results of EE2-Test1.4 is summarised in Table 1. And, the detailed results can be found in accompanied Excel file.

Table 1. Simulation result of EE2-Test1.4

|  | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 CTC |  |  |  |  | Over ECM-7.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.09% | -0.01% | 105% | 100% | -0.03% | -0.31% | 0.04% | 101% | 101% |
| Class A2 | -0.04% | -0.06% | 0.06% | 104% | 101% | -0.02% | 0.11% | -0.02% | 101% | 101% |
| Class B | -0.05% | -0.06% | -0.12% | 105% | 100% | -0.03% | -0.05% | -0.26% | 101% | 101% |
| Class C | -0.06% | -0.04% | -0.03% | 105% | 100% | -0.01% | 0.11% | -0.09% | 101% | 101% |
| Class E | -0.10% | 0.04% | -0.03% | 105% | 100% | N/A | N/A | N/A | N/A | N/A |
| Overall | -0.06% | -0.04% | -0.04% | 105% | 100% | -0.02% | -0.02% | -0.11% | 101% | 101% |
| Class D | -0.06% | -0.03% | -0.05% | 105% | 101% | -0.02% | -0.08% | -0.04% | 101% | 101% |
| Class F | -0.07% | 0.22% | -0.03% | 103% | 99% | -0.06% | -0.11% | -0.08% | 102% | 100% |

References

- V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang, "Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)," JVET-AB2024, Oct. 2022

- S. Yoo, J. Choi, J. Nam, M. Hong, J. Lim, S. Kim (LGE), K. Kim, D. Kim, J. Son, J. Kwak (WILUS), X. Li, R. Liao, J. Chen, Y. Ye (Alibaba), "EE2-related : Improved directional planar prediction," JVET-AB0257, Oct. 2022

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-7.0

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-Y2017, 25th meeting, by teleconference, Jan. 2022.

Patent rights declaration(s)

ETRI does not have any current or pending patent rights relating to the technology described in this contribution.