| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0306-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AC0095 (AHG12: Fix related to pixel copy in motion compensation) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Z. Zhang | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of crosschecking JVET-AC0095 proposed by Ericsson. JVET-AC0095 proposes to remove clipping operation on intermediate uni-prediction sample values for bi-predicted blocks. The fix is related to pixel copy function. The software change from the proponent was using the non-SIMD function which does not have clipping operation to replace the SIMD function which do clipping operation of pixel copy function.

During the crosscheck, it observed that the scaled picture boundary padding has problem under RPR configuration when introducing coding tool motion compensation boundary padding in ECM. The second fix from the proponent was to fix the scaled picture boundary padding problem in encoder side.

The update of the code had been reviewed and confirmed consistent with the proposal description.

This report reports verification result on top of the fix of scaled picture boundary padding. In the simulation, both anchor and test apply the scaled picture boundary padding fix. The test further applies the fix of removing clipping operation. The verification simulation was run for LDB RPR configuration. The verification simulation results matched the results in JVET-AC0095 in terms of BD-rate.

# Verification

Ericsson had provided the software and results of their proposed methods in JVET-AC0095 [1]. The code was updated according to the description in JVET-AC0095.

The BD-rate results match the results reported in JVET-AC0095. The simulation results are as follows:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 RPR with scaled picture boundary fix |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.09% | -0.28% | -0.13% | 103% | 100% |
| Class E | -0.20% | -0.21% | -0.08% | 103% | 103% |
| Overall |  |  |  |  |  |
| Class D | -0.05% | -0.57% | -0.03% | 94% | 94% |

# Reference

[1] K. Andersson, R. Yu (Ericsson), “AHG12: Fix related to pixel copy in motion compensation”, JVET-AC0095, January, 2023.

# Patent rights declaration(s)

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution.