| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0309-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AC0197 test 1 (EE1-1.1-related: More refinements on NN based in-loop filter) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhihuang Xie | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results for JVET-AC0197[1] test 1. There are two tests in JVET-AC0197, and this cross-check contribution only reports the first test of JVET-AC0197. In the first test of JVET-AC0197, a new well-trained model of NNVC-3.0 filter set #0 is used to conduct inference operation. The software of JVET-AC0197 is identical to NNVC-3.0[2]. Cross-check results match proponents’.

# Introduction

In JVET-AC0197, two tests were carried out. This contribution only reports the cross-check results of the first test. In particular, the first test of JVET-AC0197 used a well-retrained model instead of the one in NNVC-3.0 filter set #0. The software of JVET-AC0197 is identical to NNVC-3.0, and no other changes except for the model used in inference process. The results of the first test disclosed that the well-retrained model could have better performance compared with the model in NNVC-3.0 filter set #0.

# Results

The simulations were carried out following the NNVC common test conditions[3], and NnlfOption = 1 was configured during testing. The results of JVET-AC0197-test 1 is shown below.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -10.57% | -17.70% | -20.60% | 186% | 65105% |
| Class A2 | -10.51% | -20.92% | -15.87% | 177% | 64503% |
| Class B | -9.50% | -21.49% | -22.65% | 177% | 64515% |
| Class C | -10.22% | -23.89% | -23.60% | 151% | 50682% |
| Class E |  |  |  |  |  |
| Overall | -10.11% | -21.26% | -21.14% | 172% | 60602% |
| Class D | -11.47% | -22.97% | -24.27% | 150% | 46793% |
| Class F | -4.79% | -14.92% | -13.75% | 215% | 28271% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc-3.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -7.19% | -15.38% | -17.96% | 208% | 42908% |
| Class A2 | -6.76% | -16.87% | -14.55% | 156% | 35451% |
| Class B | -6.86% | -16.59% | -17.94% | 147% | 33539% |
| Class C | -7.91% | -17.41% | -19.91% | 123% | 22489% |
| Class E | -10.28% | -17.13% | -19.05% | 156% | 38736% |
| Overall | -7.70% | -16.71% | -18.00% | 153% | 33056% |
| Class D | -7.83% | -16.01% | -19.19% | 118% | 20171% |
| Class F | -5.04% | -13.54% | -14.03% | 120% | 26292% |

# Conclusion

Cross-check results match proponent’s.

# Reference

[1] R. Chang, L. Wang, X. Xu, S. Liu. EE1-1.1-related: More refinements on NN based in-loop filter with a single model. JVET-AC0197. JVET 29th meeting, by teleconference, Jan 2023.

[2] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM.

[3] E. Alshina, R.-L. Liao, S. Liu, A. Segall	. Common Test Conditions and evaluation procedures for neural network-based video coding technology. JVET-AB2016. JVET 28th meeting, Mainz, Oct 2022.