| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0310-v1 |
| --- | --- |

| Title: | EE1-related: Combination of EE1-1.5 and EE1-1.7 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Kai Zhang, Li Zhang (Bytedance) Du Liu, Jacob Ström, Mitra Damghanian, Per Wennersten, Kenneth Andersson (Ericsson) | Tel: Email: | yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com {du.liu, jacob.strom, mitra.damghanian, per.wennersten, kenneth.r.andersson } @ericsson.com |
| Source: | Bytedance Inc., Ericsson. |  |  |

_____________________________

- Abstract

This contribution presents the results of a combination test of EE1-1.5.3 in JVET-AC0089 and EE1-1.7.1 in JVET-AC0177. EE1-1.5.3 proposes to reduce model memory by using a shared model for the in-loop filtering of different types of slices. EE1-1.7.1 proposes to improve the in-loop filtering performance by introducing an additional CNN model. The combined test is to achieve reduced model memory and improved performance compared with NNVC-3.0 filter set #1.

BD-rate changes of {Y, Cb, Cr} of the combined test anchored on EE1-1.5.3 is {-0.70%, -0.01%, -0.01%} for RA (note that several data points of the combine test are not available yet and are approximated with the corresponding data from NNVC-3.0 filter set #1).

BD-rate changes of {Y, Cb, Cr} of the combined test anchored on NNVC-3.0 filter set #1 is {-0.83%, -0.25%, -1.43%} for RA.

BD-rate changes of {Y, Cb, Cr} of EE1-1.7.1 anchored on NNVC-3.0 filter set #1 is {-0.67%, 0.03%, 0.05%} for RA.

From results reported above, it is asserted that the method proposed in EE1-1.7.1 can bring similar gains on top of NNVC-3.0 filter set #1 and EE1-1.5.3.

Introduction

In JVET-AC0089 [1], a combined intra and inter luma model is designed with the block type I/P/B information as an additional input. In addition, a combined intra and inter chroma model is tested. Therefore, only two models are needed: a luma model and a chroma model. In JVET-AC0177 [2], two aspects are proposed to improve the CNN models with additional input information. First, collocated blocks from pictures in reference picture list 0 and list 1can be input to the in-loop filtering network. Second, the input samples of CNN can be flipped, and the output samples of CNN can be flipped back. In this test, techniques proposed in JVET-AC0089 and JVET-AC0177 are combined to achieve reduced model memory and improved performance compared with NNVC-3.0 filter set #1 [3]. In the combined test, three models are used.

Proposed method

In the proposed method, three highest temporal layers are filtered by a multiple frames-based CNN model from EE1-1.7 which takes reconstruction, prediction, collocated block from a picture in reference picture list 0, collocated block from a picture in reference picture list 1, and QP as input. For other pictures, the CNN models from EE1-1.5 are used. Other parts such as parameter selection, residual scaling, combination with deblocking, etc. remain the same as NNVC-3.0 filter set #1.

Inference

SADL is used for performing the inference of the proposed CNN filters in VTM. Both weights and feature maps are represented with int16 precision using a static quantization method. The network information in the inference stage is provided in Table 1 as suggested by [4].

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 4.66M |
|  | Parameter Precision (Bits) | 16 (I) |
|  | Memory Parameter (MB) | 9.3MB |
|  | Multiply Accumulate (kMAC/pixel) | 539K/pixel (assuming frame-level input) 682K/pixel (assuming block-level input) |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

PyTorch is used as the training platform. The DIV2K [5] and BVI-DVC [6] datasets are adopted to train the CNN filters. The network information in the training stage is provided in Table 2 as suggested by [4].

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 2 |
|  | Epoch: | 50, 400 |
|  | Batch size: | 32, 64 |
|  | Training time: | 60h, 10 days. |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Experimental results

The proposed CNN-based in-loop filtering method is integrated into NNVC-3.0 (filter set #1) and tested according to the common test conditions defined in [4]. SAO is disabled while ALF (and CCALF) is placed after the proposed CNN-based filtering.

BD-rate changes of {Y, Cb, Cr} the combined test anchored on EE1-1.5.3 are summarized in Table 3. It can be observed that the proposed method brings on average {-0.70%, -0.01%, -0.01%} BD-rate changes for {Y, Cb, Cr} under RA configurations compared with EE1-1.5.3 (note that several data points of the combine test are not available yet, and are approximated with the corresponding data from NNVC-3.0 filter set #1).

BD-rate changes of {Y, Cb, Cr} the combined test anchored on NNVC-3.0 filter set #1 are summarized in Table 4. It can be observed that the proposed method brings on average {-0.70%, -0.01%, -0.01%} BD-rate changes for {Y, Cb, Cr} under RA configurations compared with NNVC-3.0 filter set #1.

BD-rate changes of {Y, Cb, Cr} of test EE1-1.7.1 anchored on NNVC-3.0 filter set 1 are summarized in Table 5. By comparing Table 3 with Table 5, it is asserted that the method proposed in EE1-1.7.1 can bring similar gains on top of NNVC-3.0 filter set #1 and EE1-1.5.3.

Table 3. RA performance of the combined test (Anchored on EE1-1.5.3)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.68% | 0.16% | 0.47% | 92% | 90% |
| Class A2 | -0.76% | -0.41% | -0.62% | 98% | 92% |
| Class B | -0.68% | 0.12% | 0.01% | 91% | 93% |
| Class C | -0.68% | -0.02% | 0.06% | 91% | 93% |
| Class E |  |  |  |  |  |
| Overall | -0.70% | -0.01% | -0.01% | 92% | 92% |
| Class D | -0.67% | -0.32% | -0.02% | 94% | 93% |
| Class F | -0.16% | 0.06% | 0.05% | 89% | 83% |

Table 4. RA performance of the combined test (Anchored on NNVC-3.0 filter set #1)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | -0.49% | -2.04% | 87% | 90% |
| Class A2 | -0.38% | -0.46% | 1.05% | 91% | 93% |
| Class B | -0.74% | 1.27% | -1.62% | 82% | 87% |
| Class C | -1.52% | -1.81% | -2.62% | 82% | 85% |
| Class E |  |  |  |  |  |
| Overall | -0.83% | -0.25% | -1.43% | 85% | 88% |
| Class D | -2.15% | -0.81% | -4.28% | 81% | 81% |
| Class F | -0.67% | -2.99% | -2.12% | 82% | 87% |

Table 5. RA performance of EE1-1.7.1 (Anchored on NNVC-3.0 filter set #1)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.47% | 0.11% | 0.09% | 91% | 88% |
| Class A2 | -0.54% | -0.04% | 0.08% | 90% | 88% |
| Class B | -0.57% | -0.01% | 0.02% | 90% | 88% |
| Class C | -1.04% | 0.06% | 0.02% | 96% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0.67% | 0.03% | 0.05% | 92% | 90% |
| Class D | -1.59% | 0.08% | 0.07% | 93% | 91% |
| Class F | -0.38% | 0.11% | 0.02% | 93% | 96% |

Conclusions

This contribution presents the results of a combination test of EE1-1.5.3 in JVET-AC0089 and EE1-1.7.1 in JVET-AC0177. The proposed filtering method shows favorable trade-off in terms of the coding performance and complexity. It is suggested to adopt EE1-1.5.3 and EE1-1.7.2 into NNVC common software.

References

- D. Liu, J. Ström, M. Damghanian, P. Wennersten, K. Andersson, EE1-1.5: Combined intra and inter models for luma and chroma, JVET-AC0089, by teleconference, January 2023.

- Y. Li, K. Zhang, L. Zhang, EE1-1.7: Deep In-Loop Filter with Additional Input Information, JVET-AC0177, by teleconference, January 2023.

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NNVC-3.0

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AB2016, Mainz, DE, October 2022.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).