| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0315-v2 |
| --- | --- |

| Title: | Non-EE2: Cross-component merge mode for chroma intra coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Hsin-Yi Tseng, Cheng-Yen Chuang, Chih-Wei Hsu, Ching-Yeh Chen, Tzu-Der Chuang, Olena Chubach, Yi-Wen Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, ivy-hy.tseng, cheng-yen.chuang, cw.hsu, chingyeh.chen, peter.chuang, olena.chubach, yiwen-yw.chen yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

In this proposal, a cross-component merge (CCMerge) mode for chroma intra coding is introduced, comprising of cross-component model parameters inheritance for the current chroma block from its spatial adjacent and non-adjacent neighbors, or default models. The spatial adjacent and non-adjacent neighboring information is collected from the previously blocks coded by CCLM, MMLM, CCCM, GLM, chroma fusion, and CCMerge modes.

On top of ECM-7.0, simulation results of the proposed method are reported as below:
AI: {Y BD-rate = -0.12%, Cb BD-rate = -1.95%, Cr BD-rate = -2.08%, EncT = 107%, DecT = 100%}
RA: {Y BD-rate = -0.xx%, Cb BD-rate = -0.xx%, Cr BD-rate = -0.xx%, EncT = 10x%, DecT = 100%}

Introduction

Cross-component prediction (CCP) including cross-component linear model (CCLM), convolutional cross-component model (CCCM), and gradient linear model (GLM) are adopted in ECM to exploit the cross-component correlation. In this document, a cross-component merge (CCMerge) mode is proposed as a new CCP mode. Cross component model parameters of the current chroma block coded with CCMerge can be inherited from a neighboring block coded with CCP. Through CCMerge, CCP can be more efficient with less signalling overhead.

Proposed method

In the proposed CCMerge, final cross-component model parameters of the current chroma block can be inherited from its spatial adjacent and non-adjacent neighbors, or default models. A list is created, which includes CCP models from the spatial adjacent and non-adjacent neighbors coded in CCLM, MMLM, CCCM, GLM, chroma fusion, and CCMerge modes. After including neighboring CCP models, default models are further included to fill the remaining empty positions in the list. To avoid including redundant CCP models in the list, pruning operations are applied. More details are described as follows.

Fig. 1.  Positions of the adjacent spatial candidates

Spatial adjacent neighboring candidates

Positions of the spatial adjacent candidates are shown in Fig. 1. Spatial candidates are included in the following order: B1  A1  B0  A0  B2.

Spatial non-adjacent neighboring candidates

Spatial non-adjacent neighboring candidates are considered after all spatial adjacent neighbors are checked. In the current ECM design, in inter merge mode, two sets of spatial non-adjacent neighboring candidates are obtained. In the proposed method, positions and inclusion order of the spatial non-adjacent neighboring candidates from the first set are used.

CCLM candidates with default scaling parameters

CCLM candidates with default scaling parameters are considered after including the spatial adjacent and non-adjacent candidates if the list is not full. The default scaling parameters are {0, 1/8, -1/8, 2/8, -2/8, 3/8}, and the offset parameter is derived according to the selected default scaling parameter, average neighboring reconstructed luma sample value (Yavg), and average neighboring reconstructed Cb/Cr sample value (Cavg).

Merging model candidates

When merging a CCLM candidate, only the scaling parameter is inherited. The offset parameter is derived by using the inherited scaling parameter, Yavg and Cavg.

When merging a MMLM candidate, the scaling parameters and the classification threshold are inherited. The offset parameter in each class is derived according to the inherited classification threshold and the Yavg and Cavg in each class. If no neighboring reconstructed samples are available in a class, the offset parameter is directly inherited from the candidate.

When merging a CCCM candidate, all convolution parameters, offsets (i.e., offsetLuma, offsetCb, and offsetCr), and the classification threshold are inherited.

When merging a GLM candidate, if the GLM candidate is 3-parameter GLM mode, all the gradient pattern index and model parameters are inherited; otherwise, if the GLM candidate is the 2-parameter GLM mode, the offset parameter is derived by using the inherited scaling parameter, Yavg, and Cavg.

When merging a chroma fusion candidate, the derived MMLM parameters are inherited and used as merging MMLM candidate.

For a CCMerge block, if its merging candidate mode is CCLM, MMLM, CCCM, or GLM, the merging candidate mode is stored as the propagation mode of the current chroma block; otherwise, if its merging candidate mode is chroma fusion, the propagation mode is set to MMLM. When merging a CCMerge candidate, how to inherit or derive the CCP parameters depends on the propagation mode of the CCMerge candidate, as described in the above five paragraphs.

Signaling

An additional flag is signalled indicating whether CCMerge is used or not after cclm_mode_flag syntax element. If CCMerge is used, a candidate index is additionally signalled. The signalled candidate index is shared for Cb/Cr color components. Currently, the maximum number of allowed candidates is set to 6 as default. If maximum number of allowed candidates is modified to 1, candidate index does not need to be signalled. Each bin of candidate index is context coded with a separate context.

Experimental results

The proposed method is implemented on top of ECM-7.0 [1] and tested following the common test conditions [2]. The simulation results are summarized in Table 1. Currently, AI results are ready, and RA results will be updated in a later version of this document.

Table 1.  Simulation results (to be updated)

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| Class A1 | -0.21% | -2.34% | -3.36% | 105% | 99% |  |  |  |  |  |
| Class A2 | -0.18% | -3.49% | -3.56% | 107% | 100% |  |  |  |  |  |
| Class B | -0.09% | -2.24% | -2.29% | 106% | 100% |  |  |  |  |  |
| Class C | -0.07% | -0.74% | -0.78% | 108% | 100% | -0.01% | -0.26% | -0.44% | 103% | 100% |
| Class E | -0.05% | -1.17% | -0.72% | 107% | 99% |  |  |  |  |  |
| Overall | -0.12% | -1.95% | -2.08% | 107% | 100% |  |  |  |  |  |
| Class D | -0.03% | -1.11% | -0.95% | 108% | 100% | -0.02% | -0.82% | -0.84% | 104% | 100% |
| Class F | -0.16% | -1.60% | -1.64% | 104% | 100% | -0.13% | -1.10% | -1.12% | 102% | 100% |

Conclusions

This contribution presents a cross-component merge mode for chroma intra coding. The proposed method results in promising coding gains with affordable low computational complexity. It is recommended to further study this proposal in EE2.

References

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).