| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0323_r1 |
| --- | --- |

| Title: | EE2-related: Additional fixed filter for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

In the current adaptive loop filter (ALF) design, online-trained filters contain 3 kinds of taps: spatial taps, reconstruction-before-DBF based taps and fixed-filter-output based taps. In this contribution, additional fixed-filter-output based taps are introduced to provide additional texture information for ALF.

On top of ECM-7.0, simulation results of the proposed are reported as below:

AI: -0.05%, 0.01%, 0.01%, 101% 100%.

RA: -0.05%, 0.02%, -0.01%, 100%, 100%.

LB: -0.05%, -0.17%, -0.13%, 100%, 100%.

Introduction

In ALF of ECM-7.0, online-trained filters consist of 3 kinds of filter taps: spatial taps, reconstruction-before-DBF based taps and fixed-filter-output based taps, as shown in Figure 1-1. The spatial taps (i.e., tap #0 ~ #19) located in a cross shape are followed by 3 reconstruction-before-DBF based taps (i.e., tap #26, #27, #30) and the offline-filtered taps (i.e., tap #20 ~ #25, #28, #29), resulting in 25 taps in total.

Figure 1-1 The Currently Used Filter Shape in ALF.

Proposed Method

The proposed method is shown in Figure 2-1, where spatial taps (i.e., tap #0 ~ #19),  reconstruction-before-DBF based taps (i.e., tap #26, #27, #34) and fixed-filter-output based taps (i.e., tap #20 ~ #25, #32, #33) are kept the same as the ECM-7.0, and several extended taps (i.e., tap #28 ~ #31, #35) are introduced into luma online-trained filters. The extended taps take output that generated by reconstruction before DBF and an additional fixed filter as the input source.

The additional fixed filter is a diamond 7x7 filter with fixed coefficients at both encoder and decoder.

The coefficient signaling mechanism is the same as the current design in ECM. The proposed method is always enabled without filter shape switching.

Figure 2-1 The Newly Introduced Filter Shape for ALF.

Simulation Results

The proposed method was implemented on top of the ECM-7.0 [1]. Simulations were performed following the common test conditions [2]. More detailed results could be found in the attached excel sheets. The results compared to ECM-7.0 are shown in TABLE 3-1. The results on top of EE2-5.1b are shown in TABLE 3-2.

TABLE 3-1 Performance of the proposed method

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.01% | 0.01% | 100% | 100% | -0.07% | -0.18% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.06% | 0.01% | 0.01% | 100% | 99% | -0.10% | 0.03% | -0.06% | 100% | 100% |  |  |  |  |  |
| Class B | -0.05% | 0.02% | 0.02% | 101% | 101% | -0.02% | 0.16% | -0.01% | 100% | 100% | 0.02% | -0.32% | -0.40% | 100% | 100% |
| Class C | -0.05% | 0.01% | 0.01% | 102% | 100% | -0.03% | -0.02% | 0.03% | 100% | 101% | -0.11% | 0.14% | 0.17% | 100% | 100% |
| Class E | -0.04% | -0.01% | -0.01% | 103% | 102% |  |  |  |  |  | -0.10% | -0.33% | -0.06% | 102% | 101% |
| Overall | -0.05% | 0.01% | 0.01% | 101% | 100% | -0.05% | 0.02% | -0.01% | 100% | 100% | -0.05% | -0.17% | -0.13% | 100% | 100% |
| Class D | -0.02% | 0.03% | 0.03% | 104% | 102% | -0.04% | -0.01% | 0.11% | 108% | 103% | -0.11% | 0.26% | -0.56% | 105% | 99% |
| Class F | -0.09% | 0.06% | 0.06% | 101% | 99% | -0.14% | -0.03% | 0.19% | 102% | 99% | 0.01% | -0.32% | -1.15% | 102% | 99% |

TABLE 3-2 Performance of the proposed method on top of EE2-5.1b

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.05% | 0.01% | 0.01% | 98% | 102% |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.05% | 0.02% | 0.02% | 96% | 96% | -0.01% | -0.05% | 0.04% | 95% | 95% | -0.14% | 0.22% | 0.13% | 97% | 98% |
| Class E | -0.05% | 0.00% | 0.00% | 98% | 100% |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.01% | 0.03% | 0.03% | 97% | 94% | -0.04% | -0.10% | -0.02% | 98% | 94% | -0.14% | -0.51% | -0.05% | 100% | 97% |
| Class F |  |  |  |  |  | -0.12% | 0.01% | 0.26% | 99% | 96% |  |  |  |  |  |

Conclusions

In this contribution, ALF with additional fixed-filter-output based taps is proposed. The proposed method can achieve promising coding performance with limited encoding time and decoding time increase. It is recommended to include the proposed method into the next round of EE2.

Reference

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).