| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0327-v1 |
| --- | --- |

| Title: | Cross-check of JVET-AC0203 (AHG12: On LMCS luma mapping in template processing for IBC TM-based tools) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Médéric Blestel Xiaomi Technology Cesson-Sévigné, France | Email: | medericb@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results of the contribution JVET-AC203 which proposes a technique of harmonizing the luma mapping process with the mechanism of computing template matching (TM) costs for IBC TM-based tools. The proposal aims at aligning domains in which TM costs are calculated for both the templates of the current block and the candidate reference blocks.

In the contribution, two methods are proposed to perform the alignment:

- Method 1: TM costs are computed in spatial domain (i.e. after LMCS inverse luma mapping); This method follows the method used in other TM cost calculation functions.

- Method 2: TM costs are computed in LMCS (mapped) domain (i.e. before LMCS inverse luma mapping).

# Introduction

Cross-check of the source code and simulation results have been conducted. Inspected source code follows the description of the proposal.

# Cross-check results

The source code of JVET-AC0203 has been run under JVET EE2 coding test conditions.

## Method 1 cross-check results

The proposed Method 1 results against ECM-7.0 is shown in the following table. Cross-checked results match with proponent results regarding Class F, and Class TGM contents using random access scenario. Low delay B scenario was not crosschecked.

Encoder and decoder run-times are not accurate.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0,00% | 0,01% | 0,08% | 98% | 99% |
| Class TGM | -0,02% | -0,05% | -0,02% | 97% | 96% |

## EE2-2.4a cross-check results

The proposed Method 2 results against ECM-7.0 is shown in the following table. Cross-checked results match with proponent results regarding Class F and Class TGM contents using random access scenario. Low delay B scenario was not crosschecked.

Encoder and decoder run-times are not accurate.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0,01% | 0,05% | 0,06% | 96% | 96% |
| Class TGM | -0,03% | -0,05% | -0,03% | 92% | 91% |

# Conclusion

Both proposed methods were cross-checked and the BD-rates perfectly match with the result provided by the proponent. Inspection of the code shows that the implementation is consistent with the proposition documented in the contribution.

# Reference

- A. Filippov, V. Rufitskiy, “AHG12: On LMCS luma mapping in template processing for IBC TM-based tools”, JVET-AC0203, January 2023, online.

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0.

# Patent rights declaration(s)

Xiaomi does not have any current or pending patent rights relating to the technology described in this contribution.