| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0328-v2 |
| --- | --- |

| Title: | EE1-related: Combination of EE1-1.1 and EE1-1.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Chang Liqiang Wang Xiaozhong Xu Shan Liu | Email: | renjiechang@tencent.com liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the combination test results of EE1-1.1.2 in JVET-AC0194 and EE1-1.2.1 in JVET-AC0195. EE1-1.1.2 optimizes the performance by retraining the models base on new training scripts. EE1-1.2.1 proposes a NN filter which is involved in the partitioning decision process. Based on the NNVC-3.0, the combination test results are shown in order of RA and AI configurations as follows.

RA: -9.99%	-21.05%	-20.50%	EncT: 160%	DecT: 55750%

AI : -7.95%		-16.80%	-17.52%	EncT: 172%	DecT: 31594%

Introduction

In test2 of JVET-AC0194 (JVET-AB-EE1-1.1) [1], the model performance is optimized by retraining model based on new training scripts. In JVET-AC0195 (JVET-AB-EE1-1.2) [2], a NN filter is involved in the encoder decision process for partitioning and the used RDO models are retrained. In this contribution, the combination of EE1-1.1 and EE1-1.2 is evaluated and all these used models have been training crosschecked.

Network information in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Tesla V100 32GB |
|  | Framework: | Pytorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~100 |
|  | Batch size: | 64 |
|  | Loss function: | L1 |
|  | Training time (for 1 model): | ~60h |
|  | Training data information: | DIV2K, TVD, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations | 1200 |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Learning rate update strategy |  |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Training data update strategy: |  |
|  | Other information: |  |

Network information in inference stage

In the inference stage, SADL [4] is adopted as the inference library. In addition, all the models in this contribution are quantized into int16 precision.

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.9M |
|  | Total Number of Parameters (All Models) | 1.9M |
|  | Parameter Precision (Bits) | 16 |
|  | Memory Parameter (MB) | 3.8MB/model, 1 model in all |
|  | Multiply Accumulate (kMAC/pixel) | 485K (w/o block extension) 615K (w/ block extension) |
|  | Calculation Method | block basis |
| Optional |  |  |
|  | Total Conv. Layers | 101 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Results

All the experiments are performed based on the common test conditions in [5]. The test results of the proposed method are provided in Table 1. For the convenient comparison, the test results of EE1-1.1.2 are also provided here in Table 2. The average results can also be summarized as follows.

Combination test of EE-1.1.2 and EE1-1.2.1:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -9.99% | -21.05% | -20.50% | 160% | 55750% |
| AI | -7.95% | -16.80% | -17.52% | 172% | 31594% |

EE1-1.1.2:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -9.75% | -21.09% | -20.55% | 151% | 55869% |
| AI | -7.51% | -16.81% | -17.43% | 147% | 31682% |

Table 1. Performance of combination test

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NnlfOption=0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -10.09% | -16.20% | -20.25% | 170% | 59782% |
| Class A2 | -10.46% | -19.83% | -15.90% | 162% | 57248% |
| Class B | -9.45% | -22.00% | -20.98% | 166% | 59030% |
| Class C | -10.23% | -24.41% | -23.53% | 144% | 48286% |
| Class E |  |  |  |  |  |
| Overall | -9.99% | -21.05% | -20.50% | 160% | 55750% |
| Class D | -11.35% | -23.71% | -23.40% | 143% | 42810% |
| Class F | -4.76% | -14.64% | -12.57% | 196% | 24752% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NnlfOption=0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -7.28% | -15.38% | -18.01% | 222% | 42064% |
| Class A2 | -6.92% | -17.00% | -14.37% | 175% | 34191% |
| Class B | -7.11% | -16.85% | -17.16% | 166% | 31296% |
| Class C | -8.26% | -17.29% | -19.55% | 146% | 21884% |
| Class E | -10.63% | -17.29% | -18.09% | 174% | 36353% |
| Overall | -7.95% | -16.80% | -17.52% | 172% | 31594% |
| Class D | -8.14% | -16.43% | -18.99% | 140% | 19913% |
| Class F | -5.00% | -13.71% | -12.59% | 126% | 26176% |

Table 2. Performance of EE1-1.1.2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NnlfOption=0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.99% | -16.19% | -20.23% | 163% | 59732% |
| Class A2 | -10.20% | -19.85% | -15.94% | 154% | 57390% |
| Class B | -9.21% | -22.09% | -21.10% | 157% | 59179% |
| Class C | -9.92% | -24.44% | -23.54% | 135% | 48463% |
| Class E |  |  |  |  |  |
| Overall | -9.75% | -21.09% | -20.55% | 151% | 55869% |
| Class D | -10.97% | -23.77% | -23.46% | 133% | 42837% |
| Class F | -4.61% | -14.45% | -12.76% | 189% | 25003% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NnlfOption=0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.98% | -15.27% | -17.94% | 196% | 41800% |
| Class A2 | -6.52% | -16.95% | -14.22% | 152% | 34322% |
| Class B | -6.67% | -16.85% | -17.02% | 142% | 31762% |
| Class C | -7.77% | -17.62% | -19.65% | 121% | 21892% |
| Class E | -10.11% | -17.09% | -17.84% | 148% | 36132% |
| Overall | -7.51% | -16.81% | -17.43% | 147% | 31682% |
| Class D | -7.69% | -16.96% | -19.44% | 117% | 20126% |
| Class F | -4.71% | -13.65% | -12.65% | 119% | 26229% |

Conclusions

In this contribution, the performance of EE1-1.1 is further improved by introducing the proposed encoder-only method. Compared with EE1-1.1.2 under RA and AI configurations, additional 0.24% (from 9.75% to 9.99%) and 0.44% (from 7.51% to 7.95%) BD-rate luma gains are observed. In terms of decoder side, there is no any increase in both MAC result and memory size. For the runtime at encoder side of this contribution, there are about 9% (from 151% to 160%) and 25% (from 147% to 172%) increase under RA and AI configurations, respectively.

References

- R. Chang, L. Wang, X. Xu, S. Liu. EE1-1.1: More refinements on NN based in-loop filter with a single model. JVET-AC0194, 29th Meeting, by teleconference, 11–20 January 2023.

- R. Chang, L. Wang, X. Xu, S. Liu. EE1-1.2: encoder-only optimization for NN based in-loop filter with a single model. JVET-AC0195, 29th Meeting, by teleconference, 11–20 January 2023.

- H. Zhu, X. Xu, S. Liu. Deep learning based in-loop filter with QP and residual distribution information. AVS-M5654, Aug. 2020.

- F. Galpin, T. Dumas, P. Bordes, E. François. AHG11: Small Ad-hoc Deep-Learning Library (SADL) update. JVET-AA0086, 27th Meeting, by teleconference, 13–22 July 2022.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AB2016, 28th Meeting, Mainz, DE, 20–28 October 2022.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).