| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0329-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AC0096 (AHG10/12: Suggestion for new CTC for RPR in VTM and ECM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Junghak Nam 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | Junghak.nam@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results for JVET-AC0096 proposal on AHG10/12: Suggestion for new CTC for RPR in VTM and ECM. In the JVET-AC0096, proponent proposes changing the RPR CTC for better testing of RPR-related coding tools and for better viewing experiences.

Introduction

JVET-AC0096 [1] proposes four modifications to change RPR CTC for both VTM and ECM as follows,

- Add more variants of encoded resolutions, that is to allow 1(full) resolution, 1/2(half) resolution, 2/3 resolution and 4/5 resolution in both directions within the same sequence. The proposed order of resolutions is 4/5, 1, 2/3, 1, 1/2, 1, etc. Each resolution is used for encoding of 32 frames before changing to another resolution.

- Adjust the QP by -6, -4 and -2 when encoding in 1/2, 2/3 and 4/5 resolution, respectively, in both directions.

- Only evaluate performance in full (i.e., original) resolution, e.g., PSNR2 and using UpScaledOutput = 2

- Include both RA and LDB configuration. The sequence length follows VTM and ECM CTC, e.g., for low-delay 5s for ECM.

Since adaptive RPR [2] was incorporated at the last meeting, relevant codes on the first and second modifications are already included in the VTM software. For the changed CTC, the resolution can be selected periodically instead of the adaptive resolution selection.

Cross-check results

In this report, simulation of both VTM-19.0 [3] and ECM-7.0 [4] are crosschecked under RA configuration. For VTM case, the results are well matched except some classF sequences which have very minor deviations. For ECM case, there are minor deviation at the most of the results because there is an issue on pixel copy operation which reported the JVET-AC0095.

VTM

| Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-19.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.49% | 13.51% | 2.96% | 95% | 80% |
| Class A2 | 11.68% | 24.99% | 24.71% | 94% | 83% |
| Class B | 74.18% | 24.15% | 20.27% | 92% | 80% |
| Class C | 102.20% | 38.45% | 36.65% | 93% | 79% |
| Class E |  |  |  |  |  |
| Overall | 54.81% | 26.00% | 22.06% | 93% | 81% |
| Class D | 150.51% | 30.71% | 34.47% | 90% | 88% |
| Class F | 125.39% | 81.46% | 92.57% | 96% | 79% |

ECM

| Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 7.01% | 18.16% | 11.72% | 79% | 79% |
| Class A2 | 17.60% | 36.24% | 41.98% | 80% | 81% |
| Class B | 83.20% | 41.75% | 32.25% | 79% | 81% |
| Class C | 109.51% | 46.21% | 43.09% | 83% | 81% |
| Class E |  |  |  |  |  |
| Overall | 61.86% | 37.12% | 32.98% | 80% | 81% |
| Class D | 163.38% | 38.46% | 41.12% | 83% | 81% |
| Class F | 143.26% | 123.77% | 135.97% | 91% | 82% |

References

[1] K. Andersson, R. Yu, “AHG10/12: Suggestion for new CTC for RPR in VTM and ECM”, JVET-AC0096, Jan. 2023.

[2] K. Andersson, J. Ström, R. Yu, P. Wennersten, W. Ahmad, “AHG10: GOP-based RPR encoder control”, JVET-AB0080, Mainz, DE, 20–28 October 2022.

[3] VTM-19.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-19.0

[4] ECM-7.0 reference software, https://vcgit.hhi.fraunhofer.de/ecm/ECM.git