| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0331-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AC0055 (EE1-1.11: Content-adaptive post-filter) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Jacob Ström Torshamnsgatan 23 164 83 Stockholm Sweden | Tel: Email: | +46702672192 jacob.strom@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

This contribution describes a crosscheck of JVET-AC0055: “EE1-1.11: Content-adaptive post-filter”. A full training crosscheck has been performed, not based on models from the proponents. Using NNVC3.0 as the anchor, the results are:

RA: BDR -4.82% (Y) -15.25% (U) -12.86% (V)

Using the proposed method as the anchor, the results are:

RA: BDR -0.04% (Y) 3.94% (U) 4.45% (V)

Since the chroma performance differs from that of the proponent, a further investigation of this has been conducted by the crosschecker. It is reported by the crosschecker that the entire chroma difference can be attributed to overfitted models based on one of the four models. A second training has been done for that model, and in the (partial) results for that, and it is reported that the performance penalty is gone. It is the opinion of the crosschecker that the JVET-AC0055 passes the crosscheck.

# Description of algorithm

The proposed method contains two types of models; general models called “finetuned models” that are created to work with any video sequence (model0, model1, model2 and model3) and overfitted models that are created to work only with a specific videosequence/qp combination. The encoder selects one of the finetuned models as a basis for overfitting. As an example, for BQSquare qp 22, the encoder decides that model2 is the best one to overfit. This model is then trained with the BQSquare_qp22 sequence as the input and the original BQSquare sequence as the target until convergence or until 200 epochs have passed, whichever comes first. This overfitted model must then be sent to the decoder, which is done using NNR compression in an SEI message. For each frame of BQSquare qp 22, the encoder can then select between model0, model1, the overfitted model2 and model3.

# Description of training procedure

The training procedure consists of the following steps.

- Downloading training data (BVI-DVC and DIV2K)

- Generating yuv files (conversion from mp4 in the case of BVI-DVC)

- Creating training data by compressing using NNVC-1.0 (commit 4512 was used)

- Creating png files from the resulting yuv data

- Train (finetune) model0+model2 combined, train model1 and model3 individually. This step took up to 105 hours for each model on an Nvidia A100 GPU.

- Add multiplier parameters to models0 through 3 to make it possible to overfit them.

- Overfit the models from the previous step for every sequence+qp combination. This step took between an hour for classD sequences and 24 hours for classA sequences on an Nvidia A100 GPU.

- Encode and quantize the resulted models to NNR.

- Decode the resulting overfitted models to SADL files

- Run VTM encoder.

The proponent has been tremendously helpful with answering questions in a rapid fashion. A number of changes to the software were needed to make it run, but these were quickly provided by the proponent. For anyone interested in repeating this training crosscheck, these are two things to watch out for: First, the suggested paths in step 5 were created so that model0 and model3 wrote over each other’s results. Second, step 4 assumes that the JVET CTC sequences have already been converted to 10 bits, whereas the originals that JVET provides are 8-bit sequences. Using the provided software on the 8-bit sequences did in fact give valid 10-bit training data, but these were different from the 10-bit training data of the proponent, and this gave a final performance penalty of about 0.5% for luma. This was found and taken care of in the crosscheck and the performance penalty disappeared.

It is the opinion of the crosschecker that, if and when this proposal is incorporated into the NNVC software base, we should take the opportunity to make the training scripts involve less manual steps. As an example, instead of having example paths such as </path/to/finetuned/model2>, real paths such as $BASE/training/data/finetuned/model2 could be used. This would eliminate a lot of the difficulties when trying to know what path to use in what argument, and would lead to less manual errors. Furthermore, many of the scripts should be joined to reduce the number of steps needed. As an example, the overfitting training scripts could also include the encoding, quantization, and decoding steps. Another thing to fix before integration is to make sure that the encoder log produces the correct bit rate result – now correct results need to be taken from the bitstream file size with md5 checksums turned off. However, none of these things need to be fixed before adoption in the opinion of the crosschecker.

# Chroma investigation

The CTC results are very close for luma, but about 4% worse than the proponent’s for chroma on average. Looking at the results for individual sequences in class D, it is clear that the major difference is in PSNR results in cases when the overfitted model used is derived from model 3:

|  |  | diff proponent - crosschecker |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | base for overfitted |
|  |  | bitrate | Y-PSNR | U-PSNR | V-PSNR | model |
| BasketballPass | 22 | 0.29 | 0.03 | -0.06 | -0.10 | 2 |
|  | 27 | 0.28 | 0.02 | -0.06 | -0.09 | 2 |
|  | 32 | 0.29 | 0.00 | 0.37 | 0.39 | 3 |
|  | 37 | -0.01 | 0.00 | 0.35 | 0.38 | 3 |
|  | 42 | -0.34 | -0.01 | 0.30 | 0.27 | 3 |
| BQSquare | 22 | -0.06 | 0.02 | -0.06 | -0.08 | 2 |
|  | 27 | 0.30 | 0.03 | -0.08 | -0.10 | 2 |
|  | 32 | -0.34 | -0.03 | 0.21 | 0.50 | 3 |
|  | 37 | -0.01 | -0.02 | 0.20 | 0.62 | 3 |
|  | 42 | -0.35 | -0.02 | 0.13 | 0.61 | 3 |
| BlowingBubbles | 22 | -0.07 | 0.02 | -0.02 | -0.04 | 2 |
|  | 27 | -0.06 | 0.02 | -0.01 | -0.07 | 2 |
|  | 32 | -0.34 | -0.01 | 0.20 | 0.22 | 3 |
|  | 37 | 0.03 | -0.01 | 0.19 | 0.24 | 3 |
|  | 42 | -0.01 | -0.01 | 0.16 | 0.26 | 3 |
| RaceHorses | 22 | -0.05 | 0.02 | -0.07 | -0.02 | 2 |
|  | 27 | 0.01 | -0.01 | 0.27 | 0.36 | 3 |
|  | 32 | 0.00 | -0.01 | 0.30 | 0.40 | 3 |
|  | 37 | 0.29 | 0.00 | 0.28 | 0.46 | 3 |
|  | 42 | 0.00 | -0.01 | 0.23 | 0.33 | 3 |

It has further been concluded that it is not the overfitting process itself that is the problem – using the proponent’s base model3 and the crosschecker’s training data and training script execution, the chroma difference disappears. Instead, it seemed as if the crosschecker’s model3 was the weak link. The training of model3 takes about 105 hours, but fortunately the crosschecker had created two such trainings. Using this spare model3 (which we will denote model3B here) as the basis for the overfitting lowers the penalty in chroma substantially:

|  |  | diff proponent - crosschecker |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | base for overfitted |
|  |  | bitrate | Y-PSNR | U-PSNR | V-PSNR | model |
| BasketballPass | 22 | 0.29 | 0.03 | -0.06 | -0.10 | 2 |
|  | 27 | 0.28 | 0.02 | -0.06 | -0.09 | 2 |
|  | 32 | 0.03 | -0.02 | 0.16 | 0.07 | 3B |
|  | 37 | 0.01 | -0.02 | 0.16 | 0.07 | 3B |
|  | 42 | -0.31 | -0.02 | 0.13 | 0.05 | 3B |
| BQSquare | 22 | -0.06 | 0.02 | -0.06 | -0.08 | 2 |
|  | 27 | 0.30 | 0.03 | -0.08 | -0.10 | 2 |
|  | 32 | 0.02 | -0.05 | 0.01 | 0.22 | 3B |
|  | 37 | 0.02 | -0.04 | 0.12 | 0.32 | 3B |
|  | 42 | -0.32 | -0.03 | 0.14 | 0.30 | 3B |
| BlowingBubbles | 22 | -0.07 | 0.02 | -0.02 | -0.04 | 2 |
|  | 27 | -0.06 | 0.02 | -0.01 | -0.07 | 2 |
|  | 32 | 0.03 | -0.02 | 0.02 | 0.09 | 3B |
|  | 37 | 0.05 | -0.02 | 0.04 | 0.12 | 3B |
|  | 42 | 0.01 | -0.01 | 0.04 | 0.16 | 3B |
| RaceHorses | 22 | -0.05 | 0.02 | -0.07 | -0.02 | 2 |
|  | 27 | 0.01 | -0.03 | 0.03 | 0.20 | 3B |
|  | 32 | 0.02 | -0.02 | 0.08 | 0.20 | 3B |
|  | 37 | 0.02 | -0.01 | 0.07 | 0.18 | 3B |
|  | 42 | -0.33 | -0.01 | 0.06 | 0.12 | 3B |

The result for class D with the weak model3 as the basis for overfitting is

|  | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| Class D | 0.06% | 5.51% | 7.81% |

And when moving to the stronger model3B as the basis we get

|  | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| Class D | -0.17% | 1.14% | 2.66% |

which is completely satisfactory; given that the luma is slightly improved, the net result may even be better than the proponent’s results. The remaining question is then why there is such a big variance in chroma when training model3 twice. This can be due to an incorrect validation set: When finetuning model3, a validation set is used to decide when to save model3 during training. In the crosschecker’s setup, the validation set was the faulty 8-bit-to-10-bit-conversion version. If using the correctly converted data to decide when to save model3, this variance may be much smaller. When including this training software into NNVC, another set of sequences should be used to do the validation than the JVET-CTC sequences for the finetuning stage. For instance, the BVI-DVC validation set could be used.

# Results

## Results over NNVC3.0 without neural network-based loopfilters

RA:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -6.29% | -11.42% | -18.30% | 132% | 9436% |
| Class A2 | -4.35% | -15.41% | -12.05% | 127% | 8590% |
| Class B | -5.19% | -17.79% | -11.99% | 131% | 9884% |
| Class C | -3.60% | -14.82% | -10.48% | 121% | 9165% |
| Class E |  |  |  |  |  |
| Overall | -4.82% | -15.25% | -12.86% | 128% | 9332% |
| Class D | -3.55% | -13.61% | -10.67% | 122% | 9421% |
| Class F |  |  |  |  |  |

Encoder and decoder times are not reliable.

## Results over JVET-AC0055

RA:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.09% | 2.87% | 2.14% | 98% | 101% |
| Class A2 | -0.15% | 3.87% | 1.60% | 97% | 98% |
| Class B | -0.07% | 3.79% | 5.95% | 97% | 97% |
| Class C | -0.01% | 4.99% | 6.47% | 97% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 3.94% | 4.45% | 97% | 98% |
| Class D | 0.06% | 5.51% | 7.81% | 98% | 98% |
| Class F |  |  |  |  |  |

Encoder and decoder times are not reliable.

# Conclusion

It is the opinion of this crosschecker that the proposed method passes this crosscheck.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).