| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0332-v1 |
| --- | --- |

| Title: | On worst case decoder complexity of JVET-AC0130 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Gan Yue Yu | Email: | v-jonathan.gan@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution proposes reducing the number of basis vectors used in the NSPT of JVET-AC0130 for 8x16 and 16x8 block sizes from 40 to 32, which is asserted to maintain the worst-case decoder complexity at the inverse transform stage to 32 multiplications per sample. The reported results are AI -0.43% -0.57% -0.53% enc 103% dec 100%.

# Introduction

It is proposed in JVET-AC0130 [1] to replace the separable DCT plus LFNST transform combination with the NSPT for certain block sizes. In Test A of JVET-AC0130, the NSPT is used for block sizes 4x4, 4x8, 8x4 and 8x8. In Test B, the NSPT is further extended to block sizes 4x16, 16x4, 8x16 and 16x8.

This contribution is organised as follows. Section 2 is a crosscheck of the SIMD implementations of Tests A and B in [1], which were committed late to the EE2 repository. Further, the impact of extending Test A with block sizes 4x16 and 16x4, or Test A with block sizes 8x16 and 16x8, is investigated. Section 3 is a complexity analysis of the NSPT in relation to other transforms in ECM, in particular analysing the worst-case decoder burden. Finally, section 4 proposes a modification to the 16x8 and 8x16 NSPT which is asserted to address the worst-case decoder complexity.

# Crosscheck of JVET-AC0130 (informative)

All crosschecks here were performed on the latest commit of the ECM-AB-EE2 fork, TESTS_4.2 branch, which included SIMD optimisations. The tests were performed for accurate timing (test and anchor executed on the same cluster, clusters are homogenous).

Tests A and B are crosschecked and match the coding gains in [1]. The crosscheck finds slightly higher encoding times; Tests A and B have 101% and 104% EncT respectively in this crosscheck compared to 99% and 103% reported by [1].

The separate impact of NSPT on 4x16/16x4 and 8x16/16x8 block sizes are also tested. The 4x16/16x4 block sizes contribute better trade-off with approximately -0.12% luma gain on AI, while 8x16/16x8 block sizes contribute approximately -0.08% luma gain on AI.

## Test A

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.21% | -0.03% | 99% | 100% |
| Class A2 | -0.03% | -0.27% | -0.16% | 99% | 99% |
| Class B | -0.23% | -0.23% | -0.34% | 101% | 100% |
| Class C | -0.53% | -0.56% | -0.61% | 102% | 98% |
| Class E | -0.23% | -0.11% | -0.18% | 101% | 100% |
| Overall | -0.23% | -0.29% | -0.29% | 101% | 99% |
| Class D | -0.51% | -0.78% | -0.69% | 103% | 94% |
| Class F | -0.35% | -0.27% | -0.20% | 100% | 99% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

## Test B

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.36% | -0.14% | 102% | 100% |
| Class A2 | -0.19% | -0.48% | -0.43% | 103% | 100% |
| Class B | -0.44% | -0.56% | -0.58% | 104% | 100% |
| Class C | -0.85% | -0.89% | -0.83% | 106% | 100% |
| Class E | -0.40% | -0.47% | -0.52% | 103% | 100% |
| Overall | -0.43% | -0.57% | -0.53% | 104% | 100% |
| Class D | -0.61% | -0.90% | -0.81% | 106% | 100% |
| Class F | -0.57% | -0.30% | -0.26% | 102% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

## Test A + NSPT for 4x16 and 16x4

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.30% | -0.02% | 101% | 100% |
| Class A2 | -0.12% | -0.34% | -0.36% | 102% | 100% |
| Class B | -0.36% | -0.47% | -0.49% | 102% | 100% |
| Class C | -0.68% | -0.74% | -0.79% | 104% | 100% |
| Class E | -0.39% | -0.26% | -0.44% | 102% | 99% |
| Overall | -0.35% | -0.44% | -0.45% | 102% | 100% |
| Class D | -0.60% | -0.80% | -0.93% | 105% | 99% |
| Class F | -0.45% | -0.28% | -0.31% | 101% | 99% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

## Test A + NSPT for 8x16 and 16x8

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.20% | -0.12% | 101% | 100% |
| Class A2 | -0.10% | -0.32% | -0.28% | 102% | 100% |
| Class B | -0.31% | -0.37% | -0.39% | 103% | 100% |
| Class C | -0.69% | -0.69% | -0.59% | 105% | 100% |
| Class E | -0.24% | -0.19% | -0.26% | 103% | 100% |
| Overall | -0.31% | -0.37% | -0.35% | 103% | 100% |
| Class D | -0.51% | -0.79% | -0.83% | 105% | 100% |
| Class F | -0.49% | -0.16% | -0.28% | 101% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# Complexity analysis (informative)

This section examines the worst-case complexity that the decoder is required to support for the inverse transform stage, in terms of multiplications per sample. This analysis assumes the current tools adopted to ECM-7.0, plus the eight NSPT kernels proposed in Test B of [1].

- Separable DCT Type II is supported from 2-point up to 256-point transforms. The worst-case complexity occurs for 256x256 block sizes. If the DCT2 were implemented naively, matrix multiplications would result in 256x256x256x2 total multiplications for the block, or 512 multiplications per sample. However due to its regular and symmetric structure, DCT2 can be reduced in complexity by butterfly factorisations. Analysis of the true complexity is therefore beyond the scope of this contribution, but it is assumed DCT2 does not dominate the worst-case complexity.

- MTS includes selection of alternative trigonometric types: DCT8, DST7, DCT5, DST4 and DST1 [2]. Analysis of these transforms is also neglected.

- Separable KLTs are selectable in Inter-MTS [3] but only for 4-point, 8-point, and 16-point transforms. The worst-case complexity occurs for 16x16 block sizes. Assuming there is not an efficient method of implementing these KLTs, straightforward separable matrix multiplication results in 16+16 = 32 multiplications per sample.

- LFNST in ECM consists of three different kernels. For 4xN/Nx4 size blocks, the inverse LFNST requires 16x16 total multiplications. For 8xN/Nx8 size blocks, the inverse LFNST requires 64x32 total multiplications. For 16xN/Nx16 size blocks, the inverse LFNST requires 96x32 total multiplications. On a multiplications per sample basis, the worst-case complexity occurs with 8x8 size blocks, with 32 multiplications per sample. This neglects the additional complexity of performing the inverse separable DCT2 on the non-zero out region.

- The NSPT is proposed for eight different block sizes, with the following complexity:

  - For 4x4 blocks, the inverse NSPT is 16x16, resulting in 16 multiplications per sample.

  - For 4x8/8x4 blocks, inverse NSPT is 32x20, resulting in 20 multiplications per sample.

  - For 4x16/16x4 blocks, inverse NSPT is 64x24, resulting in 24 multiplications per sample.

  - For 8x8 blocks, inverse NSPT is 64x32, resulting in 32 multiplications per sample.

  - For 8x16/16x8 blocks, inverse NSPT is 128x40, resulting in 40 multiplications per sample.

Considering the worst-case complexity across all transform tools, if NSPT kernels a-d are adopted, the worst-case complexity is improved, since the NSPT for 8x8 blocks replaces the LFNST. The LFNST complexity consists of 32 multiplications per sample, plus some unknown additional complexity and/or latency introduced by the inverse DCT2. This is replaced by just 32 multiplications per sample. However, NSPT kernels e introduce a clear worst-case bottleneck. The NSPT for 8x16/16x8 require 40 multiplications per sample.

# Proposed method

This contribution tests reducing the NSPT matrices for 8x16/16x8 blocks to 128x32. That is, the first 32 basis vectors are used and the next 8 are ignored. This change would harmonise the worst-case decoder complexity across the transform options in ECM.

The method is on top of Test B in [1] and implemented by modifying the latest commit of the ECM-AB-EE2 fork, TESTS_4.2 branch with the following change in source/Lib/CommonLib/UnitTools.cpp:

dimension = ( ( width == 4 && height == 16 ) || ( width == 16 && height == 4 ) ) ? 24 : ( ( ( width == 8 && height == 16 ) || ( width == 16 && height == 8 ) ) ? 40 32 : dimension );

In a “sanity check” test, the NSPT kernels for 8x16 and 16x8 blocks were also edited, with the weights in the last 8 rows of each matrix set to 0. This produces the same results, confirming that the above minimal change is sufficient.

The test was performed for accurate timing (test and anchor executed on the same cluster, clusters are homogenous). Relative to this contribution’s crosscheck of Test B in section 2.2, observable improvement in encoder time from 104% to 103% is found, even though the software is already optimised for SIMD. Perhaps unexpectedly, there is no overall coding loss; the coding gain changes from -0.43% -0.57% -0.53% AI in Test B to -0.44% -0.53% -0.52% AI with the proposed method.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.24% | -0.09% | 101% | 99% |
| Class A2 | -0.21% | -0.45% | -0.45% | 102% | 100% |
| Class B | -0.45% | -0.57% | -0.56% | 103% | 100% |
| Class C | -0.84% | -0.86% | -0.85% | 106% | 99% |
| Class E | -0.42% | -0.40% | -0.52% | 103% | 100% |
| Overall | -0.44% | -0.53% | -0.52% | 103% | 100% |
| Class D | -0.61% | -1.00% | -0.81% | 106% | 100% |
| Class F | -0.55% | -0.25% | -0.23% | 102% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.40% | -0.55% | -0.60% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.23% | -0.69% | -0.78% | 101% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

This contribution requests verification of the above results. If the results are satisfactorily verified, and confirm an improvement in gain/complexity trade-off, this contribution recommends adopting the proposed modification on top of EE2 test 4.2b.

# References

[1] P. Garus, M. Coban, B. Ray, V. Seregin, M. Karczewicz, M. Koo, J. Zhao, J. Lim, S. H. Kim, “EE2-4.2: Non-Separable Primary Transform for Intra Coding”, JVET-AC0130, by teleconference, Jan. 2023.

[2] B. Ray, M. Coban, V. Seregin, H. Egilmez, M. Karczewicz, “EE2: Enhanced intra MTS and LFNST (tests 4.1, 4.2, and 4.4)”, JVET-W0103, by teleconference, July 2021.

[3] B. Ray, V. Seregin, M. Karczewicz, “EE2 – 4.1: Inter-MTS optimization”, JVET-AA0133, by teleconference, July 2022.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).