| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0333-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-AC0170 (Non-EE2: Fuse intra template matching prediction with intra prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Fangjun Pu 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | +1 408 330 3281 fpu@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-AC0170 [1], which proposes to fuse intra template matching prediction (Intra TMP) with intra prediction, in which the prediction block is the weighted sum of two prediction signals generated by Intra TMP and intra prediction. The simulation results match with those provided in JVET-AC0170.

# Introduction

In ECM, intra template matching prediction (Intra TMP) is a special intra prediction mode that copies the best prediction block from the reconstructed part of the current frame, whose L-shaped template matches the current template. Since the best prediction block is generated by template matching, the coding performance of Intra TMP may be limited when the template is not similar to the current block.

To improve Intra TMP, it is proposed to fuse Intra TMP with intra prediction. Specially, the prediction block is the weighted sum of two prediction signals generated by Intra TMP and intra prediction, in which the intra prediction mode derived by TIMD is used for intra prediction. A CU flag is signalled whether the proposed method is used.

# Experimental Results

The modifications on the proposal were implemented on top of ECM-7.0. The simulations were conducted under the common test conditions of EE2 [2]. The crosscheck results are shown in Table 1.

Table 1. Crosscheck results of JVET-AC0170

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.03% | -0.02% | -0.01% | 100% | 98% | 0.02% | -0.06% | 0.11% | 99% | 100% |
| Class E | -0.16% | -0.22% | -0.15% | 102% | 101% |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.00% | -0.21% | -0.12% | 98% | 100% | 0.02% | -0.30% | 0.24% | 101% | 97% |
| Class F |  |  |  |  |  |  |  |  |  |  |  | 0.00% |  | 0.13% |  | -0.19% |  | 99% |  | 98% |  |

# Reference

- Y. Wang, K. Zhang, L. Zhang, “Non-EE2: Fuse intra template matching prediction with intra prediction”, Document of Joint Video Experts Team, JVET-AC0170, by teleconference, January 2023.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.