| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0334 |
| --- | --- |

| Title: | Summary of the Evidence on Lenslet Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Jin Shenzhen International Graduate School Tsinghua University 518000 Shenzhen, China Mehrdad Teratani École Polytechnique de Bruxelles University of Brussels 1050 Brussels, Belgium | Tel: Email: Email: | +86 180 3815 3075 jin.xin@sz.tsinghua.edu.cn Mehrdad.Teratani@ulb.be |
| Source: | WG4, AHG on LVC |  |  |

_____________________________

- Abstract

This contribution provides evidence of designing coding tools dedicated to lenslet videos. Lenslet video is a general format for dense light fields that can be captured by the plenoptic cameras, e.g. Lytro and Ratrix cameras, or rearranged from dense two-dimensional multiview data. It shows high potential in both storage-oriented and communication-oriented applications. We demonstrate that designing specific coding tools dedicated to lenslet videos can improve compression efficiency obviously compared to the existing standards. We hope the summarization of lenslet video coding evidence is helpful to support the creation of a video codec extension.

- Introduction

  - Introduction of Lenslet Video

Dense light fields can be captured by different devices such as a single camera on a moving gantry, array camera, or plenoptic camera. It is found that no matter what kind of acquisition device is used, lenslet data format can be used as a general format for dense light fields [N0121].

The lenslet data organization is shown in Figure 1 for plenoptic cameras and for camera arrays. For the light field captured by a plenoptic 1.0 or 2.0 camera, the output frame is automatically a lenslet frame. For the frame captured by the Plenoptic 1.0 camera that was first developed by Ren Ng in 2005 [1], whose optical structure is shown in Fig.1(a), the output lenslet light field image sample is shown in Fig.1 (b). In it, each pixel in a lenslet unit records a ray direction at this position. For the frame captured by Plenoptic 2.0 camera that first developed by A. Lumsdaine in 2009 [2], whose relay imaging structure is shown in Fig.1(c), the output lenslet light field image is shown in Fig.1(d), in which a microimage of the target is recorded in each lenslet unit. Meanwhile, the multi-view data that are captured by moving gantry or camera arrays, like that in Fig. 1(e), can be transformed into the lenslet format as shown in Fig. 1(f). Pixels at the same spatial coordinates of each view of multi-view format are extracted and reorganized according to their angular coordinates to generate a lenslet unit. Then, a lenslet light field image can be generated by organizing these units according to their spatial coordinates.

It is found that lenslet light field format is a universal data format to represent the dense light field. Also, it is friendly to a variety of applications of light field, which are summarized in the following.

| (a) | (b) |
| --- | --- |
| (c) | (d) |
| (e) | (f) |

Figure 1. (a) optical structure of plenoptic 1.0; (b) lenslet format of plenoptic 1.0 image; (c) optical structure of plenoptic 2.0; (d) lenslet format of plenoptic 2.0 images; (e) multi-view of camera array; (f) lenslet format of camera array.

Ad hoc Group (AhG) on Lenslet Video Coding (LVC) has summarized its use cases and requirements in storage-oriented applications and communication-oriented applications [m58880]. Storage-oriented applications focus on store the lenslet data captured by the plenoptic camera and light field surveillance system, provide high quality or lossless compression for light field microscopy and endoscopy, and the in-progress applications like store the lenslet data for integral display, layered display and holographic display. Communication-oriented applications focus on fast and low bit rate compression for industrial inspection and robotics.

  - Activities of Lenslet Video Coding

WG04 started investigation on lenslet video coding from 2017 under dense light field (DLF) subgroup [m41787]. After a series of exploration experiments, the LVC AhG was established in the 134th meeting focusing on analyzing and developing the coding tools [N0094].  An overview of LVC AhG is found in [N0273].

By now, acquisition systems, test sequences, conversion tools have been investigated and designed [N0275]:

- There are two types of acquisition systems (plenoptic camera 1.0 and plenoptic 2.0) to acquire Lenslet video content. The details of the capturing system for these kinds of cameras are summarized in the related inputs [m56789] and the Activity report on dense light field [N00056]. All datasets are summarized in [N00053].

- Testing sequences in Lenslet format have been collected and summarized in the Overview of the MPEG-I-Visual Test Materials [N00053]. There are 16 test sequences used for evaluating the compression performance, which include 4 plenoptic 1.0 video sequences captured by Lytro Illum, 4 plenoptic 2.0 video sequences captured by Tsinghua plenoptic video camera, and 8 plenoptic 2.0 video sequences captured by Raytrix.

- Tools convert from lenslet video to multiview video have also been provided, i.e. Reference Lenslet content Convertor (RLC) [WG11N18567] [N0217].

- Coding Tools and Compression Performance

  - Coding Tools

For Lenslet Video Coding, coding tools designed by Tsinghua SIGS have been summarized here [m61869][N0275]. The tools designed for the plenoptic 1.0 lenslet video and the plenoptic 2.0 lenslet video are listed in Table 1 and Table 2, respectively.

Table 1. Coding tools for plenoptic 1.0 video sequences.

| Inter/Intra Tools | Coding Tools | Description |
| --- | --- | --- |
| Intra | Macropixel Based Intra Prediction (MIP) [m44685] | This method uses the reference blocks to calculate the weight value and reference block(s) weighted to predict the current PU. |
| Intra | Co-located Single-Block Prediction (CSP) [m44685] [m60998] | This mode uses one co-located reference block(s) to predict the current PU. |
| Intra | Boundary Matching based Prediction (BMP) [m46261] [m60998] | This mode uses all available weighted reference blocks to predict the current PU. |
| Inter | Macropixel-based Fast Motion Estimation (MFME) [m44685] [m56785] [m60998] | This method uses the 8-step neighborhood refinement with early termination to improve the performance based on the best reference block determined. |
| Inter | Macropixel-constrained Collocated Position Search (MCPS) [m60183] [m60992] | This method finetunes MVP by the macropixel-constrained collocated position in the nearest macropixel, and uses all the macropixel-constrained collocated positions, derived from the fine-tuned MVP, as motion search candidates. |
| Inter | Microlens-based Test Zone Search (MTZS) [m60992] | This method modifies the searching steps in conventional TZ search to multiple times of micro-image distance. |
| Inter | Local illumination compensation inter prediction (LIC) [m61863] | This method is based on a linear model for illumination changes, using a scaling factor a and an offset b. |
| Inter | Piecewise-Linear-Model-based Local Illumination Compensation inter prediction (PLMLIC) [m61864] | This method proposes a new neighboring-sample selection and classification method based on multiple reference lines for illumination changes. |

Table 2. Coding tools for plenoptic 2.0 video sequences.

| Inter/Intra Tools | Coding Tools | Description |
| --- | --- | --- |
| Intra | Pixel-Gradient-based Zooming (PGZ) [m58826] | This method realizes the zooming effect by zooming factor estimation, zooming and tailoring steps. |
| Intra | Image-correlated Intra Prediction mode (IIP) [m53793] [m60998] | This method uses block-based linear weighted prediction to predict the current PU, with the exception that the determination of the reference blocks is based on the imaging principle of the plenoptic 2.0 camera. |
| Intra | Boundary Matching based Prediction (BMP) [m53793] | This mode uses all available weighted reference block(s) to predict the current PU. |
| Intra | Co-located Single-Block Prediction (CSP) [m44685] [m60998] | This mode uses one co-located reference block(s) to predict the current PU. |
| Intra | Parameter Adaptive Image-correlated Intra Prediction (PAIIP) [m61862] | The method adoptive calculates the camera parameter for each plenoptic sequence to achieve better results. |
| Inter | Microlens-based Test Zone Search (MTZS) [m60992] | This method modifies the searching steps in conventional TZ search to multiple times of micro-image distance. |
| Inter | Micro-image Correlative Block Search (MCBS) [m60997] | This method sets the correlative blocks among adjacent micro-images to search candidates. |
| Inter | Microimage-based Two-Step Search (MTTS) [m61861] | This method defines micro-image correlative block search and refinement search to compensate for defocused inaccuracies. |
| Inter | Local illumination compensation inter prediction (LIC) [m61863] | This method is based on a linear model for illumination changes, using a scaling factor a and an offset b. |
| Inter | Piecewise-Linear-Model-based Local Illumination Compensation inter prediction (PLMLIC) [m61864] | This method proposes a new neighboring-sample selection and classification method based on multiple reference lines for illumination changes. |

  - Coding Performance

First, Table 3 compares the compression performance on the different platforms for lenslet videos for intra-coding and inter-coding with random access profile (RA). As listed in the table, VVC shows superior performance in coding lenslet videos. It outperforms HEVC and HEVC SCC profiles obviously, which is suitable to be the platform for lenslet video coding.

Table 3. The Average performance comparison on different platforms.

| Camera Type | HEVC+SCC vs. HEVC |  | VVC vs. HEVC |  | VVC vs. HEVC+SCC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Intra | RA | Intra | RA | Intra | RA |
| Plenoptic 1.0 | -19.00% | -23.76% | -41.80% | -37.16% | -22.80% | -13.40% |
| Plenoptic 2.0 | -19.53% | -15.03% | -37.34% | -25.43% | -17.81% | -10.40% |

Table 4 compares the methods on HEVC. The proposal of the reported methods and the codec version are listed in detail. BMP and PGZ achieve impressive intra-coding compression performance both for plenoptic 1.0 and plenoptic 2.0 lenslet videos. MFME and MTZS achieve good trade-offs between complexity and compression efficiency by searching according to the imaging principle of plenoptic 1.0 and plenoptic 2.0 lenslet videos, respectively. Integrating some of the tools together, higher compression efficiency can be achieved.

Table 4. Average BD-Bitrate results on the HEVC/HEVC-SCC platform.

| Tool | Proposal | Codec Version | Test Sequences | Prop. vs. HEVC |  | Prop. +SCC vs. SCC |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Intra | RA | Intra | RA |
| BMP | m46261 m47314 | HM-16.9+SCM-8.0 & HM-15.0+RExt-8.0+SCM-2.0 | Plenoptic 1.0 | -20.52% | -12.72% | -8.67% | -5.52% |
| MFME | m44685 | HM-16.15 | Plenoptic 1.0 | - | -7.11% (Lowdelay profile) | - | - |
| MCPS | m60992 | HM-16.21+ SCM-8.8 | Plenoptic 1.0 | - | - | - | -2.98% |
| CSP + BMP +MCPS | m60183 | HM-16.9+SCM-8.0 | Plenoptic 1.0 | -21.60% | -15.34% | -8.35% | -0.91% |
| IIP | m53793 |  | Plenoptic 2.0 | -6.25% | -3.99% | -1.25% | -0.28% |
| PGZ | m58826 |  | Raytrix | -15.90% | -8.07% | -3.55% | -1.64% |
| MTZS | m60992 | HM-16.21+ SCM-8.8 | Plenoptic 1.0 | - | - | - | -2.83% |
|  |  |  | Raytrix | - | - | - | -7.33% |
| CSP+IIP | m60183 | HM-16.9+SCM-8.0 | Plenoptic 2.0 | -16.08% | -10.82% | -4.39% | -3.68% |
| PAIIP | m61862 |  | Plenoptic 2.0 | -12.48% | - | - | - |

Table 5 compares the methods on VVC. Integrating some coding tools also provides obvious compression efficiency on the VVC platform.

Table 5. Average BD-Bitrate results on the VVC platform.

| Tool | Proposal | Codec Version | Test Sequences | Prop. vs. VVC |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Intra | RA |
| MFME | m56785 | VTM-10.0 | Plenoptic 1.0 | - | -4.57% (Lowdelay profile) |
| BMP | m60998 | VTM-11.0 | Plenoptic 1.0 | -2.49% | -1.92% |
| CSP | m60998 |  | Plenoptic 1.0 | -1.62% | -1.25% |
|  |  |  | Plenoptic 2.0 | -1.36% | -1.33% |
| CSP+BMP | m60998 |  | Plenoptic 1.0 | -2.87% | - |
| CSP+BMP+MFME | m60998 |  | Plenoptic 1.0 | -2.87% | -2.20% |
| IIP | m60998 |  | Plenoptic 2.0 | -2.10% | -2.05% |
| CSP+IIP | m60998 |  | Plenoptic 2.0 | -2.71% | - |
| CSP+IIP+MCBS | m60998 |  | Plenoptic 2.0 | -2.71% | -2.66% |
| MTTS | m61861 |  | Plenoptic 2.0 | - | -2.26% |

Also, a new tool called PLMLIC shows compression improvement on EMC, as listed Table 6.

Table 6. Average BD-Bitrate results on the ECM platform.

| Tool | Proposal | Codec Version | Test Sequences | Prop. vs. ECM |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Intra | RA |
| PLMLIC | m61864 | ECM-1.0 | Plenoptic 1.0 | - | -0.13% |
|  |  |  | Plenoptic 2.0 | - | -0.18% |

Since the tools are proposed in different meetings, tools like PGZ, PAIIP, MTTS and PLMLIC that present obvious compression improvements have not been integrated together. According to Table 4-6, we estimated average BD-Bitrate results of the latest methods and integrated methods on the VVC platform. We estimate the compression performance according to the individual or integrated tools on HEVC and the compression ratio between VVC and HEVC. It is possible to see that integrating the best performed methods together, like CSP+PAIIP+MTSS+PGZ+ PLMLIC, could achieve -6.55% and -6.11% bitrate reduction compared to VVC intra and RA coding for plenoptic 2.0 lenslet video coding.

Table 7. Estimated Average BD-Bitrate results on the VVC platform.

| Tool | Estimation Reference | Test Sequences | Prop. vs. VVC |  |
| --- | --- | --- | --- | --- |
|  |  |  | Intra | RA |
| PLMLIC | Estimated from the improvement compared to platform ECM [m61864] | Plenoptic 1.0 | - | -0.13% |
|  |  | Plenoptic 2.0 | - | -0.18% |
| CSP+BMP+MFME+ PLMLIC | CSP+BMP+MFME and [m60998] the estimated performance of PLMLIC | Plenoptic 1.0 | -2.87% | -2.33% |
| PAIIP | According to IIP and the performance of PAIIP [m61862] on HEVC and compression ratio of HEVC [m53793] to VVC [m60998] | Plenoptic 2.0 | -3.49% | -3.41% |
| PGZ | From the coding performance of PGZ [m58826] on HEVC and compression ratio of HEVC [m53793] to VVC [m60998] | Plenoptic 2.0 | -4.45% | -3.46% |
| CSP+PAIIP+MTSS | CSP+IIP+MCBS [m60998] and improvement between PAIIP and IIP[m60998], and MCBS[m60997] and MTTS[m61861] | Plenoptic 2.0 | -4.10% | -4.47% |
| CSP+PAIIP+MTSS+PGZ | CSP+PAIIP+MTSS and the estimated performance of PGZ | Plenoptic 2.0 | -6.55% | -5.93% |
| CSP+PAIIP+MTSS+PGZ+ PLMLIC | CSP+PAIIP+MTSS+PGZ and the estimated performance of PLMLIC | Plenoptic 2.0 | -6.55% | -6.11% |

- Conclusions

In this contribution, we summarize the evidence of the coding tools and related compression performance on different platforms and estimate the integrated compression performance on the VVC platform. The integrated methods are estimated to achieve -2.87%/-2.33% and -6.55%/-6.11% compared to VVC standards for plenoptic 1.0 and plenoptic 2.0 lenslet video coding, respectively.

- We considered that the summarization of lenslet video coding evidence is helpful to support WG04 LVC AhG to consider the creation of a video codec extension, e.g., via a profile, to support such lenslet video formats in an efficient way. Also, we call on relevant personnel of the VVC to participate in plenoptic lenslet video coding on VVC Extension.

# References

[1] R. Ng, “Digital light field photography,” Ph.D. thesis (Stanford University, 2006).

[2] A. Lumsdaine and T. Georgiev, “The focused plenoptic camera,” Computational Photography (ICCP), 2009 IEEE International Conference on, San Francisco, CA, pp. 1-8, 2009.

[N18567] Mehrdad Teratani, Toshiaki Fujii, “Manual of Reference Lenslet content Convertor (RLC 0.3)”, ISO/IEC JTC1/SC29/WG11, N18567, Gothenburg, Sweden, July 2019.

[N0053] “Overview of MPEG-I Visual Test Materials”, ISO/IEC JTC 1/SC 29/WG 04, MDS20005_WG04_N00053, Online, January 2021.

[N0056] “Activity Report on Dense Light Fields”, ISO/IEC JTC 1/SC 29/WG 04, MDS20008_WG04_N00056, Online, January 2021.

[N0094] “Overview of Dense Light Fields Activities”, ISO/IEC JTC 1/SC 29/WG 4/N0094, Online, April, 2021.

[N0121] “Overview of Dense Light Fields Activities”, ISO/IEC JTC 1/SC 29/WG 4/N0121, Online, August 2021.

[N0217] “User Manual of RLC2.0”, ISO/IEC JTC1/SC29/WG4 MPEG2021/ N0096, Online, April 2022.

[N0273] “Overview of lenslet video coding activities”, ISO/IEC JTC1/SC29/WG4 MPEG2022, Online, October 2022.

[N0275] “Collection of Evidence on Lenslet Video Coding”, ISO/IEC JTC 1/SC 29/WG 04/N0275, Mainz, October, 2022.

[m41787] Mehrdad Panahpour Tehrani, Shu Fujita, Toshiaki Fujii. “[MPEG-I Visual] Introduction to A New Test Sequence Captured by Light Field Video Camera”, ISO/IEC JTC1/SC29/WG11 MPEG2017/ m41787, Macau, China, October 2017.

[m44685] Xin Jin, Tingting Zhong, Lingjun Li, “Exploration of Lenslet Data Compression.” ISO/IEC JTC1/SC29/WG11 MPEG2018/ m44684, Macau, China, October 2018.

[m46261] Lingjun Li, Xin Jin, Haixu Han, Tingting Zhong, “Boundary matching based prediction for lenslet video compression”, ISO/IEC JTC1/SC29/WG11 MPEG2019/M46261, Marrakesh, Morocco January 2019.

[m53739] Fan Jiang, Xin Jin, Tingting Zhong, “Exploration Experiment of Plenoptic 2.0 Test Sequences for Dense Light Field Compression”, ISO/IEC JTC1/SC29/WG4 MPEG2021/ M53739, Alphach, Austria, April 2020.\

[m56785] 	Kedeng Tong, Xin Jin, Fan Jiang, Tingting Zhong, Chen Wang, “[DLF] Transfer the inter coding tool of lenslet video compression to VVC”, ISO/IECJTC1/SC29/WG04, M56785, Online, April 2021.

[m56789] Xin Jin, Kedeng Tong, Toshiaki Fujii, Mehrdad Teratani, “Applications of Lenslet Format for Dense Light Field”, ISO/IEC JTC1/SC29/WG4 MPEG2021/M56789, Online, April 2021.

[m58880] Xin Jin, Mehrdad Teratani, “[LVC] [DLF] Update on Lenslet Video Coding Use Cases”, ISO/IEC JTC 1/SC 29/WG 4 m58880, Online, January 2022.

[m60183] Yuqing Yang, Xin Jin, Kedeng Tong, Chen Wang, “[LVC][DLF] Integrated coding tools for the lenslet video on HEVC”, ISO/IEC JTC 1/SC 29/WG 4 m 60183, Online, July 2022.

[m60992] Jonghoon Yim, Thuc Nguyen Huu, Byeungwoo Jeon, “Fast and efficient microlens-based motion search for Lenslet Video Coding”, ISO/IEC JTC 1/SC 29/WG 4 m60992, Mianz, October. 2022.

[m60997] Yuqing Yang, Xin Jin, Chen Wang, Kedeng Tong, Haitian Huang, “Improved Test Zone Search for Plenoptic 2.0 video coding”, ISO/IEC JTC1/SC29/WG4 MPEG2022/M60997, Mianz, October 2022.

[m60998] Chen Wang, Xin Jin, Kedeng Tong, Yuqing Yang, “[LVC] [DLF] Integrated coding tools for the lenslet video on VVC”, ISO/IEC JTC 1/SC 29/WG 4 m60998, Mianz, October. 2022.

[m61861] Yuqing Yang, Xin Jin, Chen Wang, Kedeng Tong, Haitian Huang, “Microimage-based Two-step Search for Plenoptic 2.0 Video Coding”, Online, ISO/IEC JTC 1/SC 29/WG 4 m 61861, January 2023,

[m61862] Chen Wang, Xin Jin, Kedeng Tong, Yuqing Yang, Haitian Huang, “[LVC] [DLF] Adjusted image-correlated intra prediction (IIP) mode for the lenslet video on HEVC”, ISO/IEC JTC 1/SC 29/WG 4 m61862

Online, Jan. 2023.

[m61864] Chen Wang, Xin Jin, Kedeng Tong, Yuqing Yang, Haitian Huang, “[LVC] [DLF] Piecewise linear model based local illumination compensation inter prediction for the lenslet video on ECM”, ISO/IEC JTC 1/SC 29/WG 4 m61864, Online, Jan. 2023.

[m61869] Kedeng Tong, , Xin Jin, Chen Wang, Yuqing Yang, Haitian Huang, “[LVC] [DLF] Summary of the Coding Tools on Lenslet Video Coding”, ISO/IEC JTC 1/SC 29/WG 4 m61864, Online, Jan. 2023.