| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0335-v1 |
| --- | --- |

| Title: | Non-EE2: Content adaptive OBMC enabling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Cui, Zhi Zhang, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | kaicui@qti.qualcomm.com zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a method to adaptively control OBMC based on hash block hit percentage. It reports {-0.02%, -0.02%, -0.03%} YUV BD-rate gains for Class F and {-1.28%, -0.80%, -0.69%} YUV BD-rate gains for Class TGM under RA configuration.

# Proposal

OBMC is controlled by a SPS flag. It is not always efficient for screen content coding. In the proposed method, OBMC SPS flag is set based on the hash block hit percentage at encoder, which is already available in ECM and used in other tools. If it is larger than a threshold, the video sequence is interpreted as screen content and the OBMC is not applied.

# Simulation results

The method proposed in this contribution has been implemented on top of ECM-7.0 and it is simulated under JVET common test condition (CTC) defined in [2].

Table 1 Simulation results of the proposed method over ECM-7.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.02% | -0.02% | -0.03% |  |  |
| Class TGM | -1.28% | -0.80% | -0.69% |  |  |

# Reference

[1] ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution.