| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0339-v1 |
| --- | --- |

| Title: | New higher level for multilayer coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura, Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution proposes addition of a higher level for multilayer coding. In VVC version 2, maximum level 6.3 is defined for 4K/8K multilayer coding as well as single layer coding of 12K 60Hz. However, max luma sample rate (MaxLumaSr) of level 6.3 is not sufficient for three-layer coding use case such as 2K/4K/8K multilayer coding with a frame rate of 60Hz. Therefore, new higher level 6.4 for typical three-layer coding is proposed as follows:

Table A.2 – General tier and level limits

| Level | general_level_idc value* | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slices per AU MaxSlicesPerAu | Max # of tiles MaxTilesPerAu | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | High tier |  |  |  |
| 6.3 | 105 | 80 216 064 | 240 000 | 1 600 000 | 1 000 | 990 | 30 |
| 6.4 | 108 | 80 216 064 | 240 000 | 1 600 000 | 1 000 | 990 | 30 |

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 6.3 | 4 812 963 840 | 320 000 | 1 600 000 | 8 | 4 |
| 6.4 | 9 625 927 680 | 320 000 | 1 600 000 | 8 | 4 |

Introduction

In JVET-AB0087, several use cases of VVC multilayer coding in broadcasting and streaming applications were reported [1]. Figure 1 shows one example of three-layer coding reported in JVET-AB0087, where highest 8K layer is referencing to the lower HD and 4K layers. Three-layer coding may be useful not only for broadcasting applications but also for bitrate ladder of streaming applications to save the storage.

Figure 1. Use case of three-layer coding

However, in VVC specification version 2, maximum level 6.3 limits the frame rate up to 30 Hz for 2K/4K/8K three-layer coding due to the limitation of max luma sample rate (MaxLumaSr). Specifically, MaxLumaSr is set equal to 4,812,963,840 at level 6.3. In VVC specification section A.4.2 Profile-specific level limit, requirement of bitstream is defined as follows:

- The difference between consecutive output times of pictures of different AUs from the DPB, as specified in clause ‎C.3.3, shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( AuSizeMaxInSamplesY[ n ]  MaxLumaSr, FrVal ), where MaxLumaSr is the value specified in Table A.3 for AU n, provided that AU n has a picture that is output and AU n is not the last AU of the bitstream that has a picture that is output.

where DpbOutputInterval is interval time of output picture which has PicutreOutputFlag equal to 1, and AuSizeMaxInSamplesY is equal to PicSizeMaxInSamplesY * numDecPics.

In 2K/4K/8K multilayer coding, PicSizeMaxInSamplesY is set equal to 33,177,600 (=4,320*7,680) and numDecPics is the number of pictures in AU, which means 3. This means that at level 6.3, DpbOutputInterval should be greater than or equal to 0.02068…

Due to the above constraint, the maximum frame rate is 48Hz (1/0.02068≈48.3555…) for 3-layer coding with 8K as the maximum resolution while required frame rate for broadcasting application is generally 50 or 60Hz.

Table A.3 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 1.0 | 552 960 | 128 | - | 2 | - |
| 2.0 | 3 686 400 | 1 500 | - | 2 | - |
| 2.1 | 7 372 800 | 3 000 | - | 2 | - |
| 3.0 | 16 588 800 | 6 000 | - | 2 | - |
| 3.1 | 33 177 600 | 10 000 | - | 2 | - |
| 4.0 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5.0 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.0 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 8 | 4 |
| 6.3 | 4 812 963 840 | 320 000 | 1 600 000 | 8 | 4 |

Proposal

To enable 2K/4K/8K three-layer coding with up to 60Hz, it is proposed to add a new level 6.4 as follows:

Table A.2 – General tier and level limits

| Level | general_level_idc value* | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slices per AU MaxSlicesPerAu | Max # of tiles MaxTilesPerAu | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | High tier |  |  |  |
| 6.0 | 96 | 35 651 584 | 80 000 | 240 000 | 600 | 440 | 20 |
| 6.1 | 99 | 35 651 584 | 120 000 | 480 000 | 600 | 440 | 20 |
| 6.2 | 102 | 35 651 584 | 180 000 | 800 000 | 600 | 440 | 20 |
| 6.3 | 105 | 80 216 064 | 240 000 | 1 600 000 | 1 000 | 990 | 30 |
| 6.4 | 108 | 80 216 064 | 240 000 | 1 600 000 | 1 000 | 990 | 30 |

Table A.3 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 6.0 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 8 | 4 |
| 6.3 | 4 812 963 840 | 320 000 | 1 600 000 | 8 | 4 |
| 6.4 | 9 625 927 680 | 320 000 | 1 600 000 | 8 | 4 |

Note that all the entries other than general_level_idc_value of Table A.2 and MaxLumaSr of Table A.3 are unchanged.

Conclusion

This contribution proposes new higher level 6.4 which includes doubled MaxLumaSr to enable 2K/4K/8K three-layer coding with 60Hz. It is recommended to include future versions of VVC specifications.

References

[1] S. Nemoto, S. Iwamura, A. Ichigaya, “Multilayer coding use cases for broadcasting and streaming applications,” JVET-AB0087, Oct. 2022.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).