| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0344-v1 |
| --- | --- |

| Title: | AHG9: On Neural Network Post Filter Patch Size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Ahmed Sidiya Miska M. Hannuksela Ye-Kui Wang | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com miska.hannuksela@nokia.com yekui.wang@bytedance.com |
| Source: | Sharp Labs of America, Inc., Nokia, Bytedance Inc. |  |  |

_____________________________

# Abstract

This document is a follow-up to JVET-AC0062 Proposal 1. It is proposed to signal syntax elements nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 only when nnpfc_constant_patch_size is equal to 1 and to signal two different syntax elements (nnpfc_extended_patch_width_cd_delta_minus1 and nnpfc_extended_patch_height_cd_delta_minus1) when nnpfc_constant_patch_size is equal to 0. Accordingly, the semantics are changed, to clarify that, when nnpfc_constant_patch_size is equal to 0, the requirement is that the width and height of an extended patch (i.e., a patch plus the overlapping area), not that of a patch, shall be a positive integer multiple of a horizontal and vertical common divisor, respectively. Additionally, it is proposed to move the syntax element nnpfc_overlap up.

# Introduction

This document is a follow-up to JVET-AC0062 Proposal 1.

Some modifications are proposed to the NNPFC SEI message.

# Proposals

It is asserted that nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 currently do not have any semantics when nnpfc_constant_patch_size_flag is equal to 0. It is also asserted that the syntax elements nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 serve slightly different roles depending upon the value of nnpfc_constant_patch_size_flag. For example, only when nnpfc_constant_patch_size_flag is equal to 1, the value of inpPatchWidth is set equal to nnpfc_patch_width_minus1 + 1 and the value of inpPatchHeight is set equal to nnpfc_patch_height_minus1 + 1. On the other hand, when nnpfc_constant_patch_size_flag is equal to 0, currently those two syntax elements (nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1) indicate together with overlap size (overlapSize) horizontal and vertical values, a positive integer multiple of which the post filter accepts as input. But those syntax elements (nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1) do not directly define the value of inpPatchWidth and inpPatchHeight when nnpfc_constant_patch_size_flag flag is equal to 0.

Thus, it is proposed to signal syntax elements nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 only when nnpfc_constant_patch_size is equal to 1 and to signal two different syntax elements (nnpfc_extended_patch_width_cd_delta_minus1 and nnpfc_extended_patch_height_cd_delta_minus1 ) when nnpfc_constant_patch_size is equal to 0. It is also proposed to move the syntax element nnpfc_overlap up. The proposed modifications are shown in red below.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| nnpfc_overlap | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| if( nnpfc_constant_patch_size_flag ) { |  |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| } else { |  |
| nnpfc_extended_patch_width_cd_delta_minus1 | ue(v) |
| nnpfc_ extended_patch_height_cd_delta_minus1 | ue(v) |
| } |  |
| nnpfc_overlap | ue(v) |
| … |  |
| } |  |

...

nnpfc_overlap indicates the overlapping horizontal and vertical sample counts of adjacent input tensors of the post-processing filter. The value of nnpfc_overlap shall be in the range of 0 to 16383, inclusive.

nnpfc_constant_patch_size_flag equal to 1 indicates that the post-processing filter accepts exactly the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input. nnpfc_constant_patch_size_flag equal to 0 indicates that the post-processing filter accepts as input any patch size with width inpPatchWidth and height inpPatchHeight such that the width of an extended patch (i.e., a patch plus the overlapping area), which is equal to inpPatchWidth + 2 * nnpfc_overlap, is a positive integer multiple of nnpfc_extended_patch_width_cd_delta_minus1 + 1 + 2 * nnpfc_overlap, and the height of the extended patch, which is equal to inpPatchHeight + 2 * nnpfc_overlap, is a positive integer multiple of nnpfc_extended_patch_height_cd_delta_minus1 + 1 + 2 * nnpfc_overlap. the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input.

nnpfc_patch_width_minus1 + plus 1, when nnpfc_constant_patch_size_flag equal to 1, indicates the horizontal sample counts of the patch size required for the input to the post-processing filter. The value of nnpfc_patch_width_minus1 shall be in the range of 0 to Min( 32766, CroppedWidth − 1 ), inclusive.

nnpfc_patch_height_minus1 + plus 1, when nnpfc_constant_patch_size_flag equal to 1, indicates the vertical sample counts of the patch size required for the input to the post-processing filter. The value of nnpfc_patch_height_minus1 shall be in the range of 0 to Min( 32766, CroppedHeight − 1), inclusive.

nnpfc_extended_patch_width_cd_delta_minus1 plus 1 plus 2 * nnpfc_overlap, when nnpfc_constant_patch_size_flag equal to 0, indicates a common divisor of all allowed values of the width of an extended patch required for the input to the post-processing filter. The value of nnpfc_extended_patch_width_cd_delta_minus1 shall be in the range of 0 to Min( 32766, CroppedWidth − 1 ), inclusive.

nnpfc_extended_patch_height_cd_delta_minus1 plus 1 plus 2 * nnpfc_overlap, when nnpfc_constant_patch_size_flag equal to 0, indicates a common divisor of all allowed values of the height of an extended patch required for the input to the post-processing filter. The value of nnpfc_extended_patch_height_cd_delta_minus1 shall be in the range of 0 to Min( 32766, CroppedHeight − 1 ), inclusive.

Let the variables inpPatchWidth and inpPatchHeight be the patch size width and the patch size height, respectively.

If nnpfc_constant_patch_size_flag is equal to 0, the following applies:

–	The values of inpPatchWidth and inpPatchHeight are either provided by external means not specified in this document or set by the post-processor itself.

–	The value of inpPatchWidth + 2 * nnpfc_overlap shall be a positive integer multiple of nnpfc_patch_width_minus1 nnpfc_extended_patch_width_cd_delta_minus1 + 1 + 2 * nnpfc_overlap and inpPatchWidth shall be less than or equal to CroppedWidth. The value of inpPatchHeight + 2 * nnpfc_overlap shall be a positive integer multiple of nnpfc_patch_height_minus1 nnpfc_extended_patch_height_cd_delta_minus1 + 1 + 2 * nnpfc_overlap and inpPatchHeight shall be less than or equal to CroppedHeight.

Otherwise (nnpfc_constant_patch_size_flag is equal to 1), the value of inpPatchWidth is set equal to nnpfc_patch_width_minus1 + 1 and the value of inpPatchHeight is set equal to nnpfc_patch_height_minus1 + 1.

nnpfc_overlap indicates the overlapping horizontal and vertical sample counts of adjacent input tensors of the post-processing filter. The value of nnpfc_overlap shall be in the range of 0 to 16383, inclusive.

...

[Ed. (YK): In Equation (79), remove the row of "overlapSize = nnpfc_overlap". And replace all instances of "overlapSize" in the document with "nnpfc_overlap".]

...

# Conclusion

This document proposes modifications to neural network post processing characteristics SEI. It is proposed to adopt the proposal in the next version of “Additional SEI messages for VSEI” text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] MDS21947_WG05_N00158/ JVET-AB2006, Additional SEI messages for VSEI, Online, October 2022