| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0351-v1 |
| --- | --- |

| Title: | BoG report on non-normative optimization for machine |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | BoG report |  |  |
| Author(s) or Contact(s): | Christopher Hollmann Shan Liu (BoG coordinators) | Email: | christopher.hollmann@ericsson.com shanl@tencent.com |
| Source: | BoG on non-normative optimization for machine |  |  |

_____________________________

# Abstract

This document contains the report of the BoG on non-normative optimization for machine. The BoG met on Wednesday 18 January 2023 at 1520-1720UTC, discussing subjects related to non-normative optimization for machine consumption of coded video content, including:

- Common test conditions

  - Test sequences

  - RA/LD/AI configurations

  - QP points and BD-rate calculation

- Cross-check procedure

- Reference software

  - Where to setup the repository

  - What to be included in the initial version reference software

- Other aspects

# Recommendations

The BoG recommends:

- Align common test conditions with WG 4 VCM (including test sequences, test configurations, QP points, etc.)

- Further discuss on QP point selection for calculation of the BD-rate used for tool evaluation.

- Align crosscheck procedure with WG 4 VCM.

- Further discuss on crosscheck procedure for neural network-based encoding as well as processing tools.

- Setup repository for software development and test.

- Integrate technologies proposed in JVET-AB0275 and JVET-AC0086 to the reference software (VTM12.0) and share them in git.

# BoG notes

Subjects on non-normative optimization for machine consumption of coded video content were discussed in the BoG (coordinated by C. Hollmann and S. Liu).

## Common test conditions

JVET-AC0073 AHG15: On common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content [S. Liu (Tencent), C. Hollmann (Ericsson)]

This document describes common test conditions (CTC), reference software and configurations, reporting and other information to be used for experimenting and evaluating technologies for encoders and receiving systems for machine consumption of coded video content.

### Test sequences

Discussion:

WG4 VCM AHG decided to remove class E (SFU-HW dataset) and Kimono sequence from CTC, and truncated (or split) long TVD sequences into 10sec clips.

Question was asked whether experiments can be conducted using original (un-shortened) TVD sequences. – yes, but performance might be different to truncated ones.

Related input contribution m62326.

Recommendation: align with WG4.

### RA/LD/AI configurations

Recommendation: align with WG4.

- Include RA and LD (LDB) as mandatory configurations and AI as optional.

- Do not use temporal subsampling for AI configuration.

### QP points and BD-rate calculation

Recommendation: align with WG4. Further discuss about the key BD-rate used for tool evaluation.

- QP points are adjusted to avoid saturation problem.

- BD-rate of low-4 and high-4 QPs are calculated in addition to all (6) QP BD-rate calculation.

It was commented that maybe adding more QPs would help.

It was then commented that 6 QPs are sufficient.

It was commented that multiple BD-rate measurements based on the same R-D data may not be beneficial, instead may bring confusion. Will bring this question to WG4 – the key question is which BD-rate to look at when comparing performance and making decisions.

## Cross-check procedure

Recommendation: align with WG4. Further discuss crosscheck criteria for neural network-based encoding algorithms.

I was previously discussed and decided to require identical bitstreams for crosscheck and allow small variance for mAP and MOTA (1% relative to the performance score).

It was commented that if a neural network-based encoder algorithm (e.g. for motion estimation) is used, then the bitstreams may not be identical. This issue will be further investigated and bring to WG4 for discussion.

Question was asked about crosscheck on training. We will start from crosschecking inference for now, while training information such as dataset and process should be described in the contribution. This aspect will be further discussed and bring to WG4 as well.

## Reference software

### Where to setup the repository

https://vcgit.hhi.fraunhofer.de/

jvet-ahg-mccvc (?) name to be decided in JVET.

### What to be included in the initial version reference software

JVET-AC0049 [AHG15] Draft technical report on optimizations for encoders and receiving systems for machine analysis of coded video content [C. Hollmann (Ericsson), S. Liu (Tencent), S. Wang (Alibaba)]

This contribution contains a draft for the technical report on optimizations for encoders and receiving systems for machine analysis of coded video content that JVET decided on working on during its 28th meeting.

Some items could be included in the initial reference software:

- Spatial and temporal QP configuration /adjustment on VTM (JVET-AB275)

- Pre-processing (RoI based analysis and processing) technology (JVET-AC0086)

Recommendation: Anchor remains VTM12.0. Above items will be implemented on top of VTM12.0 and shared in git for investigation.

## Other aspects

It was asked whether other VTM versions could be used for proposals. It is recommended to use VTM12.0 for both anchor and proposals for now. It is encouraged to investigate the benefit of using of newer VTM versions and bring input contributions to the next meeting.