| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0352-v1 |
| --- | --- |

| Title: | Comments on ITP-PQ-C2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposed action |  |  |
| Author(s) or Contact(s): | W. Husak | Email: | wjh@dolby.com |
| Source: | Dolby Labs |  |  |

_____________________________

# Abstract

The ITP color representation was introduced during the 28th JVET meeting in document JVET-AB00172.  This document updates the matrix coefficients to reflect the proper values for RGB→LMS conversions.  The conversions are different from the existing coefficients to reflect the proper amount of hue crosstalk to improve linearity.

- Introduction

This document presents an update to the formulas introduced during the 28th JVET meeting in document JVET-AB00172.  The below formulas account for crosstalk that is introduced to optimize the hue linearity discussed above.

- Proposal

The following is proposed for resolving the issues:

- Add appropriate equations in the semantics under matrix_coeffs.

- Add an additional code point in Table E.5.

The detailed proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts):

The following example is for Rec. ITU-T H.265 | ISO/IEC 23008-2.  Similar additions can be added to other VUI parameter sets.

matrix_coeffs semantics

An additional code point needs to be added.  For the purposes of this document, the assumption is the next code point is 15.  The process of conversion is a two step process where the underlying R, G, and B values are first converted to LMS and then coverted to ITP-PQ-C2.  The current conversion to LMS are described in Equations [4] through [E7].

The following describe the modifications for conversion from RGB to LMS.

If matrix_coeffs is equal to 15, the "linear-domain" real-valued signals EL, EM, and ES are determined as follows:

EL = ( 1 747 * ER + 2 169 * EG + 180 * EB ) ÷ 4 096	(E-68)

EM = ( 673 * ER + 3 029 * EG + 394 * EB ) ÷ 4 096	(E-69)

ES = ( 50 * ER + 207 * EG + 3 839 * EB ) ÷ 4 096	(E-70)

In this case, the signals E′L, E′M, and E′S are determined by application of the transfer characteristics function using equations E-7, E-8 and E-9:

The following describe the modifications for conversion from LMS to IPT matrix_coeffs is equal to 15.

E′Y = ( 1 638 * E′L + 1 638 * E′M +  820 * E′S ) ÷ 4 096	 (E-71)

E′PB = ( 18 248 * E′L − 19 870 * E′M + 1 622 * E′S ) ÷ 4 096	(E-72)

E′PR = ( 3 300 * E′L + 1 463 * E′M – 4 763 * E′S ) ÷ 4 096	(E-73)

NOTE 14 – In this case, for purposes of the ITP nomenclature used in Table E.5, E′Y, E′PB, and E′PR of Equations E-71, E-72, and E-70 may be referred to as I, T, and P, respectively.

# References

- SMPTE “Liaison statement from SMPTE to SC 29/WG 4 and 5 regarding Reserving VUI values to signal the SMPTE ST 2128 color space” m61406, Oct. 2022.

- ITU BT.2100-2 “Image parameter values for high dynamic range television for use in production and international programme exchange” Aug. 2018.

# Patent rights declaration(s)

Dolby may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).