| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0353-v1 |
| --- | --- |

| Title: | AHG9: On the repetition and activation of NNPFC SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry1 Ye-Kui Wang2 Miska Hannuksela3 Francesco Cricri3 Sachin Deshande4 | Email: | dr.hendry@lge.com1 yekui.wang@bytedance.com2 firstname.lastname@nokia.com3 sdeshpande@sharplabs.com4 |
| Source: | LG Electronics Inc.1, Bytedance Inc.2, Nokia3, and Sharp Labs of America, Inc.4 |  |  |

_____________________________

# Abstract

This contribution provides design for handling repetition, update, and activation of NNPFC SEI message in response to the asserted issues discussed in contributions JVET-AC0129 and JVET-AC0131 based on offline discussion during the meeting.

# Introduction

Contributions JVET-AC0129 and JVET-AC0131 assserted that current VSEI specification draft text is not clear regarding repetition of neural-network post-filter characteristic (NNPFC) SEI message which makes it difficult to distinguish between an NNPFC SEI message that repeats previous NNPFC SEI message and an NNPFC SEI message that provides updates relative to the NNPFC SEI message that contains the base filter.

Furthermore, it is also asserted since activation of filter carried in NNPFC is done by other SEI message (i.e., neural-network post-filter activation (NNPFA) SEI message), the current specification draft is not clear about which NNPFC SEI message is activated when some updates in involved.

The two contributions have been discussed once in breakout group session. The asserted problems were agreed to be valid and offline discussion was encouraged to find the solution for the problems. This contribution provides the design that address the problems and the text implementation is provided in the attachment of this contribution.

# Proposed design

Based on the offline discussion, the following design for handling the problems is proposed:

- Instead of overriding the current existing flag (i.e., nnpfc_formatting_and_purpose_flag) to specify whether an NNPFC SEI message contains a base filter or an update relative to the base filter, a new flag is used. This flag is called nnpfc_base_flag.

  - nnpfc_base_flag shall be equal to 1 in the first NNPFC SEI message with a particular value of nnpfc_id within a CLVS.

- When value of nnpfc_base_flag in an NNPFC SEI message with a particular valueof nnpfc_id is equal to 1 but the NNPFC SEI message is not the first SEI message with that particular value of nnpfc_id, the SEI message is a repetition of the NNPFC SEI message with that particular value of nnpfc_id that contains the base filter (i.e., the first one).

- Regarding the existing flag nnpfc_formatting_and_purpose_flag, the following applies:

  - Its value shall be equal to 1 in the first NNPFC SEI message with a particular value of nnpfc_id (i.e., no change compared to the current specification draft).

  - Its value may be equal to 1 in the NNPFC SEI message that updates the base filter; however, it is constrained that in such situation, the purpose and formatting shall not change compared to the NNPFC SEI message that contains the base filter. Complexity may change but it cannot be higher / greater than the complexity specified for the base filter.

- For activation, it is agreed to use option 1 proposed by JVET-AC0129 with the following clarification:

  - For a particular picture for which the NNPF is activated, the target NNPF is the NNPF specified by the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id, that is not a repetition of the NNPFC SEI that contains the base NNPF, that precedes the first VCL NAL unit of the current picture in decoding order.

# Patent rights declaration(s)

LG Electronics Inc., Bytedance Inc., Nokia, and Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).