| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0354 |
| --- | --- |

| Title: | Report on the methodology discussions of ECM tool off tests |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiang Li | Tel: Email: | xlxiangli@google.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document contains the report of offline discussions on the methodology of ECM tool off tests. The following three topics were discussed

- Tool grouping

- Suitable testing configurations and anchors

- Identify necessary control for TM based tools

# Recommendation

- Request JVET plenary to give guidance on the direction of tool grouping

- Schedule a conference call to further discuss tool grouping, tool control and test configurations

# Discussion details

## Granularity of testing (tool grouping)

Two grouping approaches were suggested as below.

### Grouping based on hardware implementation challenges

- Group1: Inter template matching

  - Inter template matching tools

It was commented that sign prediction should be put into this group.

- Group2:  Inter and intra coding tools that interleave the (merge/skip/AMVP/ subblock/ IBV/etc) list derivation with the intra prediction/reconstruction process (block by block)

  - LIC, CIIP

It was commented that sign prediction should be put into this group.

It was commented that CIIP may not have implementation issue as it was thoroughly studied during VVC development.

- Group3:  Intra coding tools that require an excessive amount of processing on the neighboring reconstructed samples

  - With search: template IBC, and intra TMP

  - Without search: TIMD, DIMD, TMRL, SGPM

  - Use neighboring samples to derive model parameters: CCCM

It was commented that TM-IBC and IntraTMP are with different complexities from others (DIMD, TIMD, CCCM and the like) as the first tools require a search process in the search area, while the rest needs just computational processing on the neighboring samples.

It was commented that TM IBC may search a big area while TMRL only search several lines, but still search. It may not be very fair to say search and non-search.

It was commented that TMRL is more close to TIMD.

It was commented that this activity is to understand the performance split, even if several tools will be "rounded" to one of the groups may not be a big problem. Each tool in a group can be further tested individually if needed

It was commented that template analysis tools could be considered, where the template is used not for matching but rather analysis. Examples of these include sign prediction, TIMD, DIMD, TMRL, and SGPM.

- Group4: Remaining tools

  - Intra (not in groups 1-3)

  - Inter (not in groups 1-3)

  - Transform and coefficient coding

  - In-loop filtering

  - Entropy coding

  - SCC

### Grouping based on template matching

- Inter template matching search

  - e.g. TM merge, TM AMVP, …

- Inter with template (reconstructed neighbors) processing

  - e.g. LIC

- Intra template matching search

  - e.g TM-IBC, IntraTMP

- Intra with template (reconstructed neighbors) processing

  - e.g. DIMD, TIMD

It was commented that non-template related tools can be tested when the above 4 categories are disabled.

### Reference list of new tools/improvements in ECM

To ease the tool grouping, a tool list is summarized as below based on the ECM test model JVET-AB2025

- Intra

  - multi-model LM, gradient PDPC, secondary MPM, reference sample interpolation and smoothing, DIMD, fusion of chroma intra, intra tmp, TIMD, intra prediction fusion, combination of CIIP with TIMD and TM merge, extended MRL (including TM based part), CCCM, SGPM

- Inter

  - LIC, non-adjacent merge, TM, multipass DMVR, adaptive DMVR, OBMC, history and non-adjacent affine, Sample based BDOF, interpolation, MHP, ARMC-TM, MV type based ARMC, TM based MMVD and affine MMVD, regression based affine, GPM improvement with TM, GPM improvement without TM, Bilateral matching based AMVP-merge, MVD sign prediction, enhanced bi-directional motion compensation, MC based picture boundary padding, block level reference picture list reordering, RPR, template matching based BCW index derivation, DMVR for affine merge

- Transform and coefficient coding

  - 8-state dependent quantization, larger transform, improvement of primary transform, improvement of secondary transform, sign prediction

- In-loop filter

  - ALF improvement, CCALF improvement, Bilateral Filter (luma and chroma), CCSAO

- Entropy coding

  - Extended precision, slice-type based window size, improved probability estimation for CABAC

- SCC

  - IBC improvement without TM, IBC improvement with TM, RR-IBC, IBC-MBVD

## Suitable testing configurations and anchors

- ECM anchor: most recent version is preferred

It was commented that if a new release is delayed, the previous release should be used. The same version should be used by all testers.

It was commented that the required resources for performing all simulations should be considered.

- VTM anchor: VTM11-ECM6 or the most recent version

- Configurations: AI, RA, LDB

It was commented that LDP may be better due to less runtime

It was suggested to test LDB but without class B

It was commented that high resolution like class B is important

It was commented that LDB is preferred

It was commented that if full results could be promised before next meeting, LDB is preferred

- Tool off option, cfg option looks a better choice

It was commented that macro on/off may be used

It was commented that compiling issue or software bug may be observed due to insufficient macro combination tests

## Identify necessary control for TM based tools

Two approaches were suggested as below.

- Option 1

  - Umbrella flags are SPS control flags

  - Make the tools usage of that category be dependent on the umbrella SPS flag

It was commented that the disadvantage of this approach is that if a tool is moved to another category later, the control dependency should be changed

- Option 2

  - SPS flags are introduced per tool

  - Make umbrella flag as an encoder setting, which sets the corresponding tools SPS flags.

It was commented that the disadvantage of this approach is that almost every tool should have an SPS flag, it is already happening though in many cases.