| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC1009 |
| --- | --- |

| Title: | Common Test Conditions for SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Information |  |  |
| Editor(s): | Karsten Suehring | Email: | karsten.suehring@hhi.fraunhofer.de |
| Source: | JVET |  |  |

_____________________________

# Abstract

This document defines common test conditions and software reference configurations that are recommended for use in coding experiments relating to the HEVC scalable extension (SHVC) based on the scalable HEVC model (SHM) reference software. These common test conditions are also recommended for general use in SHVC-related technical contributions to JVET meetings, as applicable. During the development of SHVC, similar common test conditions were established for use in formal experiments conducted for the testing the SHVC design and its proposed modifications.

# Introduction

Common test conditions are desirable to conduct experiments in a well-defined environment and ease the comparison of the outcome of experiments.

This document defines mandatory and optional test conditions for 2x, 1.5x, 1x, and SNR scalability ratios for HEVC and AVC base layers, and colour gamut scalability (CGS).

AVC base layer test configurations are mandatory only for the technical contributions specifically developed for the base layer coded with AVC, and all intra test is not required for the base layers coded with AVC.

Similarly, CGS test configurations are mandatory only for the contributions specifically developed for that type of scalability.

Test configurations for HEVC base layers

| Mandatory test conditions | Intra, spatial 2x Intra, spatial 1.5x Random access, spatial 2x Random access, spatial 1.5x Random access, SNR Low delay, B slices, spatial 2x Low delay, B slices, spatial 1.5x Low delay, B slices, SNR |
| --- | --- |
| Optional test conditions | Low delay, P slices only, spatial 2x Low delay, P slices only, spatial 1.5x Low delay, P slices only, SNR |

Test configurations for AVC base layers

| Mandatory test conditions | Random access, spatial 2x Random access, spatial 1.5x Random access, SNR Low delay, B slices, spatial 2x Low delay, B slices, spatial 1.5x Low delay, B slices, SNR |
| --- | --- |
| Optional test conditions | Low delay, P slices only, spatial 2x Low delay, P slices only, spatial 1.5x Low delay, P slices only, SNR |

Test configurations for colour gamut scalability

| Mandatory test conditions | Intra, spatial 2x Intra, spatial 1x Random access, spatial 2x Random access, spatial 1x |
| --- | --- |
| Optional test conditions | Low delay, B slices, spatial 2x Low delay, B slices, spatial 1x Low delay, P slices only, spatial 2x Low delay, P slices only, spatial 1x |

A subset of these conditions might be used for a particular experiment. For example, when testing an intra coding tool, only intra configurations might be used.

SHM-12.4 software (based on HM-16.10) is expected to be used for experiments. Software package can be downloaded at https://vcgit.hhi.fraunhofer.de/jvet/SHM/-/tags. HM-16.10 shall be used to generate the anchor of simulcast and single layer high quality.

The following sections define test sequences, quantization parameter values, encoder configuration files, and compile-time options to be used.

People bringing input contributions should provide a set of results as complete as possible that apply to the proposal. Results should be reported using the attached Excel sheets.

# Test sequences

Table 1 defines the set of test sequences, base (BL) and enhancement (EL) layers resolution and scalability ratios to be used for intra, random-access, and low-delay conditions. All frames (as defined by frame count in the table) shall be encoded for all sequences and test cases described below.

Test sequences are available on ftp://jvet@ftp.ient.rwth-aachen.de and ftp://jvet@ftp.hhi.fraunhofer.de in the “ctc/shvc” folder.

Test sequences are made available only to qualified JVET participants. Qualified participants may contact the JVET chair for login information. Specific license information is provided with each test sequence.

Table 1. Test sequences for HEVC and AVC coded base layers

| Class | Sequence name | Frame count | Frame rate, Hz | Intra period for RA | BL resolution | EL resolution | Intra | Random access | Low-delay P or B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Traffic | 150 | 30 | 32 | 1280x800 2560x1600 | 2560x1600 2560x1600 | Spatial 2x | Spatial 2x SNR | Spatial 2x SNR |
| A | PeopleOnStreet | 150 | 30 | 32 | 1280x800 2560x1600 | 2560x1600 2560x1600 | Spatial 2x | Spatial 2x SNR | Spatial 2x SNR |
| B | Kimono | 240 | 24 | 32 | 960x540 1280x720 1920x1080 | 1920x1080 1920x1080 1920x1080 | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR |
| B | ParkScene | 240 | 24 | 32 | 960x540 1280x720 1920x1080 | 1920x1080 1920x1080 1920x1080 | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR |
| B | Cactus | 500 | 50 | 48 | 960x540 1280x720 1920x1080 | 1920x1080 1920x1080 1920x1080 | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR |
| B | BasketballDrive | 500 | 50 | 48 | 960x540 1280x720 1920x1080 | 1920x1080 1920x1080 1920x1080 | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR |
| B | BQTerrace | 600 | 60 | 64 | 960x540 1280x720 1920x1080 | 1920x1080 1920x1080 1920x1080 | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR |

Table 2 defines the set of test sequences for colour gamut scalability tests.

Table 2. Test sequences for colour gamut scalability

| Class | Sequence name | Frame count | Frame rate, Hz | Intra period for RA | BL resolution | EL resolution | Intra | Random access | Low-delay P or B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A+ B | Birthday | 300 | 60 | 64 | 1920x1080 1920x1080 | 3840x2160 1920x1080 | Spatial 2x Spatial 1x | Spatial 2x Spatial 1x | Spatial 2x Spatial 1x |
| A+ B | BirthdayFlashPart1 | 300 | 60 | 64 | 1920x1080 1920x1080 | 3840x2160 1920x1080 | Spatial 2x Spatial 1x | Spatial 2x Spatial 1x | Spatial 2x Spatial 1x |
| A+ B | BirthdayFlashPart2 | 300 | 60 | 64 | 1920x1080 1920x1080 | 3840x2160 1920x1080 | Spatial 2x Spatial 1x | Spatial 2x Spatial 1x | Spatial 2x Spatial 1x |
| A+ B | Parakeets | 250 | 50 | 48 | 1920x1080 1920x1080 | 3840x2160 1920x1080 | Spatial 2x Spatial 1x | Spatial 2x Spatial 1x | Spatial 2x Spatial 1x |
| A+ B | TableCar | 300 | 60 | 64 | 1920x1080 1920x1080 | 3840x2160 1920x1080 | Spatial 2x Spatial 1x | Spatial 2x Spatial 1x | Spatial 2x Spatial 1x |

# Quantization parameter values

For each video sequence 8 quantization parameter sets are to be used depending upon the test configuration for both AVC and HEVC base layers, and CGS configuration.  These values define the QP values used for the I-frames of the base (BL) and enhancement (EL) layers in a sequence (configuration files further define QP values used for other frames):

| Scalability ratio | BL QP | EL delta QP |
| --- | --- | --- |
| Spatial 2x and 1.5x | 22, 26, 30, 34 | 0, 2 |
| SNR | 26, 30, 34, 38 | -6, -4 |
| Spatial 1x | 22, 26, 30, 34 | -4, 0 |

# Configuration files

This section defines encoder configuration files to be used for each test case. Only two layers are considered for the common testing conditions. Parameters to be changed for each test point are the following (where N=0 for the base layer and N=1 for the enhancement layer):

- InputFileN to reflect the location of the source video sequence for layer N on the test system.

- FrameRateN to reflect the frame rate of a given sequence for layer N as per Table 1.

- SourceWidthN to reflect the width of the source video sequence for layer N.

- SourceHeightN to reflect the height of the source video sequence for layer N.

- FramesToBeEncoded to reflect the frame count of a given sequence as per Table 1.

- IntraPeriodN to reflect the intra refresh period for layer N in the random access test cases. The intra refresh period is dependent on the frame rate of the source: a value 32 shall be used for sequences with a frame rate equal to 24fps and 30fps, 48 for 50fps, and 64 for 60fps as shown in Table 1 and Table 2.

- QPN to reflect the quantization parameter values for layer N defined in section 3.

These configuration files are provided in the cfg/ folder of the SHM software package (available at https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/).

Test sequences are configured differently for HEVC and AVC base layers and sequence specific configuration files are located in cfg/per-sequence-svc/ and cfg/per-sequence-svc-avcbase/ folders correspondingly. There is one common configuration file with layer specific parameters which is cfg/layers.cfg for HEVC base layer and CGS tests, and cfg/layers_avcbase.cfg for AVC base layer tests.

There are 4 configurations provided as follows:

- “All Intra – Main” (AI-Main): encoder_intra_scalable.cfg

- “Random access – Main” (RA-Main): encoder_randomaccess_scalable.cfg

- “Low-delay B – Main” (LB-Main): encoder_lowdelay_scalable.cfg

- “Low-delay P – Main” (LP-Main, optional): encoder_lowdelay_P_scalable.cfg

For the CGS tests, the input bitdepth is 10 bits and the following configuration files should be used:

- “All Intra – Main” (AI-Main): encoder_intra_scalable10.cfg

- “Random access – Main” (RA-Main): encoder_randomaccess_scalable10.cfg

- “Low-delay B – Main” (LB-Main, optional): encoder_lowdelay_scalable10.cfg

- “Low-delay P – Main” (LP-Main, optional): encoder_lowdelay_P_scalable10.cfg

Finally, the SEIDecodedPictureHash parameter must also be configured to use MD5 hashes, i.e. set to 1.

# Compile-time settings

Compile-time settings are defined mostly in the TypeDef.h file located in the source/Lib/TLibCommon folder of the software package. The default settings provided in the source code should be used.

# Results

Test results shall be reported using the Excel template attached to this document. Both HEVC and AVC base layers templates, and CGS anchors are provided.

In the template, the “Test” tab has to be populated with the test data to be compared against the anchor, containing reference data in the “Anchor” tab.

The format of the data is explained in Figure 1, and mandatory data to be filled correspond to the red columns, while green column is optional.

Figure 1. Data format description.