| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD_Notes_d1 |
| --- | --- |

| Title: | Meeting Report of the 30th Meeting of the Joint Video Experts Team (JVET), by teleconference, 21–28 April 2023 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the chair of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen | Tel: Email: | +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chair of JVET |  |  |

_____________________________

# Summary

The Joint Video Experts Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/‌SC 29 held its thirtieth meeting during 21–28 April 2023 in Antalya, Turkey, at Mirage Park Resort Hotel (Göynük Mahallesi, Ahu Ünal Aysal Caddesi No:29, 07994 Kemer/Antalya, Türkiye, Tel: +90 (0) 242 815 22 44, web https://www.mirageparkresort.com.tr/en). The meeting was held as a face-to-face meeting, but remote participation was provided on best-effort basis for experts who were unable to travel.

For ISO/IEC purposes, JVET is alternatively designated ISO/IEC JTC 1/‌SC 29/‌WG 5, and this was the eleventh meeting as WG 5. The JVET meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.14 of this document. It is further noted that work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued in JVET as a single joint team, and explorations towards possible future need of standardization in the area of video coding are also conducted by JVET, as negotiated by the parent bodies.

The JVET meeting began at approximately 0900 TRT (UTC+3) on Friday 21 April 2023. Meeting sessions were held on all days, including the weekend days of Saturday and Sunday 22 and 23 April 2023, until the meeting was closed at approximately XXXX hours TRT on Friday 28 April 2023. Approximately XXX people attended the JVET meeting (XXX in person and XXX remotely), and approximately XXX input documents (not counting crosschecks, reports, and summary documents), 14 AHG reports, 2 EE summary reports, X BoG reports, and X incoming liaison document were discussed. The meeting took place in coordination with a meeting of various SC29 Working Groups and Advisory Groups – where WG 5 is representing the Joint Video Coding Team(s) and their activities from the perspective of the SC 29 parent body, under whose auspices this JVET meeting was held. The subject matter of the JVET meeting activities consisted of work on further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, reference software and conformance testing packages. Further important goals were reviewing the results of the Exploration Experiment (EE) on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of other technical input on novel aspects of video coding technology, and to plan next steps for investigation of candidate technology towards further standard development.

As a primary goal, the JVET meeting reviewed the work that had been performed in the interim period since the twenty-ninth JVET meeting in producing the following documents:

- JVET-AC1001 Guidelines for HM-based software development

- JVET-AC1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

- JVET-AC1008 Additional colour type identifiers for AVC, HEVC and Video CICP (Draft 3), also issued as WG 5 preliminary WD

- JVET-AC1009 Common test conditions for SHVC

- JVET-AC1013 Common test conditions of 3DV experiments

- JVET-AC1015 Common test conditions for SCM-based screen content coding

- JVET-AC2002 Algorithm description for Versatile Video Coding and Test Model 19 (VTM 19)

- JVET-AC2003 Guidelines for VTM-based software development

- JVET-AC2005 New level and systems-related supplemental enhancement information for VVC (Draft 4)

- JVET-AC2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video

- JVET-AC2016 Common test conditions and evaluation procedures for neural network-based video coding technology

- JVET-AC2019 Description of algorithms and software in neural network-based video coding (NNVC) version 2

- JVET-AC2020 Film grain synthesis technology for video applications (Draft 4), also issued as WG 5 CDTR

- JVET-AC2021 Draft verification test plan for VVC multilayer coding

- JVET-AC2022 Draft plan for subjective quality testing of FGC SEI message

- JVET-AC2023 Exploration experiment on neural network-based video coding (EE1)

- JVET-AC2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

- JVET-AC2025 Algorithm description of Enhanced Compression Model 8 (ECM 8)

- JVET-AC2026 Conformance testing for VVC operation range extensions (Draft 4)

- JVET-AC2027 SEI processing order SEI message in VVC (draft 3), also issued as WG 5 preliminary WD

- JVET-AC2028 Additional conformance bitstreams for VVC multilayer configurations

- JVET-AC2030 Optimization of encoders and receiving systems for machine analysis of coded video content (draft 1), also issued as WG 5 preliminary WD

- JVET-AC2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content

- JVET-AC2032 Improvements under consideration for neural network post filter SEI messages

As main results, the JVET produced XX output documents from the current meeting (update):

- JVET-AC1001 Guidelines for HM-based software development

- JVET-AC1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

- JVET-AC1008 Additional colour type identifiers for AVC, HEVC and Video CICP (Draft 3), also issued as WG 5 preliminary WD

- JVET-AC1009 Common test conditions for SHVC

- JVET-AC1013 Common test conditions of 3DV experiments

- JVET-AC1015 Common test conditions for SCM-based screen content coding

- JVET-AC2002 Algorithm description for Versatile Video Coding and Test Model 19 (VTM 19)

- JVET-AC2003 Guidelines for VTM-based software development

- JVET-AC2005 New level and systems-related supplemental enhancement information for VVC (Draft 4)

- JVET-AC2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video

- JVET-AC2016 Common test conditions and evaluation procedures for neural network-based video coding technology

- JVET-AC2019 Description of algorithms and software in neural network-based video coding (NNVC) version 2

- JVET-AC2020 Film grain synthesis technology for video applications (Draft 4), also issued as WG 5 CDTR

- JVET-AC2021 Draft verification test plan for VVC multilayer coding

- JVET-AC2022 Draft plan for subjective quality testing of FGC SEI message

- JVET-AC2023 Exploration experiment on neural network-based video coding (EE1)

- JVET-AC2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

- JVET-AC2025 Algorithm description of Enhanced Compression Model 8 (ECM 8)

- JVET-AC2026 Conformance testing for VVC operation range extensions (Draft 4)

- JVET-AC2027 SEI processing order SEI message in VVC (Draft 3), also issued as WG 5 preliminary WD

- JVET-AC2028 Additional conformance bitstreams for VVC multilayer configurations

- JVET-AC2030 Optimization of encoders and receiving systems for machine analysis of coded video content (draft 1), also issued as WG 5 preliminary WD

- JVET-AC2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content

- JVET-AC2032 Improvements under consideration for neural network post filter SEI messages

- The following documents were produced as WG 5 documents only:

- MDS22362 Disposition of comments received on ISO/IEC DIS 23008-2:202x (5th edition)

- MDS22363 Text of ISO/IEC FDIS 23008-2:202x High efficiency video coding (5th edition)

- MDS22364 Draft disposition of comments received on ISO/IEC 23090-3:2022 DAM 1

- MDS22365 Preliminary text of ISO/IEC FDIS 23090-3:202x Versatile video coding (3rd edition)

- MDS22368 Disposition of comments received on ISO/IEC 23090-15 DAM 1

- MDS22369 Text of ISO/IEC FDIS 23090-15:202x Conformance testing for versatile video coding (2nd edition)

For the organization and planning of its future work, the JVET established XX “ad hoc groups” (AHGs) to progress the work on particular subject areas. At this meeting, X Exploration Experiments (EE) were defined. The next ten JVET meetings were planned for 11 – 19 July 2023 under ITU-T SG16 auspices in Geneva, CH; during 13 – 20 October 2023 under ISO/IEC JTC 1/‌SC 29 auspices in Hannover, DE; during 17 – 26 January 2024 under ISO/IEC JTC 1/‌SC 29 auspices, to be conducted as teleconference meeting; during April 2024 under ITU-T SG16 auspices, date and location t.b.d.; during 12 – 19 July 2024 under ISO/IEC JTC 1/‌SC 29 auspices in Sapporo, JP; during October 2024 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.; during January 2025 under ITU-T SG16 auspices, date and location t.b.d.; during April 2025 under ITU-T SG16 auspices, date and location t.b.d.; during 11 – 18 July 2025 under ISO/IEC JTC 1/‌SC 29 auspices in Daejeon, KR; and during October 2025 under ITU-T SG16 auspices, date and location t.b.d..

The document distribution site https://jvet-experts.org/ was used for distribution of all documents. It was noted that the previous sites http://phenix.int-evry.fr/jvet/, http://phenix.int-evry.fr/jct/, and http://phenix.int-evry.fr/jct3v/ are still accessible, but were converted to read-only.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Experts Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and ISO/IEC JTC 1/‌SC 29/‌WG 5. The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/‌SC 29.

The Joint Video Experts Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/‌SC 29 held its thirtieth meeting during 21–28 April 2023 in Antalya, Turkey, at Mirage Park Resort Hotel (Göynük Mahallesi, Ahu Ünal Aysal Caddesi No:29, 07994 Kemer/Antalya, Türkiye, Tel: +90 (0) 242 815 22 44, web https://www.mirageparkresort.com.tr/en). The meeting was held as a face-to-face meeting, but remote participation was provided on best-effort basis for experts who were unable to travel. For ISO/IEC purposes, JVET is alternatively designated ISO/IEC JTC 1/‌SC 29/‌WG 5, and this was the eleventh meeting as WG 5. The JVET meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of the Versatile Video Coding (VVC) and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI) standards, as well as associated conformance test sets, reference software, verification testing, and non-normative guidance information. Furthermore, starting from the twentieth meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on:

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

Furthermore, explorations towards possible future need of standardization in the area of video coding are also conducted by JVET. Currently, the following topics are under investigation:

- Exploration on Neural Network-based Video Coding

- Exploration on Enhanced Compression beyond VVC capability

This report contains three important annexes, as follows:

- Annex A contains a list of the documents of the JVET meeting

- Annex B contains a list of the meeting participants, as recorded by the teleconferencing tool used for the meeting

- Annex C contains the meeting recommendations of ISO/IEC JTC 1/‌SC 29/‌WG 5 for purposes of results reporting to ISO/IEC.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2023_04_AD_Antalya/.

## Primary goals

As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the twenty-eighth JVET meeting in producing the following documents:

- JVET-AC1001 Guidelines for HM-based software development

- JVET-AC1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

- JVET-AC1008 Additional colour type identifiers for AVC, HEVC and Video CICP (Draft 3), also issued as WG 5 preliminary WD

- JVET-AC1009 Common test conditions for SHVC

- JVET-AC1013 Common test conditions of 3DV experiments

- JVET-AC1015 Common test conditions for SCM-based screen content coding

- JVET-AC2002 Algorithm description for Versatile Video Coding and Test Model 19 (VTM 19)

- JVET-AC2003 Guidelines for VTM-based software development

- JVET-AC2005 New level and systems-related supplemental enhancement information for VVC (Draft 4)

- JVET-AC2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video

- JVET-AC2016 Common test conditions and evaluation procedures for neural network-based video coding technology

- JVET-AC2019 Description of algorithms and software in neural network-based video coding (NNVC) version 2

- JVET-AC2020 Film grain synthesis technology for video applications (Draft 4), also issued as WG 5 CDTR

- JVET-AC2021 Draft verification test plan for VVC multilayer coding

- JVET-AC2022 Draft plan for subjective quality testing of FGC SEI message

- JVET-AC2023 Exploration experiment on neural network-based video coding (EE1)

- JVET-AC2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

- JVET-AC2025 Algorithm description of Enhanced Compression Model 8 (ECM 8)

- JVET-AC2026 Conformance testing for VVC operation range extensions (Draft 4)

- JVET-AC2027 SEI processing order SEI message in VVC (Draft 3), also issued as WG 5 preliminary WD

- JVET-AC2028 Additional conformance bitstreams for VVC multilayer configurations

- JVET-AC2030 Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 1), also issued as WG 5 preliminary WD

- JVET-AC2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content

- JVET-AC2032 Improvements under consideration for neural network post filter SEI messages

Further important goals were reviewing the results of the EE on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of other technical input on novel aspects of video coding technology, and planning next steps for investigation of candidate technology towards further standard development.

## Documents and document handling considerations

### General

The document distribution site https://jvet-experts.org/ was used for distribution of all documents. It was noted that the previous site http://phenix.int-evry.fr/jvet/ was still accessible, but had been converted to read-only.

Document registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting follow the TRT timezone (local time in Antalya), except as otherwise noted.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect one of the various software packages but have no normative effect on text are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in one of the test model descriptions such as VTM, HM, etc. (an error, oversight, or messiness) or in the associated software package are marked by the string “Decision (BF):”.

- Decisions that are merely editorial without effect on the technical content of a draft standard are marked by the string "Decision (Ed.):". Such editorial decisions are merely suggestions to the editor, who has the discretion to determine the final action taken if their judgment differs.

- Some decisions are recorded with the word “agreed” rather than “Decision:”, especially for non-normative, editorial and planning matters.

This meeting report is based primarily on notes taken by the JVET chair. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report. Expressions such as “X.XX%” indicate that the desired results were not available at the time the information was recorded.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Friday, 14 April 2023. Any documents uploaded after 1159 hours Paris/Geneva time on Saturday 15 April 2023 were considered “officially late”, with a grace period of 12 hours (to accommodate those living in different time zones of the world). The deadline does not apply to AHG reports and other such reports which can only be produced after the availability of other input documents.

All contribution documents with registration numbers higher than JVET-AD0243 were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “late” range might include break-out activity reports that were generated during the meetings, or documents which were requested to be produced for the purpose of improving specification text, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions has been to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This has been agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered and/or uploaded late:

- JVET-AD0XXX (a proposal on …), uploaded 04-XX,

- … .

It may be observed that some of the above-listed contributions were submissions made in response to issues that arose in discussions during the meeting or from the study of other contributions, and thus could not have been submitted by the ordinary deadline.

The following other documents not proposing normative technical content, but with some need for consideration, were registered and/or uploaded late:

- JVET-AD0XXXX (a document discussing …), uploaded 04-XX,

- … .

Most cross-verification reports at this meeting were registered late, and/or uploaded late (except for JVET-AD0139). In the interest of brevity, the late ones are not specifically identified here. Initial upload times for each document are recorded in Annex A of this report.

The following contribution registrations were noted that were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-AD0046, JVET-AD0049, JVET-AD0098, JVET-AD0132, JVET-AD0245, JVET-AD0301, JVET-0319, … .

The following cross-verification report was still missing three weeks after the end of the meeting: JVET-AD0XXX. This was thus marked as withdrawn by the JVET chair.

“Placeholder” contribution documents that were basically empty of content, or lacking any results showing benefit for the proposed technology, and obviously uploaded with an intent to provide a more complete submission as a revision, had been agreed to be considered unacceptable and to be rejected in the document management system until a more complete version was available (which would then be counted as a late contribution if the update was after the document deadline). At the current meeting, this situation did apply with documents JVET-AD0212 and JVET-AD0218, which were categorized as late in the list above, based on the time of the first reasonable document upload.

Contributions that had significant problems with uploaded versions were not observed.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, CE and HLS topic summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there may have been some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload can also be a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This can especially be a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision, although participants tend to seldom notice what is recorded there.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). Any such issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors may have also noticed in other initial document uploads (wrong document numbers or meeting dates or meeting locations in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

All output documents of the previous meeting, particularly the meeting report JVET-AC1000, the Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP JVET-AC1004, the Additional colour type identifiers for AVC, HEVC and Video CICP (Draft 3) JVET-AC1008, the Common test conditions for SHVC JVET-AC1009, the Common test conditions of 3DV experiments JVET-AC1013, the Common test conditions for SCM-based screen content coding JVET-AC1015, the Algorithm description for Versatile Video Coding and Test Model 19 (VTM 19) JVET-AC2002, the Guidelines for VTM-based software development JVET-AC2003, the New level and systems-related supplemental enhancement information for VVC (Draft 4) JVET-AC2005, the VTM and HM common test conditions and evaluation procedures for HDR/WCG video JVET-AC2011, the Common test conditions and evaluation procedures for neural network-based video coding technology JVET-AC2016, the Description of algorithms and software in neural network-based video coding (NNVC) version 2 JVET-AC2019, the Film grain synthesis technology for video applications (Draft 4) JVET-AC2020, the Draft verification test plan for VVC multilayer coding JVET-AC2021, the Draft plan for subjective quality testing of FGC SEI message JVET-AC2022, the Description of the EE on Neural Network-based Video Coding JVET-AC2023, the Description of the EE on Enhanced Compression beyond VVC capability JVET-AC2024, the Algorithm description of Enhanced Compression Model 8 (ECM 8) JVET-AC2025, the Conformance testing for VVC operation range extensions (Draft 4) JVET-AC2026, the SEI processing order SEI message in VVC (Draft 2) JVET-AC2027, the Additional conformance bitstreams for VVC multilayer configurations JVET-AC2028, the Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 1) JVET-AC2030, the Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content JVET-AC2031, and the Improvements under consideration for neural network post filter SEI messages JVET-AC2032, had been completed and were approved. The software implementations of HM (version 17.0), VTM (version 20.0), ECM (version 8.0), and NNVC (version 4.0) were also approved.

Only minor editorial issues were found in the meeting report JVET-AC1000; no need to produce an update was identified (see section 2.12 for details).

The available output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 5 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

It was further announced that it is necessary to register for the meeting through the ISO Meetings website for ISO/IEC experts or through the Q6/16 rapporteur for ITU-T experts. The password for meeting access had been sent to registered participants via these channels. Links to the Zoom sessions (without the necessary password) were available in the posted meeting logistics information and the calendar of meeting sessions in the JVET web site.

The following rules were established for those participating remotely via Zoom teleconference meeting:

- Use the “hand-raising” function to enter yourself in the queue to speak (unless otherwise instructed by the session chair). If you are dialed in by phone, request your queue position verbally. The online queue will be interleaved with the room queue, though it may not always be guaranteed that the sequence perfectly follows the sequence by which hand raising occurred.

- Stay muted unless you have something to say. People are muted by default when they join and need to unmute themselves to speak. The chair may mute anyone who is disrupting the proceedings (e.g. by forgetting they have a live microphone while chatting with their family or by causing bad noise or echo).

- Identify who you are and your affiliation when you begin speaking. The same applies for speakers in the room to let online participants know who is speaking.

- Use your full name and company/organization and country affiliation in your joining information, since the participation list of Zoom would also be used to compile the online part of attendance records.

- Turn on the chat window and watch for chair communication and side commentary there as well as by audio.

- Generally do not use video for the teleconferencing calls in order to avoid overloading people’s internet connections; enable only voice and screen sharing.

- Extensive use of screen sharing is encouraged, to enable participants to view the presented material and the meeting notes. At times, multiple sources of screen sharing may be enabled, so it may be necessary for participants to understand that this is happening and to understand how to select which one they want to watch.

## Agenda

The agenda for the meeting, for the further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, software and conformance packages, was as follows:

Opening remarks and review of meeting logistics and communication practices

Roll call of participants

Adoption of the agenda

Code of conduct policy reminder

IPR policy reminder and declarations

Contribution document allocation

Review of results of the previous meeting

Review of target dates

Reports of ad hoc group (AHG) activities

Report of exploration experiments on neural-network-based video coding

Report of exploration experiments on enhanced compression beyond VVC capability

Consideration of contributions on high-level syntax

Consideration of contributions and communications on project guidance

Consideration of video coding technology contributions

Consideration of contributions on conformance and reference software development

Consideration of contributions on coding-independent code points for video signal type identification

Consideration of contributions on film grain synthesis technology

Consideration of contributions on optimization of encoders and receiving systems for machine analysis of coded video content

Consideration of contributions on errata relating to standards in the domain of JVET

Consideration of contributions on technical reports relating to standards and exploration study activities in the domain of JVET

Consideration of contributions providing non-normative guidance relating to standards and exploration study activities in the domain of JVET

Consideration of information contributions

Consideration of future work items

Coordination of visual quality testing

Liaisons, coordination activities with other organizations

Review of project editor and liaison assignments

Approval of output documents and associated editing periods

Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments (if any), establishment of AHGs, future meeting planning, other planning issues

Other business as appropriate for consideration

Closing

- The agenda was approved as suggested.

- The times of meeting sessions will follow the needs of the face-to-face meeting, with highest priority given to the aim of achieving the goals of the meeting. Typical meeting hours will be 0900-1900 TRT with coffee breaks and lunch breaks as appropriate, however some early morning or late-night sessions may be necessary. Sessions will be announced in the JVET calendar in advance as far as possible, but it might happen that some activities (such as breakout sessions) will be held at short notice.

## ISO and IEC Code of Conduct reminders

Participants were reminded of the ISO and IEC Codes of Conduct, found at

https://www.iso.org/publication/PUB100397.html.

https://www.iecapc.jp/F/IEC_Code_of_Conduct.pdf

These include points relating to:

- Respecting others

- Behaving ethically

- Escalating and resolving disputes

- Working for the net benefit of the international community

- Upholding consensus and governance

- Agreeing to a clear purpose and scope

- Participating actively and managing effective representation

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that the VTM and ECM software implementation packages use the same software copyright license header as the HEVC reference software, where the latter had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time. This license header language is based on the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N 10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is also used in the process of designing the VTM and ECM software, and for evaluating proposals for technology to be potentially included in these designs. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

These considerations apply to the 360Lib video conversion software and HDRTools as well. The SADL and NNVC packages for neural network-based video coding use the same licensing terms.

Software packages that had been developed in prior work of the JVT, the JCT-VC and the JCT-3V have similar considerations and are maintained according to the past practice in that work.

## Communication practices

The documents for the meeting can be found at https://jvet-experts.org/. It was noted that the previous site http://phenix.int-evry.fr/jvet/ is still accessible, but was converted to read-only. It was reminded to send a notice to the chairs in cases of changes to document titles, authors, etc.

JVET email lists are managed through the site https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers on the JVET email list was 1199 (as of 19 April 2023). All discussions (including those on AVC, HEVC, VVC, CICP, etc.) shall be conducted on the JVET reflector rather than any of the old reflectors (including JVT, JCT-VC, and JCT-3V) which are retained for archiving purposes.

For distribution of test sequences, a password-protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI. Accredited members of JVET may contact the responsible JVET coordinators to obtain the password information (but the site is not open for use by others).

It is further emphasized that the document JVET-AB1012 gives valuable hints about communication practices as well as other IT resources used in JVET, such as software, conformance, and test materials.

## Terminology

- ACT: Adaptive colour transform

- AFF: Adaptive frame-field

- AI: All-intra

- AIF: Adaptive interpolation filtering

- ALF: Adaptive loop filter

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component)

- AMVP: Adaptive motion vector prediction

- AMT or MTS: Adaptive multi-core transform, or multiple transform selection

- AMVR: (Locally) adaptive motion vector resolution

- APS: Adaptation parameter set

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR)

- ARMC: Adaptive re-ordering of merge candidates

- ARSS: Adaptive reference sample smoothing

- ATM: AVC-based multiview and 3D test model

- ATMVP or “subblock-based temporal merging candidates”: Alternative temporal motion vector prediction

- AU: Access unit

- AUD: Access unit delimiter

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10

- BA: Block adaptive

- BC: See CPR or IBC

- BCW: Biprediction with CU based weighting

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001)

- BDOF: Bi-directional optical flow (formerly known as BIO)

- BDPCM: Block-wise DPCM

- BL: Base layer

- BMS: Benchmark set (no longer used), a former preliminary compilation of coding tools on top of VTM, which provide somewhat better compression performance, but are not deemed mature for standardzation

- BoG: Break-out group

- BR: Bit rate

- BT: Binary tree

- BV: Block vector (used for intra BC prediction)

- CABAC: Context-adaptive binary arithmetic coding

- CBF: Coded block flag(s)

- CC: May refer to context-coded, common (test) conditions, or cross-component

- CCALF: Cross-component ALF

- CCLM: Cross-component linear model

- CCCM: Cross-component convolutional model

- CCP: Cross-component prediction

- CCSAO: Cross-component SAO

- CE: Core Experiment – a coordinated experiment conducted toward assessment of coding technology

- CG: Coefficient group

- CGS: Colour gamut scalability (historically, coarse-grained scalability)

- CIIP: Combined inter/intra prediction

- CIPF: CABAC initialization from the previous frame

- CL-RAS: Cross-layer random-access skip

- CPB: Coded picture buffer

- CPMV: Control-point motion vector

- CPMVP: Control-point motion vector prediction (used in affine motion model)

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- CST: Chroma separate tree

- CTC: Common test conditions

- CVS: Coded video sequence

- DCI: Decoder capability information

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics)

- DCTIF: DCT-derived interpolation filter

- DF: Deblocking filter

- DIMD: Decoder intra mode derivation

- DMVR: Decoder motion vector refinement

- DoCR: Disposition of comments report

- DPB: Decoded picture buffer

- DPCM: Differential pulse-code modulation

- DPS: Decoding parameter sets

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR)

- DT: Decoding time

- DQ: Dependent quantization

- ECS: Entropy coding synchronization (typically synonymous with WPP)

- EMT: Explicit multiple-core transform

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element)

- ECM: Enhanced compression model – a software codebase for future video coding exploration

- ECV: Extended Colour Volume (up to WCG)

- EL: Enhancement layer

- EOS: End of (coded video) sequence

- ET: Encoding time

- FRUC: Frame rate up conversion (pattern matched motion vector derivation)

- GCI: General constraints information

- GDR: Gradual decoding refresh

- GLM: Gradient linear model

- GOP: Group of pictures (somewhat ambiguous)

- GPM: Geometry partitioning mode

- GRA: Gradual random access

- HBD: High bit depth

- HDR: High dynamic range

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2

- HLS: High-level syntax

- HM: HEVC Test Model – a video coding design containing selected coding tools that conforms to the HEVC standard design (possibly with under-development extensions) – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM)

- HMVP: History based motion vector prediction

- HRD: Hypothetical reference decoder

- HTM: HEVC-based multiview and 3D test model (developed by JCT-3V)

- HyGT: Hyper-cube Givens transform (a type of NSST)

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample)

- IBF: Intra boundary filtering

- ILP: Inter-layer prediction (in scalable coding)

- ILRP: Inter-layer reference picture

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC)

- IRAP: Intra random access picture

- ISP: Intra subblock partitioning

- JCCR: Joint coding of chroma residuals

- JCT-3V: Joint collaborative team on 3D video (for AVC and HEVC)

- JCT-VC: Joint collaborative team on video coding (for HEVC)

- JEM: Joint exploration model – a software codebase previously used for video coding exploration

- JM: Joint model – the primary software codebase that has been developed for the AVC standard

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions

- JVET: Joint video experts team (initially for VVC, later expanded)

- JVT: Joint video team (for AVC)

- KLT: Karhunen-Loève transform

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP

- LFNST: Low-frequency non-separable transform

- LIC: Local illumination compensation

- LM: Linear model

- LMCS: Luma mapping with chroma scaling (formerly sometimes called “in-loop reshaping”)

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames

- LUT: Look-up table

- LTRP: Long-term reference picture

- MANE: Media-aware network element

- MC: Motion compensation

- MCP: Motion compensated prediction

- MCTF: Motion compensated temporal pre-filtering

- MDNSST: Mode dependent non-separable secondary transform

- MIP: Matrix-based intra prediction

- MMLM: Multi-model (cross component) linear mode

- MMVD: Merge with MVD

- MPEG: Moving picture experts group (an alliance of working groups and advisory groups in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET)

- MPM: Most probable mode (in intra prediction)

- MRL: Multiple reference line intra prediction

- MV: Motion vector

- MVD: Motion vector difference

- NAL: Network abstraction layer

- NNVC: Neural network-based video coding (experimental software package)

- NSQT: Non-square quadtree

- NSST: Non-separable secondary transform

- NUH: NAL unit header

- NUT: NAL unit type (as in AVC and HEVC)

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F)

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value

- OLS: Output layer set.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- operation point: A temporal subset of an OLS.

- PDPC: Position-dependent (intra) prediction combination.

- PERP: Padded equirectangular projection (a 360° projection format).

- PH: Picture header.

- PHEC: Padded hybrid equiangular cubemap (a 360° projection format).

- PMMVD: Pattern-matched motion vector derivation.

- POC: Picture order count.

- PoR: Plan of record.

- PROF: Prediction refinement with optical flow

- PPS: Picture parameter set (as in AVC and HEVC).

- PTL: Profile/tier/level combination.

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading (type of picture).

- RASL: Random-access skipped leading (type of picture).

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- RDPCM: Residual DPCM

- ROT: Rotation operation for low-frequency transform coefficients.

- RPL: Reference picture list.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SADL: Small adhoc deep learning library

- SAO: Sample-adaptive offset.

- SBT: Subblock transform.

- SbTMVP: Subblock based temporal motion vector prediction.

- SCIPU: Smallest chroma intra prediction unit.

- SD: Slice data; alternatively, standard-definition.

- SDH: Sign data hiding.

- SDT: Signal-dependent transform.

- SE: Syntax element.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIF: Switchable (motion) interpolation filter.

- SIMD: Single instruction, multiple data.

- SMVD: Symmetric MVD.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatial-temporal motion vector prediction.

- STRP: Short-term reference picture.

- STSA: Step-wise temporal sublayer access.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- TIMD: Template-based intra mode derivation

- TM: Template matching.

- TMVP: Temporal motion vector prediction.

- TS: Transform skip.

- TSRC: Transform skip residual coding.

- TT: Ternary tree.

- UCBDS: Unrestricted center-biased diamond search.

- UGC: User-generated content.

- UWP: Unequal weight prediction.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VCM: Video coding for machines.

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- VQA: Visual quality assessment.

- VT: Verification testing.

- VTM: VVC Test Model.

- VUI: Video usability information.

- VVC: Versatile Video Coding, the standardization project developed by JVET.

- WAIP: Wide-angle intra prediction

- WCG: Wide colour gamut.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in VVC:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in a P or B slice or in an I slice that uses a single tree, and one CTB per luma CTU and two CTBs per chroma CTU in an I slice that uses separate trees.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in a P or B slice or in an I slice that uses a single tree, containing only luma or only chroma in an I slice that uses separate trees), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size as a CU in the VVC context.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size as a CU in the VVC context.

## Standards, TRs, supplements and technical papers approval and publication status

- AVC

  - ITU-T H.264 V14 Consented at 22nd meeting on 2021-04-30 (with annotated regions, shutter interval, and miscellaneous corrections), approved 2021-08-22, published 2021-10-13

  - ISO/IEC 14496-10:2020 (Ed. 9) FDIS ballot closed 2020-11-27, published 2020-12-15

  - ISO/IEC 14496-10:2022 (Ed. 10), had been forwarded from DIS directly for publication 2022-01-21 (with annotated regions, shutter interval, and miscellaneous corrections) with an editing period, submitted to ITTF in 2022-05 after consultation with ISO staff on format of graphics files, upgraded to “DIS approved for registration” in ISO Project system 2022-07-04, published 2022-11-07

  - Preliminary draft text for YCgCo-Re and YCgCo-Ro issued at 26th meeting, second draft including SMPTE ST 2128 issued at 28th meeting (not yet formally requested as a project)

  - Conformance testing

    - ITU-T H.264.1 V6 Approved 2016-02-13, published 2016-06-17

    - Various amendments of ISO/IEC 14496-4:2004, including:

      - ISO/IEC 14496-4:2004/AMD 6:2005 Advanced Video Coding conformance

      - ISO/IEC 14496-4:2004/AMD 9:2006 AVC fidelity range extensions conformance

      - ISO/IEC 14496-4:2004/AMD 30:2009 Conformance testing for new profiles for professional applications

      - ISO/IEC 14496-4:2004/AMD 31:2009 Conformance testing for SVC profiles

      - ISO/IEC 14496-4:2004/AMD 38:2010 Conformance testing for Multiview Video Coding

      - ISO/IEC 14496-4:2004/AMD 41:2014 Conformance testing of MVC plus depth extension of AVC

      - ISO/IEC 14496-4:2004/AMD 42:2014 Conformance testing of Multi-Resolution Frame Compatible Stereo Coding extension of AVC

      - ISO/IEC 14496-4:2004/AMD 43:2015 3D-AVC conformance testing

      - ISO/IEC 14496-4:2004/AMD 45:2016 Conformance Testing for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps Extension of AVC

  - Reference software

    - ITU-T H.264.2 V7 Approved 2016-02-13, published 2016-05-30

    - Various amendments of ISO/IEC 14496-5:2001, including:

      - ISO/IEC 14496-5:2001/AMD 6:2005 Advanced Video Coding (AVC) and High Efficiency Advanced Audio Coding (HE AAC) reference software

      - ISO/IEC 14496-5:2001/AMD 8:2006 AVC fidelity range extensions reference software

      - ISO/IEC 14496-5:2001/AMD 15:2010 Reference software for Multiview Video Coding

      - ISO/IEC 14496-5:2001/AMD 18:2008 Reference software for new profiles for professional applications

      - ISO/IEC 14496-5:2001/AMD 19:2009 Reference software for Scalable Video Coding

      - ISO/IEC 14496-5:2001/AMD 33:2015 Reference software for MVC plus depth extension of AVC

      - ISO/IEC 14496-5:2001/AMD 34:2014 Reference software of the multi-resolution frame compatible stereo coding of AVC

      - ISO/IEC 14496-5:2001/AMD 35:2015 3D-AVC Reference software

      - ISO/IEC 14496-5:2001/AMD 39:2016 Reference software for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps of AVC

      - ISO/IEC 14496-5:2001/AMD 42:2017 Reference software for the alternative depth information SEI message extension of AVC

- HEVC

  - ITU-T H.265 V7 approved 2019-11-29, published 2020-01-10

  - ISO/IEC 23008-2:2020 (Ed. 4) FDIS ballot closed 2020-07-16, published 2020-08-27

  - ITU-T H.265 V8 Consented at the 22nd meeting (shutter interval information SEI message and miscellaneous corrections), published 2020-10-13

  - ISO/IEC 23008-2:2020/AMD 1:2021 (shutter interval information SEI message) published 2021-07-12

  - ISO/IEC 23008-2:202x (Ed. 5) began as CDAM 2 High-range levels output of 25th meeting of January 2022, CDAM ballot closed 2022-04-15, conversion to 5th edition with miscellaneous corrections planned at 26th meeting of April 2022, text submitted for DIS ballot 2022-07-10, DIS ballot closed 2023-01-10

  - Preliminary draft text for YCgCo-Re and YCgCo-Ro issued at 26th meeting, second draft including SMPTE ST 2128 issued at 28th meeting (not yet formally requested as a project)

  - Conformance testing

    - ITU-T H.265.1 V3 approved 2018-10-14, published 2019-01-15

    - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06

    - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15

  - Reference software

    - ITU-T H.265.2 V4 approved 2016-12-22, published 2017-04-10

    - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for high efficiency video coding, published 2017-03-01

    - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for screen content coding extensions, published 2017-11-09

- VVC

  - ITU-T H.266 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23090-3:2021 (Ed. 1) published 2021-02-16

  - ITU-T H.266 V2 with operation range extensions, Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-06-06, published 2022-07-12

  - ISO/IEC 23090-3:2022 (Ed. 2) with operation range extensions, approval at WG level to proceed to FDIS 2022-01-21, published 2022-09-25

  - ISO/IEC 23090-3:202x (Ed. 2) / Amd.1 New level and systems-related supplemental enhancement information, CDAM 1 issued from 26th meeting, ballot closed 2022-07-14, DAM 1 issued from 27th meeting, ballot closed 2023-01-03 (ready for action at this meeting)

  - Conformance testing

    - ITU-T H.266.1 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-15:2022 V1 approval at WG level to proceed to FDIS 2022-10-15, upgraded to “DIS approved for registration” in ISO Projects system 2021-10-24, upgraded to “FDIS registered for formal approval” 2022-07-11, FDIS ballot closed 2022-11-04, published 2022-11-24

    - ISO/IEC 23090-15:202x Amd.1 Operation range extensions – DAM 1 issued from 25th meeting 2022-01-21, upgraded to “CD approved for registration as DIS” status in ISO Projects system 2022-05-31, upgraded to “DIS registered” 2022-06-22, DAM ballot closed 2022-11-15 (ready for action at this meeting)

  - Reference software

    - ITU-T H.266.2 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-16:2022 V1 approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-04-21, upgraded to “FDIS registered for formal approval” 2022-04-22, FDIS ballot initiated 2022-07-24, FDIS ballot closed 2022-09-19, published 2022-10-23

- VSEI

  - ITU-T H.274 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23002-7:2021 (Ed. 1) published 2021-01-28

  - ITU-T H.274 V2 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-05-22 (after 1 Last Call comment and Additional Review), pre-published 2022-06-17, published 2022-07-25

  - ISO/IEC 23002-7:2022 (Ed. 2) approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-05-05 and “FDIS registered for formal approval” 2022-05-08, FDIS ballot closed 2022-09-27, published 2022-10-30

  - ISO/IEC 23002-7:202x (2nd Ed.) Amd.1 Request for new edition and CD for additional SEI messages issued at 27th meeting, ballot closed 2022-10-10, DAM registered 2022-11-13, DAM ballot closed 2022-04-06: comments in m62571 from Finland, Japan, US, ISO CS (minor)

- CICP

  - ISO/IEC 23091-2:2021 (Ed. 2) had been forwarded from DIS directly for publication in 2021-04 and published 2021-10-18

  - ITU-T H.273 V2 (with 4:2:0 sampling alignment and corrections for range of values for sample aspect ratio, ICTCP equations for HLG, and transfer characteristics function for sYCC of IEC 61966-2-1) Consented on 2021-04-30, Last Call closed during the 23rd meeting with approval on 2021-07-14, published 2021-09-24

  - ISO/IEC 23091-2:202x (Ed. 3) Request for new edition and CD for new edition (including YCgCo-Re and YCoCg-Ro) issued at 27th meeting, ballot closed 2022-10-10, DIS registered 2022-11-13, pending DIS ballot (no action at this meeting)

  - Preliminary draft text for including SMPTE ST 2128 issued at 28th meeting (not yet formally requested as a project)

- Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics

  - H.Sup15 V1, approved 2017-01-27, published 2017-04-12

  - ISO/IEC TR 23008-14:2018 published 2018-08

- Signalling, backward compatibility and display adaptation for HDR/WCG video coding

  - H.Sup18 V1, approved 2017-10-27, published 2018-01-18

  - ISO/IEC TR 23008-15:2018 published 2018-08

- Usage of video signal type code points

  - H.Sup19 V3 approved 2021-04-30, published 2021-06-04

  - ISO/IEC TR 23091-4 (Ed. 3) published 2021-05-23

- Working practices using objective metrics for evaluation of video coding efficiency experiments

  - HSTP-VID-WPOM V1: approved 2020-07-03, published 2020-11

  - ISO/IEC TR 23002-8 (Ed. 1) published 2021-05-20

- Film grain synthesis technologies for video applications

  - ISO/IEC TR 23002-9 Request for subdivision and WD 1 issued at 25th meeting 2022-01-21, WD 2 issued at 27th meeting, WD 3 issued at 28th meeting

- The following freely available standards are published here in ISO/IEC:
https://standards.iso.org/ittf/PubliclyAvailableStandards/index.html

  - Various amendments of ISO/IEC 14496-4:2004

  - Various amendments of ISO/IEC 14496-5:2001

  - ISO/IEC 23008-2:2020 (Ed. 4) HEVC

- The following standards that have been intended by JVET to be publicly available were not available at https://standards.iso.org/ittf/PubliclyAvailableStandards/index.html as of 2023-01-08. (Please see below for record of previously issued requests.)

  - ISO/IEC 23091-2:2021 (Ed. 2) Video CICP (was requested in April 2021, and the 2019 previous edition was also not available there)

  - ISO/IEC 23008-2:2020 (Ed. 4) Amd.1:2021: Shutter interval information SEI message, published 2021-07-12 (has not been requested)

  - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for high efficiency video coding, published 2017-03-01

  - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for screen content coding extensions, published 2017-11-09

  - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08, published 2018-08-06

  - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15

  - ISO/IEC 14496-10:2022 (Ed. 10) – AVC – final text issued and public availability requested at the 25th meeting (January 2022)

  - ISO/IEC 23002-7:2022 (Ed. 2) – VSEI – FDIS issued and public availability requested at the 25th meeting (January 2022)

  - ISO/IEC 23090-3:2022 (Ed. 2) – VVC – FDIS issued and public availability requested at the 25th meeting (January 2022)

  - ISO/IEC 23090-15:2022 (Ed. 1) – VVC conformance – FDIS issued and public availability requested at the 24th meeting (October 2021)

  - ISO/IEC 23090-16:2022 (Ed. 1) – VVC reference software – FDIS issued and public availability requested at the 25th meeting (January 2022)

- It appears necessary to check if all older software and conformance packages are publicly available – it might be that it was never requested, e.g. for those that were produced by JCT-3V. This topic was left TBD until the next meeting – perhaps it would be best to compile a list of all relevant software and conformance parts of AVC, HEVC, MPEG-2 aka H.262, CICP, and request these in bulk.

## Draft standards progression status

- AVC and HEVC colour type indicators for YCgCo-Re, YCgCo-Ro, and SMPTE ST 2128 are drafted and pending formal action

- HEVC new levels (from JVET-Z1005) – ISO/IEC 23008-2 DIS of new edition of HEVC was issued from the April 2022 26th meeting, incorporating Amd.1 and corrigenda items (ballot closed 2023-01-10, ballot comments in the Summary of Voting document m61834); note that Amd.1 = shutter interval SEI is already included in latest ITU-T edition of H.265. It is noted that there are potential additional items (corrigenda+tickets, YCgCo-Re and YCgCo-Ro draft, SMPTE ST 2128, multiview profiles draft) where only corrigenda items were included in the FDIS text based on ballot comments, ballot has not been started yet. ITU-T consent for a new edition is planned for July. It was noted that the referencing of VSEI is also somewhat different in the ITU-T and ISO/IEC versions of HEVC and AVC, which might be aligned at the next convenient time (basically editorial – e.g., the ITU version of AVC specifies the annotated regions SEI message without referencing VSEI, whereas the ISO/IEC version references VSEI for the syntax and semantics of that SEI message). Proceeding to FDIS should be considered at the current meeting.

- VVC new level and systems-related supplemental enhancement information (from JVET-AA2005) – VVC DAM was issued from 27th meeting, ballot closed 2023-01-03, ballot comments in the Summary of Voting document m61833. This was converted into a preliminary FDIS of VVC 3rd edition (WG 5 N 183) at the 29th meeting, anticipating that some alignment will be necessary with the ongoing VSEI amendment. ITU-T consent is planned for July 2023 when a new edition of VSEI is also planned to be consented, to keep VVC and VSEI aligned.

- VVC Conformance testing for operation range extensions – (from JVET-Y2026) – the DAM ballot closed 2022-11-15 (ballot comments in the Summary of Voting document m61832), and this was issued as FDIS at the 29th meeting, but ballot has not been started yet. ITU-T consent is planned for July 2023.

- VSEI additional SEI messages (from JVET-AB2006) – VSEI DAM (JVET draft 3) was issued from the 28th meeting and a DAM ballot was issued, Summary of Voting document is available as m62571. It was planned to be converted into FDIS of new edition in April (if possible) or July, ITU-T consent in July 2023. There are numerous input contributions in particular on NNPF.

- Film grain synthesis technology for video applications – JVET draft 4 was issued at the 29th meeting (JVET-AC2020), and the ISO/IEC 23002-9 CDTR was issued (a request to start work on the TR had been made at the 25th meeting), but ballot has not been started yet. The publication limit date is 2023-08-09, so an extension request may be needed, but perhaps not at the current meeting.

- Video CICP new edition with for YCgCo-Re and YCgCo-Ro (from JVET-Z1003), an ISO/IEC 23091-2 DIS was issued from the 28th meeting and the Summary of Voting document is available as m62572. ITU-T consent is planned for July 2023.

- Video CICP colour type indicator for SMPTE ST 2128 is drafted and pending formal action. It was reported that it is expected to become finalized in the SMPTE meeting in March. It was further reported that some clarification on conversion equations is necessary.

- A request for free availability in ISO/IEC has to be made for each edition, amendment and corrigendum, and the request needs to be approved in the WG 5 Recommendations. A request form also needs to be filled out (but the form does not need to be issued as a WG 5 document). A freely available URL for the ITU publication should be provided for the following parts:

  - For the ongoing work items, when they become finalized

  - ISO/IEC 23008-2:2020/Amd.1:2021 – HEVC FDAM issued 20th meeting (October 2020), public availability not yet requested but may not be necessary as it becomes included in next edition

## Opening remarks

Remarks during the opening session of the meeting Friday 21 April at 0900 TRT were as follows.

- Timing and organization of the meeting and online access, calendar posting of session plans

  - The initial number of documents was was again higher than for the previous meeting (160->200) – parallel sessions appear to be necessary

  - Scheduling of NNVC discussions – JPEG meeting and JPEG-AI sessions in parallel (virtual, typically in afternoon time TRT, not over the weekend)

- Plans for subsequent F2F meetings in July (Geneva) and October (Hannover)

- The meeting logistics, agenda, working practices, policies, and document allocation considerations were reviewed.

  - Remote access to the meeting was provided using Zoom. This requires discipline in the meeting room (no microphone to be switched on, podium and room microphones to be under central control).

  - Having text and software available is crucial (and not just arriving at the end of the meeting).

  - There were no objections voiced in the opening plenary to the consideration of late contributions.

- The results of the previous meeting and the meeting report JVET-AC1000 were reviewed. The following small issues in the meeting report were noted and were not considered sufficient to warrant issuing a revision. These are obviously left over from a previous report, and the correct information can be found in other places of the report:

  - In section 2.10, it is mentioned that JVET-AB1012 would become updated, but this did not happen.

  - In section 2.10, the number of email list subscribers relates to the status of January 2023 rather than 2022.

  - In section 7.2, the times for the first two information sharing sessions should have been Monday 16 Jan. 0500-0730 UTC, and Wednesday 18 Jan. 0500-0600 UTC

  - In Annex B2, the twenty-ninth meeting should be referred rather than the twenty-eighth.

  - …

- There was somewhat less of a problem of late non-cross-check documents.

- There were again a few documents registered where authors’ given names were not abbreviated, and/or company affiliation was missing in the authors’ list. Participants were reminded to stick to JVET’s conventions.

- Experts are asked to inform the chair when the title of a document is changed, or if authors are added. Otherwise, that might not be correct in the meeting notes.

- The primary goals of the meeting were

  - VVC and VSEI third editions FDIS (and plans for next ITU editions in July)

    - VVC DAM draft DoCR (from last meeting) in MDS22364

    - VSEI DAM ballot response in m62571: Relatively large number of comments, and numerous input contributions in particular on NNPF; relation with proposed processing order SEI message (JVET-AC2027) ?

    - Stay with preliminary FDIS as output from this meeting?

  - New edition video CICP FDIS (DIS ballot response in m62572) (and plans for next ITU edition in July) – ballot comment on including ST 2128 descriptor may need consideration of the status, would be good to include but this may impose delay

  - Optimization of encoders and receiving systems for machine analysis of coded video content – WD of TR to be issued

  - Preparation of verification tests film grain and multilayer

  - Any action items on reference software JM/HM/VTM? Status of MV-HEVC software and test conditions?

  - Additional colour type identifiers for AVC and HEVC (Draft 2 in JVET-AC1008 was issued at the last meeting)

    - there is a ballot comment for including identifier for SMPTE ST 2128 in new edition of CICP

    - might also be included in new edition of H.265 in July (there was no ISO/IEC 23008-2 DIS ballot comment on this, so it is not in the newest edition of 23008-2)

    - no action yet to be taken for AVC at this moment. It was suggested to consider new editions of ITU-T H.264 and ISO/IEC 14496-10 in the near future.

  - Exploration Experiments

    - Neural network-based video coding

    - Enhanced compression beyond VVC

- Liaison communication:

  - TBD – none at this moment

- Joint meetings: with AG 5 on subjective testing, with WG 4 on VCM, any other?

- Principles of standards development were discussed.

## Scheduling of discussions

- The times of meeting sessions followed the needs of the face-to-face meeting, with highest priority given to the aim of achieving the goals of the meeting. Typical meeting hours were in the range of 0900-1900 TRT with coffee breaks and lunch breaks as appropriate, however some early morning or late-night sessions may be necessary. Sessions were announced in the JVET calendar in advance as far as possible, but it might happen that some activities (such as breakout sessions) will be held at short notice.

Particular scheduling notes are shown below, although not necessarily 100% accurate or complete. Times are recorded in the local timzone of the meeting venue, except as otherwise noted:

- Fri. 21 April, 1st day

  - Morning session:

    - 0900–45 Opening remarks, review of practices, agenda, IPR policy reminder

    - 0945–1245 Reports of AHGs 1–X

  - Afternoon session:

    - 1400–2000 EE2 summary report

    - 1400-1900 BoG on NNPF (Y.-K. Wang, M. Hannuksela, S. Deshpande)

- Sat. 22 April, 2nd day

  - Morning session:

    - 0900–XXXX Review of …

  - Afternoon session:

    - XXXX–XXXX …

- Sun. 23 April, 3rd day

  - Morning session:

    - 0900–XXXX Review of …

  - Afternoon session:

    - XXXX–XXXX …

- Mon. 24 April, 4th day

  - 0900–1130 MPEG information sharing session

  - Morning session:

    - 1200–1300 Further planning, Review of …

  - Afternoon session:

    - XXXX–XXXX …

- Tue. 25 April, 5th day

  - Morning session:

    - 0900–XXXX Review of …

  - Afternoon session:

    - XXXX–XXXX …

- Wed. 26 April, 6th day

  - 0900–1000 MPEG information sharing session

  - Morning session:

    - 1030–XXXX Further planning, Review of …

  - Afternoon session:

    - XXXX–XXXX …

- Thu. 27 April, 7th day

  - Morning session:

    - 0900–XXXX Review of …

  - Afternoon session:

    - XXXX–XXXX …

- Fri. 28 April, 8th day

  - Plenary:

    - 0900–XXXX Remaining doc review, review and approval of output docs, AHGs

    - XXXX–XXXX Recommendations, future planning, a.o.b.

  - 1400–XXXX MPEG information sharing session

  - XXXX–XXXX WG 5 Closing plenary: Approval of meeting recommendations

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows (note that the noted document counts do not include crosschecks and summary reports, and may not be completely accurate; documents which are allocated to multiple sections are only counted in one of them):

- AHG reports (14) (section 3)

- Project development (section 4)

  - AHG1: Deployment and advertisement of standards (2)

  - AHG2: Text development and errata reporting (1)

  - AHG3: Test conditions (0)

  - AHG3: Software development (1)

  - AHG4: Subjective quality testing and verification testing (3)

  - AHG4: Test Material (3)

  - AHG4: Subjective quality assessment methodology (0)

  - AHG5: Conformance test development (0)

  - AHG7: ECM tool assessment (4)

  - AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (12)

  - AHG10: Encoding algorithm optimization (3)

  - AHG13: Film grain synthesis (2)

  - Implementation studies (2)

  - Profile/tier/level specification (1)

  - Proposed modification of system interface (0)

  - General aspects of standards development and applications of standards (0)

- Low-level tool technology proposals (section 5) with subtopics (number counts excluding BoG and summary reports)

  - AHG11/AHG14 and EE1: Neural network-based video coding (25) (section 5.1)

  - AHG6/AHG12 and EE2: Enhanced compression beyond VVC capability (103) (section 5.2)

- AHG9: High-level syntax (HLS) proposals (section 6) with subtopics

  - SEI messages on neural-network post filter (32) (section 6.1)

  - SEI messages on topics other than NNPF (8) (section 6.2)

  - Non-SEI HLS aspects (0) (section 6.3)

- Joint meetings, plenary discussions, BoG reports (1), liaison (0), summary of actions (section 7)

- Project planning (section 8)

- Establishment of AHGs (section 9)

- Output documents (section 10)

- Future meeting plans and concluding remarks (section 11)

The document counts above do not include cross-checks and summary reports.

# AHG reports (14)

These reports were discussed during XXXX–XXXX on Friday 21 April 2023 (chaired by JRO).

JVET-AD0001 JVET AHG report: Project Management (AHG1) [J.-R. Ohm (chair), G. J. Sullivan (vice-chair)]

- Goals and activity

The work of the JVET overall had proceeded well in the interim period with approximately 20% higher number of input documents (as compared to the previous meeting) submitted to the current meeting. Intense discussion had been carried out on the group email reflector, and most output documents from the preceding meeting had been produced as planned.

Output documents from the preceding meeting had been made initially available at the JVET web site (https://jvet-experts.org/) or the ITU-based JVET site (http://wftp3.itu.int/av-arch/jvet-site/2023_01_AC_Virtual/). It is noted that the previous document sites http://phenix.int-evry.fr/jvet/, http://phenix.int-evry.fr/jct/, and http://phenix.int-evry.fr/jct3v/ are still accessible, but were converted to read-only.

The list of documents produced included the following, particularly:

- The meeting report (JVET-AC1000) [Posted 2023-02-17, also submitted as WG 5 N 173]

- Guidelines for HM-based software development (JVET-AC1001) [Posted 2023-01-24]

- Errata report items for VVC, HEVC, AVC, and Video CICP (JVET-AC1004) [Posted 2023-03-21]

- Additional colour type identifiers for AVC, HEVC and Video CICP (Draft 3) (JVET-AC1008) [Posted 2023-02-24, last update 2023-03-08, also submitted as WG 5 preliminary WD N 186]

- Common test conditions for SHVC (JVET-AC1009) [Posted 2023-04-04]

- Common test conditions of 3DV experiments (JVET-AC1013) [not posted yet, situation about unavailable sequences needs to be clarified]

- Common test conditions for SCM screen content coding (JVET-AC1015) [Posted 2023-04-04]

- Algorithm description for Versatile Video Coding and Test Model 19 (VTM 19) (JVET-AC2002) [Posted 2023-04-07, also submitted as WG 5 WD N 161]

- Guidelines for VTM-based software development (JVET-AC2003) [Posted 2023-01-25]

- New level and systems-related supplemental enhancement information for VVC (Draft 4) (JVET-AC2005) [Posted 2023-01-31]

- VTM and HM common test conditions and evaluation procedures for HDR/WCG video (JVET-AC2011) [Posted 2023-02-25]

- Common Test Conditions and evaluation procedures for neural network-based video coding technology (JVET-AC2016) [Posted 2023-02-15]

- Description of algorithms and software in neural network-based video coding (NNVC) version 2 (JVET-AC2019) [Posted 2023-03-09, also submitted as WG 5 N 165]

- Film grain synthesis technology for video applications (Draft 4) (JVET-AC2020) [Posted 2023-03-27, last update 2023-03-31, to be submitted as WG 5 CDTR N 188]

- Draft verification test plan for VVC multilayer coding (JVET-AC2021) [Posted 2023-03-07]

- Draft plan for subjective quality testing of FGC SEI message (JVET-AC2022) [Posted 2023-03-07]

- Exploration experiment on Neural Network-based Video Coding (EE1) (JVET-AC2023) [Posted 2023-01-20, last update 2023-02-06, also submitted as WG 5 N 187]

- Exploration experiment on Enhanced Compression beyond VVC capability (EE2) (JVET-AC2024) [Posted 2023-01-20, last update 2023-02-20, also submitted as WG 5 N 189]

- Algorithm description of Enhanced Compression Model 8 (ECM 8) (JVET-AC2025) [Posted 2023-04-06, also submitted as WG 5 N 190]

- Conformance testing for VVC operation range extensions (Draft 4) (JVET-AC2026) [Posted 2023-04-XX]

- SEI processing order SEI message in VVC (Draft 1) (JVET-AC2027) [Posted 2023-03-23, also submitted as WG 5 preliminary WD N 182]

- Additional conformance bitstreams for VVC multilayer configurations (JVET-AC2028) [Posted 2023-04-XX]

- Optimization of encoders and receiving systems for machine analysis of coded video content (draft 1) (JVET-AC2030) [Posted 2023-03-20, also submitted as WG 5 preliminary WD N 177]

- Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content (JVET-AC2031) [Posted 2023-02-02, last update 2023-03-30]

- Improvements under consideration for neural network post filter SEI messages (JVET-AC2032) [Posted 2023-01-27, last update 2023-02-01, also submitted as WG 5 N 175]

Furthermore, the following documents were submitted to the ISO/IEC JTC1/SC29 parent body on behalf of its WG 5:

- Recommendations of the 10th WG 5 meeting (WG 5 N 173)

- Disposition of comments received on ISO/IEC DIS 23008-2:202x (5th edition) (WG 5 N 178)

- Text of ISO/IEC FDIS 23008-2:202x High efficiency video coding (5th edition) (WG 5 N 179)

- Draft disposition of comments received on ISO/IEC 23090-3:2022 DAM 1 (WG 5 N 180)

- Preliminary text of ISO/IEC FDIS 23090-3:202x Versatile video coding (3rd edition) (WG 5 N 181)

- Disposition of comments received on ISO/IEC 23090-15 DAM 1 (WG 5 N 184)

- Text of ISO/IEC FDIS 23090-15:202x Conformance testing for versatile video coding (2nd edition) (WG 5 N 185) – submission under preparation

- Liaison statement to SC 29/WG 1 (JPEG) on JPEG AI and neural network-based video coding (WG 5 N 191)

- List of AHGs established at the 10th WG 5 meeting (WG 5 N 192)

The fourteen ad hoc groups had made progress, and reports from those activities had been submitted. Furthermore, two exploration experiments (EE) on neural network-based video coding and on enhanced compression beyond VVC capability were conducted.

The arrangements for the 30th meeting had been announced in the JVET reflector, in the JVET logistics document (https://www.itu.int/wftp3/av-arch/jvet-site/2023_04_AD_Antalya/JVET-AD_Logistics.docx), and in the WG 5 calling notice (N 193) and agenda (N 194) for the 11th WG 5 meeting.

Software integration was finalized approximately according to the plan.

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system. That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful.

Roughly 200 input contributions (not counting the AHG reports, summary reports and crosschecks) had been registered for consideration at the current meeting.

It is further noted that, starting from the twentieth JVET meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and minor enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

To retain a consistent numbering scheme, the number range of output documents starting from 1001 was reserved for the previous JCT-VC topic items listed above, whereas the number range starting from 2001 was retained for VVC, VSEI and exploration activities.

A preliminary basis for the document subject allocation and meeting notes for the 30th meeting had been made publicly available on the ITU-hosted ftp site as http://wftp3.itu.int/av-arch/jvet-site/2023_04_AD_Antalya/JVET-AD_notes_d0.docx.

- Recommendations

The AHG recommends its continuation.

JVET-AD0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice-chairs)]

2.1	Output documents produced

2.1.1	JVET-AC1004 - Draft Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

Incorporated items at the JVET-AC meeting:

•	For VVC, all errata items have been incorporated in JVET-AC2005: New level and systems-related supplemental enhancement information for VVC (Draft 4).

•	For HEVC, incorporated the following items:

o	Confirmed redudnant call to clause 8.6.1 (reported by Cliff Reader – thanks!).

o	Confirmed JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset".

o	JVET-AC0311: On number of conformance tests (option 1.)

o	Updated issues 1 to 4 in Subsection 4.4 per inputs from Miska M. Hannuksela (thanks!).

o	Change the font size from 8 pt to 10 pt for "Table 9-4" in a paragraph in clause 9.3.2.2 (reported by Cliff Reader – thanks!).

•	For AVC, incorporated the following items:

o	Confirmed JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset".

o	An issue of talking about VUI in subclause 0.6.1 in ISO/IEC 14496-10:2022 (Edition 10) while the sentence should only talk about SEI messages.

2.1.2	JVET-AC1008 - Additional colour type identifiers for AVC, HEVC and Video CICP (Draft 3)

This document contains the draft text for the specification of additional colour type identifiers for AVC (Rec. ITU-T H.264 | ISO/IEC 14496-10), HEVC (Rec. ITU-T H.265 | ISO/IEC 23008-2), and CICP (Rec. ITU-T H.273 | ISO/IEC 23091-2). Text modifications are provided for specification of code point identifiers for YCgCo-R colour representation with equal luma and chroma bit depths and for Draft SMPTE ST 2128. The new code points for for YCgCo-R are referred to as YCgCo-Re and YCgCo-Ro, where the number of bits added to a source RGB bit depth is 2 (i.e., even) and 1 (odd), respectively. Draft SMPTE ST 2128 specifies a colour representation referred to as IPT-PQ-C2. The affected sections are provided relative to corresponding basis texts (based on the 2021-08 edition of Rec. ITU-T H.264 (the most recent edition), the 2020 edition of ISO/IEC 23008-2 (not the 5th edition currently under preparation), and ISO/IEC DIS 23091-2:202x (3rd Ed.), respectively). Equation numbers and their cross-references have been updated without revision marking. Change marks are included to show the changes since the October output (JVET-AB1008), which reflect the LMS equations for SMPTE ST 2128 as provided in JVET-AC0352 and some minor refinements and corrections since the previous draft.

2.1.3	JVET-AC2002 - Algorithm description for Versatile Video Coding and Test Model 19 (VTM 19)

The JVET established the VVC Test Model 20 (VTM20) software at its 29th meeting (11-20 January 2023, teleconference). This document serves as a source of general tutorial information on the VVC design and also provides an algorithm description and encoding method description of VTM20 software. It is noted that, as no update of the algorithm description document had been released at the 27th JVET meeting, the numbering of the VTM software version is one higher than that of this description document. In the main body of the text, numbering refers to the software version. The VVC has been developed by a joint collaborative team of ITU-T and ISO/IEC experts known as the Joint Video Experts Team (JVET), which is a partnership of ITU-T Study Group 16 Question 6 (known as VCEG) and ISO/IEC JTC 1/SC 29/WG 11 (known as MPEG). This draft new standard has been designed with two primary goals. The first of these is to specify a video coding technology with a compression capability that is substantially beyond that of the prior generations of such standards, and the second is for this technology to be highly versatile for effective use in a broadened range of applications. Some key application areas for the use of this standard particularly include ultra-high-definition video (e.g., with 3840×2160 or 7620×4320 picture resolution and bit depth of 10 or 12 bits as specified in Rec. ITU-R BT.2100), video with a high dynamic range and wide colour gamut (e.g., with the perceptual quantization or hybrid log-gamma transfer characteristics specified in Rec. ITU-R BT.2100), and video for immersive media applications such as 360° omnidirectional video projected using a common projection format such as the equirectangular or cubemap projection format, in addition to the applications that have commonly been addressed by prior video coding standards.

Ed. Notes:

VVC Test Model 19 (VTM20) algorithm description and encoding method v1

•	General improvements to neural network-based post filtering SEI messages (section 4.4) to bring the section up to date with the current filtering purposes of the NNPFC and NNPFA messages

•	Incorporated JVET-AC0096: AHG10/12: Suggestion for new CTC for RPR in VTM and ECM

2.1.4	JVET-AC2005 - New level and systems-related supplemental enhancement information for VVC (Draft 4)

This document contains a draft amendment for changes to the Versatile Video Coding (VVC) standard (ITU T H.266 | ISO/IEC 23090-3). This amendment includes the following: 1) the support of an unlimited level for video profiles; 2) some technical corrections and editorial improvements to the 2nd edition text of VVC; and 3) the VVC-specific supports for some supplemental enhancement information (SEI) messages that may be included in VVC bitstreams but are not to be specified in the VVC specification. These SEI messages include two systems-related SEI messages, one for signalling of "green metadata" as to be specified in ISO/IEC 23001-11 and the other for signalling of an alternative video decoding interface for immersive media as to be specified in ISO/IEC 23090-13, and five other SEI messages, namely the shutter interval information SEI message, the neural network post-filter characteristics SEI message SEI message, the neural-network post-processing filter activation SEI message, the phase indication SEI message, and the post filter hint SEI message, that are to be specified in a new edition of the Versatile Supplemental Enhancement Information messages for coded video bitstreams (VSEI) standard (ITU T H.274 | ISO/IEC 23002-7).

Draft 4 incorporated items:

•	All errata items for VVC in Subsection 2.3 of JVET-AB1004-v1/JVET-AC0346.

•	JVET-AC0311 option 1: Remove equations and related text from both VVC and HEVC (but leave the statements about testing the buffer behaviour in HRD similar as in AVC).

•	Errata item 3 in Subsection 4.3 of JVET-AB1004-v1.

•	Added the payload type of the post filter hint SEI message.

•	Modified the subclause specifying the use of the neural-network post-filter characteristics SEI message as noted in the BoG report on neural-network based post-filtering contributions (JVET-AC0324-v4):

o	Action on Item 14 (cleanup): Adopt Option A approach in JVET-AC0074-v2 with editorial rephrasing as noted in discussion.

o	Action on Item 16 (BF/cleanup): Adopt to specify that when picture rate upsampling is applied together with the frame packing arrangement SEI message with fp_arrangement_type equal to 5 (temporal interleaving), the pictures are the pictures from the same view, based on text in JVET-AC0074-v2 as expressed under Item 14.

o	Action on Item 17 (sensibility constraint): Specify that frame rate upsampling can only be used when all input pictures have the same cropped dimensions, after applying the super resolution post-filter applicable to the input picture, if any, based on text in JVET-AC0074-v2 as expressed under Item 14.

•	DAM ballot comment US-008/FI-06: add a subclause specifying the use of the phase indication SEI message.

•	DAM ballot comments FI-07 and FI-08: add a subclause specifying the use of the post-filter hint SEI message and specify constraints for its syntax element filter_hint_chroma_coeff_present_flag.

2.1.5	JVET-AC2027 - SEI processing order SEI message in VVC (draft 3)

This document contains the draft text for changes to the Versatile Video Coding (VVC) standard (Rec. ITU-T H.266 | ISO/IEC 23090-3), to specify the SEI processing order SEI message.

2.1.6	JVET-AC2032 - Improvements under consideration for neural network post filter SEI messages

This document contains the draft text for changes to the versatile supplemental enhancement information messages for coded video bitstreams (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7), to specify additional SEI messages, including the shutter interval information SEI message, neural-network post-filter characteristics SEI message, neural-network post-filter activation SEI message, phase indication SEI message, and post-filter hint SEI message.

3	Related input contributions

The following input contribution was noted as relevant to the work of this ad hoc group:

•	JVET-AD0077 - AHG2: On num_l0_weights and num_l1_weights for VVC specification

4	Remaining bug tickets

•	#1593 Figure 5 in the VVC specification shows the incorrect number of slices

•	#1594 Mismatch between VVC spec and VTM for sample generation in CCLM process

5	Recommendations

The AHG recommends to:

•	Approve JVET-AC1004, JVET-AC1008, JVET-AC2002, JVET-AC2005, JVET-AC2027, and JVET-AC2032 documents as JVET outputs,

•	Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

•	Encourage the use of the issue tracker to report issues with the text of both the VVC specification text and the algorithm and encoder description,

•	Continue to improve the editorial consistency of VVC text specification and Test Model documents,

•	Ensure that, when considering changes to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC specification text is made available in a timely manner,

•	Review bug tickets, and other AHG2 related inputs and act on them if found to be necessary.

JVET-AD0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice-chairs)]

The software model versions prior to the start of the meeting were:

•	VTM 20.0 (Apr. 2023)

•	HM-17.0 (Jan. 2023)

•	HM-16.21+SCM-8.8 (Mar. 2020)

•	SHM 12.4 (Jan. 2018)

•	HTM 16.3 (Jul. 2018)

•	JM 19.1 (Apr. 2023)

•	JSVM 9.19.15

•	JMVC 8.5

•	3DV ATM 15.0 (no version history)

•	HDRTools 0.23 (October 2021)

Software for MFC and MFCD is only available as published by ITU-T and ISO/IEC. It is planned to create repositories with the latest versions available in ITU-T H.264.2 (02/2016). All development history is lost.

2	Software development

Development was continued on the GitLab server, which allows participants to register accounts and use a distributed development workflow based on git.

The server is located at:

https://vcgit.hhi.fraunhofer.de

The registration and development workflow are documented at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/VVC-Software-Development-Workflow

Although the development process is described in the context of the VTM software, it can be applied to all other software projects hosted on the GitLab server as well.

3	VTM related activities

The VTM software can be found at

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

The software development continued on the GitLab server. VTM versions 19.1 and 19.2 were tagged on Jan. 18, and VTM version 20.0 was tagged on Apr. 4. VTM 20.1 is expected during the 30th JVET meeting.

VTM 19.1 was tagged on Jan. 18, 2023. Changes include:

•	Remove unused functions

•	Use constant for subblock size in deblocking filter

•	Fix variable names

•	Define constant for VDPU size and fix function name

•	Clean up intra and CIIP code

•	Fix variable names

•	Fixes in SubpicMergeApp for NumSubPics and SubPicId setters

•	Use strong-typed enum for MtsType

•	Clean deblocking filter code

•	SubPicMergeApp: remove duplicate ceilLog2 function

•	Clean up DMVR code

•	Fix compilation when disabling REUSE_CU_RESULTS

•	Use named constants

•	Use named variable for unset cost

•	Clean up reading/parsing of number of active refs

•	JVET-T0056: SEI manifest and SEI prefix indication

•	Define named reference index for IBC

•	Clean up ISP and use strong typed enum

•	Use strong typed enum for SEI payload type

•	Rename dynamic_cache to Pool

•	JVET-AB0069 change the length of the po_sei_processing_order[i] syntax element

•	JVET-AB0135 and BC#017 Add nnpfc_total_kilobyte_size and replace nnpfc_complexity_idc with a flag

•	Rename LFCUParam data structure

•	M60678: ballot comments of FI_03

•	Clean up MMVD related code

•	Define RefSetArray template to define arrays indexed by listIdx and refIdx

•	Move new macros into section where they are indicated to be removed in next cycle

•	Fix compilation when JVET_R0351_HIGH_BIT_DEPTH_ENABLED is enabled

•	Use python3 as interpreter for cmake/CMakeBuild/bin/cmake.py

•	Use single variable to control ENABLE_TRACING

•	JVET-AB0058: signal frame upsampling in neural network post-filter characteristics

•	Fix a gcc12 compiling warning/error

•	Add option to enable RExt__HIGH_BIT_DEPTH_SUPPORT from command line

•	Fix variable names

•	Clean up hash related functions

•	Remove Ubuntu 18.04 from auto-build pipeline

•	Use ptrdiff_t for strides

•	Use strong enum type for AffineModel

•	Clean buffer usage in merge RD checking functions

•	Clean up BCW and AMVR related code

•	Clean up merge context data structure

•	Use strong typed enum for MESearchMethod

•	Remove unused field

•	Use strong typed ChannelType

•	Use strong typed enum for MergeType

•	Fix gcc 12 compiling errors

•	Replace m_subPuMC by a local variable

•	Clean up MV precision handling functions and use strong-typed enum

•	Define MergeIdxPair type to hold pair of merge indices

•	Clean up inter merge related functions

•	Clean up width/height in InterPrediction::xPredInterBlk

•	Remove m_iRefListIdx field from InterPrediction class

•	Use strong typed enum for APS type

•	Remove unused variable

•	Use proper data type for RPR scaling

•	Fix variable names and use named constants

•	Use strong-type enum for DFunc

•	Change copyright year to 2023

•	Remove unused field

•	Fix parameter type in function definition: use ptrdiff_t instead of uint32_t

•	JVET-AB0051: Modification of SEI processing order SEI message

•	Clean up MVD encoding

•	Group partition contexts for faster copy

•	Define enum for MIP size and use Pel elements for buffers

•	Improve adherence to coding guidelines in EncGOP.cpp

•	JVET-AB0049: Modifications of NNPFC and NNPFA

•	Improve adherence to coding guidelines

•	JVET-AB0050: Add two flags for NNPFA

•	Remove mvRefine field from InterPredictionData

•	Fix macOS build with trace file enabled

•	Use Size type instead of array of two values

•	Remove g_tbMax and use floorLog2 instead

•	Move sei_processing_order.cfg into sei_vui subdirectory

•	Remove duplicate function and fix data types

•	Add Ubuntu 22.04 auto-build

•	JVET-AB0070: post-filter hint SEI

•	Remove macros for HLS syntax element read/write

•	Fix #1586 and #1584: invalid syntax and cfg params for NNPFC SEI message

•	Clean up definition and use of g_GeoParams

•	Fix xParsePrefixSEIsForUnknownVCLNal

•	Clean up handling of PTLs in VPS

•	Fix xCheckScalableNestingConstraints()

•	Fix getInterLayerRefIdc with wrong parameter

•	Fix #1587: Set NumPTLsInVPS to 2 to avoid encoder crash

•	Fix compilation when green SEI metadata is enabled

•	Fix compilation: remove unused variable

•	Fix use of a not initialized value when decoding

VTM 19.2 was tagged on Jan. 18, 2023. Changes include:

•	Remove macros from previous cycle

VTM 20.0 was tagged Apr. 4, 2023. Changes include:

•	JVET-AC0166: Set C++ version to C++17

•	JVET-AC0139: Unified merge function and updated cfg settings

•	JVET-AC0096: RPR functionality testing

•	JVET-AC0154: NNPF SEI IuC item 13, 31, and 34

•	JVET-AC0127: NNPF SEI IuC item 11, 20, 28, 30, 32

•	JVET-AC0058 indicates the SEI processing order of T35 SEI messages with

•	JVET-AC0061: Define new syntax elements to specify input and output integer...

•	JVET-AC0062: Update constraint checks for nnpfc_num_input_pics_minus1 and...

•	JVET-AC0344: Signal 2 new syntax elements,...

•	JVET-AC0074: adopted constraints for NNPF frame rate upsampling

•	JVET-AC0353: Implementation of nnpfc_base_Flag

•	Move GDR status out of PicHeader structure into Picture

•	Adapt code to new SW guidelines

•	Fix and enable SIMD SSE functions

•	Use strong-typed enum for deblocking's EdgeDir

•	Minor cleanup for Slice.h

•	Use standardized memory alignment features

•	Minor style improvements in AnnexBread

•	Avoid use of std::iterator which is deprecated in C++17

•	Clean up RPL-related code

•	Clean up adaptive QP code

•	Fix of the RelCU speed-up of ISP

•	Remove use of 'using namespace std'

•	Split ConstraintInfo class out of Slice.h/cpp

•	Update .clang-format file

•	Adjust size of be a multiple of alignment when using std::aligned_alloc

•	StreamMergeApp: fix VPS allocation issue

•	Rename m_tmpStorageLCU to m_tmpStorageCtu

•	Define AlfMode element that captures enable flag and filter index

•	Fix compilation when JVET_O0756_CALCULATE_HDRMETRICS is enabled

•	Add GCC 12 to build configuration

•	Include additional build with high bit depth enabled

•	Move parameter sets out of Slice.h/cpp

•	Ignore CLion files

•	Rename "LevelTier" to "TierLevel"

•	Move MergeCtx and related classes from ContextModelling to InterPrediction

•	add options --SourceScalingRatio to rescale source video

•	Fix parsing of unknown SEI message type.

•	Clean up GDR condition checks

•	Fix assert() statement

•	Minor cleanup for affine block checking

•	add option --PrintRefLayerMetrics to compute and display PSNR between current layer and reference layer

•	Fix compiling issues with clang-14

•	Fix memory leak in MergeItemList

•	Store latest EDRAP POC in DecLib and set it to Slice

•	Fix: Use size of reference picture instead of using picture size from PPS...

•	Fix rpl_idx parsing when numRplsInSps == 1 && rplSpsFlag == 1

•	Use strong-typed enum for chroma format

•	Fix rpl use in checkSubpicTypeConstraints function

•	Fix for sequential coding of PPS for RPR

•	Clean up BCW target block computation for bipred

•	Align variable name to JVET-AC2032 text

•	Fix issue with m_olsHrdParams

•	Fix PicHeader::m_maxNumAffineMergeCand value when Affine is OFF and RDO for merge if no affine and no sbTMVP for picture

•	Fix: Related to HashME and sequential coding

•	Fix to conformance window usage for RPR

•	Fix multilayer with fields

•	Update software documentation for several SEINNPostFilterCharacteristics parameters.

•	Fix formatting of Neural network post-filter characteristics SEI parameter table

•	Add CI build for PDF manual

•	Update version number in SW manual

VTM 20.1 is expected to be tagged during the 30th JVET meeting. Changes include so far:

•	Add command line option to print per-frame encoding time in float format

•	Fix parsing of multiple SEI payloads in one NAL unit

•	Fix parsing of MAI SEI message

•	Fix parsing of SDI SEI message

•	Fix m_maxChromaFormatConstraintIdc derivation

•	Fix subpictures compliance check on monochrome streams after !2541

•	Fix build when GDR_ENABLED is 0

•	Undefine macro from HDRTools

3.1	CTC Performance

The following tables shows VTM 20.0 performance over HM 17.0.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-17.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -29.50% | -32.81% | -33.83% | 1273% | 166% |
| Class A2 | -29.73% | -24.39% | -21.58% | 2142% | 174% |
| Class B | -22.32% | -27.21% | -30.98% | 2397% | 175% |
| Class C | -22.89% | -19.53% | -23.19% | 3348% | 179% |
| Class E | -26.04% | -25.90% | -24.15% | 1869% | 158% |
| Overall | -25.50% | -25.75% | -27.02% | 2187% | 171% |
| Class D | -18.80% | -13.85% | -13.68% | 4270% | 189% |
| Class F | -39.49% | -40.23% | -42.90% | 4220% | 171% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over HM-17.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -40.60% | -40.71% | -47.16% | 586% | 158% |
| Class A2 | -44.02% | -41.73% | -40.78% | 679% | 170% |
| Class B | -37.41% | -50.12% | -48.51% | 672% | 161% |
| Class C | -33.86% | -36.28% | -38.21% | 923% | 164% |
| Class E |  |  |  |  |  |
| Overall | -38.42% | -42.87% | -43.95% | 713% | 163% |
| Class D | -31.72% | -32.60% | -31.95% | 970% | 161% |
| Class F | -46.07% | -49.76% | -50.65% | 490% | 145% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over HM-17.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -30.40% | -37.43% | -35.12% | 629% | 152% |
| Class C | -28.18% | -21.23% | -21.06% | 817% | 167% |
| Class E | -31.16% | -35.72% | -28.79% | 309% | 140% |
| Overall | -29.85% | -31.60% | -28.85% | 575% | 154% |
| Class D | -26.75% | -16.50% | -14.88% | 881% | 179% |
| Class F | -42.17% | -44.02% | -44.14% | 435% | 137% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over HM-17.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -34.52% | -38.34% | -36.12% | 526% | 160% |
| Class C | -28.07% | -17.73% | -18.09% | 670% | 176% |
| Class E | -33.06% | -37.89% | -31.10% | 280% | 146% |
| Overall | -32.01% | -31.36% | -28.85% | 487% | 161% |
| Class D | -26.68% | -12.73% | -11.47% | 762% | 175% |
| Class F | -40.13% | -41.24% | -41.72% | 437% | 145% |

According to common test conditions in random access configuration HM is using a GOP size of 16 pictures compared to VTM using a GOP of 32 pictures. Random access points are inserted approximately every second aligned with a GOP boundary of GOP 32 in both VTM and HM. VTM uses two more reference pictures in random access than HM (due to more memory being availably in typical level settings).

The following tables show VTM 20.0 performance compared to VTM 19.0 using SDR CTC:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-19.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.06% | -0.03% | 99% | 101% |
| Class A2 | 0.00% | 0.03% | 0.00% | 99% | 101% |
| Class B | 0.00% | 0.01% | 0.02% | 99% | 99% |
| Class C | 0.00% | 0.02% | 0.04% | 100% | 97% |
| Class E | 0.00% | -0.01% | 0.23% | 99% | 99% |
| Overall | 0.00% | 0.02% | 0.05% | 99% | 99% |
| Class D | -0.01% | -0.05% | 0.20% | 100% | 98% |
| Class F | -0.02% | -0.09% | -0.02% | 101% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-19.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | -0.07% | 96% | 100% |
| Class A2 | -0.03% | -0.03% | 0.01% | 95% | 100% |
| Class B | 0.05% | 0.05% | 0.01% | 96% | 98% |
| Class C | 0.05% | 0.02% | 0.11% | 95% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | 0.02% | 96% | 98% |
| Class D | 0.01% | 0.14% | 0.09% | 96% | 94% |
| Class F | 0.01% | 0.09% | 0.10% | 96% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-19.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.05% | -0.45% | 98% | 97% |
| Class C | 0.03% | 0.04% | 0.00% | 98% | 93% |
| Class E | -0.08% | -0.27% | 0.81% | 99% | 96% |
| Overall | 0.01% | -0.07% | 0.01% | 98% | 95% |
| Class D | 0.07% | -0.40% | 0.87% | 99% | 93% |
| Class F | 0.13% | 0.05% | 0.03% | 98% | 94% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-19.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.06% | 0.01% | 91% | 97% |
| Class C | 0.12% | -0.10% | 0.20% | 91% | 97% |
| Class E | -0.09% | 0.45% | 0.24% | 92% | 95% |
| Overall | 0.03% | 0.06% | 0.13% | 91% | 96% |
| Class D | 0.02% | 0.06% | 0.24% | 93% | 91% |
| Class F | 0.09% | 0.45% | 0.02% | 93% | 94% |

The following tables show VTM 20.0 performance over HM 17.0 using HDR CTC:

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM17.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -39.54% | -37.56% | -36.67% | -54.98% | -48.45% | -33.68% | -49.48% | -40.84% | 286% | 97% |
| Class H2 |  |  |  |  |  | -31.97% | -57.69% | -63.36% | 271% | 97% |
| Overall | -39.54% | -37.56% | -36.67% | -54.98% | -48.45% | -33.06% | -52.47% | -49.03% | 280% | 97% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over HM17.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -41.45% | -27.31% | -26.79% | -57.74% | -52.79% | -24.00% | -52.93% | -45.13% | 1538% | 113% |
| Class H2 |  |  |  |  |  | -21.76% | -47.21% | -50.57% | 1320% | 109% |
| Overall | -41.45% | -27.31% | -26.79% | -57.74% | -52.79% | -23.18% | -50.85% | -47.11% | 1455% | 112% |

The following tables show VTM 20.0 performance compared to VTM 19.0 using HDR CTC:

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM19.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.04% | 0.00% | -0.05% | 0.00% | -0.01% | -0.05% | 0.00% | 95% | 95% |
| Class H2 |  |  |  |  |  | 0.07% | 0.08% | -0.06% | 95% | 97% |
| Overall | 0.00% | 0.04% | 0.00% | -0.05% | 0.00% | 0.02% | -0.01% | -0.02% | 95% | 96% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM19.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.04% | 0.00% | 0.01% | 0.00% | 0.00% | 0.00% | 0.04% | -0.04% | 98% | 96% |
| Class H2 |  |  |  |  |  | 0.00% | -0.01% | -0.01% | 98% | 99% |
| Overall | -0.04% | 0.00% | 0.01% | 0.00% | 0.00% | 0.00% | 0.02% | -0.03% | 98% | 97% |

The following tables show VTM 20.0 performance compared to VTM 19.0 using high bit depth CTCs at ‘low’ QP ranges. The simulations for VTM20.0 included two additional merge requests (2566 and 2567) to fix chroma format errors and a compilation error regarding HDR tools integration.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM19.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 98% |
| PQ422 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 98% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM19.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.07% | 0.13% | 0.06% | 0.07% | 0.13% | 0.06% | 97% | 100% |
| PQ422 | 0.03% | 0.01% | 0.06% | 0.03% | 0.01% | 0.05% | 96% | 101% |
| Overall | 0.05% | 0.07% | 0.06% | 0.05% | 0.07% | 0.06% | 96% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM19.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.03% | 0.03% | 0.02% | 0.03% | 0.03% | 0.02% | 97% | 100% |
| PQ422 | 0.00% | -0.02% | 0.02% | 0.00% | -0.01% | 0.01% | 96% | 99% |
| Overall | 0.01% | 0.01% | 0.02% | 0.01% | 0.01% | 0.02% | 96% | 100% |
| Overall PQ | 0.02% | 0.03% | 0.03% | 0.02% | 0.03% | 0.02% | 97% | 99% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM19.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| HLG422 | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 98% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM19.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | 0.00% | 0.00% | 98% | 101% |
| HLG422 | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Overall | -0.01% | 0.00% | 0.00% | 98% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM19.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | -0.01% | 0.00% | 98% | 97% |
| HLG422 | 0.00% | 0.01% | -0.01% | 99% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Overall HLG | 0.00% | 0.00% | 0.00% | 98% | 99% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM19.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| SVT12 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM19.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 102% |
| SVT12 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM19.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 102% |
| SVT12 | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Overall RGB | 0.00% | 0.00% | 0.00% | 100% | 101% |

The following tables show VTM 20.0 performance compared to VTM 19.0 using high bit depth CTCs at ‘standard’ QP ranges. The simulations for VTM20.0 included the two aforementioned merge requests (2566, 2567).

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM19.0 |  |  |  |  |
|  | PSNR |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.04% | -0.17% | 96% | 100% |
| Class H2 | -0.02% | -0.12% | -0.16% | 97% | 101% |
| Overall | -0.01% | -0.04% | -0.16% | 97% | 100% |
|  | All Intra |  |  |  |  |
|  | Over VTM19.0 |  |  |  |  |
|  | PSNR |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class H1 | 0.01% | -0.09% | -0.02% | 100% | 100% |
| Class H2 | 0.01% | -0.03% | 0.01% | 101% | 101% |
| Overall | 0.01% | -0.06% | -0.01% | 100% | 101% |

Full results are attached to this AHG report as Excel files.

3.2	Issues in VTM affecting conformance

The following issues in VTM master branch may affect conformance:

•	Missing HLS features (see sections below)

3.3	Status of implementation of proposals of previous JVET meetings

The following list contains all adoptions of the Q and R meetings that were not marked as merged (or submitted) or specification only change in the software coordinator tracking sheet:

•	JVET-Q0112

•	JVET-Q0154: Disallow mixing of GDR and IRAP (Disallow mixing of GDR with any non-GDR).

•	JVET-Q0164

•	JVET-Q0402

•	JVET-R0178: Require that when no_aps_constraint_flag is equal to 1, sps_lmcs_enabled_flag and sps_scaling_list_enabled_flag shall be equal to 0

•	JVET-R0221

•	JVET-R0046: Change the description of the bitstream extraction process per the value of max_tid_il_ref_pics_plus1[ ][ ] (aspect 1.2 per JVET-R0046-v4).

•	JVET-R0065: Specify that GDR AUs shall be complete – i.e., all of the layers in the CVS shall have a picture in the AU (as with IRAP AUs).

•	JVET-R0191: Update the range value for num_ols_hrd_params_minus1.

•	JVET-R0222 aspect 1: Infer vps_max_sublayers_minus1 to be equal to 6 when sps_video_parameter_set_id is equal to 0 (i.e. VPS is not present). The exact editorial expression is at the discretion of the editor.

•	JVET-S0196 (JVET-S0144 item 17)

•	JVET-S0227 (JVET-S0144 item 22)

•	JVET-S0077 (JVET-S0139 item 5)

•	JVET-S0174 aspect 2 (JVET-S0139 item 18.b)

•	JVET-S0156 aspect 3 (JVET-S0139 item 21)

•	JVET-S0139 item 26 (no source listed, text only?)

•	JVET-S0188 aspect 1 (JVET-S0139 item 28)

•	JVET-S0139 item 40 (item does not exist)

•	JVET-S0042 (JVET-S0142 item 1.b)

•	JVET-S0174 aspect 1 (JVET S0143 item 19)

•	JVET-S0096 aspect 3 (JVET-S0140 item 10)

•	JVET-S0096 aspect 4 (JVET-S0140 item 13)

•	JVET-S0159 aspect 3 (JVET-S0140 item 16)

•	JVET-S0171 (JVET-S0256)

•	JVET-S0118 (JVET-S0141 item 7)

•	JVET-S0102 (JVET-S0141 item 9.a)

•	JVET-S0157 item 2 (JVET-S0141 item 13)

•	JVET-S0157 item 4 (JVET-S0141 item 14)

•	JVET-S0175 aspect 3 (JVET-S0141 item 16)

•	JVET-S0175 aspect 1, 2 (JVET-S0141 item 17)

•	JVET-S0175 aspects 4 and 5 (JVET-S0141 item 18)

•	JVET-S0175 aspect 6 (JVET-S0141 item 19)

•	JVET-S0198/ JVET-S0223 (JVET-S0141 item 24)

•	JVET-S0173 aspect 2 (JVET-S0141 item 40.b)

•	JVET-S0173 item 1 (JVET-S0141 item 51)

•	JVET-S0173 item 3 (JVET-S0141 item 52)

•	JVET-S0173 item 5 (JVET-S0141 item 53)

•	JVET-S0173 item 6 (JVET-S0141 item 54)

•	JVET-S0173 item 4 (JVET-S0141 item 56)

•	JVET-S0176 item 4 (JVET-S0141 item 60)

•	JVET-S0154 aspect 5 (JVET-S0141 item 68)

•	JVET-S0154 aspect 6 (JVET-S0141 item 69)

•	JVET-S0154 aspect 8 (JVET-S0141 item 71)

•	JVET-S0095 aspect 5 (JVET-S0145 item 5)

•	JVET-S0095 aspect 6 (JVET-S0145 item 6)

•	JVET-S0100 aspect 1, depends on JVET-R0193 (JVET-S0147 item 2)

•	FINB ballot comments

•	Make high tier support up to 960.

4	HM related activities

HM 18.0 was tagged on Apr. 20, 2023. Changes include:

•	Update to Ubuntu 22.04 build server

•	Update the HDR configuration files to align with the HDR CTC document (JVET-AC2011)

•	Update to FilmGrainCharacteristics SEI support, including applying film grain noise onto the decoded output.

The performance of HM 18.0 is identical to HM 17.0 under SDR CTCs. Initial results also indicate that the performance is identical to HM 17.0 under HDR, RExt non-4:2:0 and RExt high bit depth CTCs.

The following MRs are pending [with status indicated]:

•	Port the Y4M support [one issue remains]

•	Mark the current picture as short-term ref (for SCM) [need SCC expert reviewer]

The HM SCC (SCM) branch (HM-16.21+SCM-8.8) has not been updated for the recent HM versions. Updating SCM to, for example, HM-18.0+SCM-8.8 should be considered. It may though be helpful to move SCC related functionality into separate source files. Volunteer work towards merging the branches would be appreciated.

As reported in the previous reports, further information on lambda optimisation in HM would be appreciated, including comparison of allocation of bits within the GOP structures between HM and VTM.

The HEVC bug tracker lists:

•	43 tickets for “HM”, most of which are more than 5 years,

•	1 ticket for “HM RExt”,

•	9 tickets for “HM SCC”, most of which are at least 3 years old,

Help to address these tickets would be appreciated.

One merge request (MR#40) is available related to HM SCC for ticket #1511 on SCC reference picture marking. We would appreciate help to confirm that the proposed change matches the SCC text.

5	360Lib related activities

The latest 360Lib software can be found at https://vcgit.hhi.fraunhofer.de/jvet/360lib

The following table is for the projection formats comparison using VTM-20.0 according to 360-degree video CTC (JVET-U2012) compared to that using VTM-19.0 (VTM-19.0 as anchor).

|  | PERP: VTM-20.0 over VTM-19.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -0.01% | -0.02% | 0.02% | 0.00% | -0.02% | 0.02% |
| Class S2 | -0.02% | 0.07% | 0.06% | -0.02% | 0.07% | 0.06% |
| Overall | -0.01% | 0.02% | 0.04% | -0.01% | 0.02% | 0.04% |

The following table compares generalized cubemap (GCMP) coding and padded equi-rectangular projection (PERP) coding using VTM-20.0 (PERP as anchor).

|  | GCMP Over PERP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -11.50% | -5.44% | -6.08% | -11.49% | -5.37% | -6.02% |
| Class S2 | -3.69% | 0.81% | 1.28% | -3.67% | 0.90% | 1.37% |
| Overall | -8.38% | -2.94% | -3.13% | -8.36% | -2.86% | -3.07% |

The following tables are for PERP and GCMP coding comparison between VTM-20.0 and HM-16.22 (HM as anchor), respectively.

|  | VTM-20.0 PERP Over HM-16.22 PERP (anchor) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -30.91% | -38.82% | -41.28% | -30.91% | -38.88% | -41.28% |
| Class S2 | -36.89% | -37.33% | -39.65% | -36.88% | -37.36% | -39.71% |
| Overall | -33.30% | -38.23% | -40.63% | -33.29% | -38.27% | -40.65% |

|  | VTM-20.0 GCMP Over HM-16.22 PCMP (anchor) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -35.60% | -40.82% | -42.80% | -35.56% | -40.75% | -42.75% |
| Class S2 | -39.84% | -39.52% | -41.44% | -39.85% | -39.52% | -41.48% |
| Overall | -37.29% | -40.30% | -42.25% | -37.27% | -40.26% | -42.24% |

6	SCM related activities

There had not been any further developments to SCC’s SCM during this meeting cycle.

7	SHM related activities

There had not been any further developments to SHVC’s SHM during this meeting cycle.

8	HTM related activities

There had not been any releases of HTM of MV-HEVC and 3D-HEVC.

The next release will include the following changes:

•	JVET-Z0209: Early termination during calculating RDcost of depth

9	HDRTools related activities

There had not been any updates of HDRTools.

New development is being added under the branch named 0.24-dev.

10	JM, JSVM, JMVM related activities

A new version of JM was tagged on April 20, 2023. Changes include:

•	Change build system to cmake

•	fix build of rtpdump and rtploss on linux

•	add config file for automatic building

•	remove VS2015 from build spec

•	Compile fixes for newer compilers

Basic testing was performed to confirm that the code still works, but performance testing is pending. Performance changes are unlikely but may occur due to bug fixes.

There had not been any updates to the JSVM and JMVM software.

11	Bug tracking

The bug tracker for VTM and specification text is located at:

https://jvet.hhi.fraunhofer.de/trac/vvc

The bug tracker uses the same accounts as the HM software bug tracker. Users may need to log in again due to the different sub-domain. For spam fighting reasons account registration is only possible at the HM software bug tracker at

https://hevc.hhi.fraunhofer.de/trac/hevc

Bug tracking for HDRTools is located at:

https://gitlab.com/standards/HDRTools/-/issues

Please file all issues related to the VVC reference software and HDRTools into the appropriate bug tracker. Try to provide all the details, which are necessary to reproduce the issue. Patches for solving issues and improving the software are always appreciated.

12	Software repositories

Git repositories that were previously assigned to the JCT-VC group on the GitLab server were re-assigned to the JVET group. The old URLs are still working and will forward the user to the new location, with the display of a warning suggesting to update bookmarks to the new location. The SVN repository for 360Lib was converted to git and development was moved to the GitLab server. Historical branches can still be accessed in the SVN repository.

13	CTC alignment and merging

There are currently 8 JVET CTC documents:

JVET-Y2010		VTM/HM 4:2:0 test conditions

JVET-Z2011		VTM/HM HDR test conditions

JVET-AA2018	VTM/HM high bit depth test conditions (without spreadsheet)

JVET-T2013		VTM non-4:2:0 test conditions

JVET-AA1100	HM non-4:2:0 test conditions

JVET-U2012		VTM 360 video test conditions

JVET-AC1009	SHVC test conditions

JVET-AC1015	SCM test conditions

Older CTC documents are:

JCT3V-G1100		3DV test conditions

JVET-AC1013 was a planned output document for updating JCT3V-G1100, which was not produced. JVET-AD0201 was submitted with a description of the problems that were found during editing.

Further merging of HM RExt CTC into the appropriate VVC CTC was investigated (non 4:2:0 chroma formats), but proper comparable HM configuration files were not yet available by the beginning of this meeting.

The HM HDR test conditions require additional post-processing stages, due to WPSNR, deltaE100 and PSNRL100 values not being generated directly by the HM encoder. Merging of respective functions from VTM is recommended to streamline the process.

14	Guidelines for reference software development

Guidelines for VVC and HEVC reference software development have been released as output document JVET-AC2003 and JVET-AC1001.

15	Recommendations

The AHG recommends to:

-	Continue to develop reference software.

-	Improve documentation, especially the software manual.

-	Encourage people to test VTM and other reference software more extensively outside of common test conditions.

-	Encourage people to report all (potential) bugs that they are finding.

-	Encourage people to submit bit-streams/test cases that trigger bugs in VTM and other reference software.

-	Encourage people to submit non-normative changes that either reduce encoder run time without significantly sacrificing compression performance or improve compression performance without significantly increasing encoder run time.

-	Design and add configuration files to the VTM software for testing of HLS features.

-	Review VTM-related contributions and determine whether features should be added (or removed) from the software.

-	Continue to investigate the merging of branches.

-	Continue to investigate merging of CTC documents.

-	Verify correctness of CTC documents and issue updates as appropriate

-	Keep common test conditions aligned for the different standards.

-	Consider documents (including late documents) related to AHG3 activities.

JVET-AD0004 JVET AHG report: Test material and visual assessment (AHG4) [V. Baroncini, T. Suzuki, M. Wien (co-chairs), S. Liu, S. Puri, A. Segall, P. Topiwala, S. Wenger, J. Xu, Y. Ye (vice-chairs)]

    - Ad hoc group activity

    - Output documents produced

    - JVET-AC1004 - Draft Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

Incorporated items at the JVET-AC meeting:

- For VVC, all errata items have been incorporated in JVET-AC2005: New level and systems-related supplemental enhancement information for VVC (Draft 4).

- For HEVC, incorporated the following items:

  - Confirmed redudnant call to clause 8.6.1 (reported by Cliff Reader – thanks!).

  - Confirmed JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset".

  - JVET-AC0311: On number of conformance tests (option 1.)

  - Updated issues 1 to 4 in Subsection 4.4 per inputs from Miska M. Hannuksela (thanks!).

  - Change the font size from 8 pt to 10 pt for "Table 9-4" in a paragraph in clause 9.3.2.2 (reported by Cliff Reader – thanks!).

- For AVC, incorporated the following items:

  - Confirmed JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset".

  - An issue of talking about VUI in subclause 0.6.1 in ISO/IEC 14496-10:2022 (Edition 10) while the sentence should only talk about SEI messages.

    - JVET-AC1008 - Additional colour type identifiers for AVC, HEVC and Video CICP (Draft 3)

This document contains the draft text for the specification of additional colour type identifiers for AVC (Rec. ITU-T H.264 | ISO/IEC 14496-10), HEVC (Rec. ITU-T H.265 | ISO/IEC 23008-2), and CICP (Rec. ITU-T H.273 | ISO/IEC 23091-2). Text modifications are provided for specification of code point identifiers for YCgCo-R colour representation with equal luma and chroma bit depths and for Draft SMPTE ST 2128. The new code points for for YCgCo-R are referred to as YCgCo-Re and YCgCo-Ro, where the number of bits added to a source RGB bit depth is 2 (i.e., even) and 1 (odd), respectively. Draft SMPTE ST 2128 specifies a colour representation referred to as IPT-PQ-C2. The affected sections are provided relative to corresponding basis texts (based on the 2021-08 edition of Rec. ITU-T H.264 (the most recent edition), the 2020 edition of ISO/IEC 23008-2 (not the 5th edition currently under preparation), and ISO/IEC DIS 23091-2:202x (3rd Ed.), respectively). Equation numbers and their cross-references have been updated without revision marking. Change marks are included to show the changes since the October output (JVET-AB1008), which reflect the LMS equations for SMPTE ST 2128 as provided in JVET-AC0352 and some minor refinements and corrections since the previous draft.

    - JVET-AC2002 - Algorithm description for Versatile Video Coding and Test Model 19 (VTM 19)

The JVET established the VVC Test Model 20 (VTM20) software at its 29th meeting (11-20 January 2023, teleconference). This document serves as a source of general tutorial information on the VVC design and also provides an algorithm description and encoding method description of VTM20 software. It is noted that, as no update of the algorithm description document had been released at the 27th JVET meeting, the numbering of the VTM software version is one higher than that of this description document. In the main body of the text, numbering refers to the software version. The VVC has been developed by a joint collaborative team of ITU-T and ISO/IEC experts known as the Joint Video Experts Team (JVET), which is a partnership of ITU-T Study Group 16 Question 6 (known as VCEG) and ISO/IEC JTC 1/SC 29/WG 11 (known as MPEG). This draft new standard has been designed with two primary goals. The first of these is to specify a video coding technology with a compression capability that is substantially beyond that of the prior generations of such standards, and the second is for this technology to be highly versatile for effective use in a broadened range of applications. Some key application areas for the use of this standard particularly include ultra-high-definition video (e.g., with 3840×2160 or 7620×4320 picture resolution and bit depth of 10 or 12 bits as specified in Rec. ITU-R BT.2100), video with a high dynamic range and wide colour gamut (e.g., with the perceptual quantization or hybrid log-gamma transfer characteristics specified in Rec. ITU-R BT.2100), and video for immersive media applications such as 360° omnidirectional video projected using a common projection format such as the equirectangular or cubemap projection format, in addition to the applications that have commonly been addressed by prior video coding standards.

Ed. Notes:

VVC Test Model 19 (VTM20) algorithm description and encoding method v1

- General improvements to neural network-based post filtering SEI messages (section 4.4) to bring the section up to date with the current filtering purposes of the NNPFC and NNPFA messages

- Incorporated JVET-AC0096: AHG10/12: Suggestion for new CTC for RPR in VTM and ECM

    - JVET-AC2005 - New level and systems-related supplemental enhancement information for VVC (Draft 4)

This document contains a draft amendment for changes to the Versatile Video Coding (VVC) standard (ITU-T H.266 | ISO/IEC 23090-3). This amendment includes the following: 1) the support of an unlimited level for video profiles; 2) some technical corrections and editorial improvements to the 2nd edition text of VVC; and 3) the VVC-specific supports for some supplemental enhancement information (SEI) messages that may be included in VVC bitstreams but are not to be specified in the VVC specification. These SEI messages include two systems-related SEI messages, one for signalling of "green metadata" as to be specified in ISO/IEC 23001-11 and the other for signalling of an alternative video decoding interface for immersive media as to be specified in ISO/IEC 23090-13, and five other SEI messages, namely the shutter interval information SEI message, the neural network post-filter characteristics SEI message SEI message, the neural-network post-processing filter activation SEI message, the phase indication SEI message, and the post filter hint SEI message, that are to be specified in a new edition of the Versatile Supplemental Enhancement Information messages for coded video bitstreams (VSEI) standard (ITU-T H.274 | ISO/IEC 23002-7).

Draft 4 incorporated items:

- All errata items for VVC in Subsection 2.3 of JVET-AB1004-v1/JVET-AC0346.

- JVET-AC0311 option 1: Remove equations and related text from both VVC and HEVC (but leave the statements about testing the buffer behaviour in HRD similar as in AVC).

- Errata item 3 in Subsection 4.3 of JVET-AB1004-v1.

- Added the payload type of the post filter hint SEI message.

- Modified the subclause specifying the use of the neural-network post-filter characteristics SEI message as noted in the BoG report on neural-network based post-filtering contributions (JVET-AC0324-v4):

  - Action on Item 14 (cleanup): Adopt Option A approach in JVET-AC0074-v2 with editorial rephrasing as noted in discussion.

  - Action on Item 16 (BF/cleanup): Adopt to specify that when picture rate upsampling is applied together with the frame packing arrangement SEI message with fp_arrangement_type equal to 5 (temporal interleaving), the pictures are the pictures from the same view, based on text in JVET-AC0074-v2 as expressed under Item 14.

  - Action on Item 17 (sensibility constraint): Specify that frame rate upsampling can only be used when all input pictures have the same cropped dimensions, after applying the super resolution post-filter applicable to the input picture, if any, based on text in JVET-AC0074-v2 as expressed under Item 14.

- DAM ballot comment US-008/FI-06: add a subclause specifying the use of the phase indication SEI message.

- DAM ballot comments FI-07 and FI-08: add a subclause specifying the use of the post-filter hint SEI message and specify constraints for its syntax element filter_hint_chroma_coeff_present_flag.

    - JVET-AC2027 - SEI processing order SEI message in VVC (draft 3)

This document contains the draft text for changes to the Versatile Video Coding (VVC) standard (Rec. ITU-T H.266 | ISO/IEC 23090-3), to specify the SEI processing order SEI message.

    - JVET-AC2032 - Improvements under consideration for neural network post filter SEI messages

This document contains the draft text for changes to the versatile supplemental enhancement information messages for coded video bitstreams (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7), to specify additional SEI messages, including the shutter interval information SEI message, neural-network post-filter characteristics SEI message, neural-network post-filter activation SEI message, phase indication SEI message, and post-filter hint SEI message.

    - Related input contributions

The following input contribution was noted as relevant to the work of this ad hoc group:

- JVET-AD0077 - AHG2: On num_l0_weights and num_l1_weights for VVC specification

    - Remaining bug tickets

- #1593 Figure 5 in the VVC specification shows the incorrect number of slices

- #1594 Mismatch between VVC spec and VTM for sample generation in CCLM process

    - Recommendations

The AHG recommends to:

- Approve JVET-AC1004, JVET-AC1008, JVET-AC2002, JVET-AC2005, JVET-AC2027, and JVET-AC2032 documents as JVET outputs,

- Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

- Encourage the use of the issue tracker to report issues with the text of both the VVC specification text and the algorithm and encoder description,

- Continue to improve the editorial consistency of VVC text specification and Test Model documents,

- Ensure that, when considering changes to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC specification text is made available in a timely manner,

- Review bug tickets, and other AHG2 related inputs and act on them if found to be necessary.

The report has an annex with an extensive list of available test materials. This annex should be put into a new version of AD1012.

JVET-AD0005 JVET AHG report: Conformance testing (AHG5) [D. Rusanovskyy, I. Moccagatta (co-chairs), F. Bossen, K. Kawamura, T. Ikai, H.-J. Jhu, K. Sühring, Y. Yu (vice-chairs)]

    - Activities

The AHG communication is conducted through the main JVET reflector, jvet@lists.rwth-aachen.de, with [AHG5] in message headers. However, no correspondence marked as AHG5 was sent between the 29th and 30th meetings.

    - Timeline

The progress on the Conformance testing specification is proceeding per the timeline below:

- VVCv1 conformance:

  - ISO/IEC FDIS 23090-15 issued from 2021-10 meeting, FDIS registered for formal approval 2022-07-11, FDIS ballot closed 2022-11-04, standard published 2022-11-24

  - H.266.1 V1 Consent 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, standard published 2022-07-12.

- VVCv2 conformance:

  - ISO/IEC 23090-15/Amd.1 CDAM: 2021-10

  - ISO/IEC 23090-15/Amd.1 DAM: 2022-01

  - DAM ballot closed 2022-11-15, ready to proceed to FDAM at current meeting 2023-01

  - H.266.1 V2 Consent planned for 2023-07

    - Status on bitstream submission

There were no changes to the submitted bitstreams. The status at the time of preparation of this report is as follows:

- conformance bitstreams for VVC:

  - 104 bitstream categories have been identified

  - At least one bitstream has been submitted in each identified category

  - 283 total bitstreams have been provided, checked, and made available

  - As reported in JVET-AB0005, HRD_A_Fujitsu (HRD_A_Fujitsu_4) has been regenerated to avoid CPB underflow

  - 7 of the 10 bitstreams reported in Ticket #1581 have been re-generated

- conformance bitstreams for VVC operation range extensions:

  - 57 bitstream categories have been identified

  - 1 bitstream of 1 identified category has been re-generated

  - 128 bitstreams of 57 identified categories have been cross-checked and uploaded.

  - No changes between 29th and 30th meeting.

    - Activities and Discussion

The AHG activities are on schedule with the preliminary timeline shown in section 2.

VVC activities:

Seven of the 10 bitstreams reported in Ticket #1581 have been re-generated:

- ILRPL_A_Huawei (ILRPL_A_Huawei_3)

- OPI_B_Nokia (OPI_B_Nokia_4)

- SPATSCAL444_A_Qualcomm (SPATSCAL444_A_Qualcomm_3)

- SPATSCAL_A_Qualcomm (SPATSCAL_A_Qualcomm_4)

- VPS_A_INTEL (VPS_A_INTEL_4)

- VPS_B_ERICSSON (VPS_B_ERICSSON_2)

- VPS_C_ERICSSON (VPS_C_ERICSSON_3)

and are available in the dropbox.

The remaining 3 bitstreams

- OLS_A_Tencent_5

- OLS_B_Tencent_5

- OLS_C_Tencent_5

have been re-generated with unmodified VTM-20.0 encoder, but VTM-20.0 decoder cannot decode them correctly. Ticket #1595 has been filed to report this problem. The problem may be related to #1581.

The 7 re-generated bitstreams, HRD_A_Fujitsu_4, and the remaining conformance streams are decoded correctly using VTM-20.0 w/o range extension support.

VVC operation range extensions activities:

All bitstreams are decoded correctly using VTM-20.0.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de).

The AHG5 chairs and JVET chairs can be reached at jvet-conformance@lists.rwth-aachen.de. Participants should not subscribe to this list but may send emails to it.

    - Contributions

JVET-AD0101 - Additional conformance tests of scaling window functionality for multilayer coding [Shunsuke Iwamura, Shimpei Nemoto, Atsuro Ichigaya].

JVET-AD0232 - HEVC Multiview Profiles: Implementation Status [Seethal Paluri, Dimitri Podborski, Alexis Tourapis].

    - Ftp site information

The procedure to exchange the bitstream (ftp cite, bitstream files, etc.) is specified in Sec 2 “Procedure” of JVET-R2008. The ftp and http sites for downloading bitstreams are

- VVC:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVC

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC/

- VVC operation range extensions:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVCv2

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCv2

The ftp site for uploading bitstream file is as follows.

ftp://ftp3.itu.int/jvet-site/dropbox/

(user id: avguest, passwd: Avguest201007)

If using FileZilla, the following configuration is suggested:

In the Filezilla Edit  Settings  Connection menu, it may also be necessary to set the minimum TLS level to 1.0.

    - Recommendations

The AHG recommends the following:

- Proceed with the generation, cross-checking, and documentation of the additional conformance bitstreams for VVC, in particular conformance bitstreams for VVC multilayer configurations (JVET-AC2028).

- Maintain and update the conformance bitstream database and contribute to report problems in JVET document 1004.

- Start the generation, cross-checking, and documentation of the conformance streams for the HEVC multiview profiles supporting extended bit depth (JVET-AA1011).

JVET-AD0006 JVET AHG report: ECM software development (AHG6) [V. Seregin (chair), J. Chen, F. Le Léannec, K. Zhang (vice-chairs)]

    - Activities

The AHG communication is conducted through the main JVET reflector, jvet@lists.rwth-aachen.de, with [AHG5] in message headers. However, no correspondence marked as AHG5 was sent between the 29th and 30th meetings.

    - Timeline

The progress on the Conformance testing specification is proceeding per the timeline below:

- VVCv1 conformance:

  - ISO/IEC FDIS 23090-15 issued from 2021-10 meeting, FDIS registered for formal approval 2022-07-11, FDIS ballot closed 2022-11-04, standard published 2022-11-24

  - H.266.1 V1 Consent 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, standard published 2022-07-12.

- VVCv2 conformance:

  - ISO/IEC 23090-15/Amd.1 CDAM: 2021-10

  - ISO/IEC 23090-15/Amd.1 DAM: 2022-01

  - DAM ballot closed 2022-11-15, ready to proceed to FDAM at current meeting 2023-01

  - H.266.1 V2 Consent planned for 2023-07

    - Status on bitstream submission

There were no changes to the submitted bitstreams. The status at the time of preparation of this report is as follows:

- conformance bitstreams for VVC:

  - 104 bitstream categories have been identified

  - At least one bitstream has been submitted in each identified category

  - 283 total bitstreams have been provided, checked, and made available

  - As reported in JVET-AB0005, HRD_A_Fujitsu (HRD_A_Fujitsu_4) has been regenerated to avoid CPB underflow

  - 7 of the 10 bitstreams reported in Ticket #1581 have been re-generated

- conformance bitstreams for VVC operation range extensions:

  - 57 bitstream categories have been identified

  - 1 bitstream of 1 identified category has been re-generated

  - 128 bitstreams of 57 identified categories have been cross-checked and uploaded.

  - No changes between 29th and 30th meeting.

    - Activities and Discussion

The AHG activities are on schedule with the preliminary timeline shown in section 2.

VVC activities:

Seven of the 10 bitstreams reported in Ticket #1581 have been re-generated:

- ILRPL_A_Huawei (ILRPL_A_Huawei_3)

- OPI_B_Nokia (OPI_B_Nokia_4)

- SPATSCAL444_A_Qualcomm (SPATSCAL444_A_Qualcomm_3)

- SPATSCAL_A_Qualcomm (SPATSCAL_A_Qualcomm_4)

- VPS_A_INTEL (VPS_A_INTEL_4)

- VPS_B_ERICSSON (VPS_B_ERICSSON_2)

- VPS_C_ERICSSON (VPS_C_ERICSSON_3)

and are available in the dropbox.

The remaining 3 bitstreams

- OLS_A_Tencent_5

- OLS_B_Tencent_5

- OLS_C_Tencent_5

have been re-generated with unmodified VTM-20.0 encoder, but VTM-20.0 decoder cannot decode them correctly. Ticket #1595 has been filed to report this problem. The problem may be related to #1581.

The 7 re-generated bitstreams, HRD_A_Fujitsu_4, and the remaining conformance streams are decoded correctly using VTM-20.0 w/o range extension support.

VVC operation range extensions activities:

All bitstreams are decoded correctly using VTM-20.0.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de).

The AHG5 chairs and JVET chairs can be reached at jvet-conformance@lists.rwth-aachen.de. Participants should not subscribe to this list but may send emails to it.

    - Contributions

JVET-AD0101 - Additional conformance tests of scaling window functionality for multilayer coding [Shunsuke Iwamura, Shimpei Nemoto, Atsuro Ichigaya].

JVET-AD0232 - HEVC Multiview Profiles: Implementation Status [Seethal Paluri, Dimitri Podborski, Alexis Tourapis].

    - Ftp site information

The procedure to exchange the bitstream (ftp cite, bitstream files, etc.) is specified in Sec 2 “Procedure” of JVET-R2008. The ftp and http sites for downloading bitstreams are

- VVC:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVC

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC/

- VVC operation range extensions:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVCv2

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCv2

The ftp site for uploading bitstream file is as follows.

ftp://ftp3.itu.int/jvet-site/dropbox/

(user id: avguest, passwd: Avguest201007)

If using FileZilla, the following configuration is suggested:

In the Filezilla Edit  Settings  Connection menu, it may also be necessary to set the minimum TLS level to 1.0.

    - Recommendations

The AHG recommends the following:

- Proceed with the generation, cross-checking, and documentation of the additional conformance bitstreams for VVC, in particular conformance bitstreams for VVC multilayer configurations (JVET-AC2028).

- Maintain and update the conformance bitstream database and contribute to report problems in JVET document 1004.

- Start the generation, cross-checking, and documentation of the conformance streams for the HEVC multiview profiles supporting extended bit depth (JVET-AA1011).

It is noted that in AI the encoder runtime increase is significantly larger than expected from the last meeting’s EE reports based in which new tools were adopted. This requires more analysis.

It is noted that the memory consumption of ECM was never reported before. This has probably increased since the beginning, but has never been optimized when new tools were integrated. This requires more analysis.

It was pointed out that the luma/chroma gain unbalance has again increased, and so far never action was taken on that.

JVET-AD0007 JVET AHG report: ECM tool assessment (AHG7) [X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, H.-J. Jhu, R.-L. Liao, H. Wang (vice-chairs)]

    - AhG meeting summary

The AhG meeting was held on Feb. 17, 2023. The meeting report was uploaded as JVET-AD0041. The agreed items during the meeting are summarized as below.

- ECM tool grouping

  - Group 1: Inter template matching tools

  - Group 2: Coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process

  - Group 3: Intra and IBC template matching (with search) related tools

  - Group 4: Tools that require more processing on the neighboring reconstructed samples than VVC

  - Group 5: Remaining tools

- Tool grouping list

| Group | Tools |
| --- | --- |
| 1 | TM, TM OBMC, ARMC-TM, TM GPM, TM-Merge, TM-AMVP, TM-MMBD, block level reference picture list reordering, TM based BCW index derivation, MVD sign prediction |
| 2 | LIC, CIIP with TIMD and TM merge, coefficient sign prediction |
| 3 | TM IBC, IBC BVD prediction, IBC BVP clustering, intra TMP, TIMD, TMRL, SGPM, Direct BV for chroma prediction |
| 4 | DIMD, CCCM |
| 5 | Remaining tools |

- The missing cfg options of tools in group 1-4 are as below. The cfg options of tools in group5 are to be further identified. Note that the missing control for TM IBC and TMRL affected testing for Group 3 off and Group 1-4 off. The controls of TM based BCW, CIIP with TM merge and IBC BVP clustering have been added to ECM 8.1 and included in the following tests.

  - TM based BCW index derivation, CIIP with TM merge, TM IBC, IBC BVP clustering, TMRL

- Testing configurations

  - ECM anchor: ECM-8.1

  - VTM anchor: VTM11-ecm8.1

  - Configurations: AI, RA, LDB (with TGM class), do not test LDB for group 3 and 4

  - Cfg options are used to disable tools

  - Test each group on their own, i.e., anchor vs anchor + disable tool group x (1-5)

    - Issues during testing

During the testing, several issues were identified.

ECM-8 decoding mismatch was observed for SCC sequences during tool off tests. In addition, a tool control issue and several compiling issues were reported. These issues which blocked the tool off tests were resolved in a short time. We thank the effort from the proponents and AHG6 to resolve these issues.

Minor issues on test setting and bitrate/PSNR calculating were observed and resolved during the crosschecking.

There are still open software issues related to tool off tests, such as issue #27, #36, #42, #43 (https://vcgit.hhi.fraunhofer.de/groups/ecm/-/issues). These issues didn’t block AHG7 tests. But it would be good to have them fixed.

    - Results of group off tests

    - Test anchor and crosschecking

As ECM-8.0 has software issues when some tools are disabled, ECM-8.1 is used as the first anchor for the group off tests. The second anchor is VTM-11ecm8.1. As the second anchor data is directly copied from AHG6 report, the runtime info is removed to avoid confusion.

The testers and crosscheckers are summarized in the table below. All the tests below have been crosschecked.

| Tests | Tester | Crosschecker |
| --- | --- | --- |
| Group 1 off | Jonathan Gan (v-jonathan.gan@oppo.com) | Charles Salmon-Legagneur (charles.salmon-legagneur@interdigital.com) |
| Group 2 off | Jonathan Gan (v-jonathan.gan@oppo.com) | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) |
| Group 3 off | Xiang Li (xlxiangli@google.com) | Zhipin Deng (zhipin.deng@bytedance.com) |
| Group 4 off | Xiang Li (xlxiangli@google.com) | Hong-Jheng Jhu (jhuhong-jheng@kwai.com) |
| Group 1-4 off | Xiang Li (xlxiangli@google.com) | Lien-Fei Chen (lienfei.chen@global.tencent.com), Hongtao Wang (hongtaow@qti.qualcomm.com) |

    - Group 1 off

Group 1 includes inter template matching tools.

The command line setting of this test is: --EnableTMTools=0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% | -9.04% | -18.79% | -25.18% | #DIV/0! | #DIV/0! |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% | -13.53% | -22.78% | -25.89% | #DIV/0! | #DIV/0! |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% | -8.31% | -21.80% | -20.07% | #DIV/0! | #DIV/0! |
| Class C | -0.01% | -0.01% | -0.01% | 100% | 100% | -9.23% | -14.01% | -14.35% | #DIV/0! | #DIV/0! |
| Class E | -0.01% | -0.01% | -0.01% | 100% | 100% | -10.50% | -18.87% | -17.71% | #DIV/0! | #DIV/0! |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 100% | -9.87% | -19.24% | -20.23% | #DIV/0! | #DIV/0! |
| Class D | -0.03% | -0.02% | -0.02% | 100% | 100% | -7.53% | -11.50% | -11.55% | #DIV/0! | #DIV/0! |
| Class F | 0.32% | 0.24% | 0.74% | 100% | 97% | -21.97% | -30.46% | -29.87% | #DIV/0! | #DIV/0! |
| Class TGM | 1.48% | 1.45% | 1.48% | 100% | 88% | -34.68% | -42.11% | -41.45% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 3.63% | 4.06% | 4.42% | 89% | 84% | -16.99% | -19.85% | -26.72% | #DIV/0! | #DIV/0! |
| Class A2 | 3.93% | 4.25% | 4.61% | 86% | 78% | -19.33% | -26.63% | -29.84% | #DIV/0! | #DIV/0! |
| Class B | 3.40% | 3.66% | 3.62% | 86% | 78% | -15.47% | -26.52% | -24.41% | #DIV/0! | #DIV/0! |
| Class C | 3.94% | 4.12% | 4.39% | 81% | 70% | -16.77% | -20.48% | -20.12% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 3.70% | 3.98% | 4.18% | 85% | 77% | -16.89% | -23.60% | -24.81% | #DIV/0! | #DIV/0! |
| Class D | 3.23% | 3.79% | 3.80% | 80% | 65% | -18.15% | -22.11% | -22.24% | #DIV/0! | #DIV/0! |
| Class F | 2.77% | 2.74% | 2.67% | 90% | 82% | -22.80% | -30.73% | -30.86% | #DIV/0! | #DIV/0! |
| Class TGM | 4.47% | 4.45% | 4.39% | 91% | 82% | -30.30% | -35.90% | -36.18% | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 4.18% | 4.84% | 5.12% | 77% | 69% | -12.85% | -27.67% | -25.87% | #DIV/0! | #DIV/0! |
| Class C | 4.43% | 4.86% | 5.46% | 75% | 60% | -14.03% | -22.60% | -22.14% | #DIV/0! | #DIV/0! |
| Class E | 4.28% | 5.75% | 5.37% | 78% | 78% | -11.04% | -20.42% | -19.67% | #DIV/0! | #DIV/0! |
| Overall (Ref) | 4.29% | 5.07% | 5.30% | 77% | 68% | -12.79% | -24.17% | -23.08% | #DIV/0! | #DIV/0! |
| Class D | 3.73% | 3.63% | 4.80% | 73% | 58% | -16.80% | -24.23% | -23.72% | #DIV/0! | #DIV/0! |
| Class F | 3.63% | 3.04% | 2.93% | 87% | 75% | -19.17% | -28.87% | -27.57% | #DIV/0! | #DIV/0! |
| Class TGM | 5.98% | 6.05% | 5.85% | 88% | 78% | -28.11% | -34.34% | -34.26% | #DIV/0! | #DIV/0! |

    - Group 2 off

Group 2 includes coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process.

The command line setting of this test is: --LIC=0 --CIIPTM=0 --CIIPTIMD=0 --NumSignPred=0 --IBCLIC=0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.45% | 0.71% | 0.73% | 87% | 91% | -8.62% | -18.26% | -24.61% | #DIV/0! | #DIV/0! |
| Class A2 | 0.50% | 0.59% | 0.70% | 84% | 91% | -13.09% | -22.32% | -25.37% | #DIV/0! | #DIV/0! |
| Class B | 0.45% | 0.58% | 0.63% | 83% | 89% | -7.89% | -21.35% | -19.56% | #DIV/0! | #DIV/0! |
| Class C | 0.37% | 0.44% | 0.50% | 80% | 87% | -8.89% | -13.62% | -13.92% | #DIV/0! | #DIV/0! |
| Class E | 0.66% | 0.82% | 0.91% | 83% | 90% | -9.90% | -18.24% | -16.96% | #DIV/0! | #DIV/0! |
| Overall | 0.47% | 0.61% | 0.68% | 83% | 89% | -9.44% | -18.76% | -19.68% | #DIV/0! | #DIV/0! |
| Class D | 0.30% | 0.33% | 0.48% | 78% | 85% | -7.23% | -11.19% | -11.11% | #DIV/0! | #DIV/0! |
| Class F | 2.47% | 2.44% | 2.86% | 88% | 92% | -20.39% | -29.10% | -28.55% | #DIV/0! | #DIV/0! |
| Class TGM | 0.49% | 0.56% | 0.56% | 88% | 98% | -35.30% | -42.63% | -41.99% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 2.14% | 1.29% | 1.77% | 89% | 98% | -18.18% | -21.79% | -28.57% | #DIV/0! | #DIV/0! |
| Class A2 | 1.60% | 1.06% | 1.56% | 89% | 97% | -21.14% | -28.86% | -31.85% | #DIV/0! | #DIV/0! |
| Class B | 1.50% | 1.05% | 1.23% | 88% | 96% | -17.03% | -28.35% | -26.17% | #DIV/0! | #DIV/0! |
| Class C | 1.14% | 0.70% | 0.92% | 86% | 95% | -19.05% | -23.14% | -22.82% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.55% | 1.01% | 1.32% | 88% | 96% | -18.62% | -25.75% | -26.89% | #DIV/0! | #DIV/0! |
| Class D | 0.71% | 0.63% | 0.52% | 85% | 94% | -20.18% | -24.57% | -24.82% | #DIV/0! | #DIV/0! |
| Class F | 2.91% | 2.39% | 2.72% | 91% | 95% | -22.81% | -31.17% | -31.01% | #DIV/0! | #DIV/0! |
| Class TGM | 1.52% | 1.39% | 1.50% | 90% | 99% | -32.21% | -37.78% | -37.91% | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.57% | 0.68% | 1.11% | 86% | 95% | -15.04% | -30.43% | -28.52% | #DIV/0! | #DIV/0! |
| Class C | 1.08% | 1.16% | 1.27% | 85% | 95% | -16.82% | -25.36% | -25.26% | #DIV/0! | #DIV/0! |
| Class E | 1.10% | 2.15% | 0.47% | 87% | 95% | -13.73% | -23.05% | -23.41% | #DIV/0! | #DIV/0! |
| Overall (Ref) | 1.29% | 1.21% | 1.00% | 86% | 95% | -15.31% | -26.90% | -26.16% | #DIV/0! | #DIV/0! |
| Class D | 0.59% | 0.17% | 1.21% | 85% | 96% | -19.33% | -26.86% | -26.34% | #DIV/0! | #DIV/0! |
| Class F | 2.64% | 1.10% | 1.12% | 90% | 97% | -20.03% | -30.26% | -28.94% | #DIV/0! | #DIV/0! |
| Class TGM | 1.68% | 1.70% | 1.86% | 92% | 99% | -31.01% | -37.15% | -36.76% | #DIV/0! | #DIV/0! |

    - Group 3 off

Group 3 includes intra and IBC template matching (with search) related tools. Note that the control of TM IBC and TMRL are still missing. The tests were performed with these two tools on.

The command line setting of this test is: --BvdPred=0 --IntraDBV=0 --BvpCluster=0 --IntraTMP=0 --TIMD=0 --SGPM=0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.45% | 0.29% | 0.35% | 83% | 93% | -8.62% | -18.55% | -24.91% | #DIV/0! | #DIV/0! |
| Class A2 | 1.03% | 1.10% | 1.19% | 84% | 88% | -12.59% | -21.88% | -24.97% | #DIV/0! | #DIV/0! |
| Class B | 1.15% | 1.04% | 1.23% | 83% | 86% | -7.24% | -20.96% | -19.06% | #DIV/0! | #DIV/0! |
| Class C | 0.95% | 0.78% | 0.80% | 84% | 83% | -8.36% | -13.33% | -13.64% | #DIV/0! | #DIV/0! |
| Class E | 1.89% | 1.88% | 1.88% | 84% | 84% | -8.80% | -17.33% | -16.14% | #DIV/0! | #DIV/0! |
| Overall | 1.09% | 1.01% | 1.09% | 84% | 86% | -8.87% | -18.41% | -19.33% | #DIV/0! | #DIV/0! |
| Class D | 0.72% | 0.38% | 0.53% | 85% | 82% | -6.84% | -11.13% | -11.06% | #DIV/0! | #DIV/0! |
| Class F | 1.79% | 1.69% | 2.05% | 92% | 92% | -20.84% | -29.45% | -28.95% | #DIV/0! | #DIV/0! |
| Class TGM | 4.10% | 4.52% | 4.91% | 93% | 98% | -33.05% | -40.44% | -39.56% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.26% | 0.13% | 0.39% | 96% | 99% | -19.67% | -22.70% | -29.57% | #DIV/0! | #DIV/0! |
| Class A2 | 0.54% | 0.22% | 0.71% | 96% | 99% | -21.96% | -29.46% | -32.43% | #DIV/0! | #DIV/0! |
| Class B | 0.61% | 0.40% | 0.55% | 95% | 99% | -17.76% | -28.83% | -26.67% | #DIV/0! | #DIV/0! |
| Class C | 0.40% | 0.30% | 0.63% | 96% | 98% | -19.63% | -23.42% | -23.01% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 0.47% | 0.28% | 0.57% | 96% | 99% | -19.48% | -26.29% | -27.43% | #DIV/0! | #DIV/0! |
| Class D | 0.38% | 0.24% | 0.37% | 96% | 98% | -20.45% | -24.87% | -24.96% | #DIV/0! | #DIV/0! |
| Class F | 1.01% | 1.14% | 0.96% | 97% | 99% | -24.22% | -31.97% | -32.16% | #DIV/0! | #DIV/0! |
| Class TGM | 2.04% | 2.28% | 2.40% | 96% | 100% | -31.92% | -37.31% | -37.43% | #DIV/0! | #DIV/0! |

    - Group 4 off

Group 4 includes tools that require more processing on the neighboring reconstructed samples than VVC.

The command line setting of this test is: --DIMD=0 --CCCM=0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 1.53% | 4.43% | 5.96% | 88% | 96% | -7.64% | -15.40% | -20.83% | #DIV/0! | #DIV/0! |
| Class A2 | 2.41% | 4.37% | 4.80% | 87% | 98% | -11.55% | -19.47% | -22.51% | #DIV/0! | #DIV/0! |
| Class B | 1.00% | 4.75% | 3.80% | 88% | 97% | -7.39% | -18.37% | -17.18% | #DIV/0! | #DIV/0! |
| Class C | 0.87% | 2.21% | 1.97% | 87% | 98% | -8.44% | -12.20% | -12.72% | #DIV/0! | #DIV/0! |
| Class E | 1.11% | 4.73% | 2.84% | 88% | 97% | -9.50% | -15.26% | -15.52% | #DIV/0! | #DIV/0! |
| Overall | 1.31% | 4.07% | 3.76% | 87% | 97% | -8.71% | -16.17% | -17.41% | #DIV/0! | #DIV/0! |
| Class D | 0.79% | 1.79% | 1.75% | 87% | 98% | -6.77% | -9.91% | -9.99% | #DIV/0! | #DIV/0! |
| Class F | 1.15% | 2.95% | 2.87% | 93% | 96% | -21.31% | -28.65% | -28.46% | #DIV/0! | #DIV/0! |
| Class TGM | 1.74% | 2.69% | 2.84% | 92% | 98% | -34.53% | -41.48% | -40.76% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 1.36% | 2.96% | 4.83% | 93% | 99% | -18.78% | -20.62% | -26.72% | #DIV/0! | #DIV/0! |
| Class A2 | 1.13% | 2.55% | 3.37% | 94% | 99% | -21.50% | -27.88% | -30.73% | #DIV/0! | #DIV/0! |
| Class B | 0.58% | 4.28% | 3.64% | 93% | 99% | -17.79% | -26.18% | -24.55% | #DIV/0! | #DIV/0! |
| Class C | 0.34% | 1.41% | 1.39% | 95% | 100% | -19.68% | -22.56% | -22.43% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 0.78% | 2.91% | 3.22% | 94% | 99% | -19.23% | -24.45% | -25.65% | #DIV/0! | #DIV/0! |
| Class D | 0.35% | 2.04% | 1.32% | 96% | 100% | -20.47% | -23.43% | -24.25% | #DIV/0! | #DIV/0! |
| Class F | 0.61% | 1.96% | 1.60% | 97% | 100% | -24.50% | -31.39% | -31.66% | #DIV/0! | #DIV/0! |
| Class TGM | 0.53% | 1.09% | 1.02% | 94% | 100% | -32.93% | -37.99% | -38.25% | #DIV/0! | #DIV/0! |

    - Group 5 off

Group 5 includes all coding tools which are not in group 1-4. Unfortunately, many tools in this group cannot be switched off with cfg options. Considering these limitations, the tools off test of group 5 was not performed.

    - Group 1-4 off

This test was not discussed during the AHG interim meeting. As no progress was achieved for group 5 off test, this test was suggested.

The command line setting of this test is: --EnableTMTools=0 --LIC=0 --CIIPTM=0 --CIIPTIMD=0 --NumSignPred=0 --IBCLIC=0 --BvdPred=0 --IntraDBV=0 --BvpCluster=0 --IntraTMP=0 --TIMD=0 --SGPM=0 --DIMD=0 --CCCM=0

Note that the control of TM IBC and TMRL are still missing. The tests were performed with these two tools on.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 2.72% | 5.93% | 7.43% | 61% | 81% | -6.55% | -14.21% | -19.69% | #DIV/0! | #DIV/0! |
| Class A2 | 4.20% | 6.34% | 6.84% | 58% | 75% | -9.95% | -17.91% | -20.96% | #DIV/0! | #DIV/0! |
| Class B | 2.83% | 6.57% | 5.78% | 58% | 70% | -5.71% | -16.93% | -15.57% | #DIV/0! | #DIV/0! |
| Class C | 2.50% | 3.63% | 3.52% | 56% | 64% | -6.97% | -10.97% | -11.38% | #DIV/0! | #DIV/0! |
| Class E | 4.13% | 7.65% | 5.95% | 59% | 70% | -6.79% | -12.87% | -12.95% | #DIV/0! | #DIV/0! |
| Overall | 3.18% | 5.95% | 5.76% | 58% | 71% | -7.02% | -14.64% | -15.79% | #DIV/0! | #DIV/0! |
| Class D | 2.15% | 3.11% | 3.11% | 56% | 60% | -5.52% | -8.74% | -8.79% | #DIV/0! | #DIV/0! |
| Class F | 5.85% | 7.74% | 7.60% | 76% | 75% | -17.79% | -25.61% | -25.40% | #DIV/0! | #DIV/0! |
| Class TGM | 9.12% | 10.07% | 10.93% | 76% | 81% | -29.90% | -37.36% | -36.20% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 7.85% | 9.43% | 12.35% | 69% | 81% | -13.60% | -15.79% | -21.40% | #DIV/0! | #DIV/0! |
| Class A2 | 7.55% | 9.08% | 10.44% | 68% | 76% | -16.51% | -23.31% | -26.02% | #DIV/0! | #DIV/0! |
| Class B | 6.31% | 9.83% | 9.65% | 66% | 74% | -13.07% | -22.23% | -20.12% | #DIV/0! | #DIV/0! |
| Class C | 6.11% | 7.38% | 7.61% | 62% | 65% | -15.02% | -17.98% | -17.62% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 6.81% | 8.95% | 9.80% | 66% | 73% | -14.39% | -20.03% | -20.89% | #DIV/0! | #DIV/0! |
| Class D | 4.89% | 6.34% | 6.41% | 62% | 61% | -16.80% | -20.21% | -20.31% | #DIV/0! | #DIV/0! |
| Class F | 7.67% | 8.82% | 8.71% | 76% | 78% | -19.25% | -26.86% | -26.97% | #DIV/0! | #DIV/0! |
| Class TGM | 9.51% | 10.19% | 10.44% | 77% | 82% | -26.93% | -32.44% | -32.52% | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-8.1 |  |  |  |  | Over VTM-11ecm8.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 6.25% | 7.49% | 8.35% | 60% | 63% | -11.10% | -25.86% | -23.62% | #DIV/0! | #DIV/0! |
| Class C | 6.11% | 6.82% | 7.45% | 58% | 53% | -12.65% | -21.18% | -20.70% | #DIV/0! | #DIV/0! |
| Class E | 5.76% | 10.68% | 8.79% | 63% | 73% | -9.76% | -16.83% | -17.10% | #DIV/0! | #DIV/0! |
| Overall (Ref) | 6.08% | 8.06% | 8.16% | 60% | 62% | -11.29% | -22.04% | -21.02% | #DIV/0! | #DIV/0! |
| Class D | 4.71% | 5.31% | 7.21% | 58% | 52% | -15.99% | -22.98% | -21.93% | #DIV/0! | #DIV/0! |
| Class F | 7.48% | 8.07% | 7.73% | 73% | 70% | -16.24% | -25.50% | -24.25% | #DIV/0! | #DIV/0! |
| Class TGM | 10.15% | 10.71% | 11.08% | 79% | 75% | -25.27% | -31.58% | -31.10% | #DIV/0! | #DIV/0! |

    - Result summary

The average results compared to ECM-8.1are summarized in the table below. Note that SCC results are not included.

| AI | PRSNY | PSNRU | PSNRV | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Off1 | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Off2 | 0.47% | 0.61% | 0.68% | 83% | 89% |
| Off3 | 1.09% | 1.01% | 1.09% | 84% | 86% |
| Off4 | 1.31% | 4.07% | 3.76% | 87% | 97% |
| Test1 (off1-4) | 3.18% | 5.95% | 5.76% | 58% | 71% |
| RA | PRSNY | PSNRU | PSNRV | EncT | DecT |
| Off1 | 3.70% | 3.98% | 4.18% | 85% | 77% |
| Off2 | 1.55% | 1.01% | 1.32% | 88% | 96% |
| Off3 | 0.47% | 0.28% | 0.57% | 96% | 99% |
| Off4 | 0.78% | 2.91% | 3.22% | 94% | 99% |
| Test1 (off1-4) | 6.81% | 8.95% | 9.80% | 66% | 73% |
| LDB | PRSNY | PSNRU | PSNRV | EncT | DecT |
| Off1 | 4.29% | 5.07% | 5.30% | 77% | 68% |
| Off2 | 1.29% | 1.21% | 1.00% | 86% | 95% |
| Test1 (off1-4) | 6.08% | 8.06% | 8.16% | 60% | 62% |

    - Open Questions

The following questions were raised during AHG teleconference and testing.

- How to further improve the methodology of tool grouping and assessment?

- Should cfg controls for tools in group 5 be added? If not, how to assess tools in group 5?

- Should the visual quality of group off tests be evaluated?

    - Input contributions

| JVET-AD0041 | m62485 | 2023-02-27 18:21:13 | 2023-02-27 22:19:31 | 2023-02-27 22:19:31 | AHG7: Report on AHG meeting on ECM tool assessment | X. Li |
| --- | --- | --- | --- | --- | --- | --- |
| JVET-AD0082 | m62708 | 2023-04-14 05:52:30 | 2023-04-14 19:59:34 | 2023-04-14 19:59:34 | [AHG7] A configuration option for TMRL mode | L. Xu, J. Gan, Y. Yu, H. Yu, D. Wang(OPPO) |
| JVET-AD0131 | m62775 | 2023-04-14 12:37:51 | 2023-04-14 21:48:52 | 2023-04-14 21:48:52 | AhG 7: Coding efficiency/runtime tradeoff metrics | P. Onno, G. Laroche (Canon) |
| JVET-AD0229 | m62914 | 2023-04-15 00:15:43 | 2023-04-15 00:19:30 | 2023-04-17 07:34:52 | AHG4/AHG12/AHG7: Mixed-Content Sequences for ECM CTC or Tool Assessment | X. Li (Google), Z. Deng, K. Zhang, L. Zhang (Bytedance) |
| JVET-AD0246 | m62973 | 2023-04-17 02:10:57 | 2023-04-17 03:19:18 | 2023-04-17 03:19:18 | [AHG7] ECM tool assessment with HDR dataset | Z. Fan, Y. Yausig, T. Chujoh, T. Ikai (Sharp) |

    - Recommendations

- Continue and improve tool assessment

- Given the existing and potential software issues related to tool assessment, it is recommended to add a new mandate “Coordinate with AHG6 on resolving tool assessment related ECM software issues”

- Discuss the open questions

- Review all the input contributions

JVET-AD0008 JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) [C. Hollmann, S. Liu, S. Wang, M. Zhou (AHG chairs)]

  - Common Test Conditions

Common test conditions (CTC) for optimization of encoders and receiving systems for machine analysis of coded video content, are summarized in output document JVET-AC2031. This document includes detailed descriptions of test datasets, anchor software and configurations, anchor generation processes, machine task networks used, test and training conditions, evaluation methodologies and metrics. A reporting template in Excel format is enclosed in the document package. The first version of this output document was uploaded on 2023-02-02, followed by a couple of revisions, the final verison includes crosschecked CTC results.

    - Test datasets

Two video datasets, referred as SFU-HW and TVD, are included in the CTC and results on both are expected to be reported. In addition, three image datasets, referred as TVD (image), OpenImageV6 and FLIR, are included in the CTC and can be tested using intra-only configuration. The results on the image datasets are optional.

It was decided in the last JVET meeting to remove four sequences from the originally proposed SFU-HW (SFU-HW-objects-v1) dataset, including one class B sequence (Kimono) and three class E sequences, and thus the SFU-HW dataset now consist of 13 test video sequences, as shown in Table 1. The sequences can be found on ftp://hevc@mpeg.tnt.uni-hannover.de. The annotations are available at https://data.mendeley.com/datasets/hwm673bv4m/1.

Table 1. Test sequences in SFU-HW dataset

| Class | Sequence name | Frame count | Frame rate | Bit depth | Frames skipped | Frames coded | Intra | RA | LD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Traffic | 150 | 30 | 8 | 117 | 33 | M | M | M |
| B | ParkScene | 240 | 24 | 8 | 207 | 33 | M | M | M |
| B | Cactus | 500 | 50 | 8 | 403 | 97 | M | M | M |
| B | BasketballDrive | 500 | 50 | 8 | 403 | 97 | M | M | M |
| B | BQTerrace | 600 | 60 | 8 | 471 | 129 | M | M | M |
| C | RaceHorses | 300 | 30 | 8 | 235 | 65 | M | M | M |
| C | BQMall | 600 | 60 | 8 | 471 | 129 | M | M | M |
| C | PartyScene | 500 | 50 | 8 | 403 | 97 | M | M | M |
| C | BasketballDrill | 500 | 50 | 8 | 403 | 97 | M | M | M |
| D | RaceHorses | 300 | 30 | 8 | 235 | 65 | M | M | M |
| D | BQSquare | 600 | 60 | 8 | 471 | 129 | M | M | M |
| D | BlowingBubbles | 500 | 50 | 8 | 403 | 97 | M | M | M |
| D | BasketballPass | 500 | 50 | 8 | 403 | 97 | M | M | M |

Note: M – mandatory; O – optional.

It was decided in the last JVET meeting to split the originally proposed long sequences in Tencent Video Dataset (TVD) into multiple short clips. With some investigations, experiments and validations, seven clips are extracted from the originally proposed TVD sequences, as shown in Table 2. The dataset with corresponding annotations is available at https://multimedia.tencent.com/resources/tvd.

Table 2. Test sequences in TVD

| Sequence name | Frame count | Frame rate | Bit depth | Frames skipped | Frames coded | Intra | Random access | Low delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TVD-01_1 | 3000 | 50 | 8 | 1500 | 500 | M | M | M |
| TVD-01_2 | 3000 | 50 | 8 | 2000 | 500 | M | M | M |
| TVD-01_3 | 3000 | 50 | 8 | 2500 | 500 | M | M | M |
| TVD-02_1 | 636 | 50 | 8 | 0 | 636 | M | M | M |
| TVD-03_1 | 2334 | 50 | 8 | 0 | 500 | M | M | M |
| TVD-03_2 | 2334 | 50 | 8 | 500 | 500 | M | M | M |
| TVD-03_3 | 2334 | 50 | 8 | 1000 | 500 | M | M | M |

The original sequences are available in .mp4 format and shall be converted to YUV420 using FFmpeg:

ffmpeg -i {input.mp4} -f rawvideo -pix_fmt yuv420p -dst_range 1 {output.yuv}

The TVD (image) dataset is an image dataset of 166 images of 1920x1080 resolution that have annotations for object detection and instance segmentation. The dataset with corresponding annotations is available at https://multimedia.tencent.com/resources/tvd.

The OpenImages dataset consists of around 9 million images. A subset of the validation set of its version 6 containing 5000 images are selected for testing object detection in this activity. The dataset with corresponding annotations is available at https://storage.googleapis.com/openimages/web/index.html.

The FLIR dataset used in the WG 4 VCM activities is a dataset consisting of 300 infrared images. The images, annotations and the fine-tuned model for thermal images can be found on the MPEG content repository (https://content.mpeg.expert/data/).

More detailed information about these test datasets can be found in JVET-AC2031.

    - Anchor software and configuration

Version 12.0 of the VTM software is used for generating anchor results. The VTM software is available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/.

In addition, version 4.2.2 of the FFmpeg is used for the format conversion. The FFmpeg software is available at https://ffmpeg.org/releases.

Three test conditions are used for experimenting encoder and receiving system optimizations on video coding for machine consumptions, i.e., random-access, low-delay and all-intra. The default configuration files provided with the VTM software are used for anchor generation.

- “Random access” (RA): encoder_randomaccess_vtm.cfg

- “Low delay” (LD): encoder_lowdelay_vtm.cfg

- “All Intra” (AI): encoder_intra_vtm.cfg

It was decided in the last JVET meeting that temporal subsampling is disabled for “All Intra” configuration for this activity, thus the following line

TemporalSubsampleRatio : 8

in “encoder_intra_vtm.cfg” needs to be modified to

TemporalSubsampleRatio : 1

A subset of these configurations might be used for a particular experiment. However, as these test conditions are for video coding experiments, results for at least one of the random access or low delay configurations are expected to be provided.

    - Anchor generation

The anchor generation process is illustrated in Figure 1. The task network Faster R-CNN X101-FPN which is part of Detectron2 (https://github.com/facebookresearch/detectron2) is used to evaluate object detection (SFU-HW dataset and image datasets), the model is available here. The task network JDE-1088x608 which is part of the Towards Realtime MOT framework (https://github.com/Zhongdao/Towards-Realtime-MOT) is used to evaluate object tracking (TVD), the model is available here. More details about anchor generation process and these machine task networks are referred to JVET-AC2031.

Figure 1. Anchor generation pipeline

Experiments were conducted after the last JVET meeting to carefully select the QP values for future tool development, testing and evalution. The updated QP values are listed in Table 3, Table 4 and Table 5.

Table 3. QPs for sequences in SFU-HW dataset

| Class | Sequence name | Configuration | QP values |
| --- | --- | --- | --- |
| A | Traffic | RA, LD, AI | {39, 45, 48, 51, 54, 58} |
| B | ParkScene | RA, LD, AI | {32, 36, 40, 44, 48, 52} |
| B | Cactus | RA, LD, AI | {43, 46, 49, 52, 55, 58} |
| B | BasketballDrive | RA, LD, AI | {40, 43, 46, 49, 52, 55} |
| B | BQTerrace | RA, LD, AI | {40, 43, 46, 49, 52, 55} |
| C | RaceHorses | RA, LD, AI | {27, 31, 35, 39, 43, 47} |
| C | BQMall | RA, LD, AI | {27, 32, 37, 42, 47, 52} |
| C | PartyScene | RA, LD, AI | {31, 35, 39, 43, 47, 51} |
| C | BasketballDrill | RA, LD, AI | {27, 31, 35, 39, 43, 47} |
| D | RaceHorses | RA, LD, AI | {22, 26, 30, 34, 38, 42} |
| D | BQSquare | RA, LD, AI | {24, 28, 31, 34, 38, 41} |
| D | BlowingBubbles | RA, LD, AI | {27, 31, 34, 37, 40, 43} |
| D | BasketballPass | RA, LD, AI | {22, 26, 30, 34, 38, 42} |

Table 4. QPs for sequences in TVD

| Sequence name | Configuration | QP values |
| --- | --- | --- |
| TVD-01_1 | RA, LD, AI | {20, 23, 26, 29, 32, 35} |
| TVD-01_2 | RA, LD, AI | {21, 24, 28, 31, 34, 37} |
| TVD-01_3 | RA, LD, AI | {23, 26, 30, 34, 38, 42} |
| TVD-02_1 | RA, LD, AI | {26, 32, 37, 42, 48, 53} |
| TVD-03_1 | RA, LD, AI | {34, 39, 42, 46, 50, 54} |
| TVD-03_2 | RA, LD, AI | {28, 32, 36, 40, 43, 49} |
| TVD-03_3 | RA, LD, AI | {22, 27, 32, 37, 42, 47} |

Table 5. QPs for image datasets

| Dataset | Configuration | QP values |
| --- | --- | --- |
| OpenImageV6 | AI | {22, 27, 32, 37, 42, 47} |
| FLIR (IR) | AI | {22, 27, 32, 37, 42, 47} |
| TVD-I | AI | {22, 27, 32, 37, 42, 47} |

Anchor results were generated following the CTC described in JVET-AC2031 and crosschecked by Ericsson and Tencent.

    - Evaluation and reporting

Proposed technologies are evaluated based on their compression performance, measured by bitrate, PSNR, mAP and MOTA, as well as encoding and decoding runtime to reflect the complexity of the proposed technology, to some extent. Definitions and detailed descriptions of these metrics can be found in JVET-AC2031.

It is noted that the mean Average Precision (mAP) is used to measure object detection performance, and Multiple Object Tracking Accuracy (MOTA) is used to measure object tracking performance. For the purpose of reporting encoding and decoding running times, the anchor and proposal should be simulated on the same platform, e.g. similar CPU and GPU configuration, to have reliable time comparison. Parallel encoding and decoding as described in JVET-B0036 may be applied for RA configurations.

In addition, relevant inference and training information should be reported if the proposed technology consists of learning-based components, such as network structure, number of network parameters, precision of network parameters, number of multiply–accumulate operations (MAC) per pixel, patch size, batch size, epoch, training time, training datasets, lost function, number of iterations, optimizer, and any pre- and post-processings used. More details can be found in JVET-AC2031.

A reporting template in Excel file format has been prepared to illustrate results following CTC and evaluation methodology described in JVET-AC2031. It is enclosed in JVET-AC2031 package. Some cleanups of the reporting template are suggested in input contribution JVET-AD0043.

Figure 2. Reporting template summary

  - Technical Report

The draft 1 of the technical report (TR) has been prepared and uploaded as JVET-AC2030 based on discussions in the last JVET meeting on 2023-03-20. An input contribution JVET-AD0042 was submitted to suggest additional content to be discussed in this meeting. Another input contribution JVET-AD0135 was submitted to provide additional details on RoI-based adaptive QP for the TR.

  - Git Management

Following the decision in the last JVET meeting, a git repository was set up for software development, experiments, document management and other activities that are related to this AHG, at https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm. This repository contains two projects, one containing instrucitons and information such as scripts for conducting experiements and evaluation, together with anchor results, while the other one containing implementation examples.

Two software example packages have been uploaded to the git, which can be found in the two branches under project https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/vtm-ofm:

- JVET-AB0275: a region of interest-based method that uses adaptive QP to reduce the quality in background areas

- JVET-AC0086: a method that uses a pre-analysis to perform content adaptive machine vision oriented preprocessing

  - AHG Coordination

Intensive and collabrative discussions were held between this AHG and the VCM AHG in WG4 to synchronize common test conditions including test datasets, anchor QP points, encoder configurations, etc. By now the common test conditions of this AHG and the VTM branch of WG4 VCM AHG are largely aligned.

- Input contributions

There are 15 input contriubtions related to AHG 8 mandates. They are listed below.

| Report |  |  |
| --- | --- | --- |
| JVET-AD0008 | JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) | C. Hollmann, S. Liu, S. Wang, M. Zhou (AHG chairs) |
| Proposal |  |  |
| JVET-AD0042 | [AHG8] Proposed text for optimization of encoders and receiving systems for machine analysis of coded video content | J. Chen (Alibaba), C. Hollmann (Ericsson), S. Liu (Tencent) |
| JVET-AD0043 | [AHG8] Clean-ups for the reporting template | C. Hollmann (Ericsson) |
| JVET-AD0047 | AHG8: A preprocessing software implementation for OFM | B. Li, S. Wang, J. Chen, Y. Ye (Alibaba), S. Wang (CityU) |
| JVET-AD0060 | AHG8/AHG9: Indications related to suboptimal user viewing | M. M. Hannuksela, F. Cricri, H. Zhang (Nokia) |
| JVET-AD0113 | AHG8: Adaptive Mask Pre-Process algorithm for Video Coding for Machines | Z. Huang, Z. Wang, C. Jia, S. Wang, S. Ma |
| JVET-AD0122 | [AHG8] Configuration changes for random access and low delay | C. Hollmann, R. Sjöberg, J. Ström (Ericsson) |
| JVET-AD0135 | [AHG8] Additional details on RoI-based adaptive QP for TR | C. Hollmann, R. Sjöberg, J. Ström (Ericsson) |
| JVET-AD0137 | [AHG8] SEI for encoder optimization information | C. Kim, Hendry, D. Gwak, J. Lim(LGE) |
| JVET-AD0138 | [AHG8] QPA with low activity threshold for machine task | C. Kim, D. Gwak, J. Lim (LGE) |
| JVET-AD0181 | AHG8: Update on TVD object tracking dataset | Z. Liu, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AD0186 | AHG8: VVC tool analysis for Machines Vision Tasks | Z. Liu, L. Wang, X. Xu, S. Liu (Tencent) |
| Information |  |  |
| JVET-AD0110 | [AHG8] Evaluating and Improving Coding Performance for General Image Classification Models | Qi Zhang, Zhao Wang, Shanshe Wang, Siwei Ma |
| Crosscheck |  |  |
| JVET-AD0252 | Cross-check of JVET-AD0181 (AHG8: Update on TVD object tracking dataset) | C. Hollmann (Ericsson) |
| JVET-AD0307 | Crosscheck of JVET-AD0122 ([AHG8] Configuration changes for random access and low delay) | Z. Liu (Tencent) |

- Recommendations

The AHG recommends:

- Review all input contributions.

- Discuss and refine test conditions, evalution and reporting procedures.

- Discuss and refine anchor generation processes and results.

- Discuss existing and continue collecting new test materials.

- Continue investigating non-normative technologies and their suitability for machine analysis applications.

- Continue developing draft technical report on optimization of encoders and receiving systems for machine analysis of coded video content.

Contributions to be reviewed in main track. Scheduling to be coordinated with WG 4 activities on VCM.

JVET-AD0009 JVET AHG report: SEI message studies (AHG9) [S. McCarthy, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, C. Fogg, Hendry, P. de Lagrange, G. J. Sullivan, A. Tourapis, S. Wenger (vice-chairs)]

- Related contributions

A total of 41 contributions are identified relating to the mandates of AHG9. Some contributions also relate to the work of other AHGs, specifically AHG8, AHG11, and AHG13.

The number of contributions relating to each AHG9 mandate is as follows

- 36 contributions relate to the mandate to study the SEI messages in VSEI, VVC, HEVC, and AVC. Some contributions relate to more than one SEI message.

  - 32 contributions relate to the neural-network post-filter SEI messages, including 1 summary of proposals on NNPF and SEI processing order SEI messages.

  - 5 contributions relate to the SEI processing order SEI message, including 1 summary of proposals on NNPF and SEI processing order SEI messages.

  - 1 contribution relates to the post filter hint SEI message.

  - 1 contribution relates to the annotated region SEI message

  - 1 contribution relates to the film grain characteristics SEI message

- 0 contributions relate to the mandate to collect software and showcase information for SEI messages;

- 6 contributions relate to the mandate to identify potential needs for additional SEI messages.

The following is a list of contributions related to the mandates of AHG9.

  - Study the SEI messages in VSEI, VVC, HEVC and AVC

JVET-AD0258 AHG9: A summary of proposals on NNPF and SEI processing order SEI messages [Y.-K. Wang (Bytedance)]

    - Neural-network post filter characteristics and activation SEI messages

      - Proposals on NNPF SEI messages

JVET-AD0052 AHG9: NNPFC SEI extension mechanism to enable usages beyond user viewing [M. M. Hannuksela, F. Cricri, H. Zhang (Nokia)]

JVET-AD0053 AHG9: On NNPFC SEI for filtering input pictures and upsampling picture rate [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

JVET-AD0054 AHG9: On NNPFC SEI for picture rate upsampling [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

JVET-AD0055 AHG9: On buffering requirements for multi-picture NNPFC SEI [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AD0056 AHG9: Miscellaneous VSEI changes on neural-network post-filter SEI messages [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

JVET-AD0057 AHG9: Miscellaneous VVC changes on SEI messages [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

JVET-AD0058 AHG9: Approaches for region-based neural-network post-filtering	 [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AD0059AHG9: Indicating processing order of NNPFs and SEI messages [M. M. Hannuksela, F. Cricri (Nokia)]

(JVET-AD0059 also relates to the SEI processing order SEI message)

JVET-AD0065 AHG9: On NNPFC Auxiliary Input Information [S. Deshpande (Sharp)]

JVET-AD0066 AHG9: On NNPFC Description and Identification [S. Deshpande (Sharp)]

JVET-AD0067 AHG9: Comments on Neural-Network Post-Filter Characteristics Message [S. Deshpande (Sharp)]

JVET-AD0078 AHG9: On comments for the draft of VSEI [T. Chujoh, Y. Yasugi, T. Ikai (Sharp)]

(JVET-AD0078 also relates to the post filter hint SEI message)

JVET-AD0079 AHG9: On the SEI processing order SEI message [P. Yin, S. McCarthy, W. Husak, K. Konstantinides, T. Lu, F. Pu, A. Arora, T. Shao (Dolby)]

(JVET-AD0078 also relates to the SEI processing order SEI message)

JVET-AD0087 AHG9: On cascading of post-processing filters [Y.-K. Wang, L. Zhang, J. Xu, C. Lin, Y. Li, J. Li, K. Zhang (Bytedance) Preparation of FDAM/FDIS text of VVC 2nd Ed.]

(JVET-AD0087 also relates to the SEI processing order SEI message)

JVET-AD0088AHG9: Overall post-processing filtering process using NNPFs [Y.-K. Wang, J. Xu, L. Zhang (Bytedance)]

JVET-AD0089 AHG9: NNPF activation parameters [Y.-K. Wang, Y. Li, J. Xu, C. Lin, J. Li, L. Zhang, K. Zhang (Bytedance)]

JVET-AD0090 AHG9: On input pictures to an NNPF [Y.-K. Wang, L. Zhang, C. Lin (Bytedance)]

JVET-AD0091 AHG9: Miscellaneous NNPF topics (set 1) [Y.-K. Wang, J. Xu, C. Lin, Y. Li, J. Li, L. Zhang, K. Zhang (Bytedance)]

JVET-AD0093 AHG9: Neural-network post-processing filter with downsampling capabilities [Y. Li, J. Xu, Y.-K. Wang, C. Lin, J. Li, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0094 AHG9: On the NNPFC SEI message involving depth pictures [C. Lin, Y.-K. Wang, J. Xu, Y. Li, J. Li, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0104 AHG9: On NNPF Overlap [A. Sidiya, S. Deshpande (Sharp)]

JVET-AD0141 AHG9: On discardable and non-output pictures for NNPF SEI messages [Hendry, J. Nam, H. Moon, S. Kim, J. Lim (LGE)]

JVET-AD0143 AHG9: On picture size for input and output pictures of NNPFC SEI message [Hendry, J. Nam, H. Moon, S. Kim, J. Lim (LGE)]

JVET-AD0145 AHG9: On non-consecutive input pictures for NNPFC SEI message [Hendry, J. Nam, H. Moon, S. Kim, J. Lim (LGE)]

JVET-AD0149 AHG9: On carriage of NNPF bitstream only in NNPFC SEI message [Hendry, J. Nam, H. Moon, S. Kim, J. Lim (LGE)

JVET-AD0151 AHG9: On output picture design in NNPFC SEI message [Hendry, J. Nam, H. Moon, S. Kim, J. Lim (LGE)]

JVET-AD0154 AHG9: Additional purposes for the NNPFC SEI message [M. Pettersson, M. Damghanian, R. Sjöberg (Ericsson)]

JVET-AD0159 AHG9: Explicit identification of NNPF input pictures [Rickard Sjöberg, Martin Pettersson, Mitra Damghanian (Ericsson)]

JVET-AD0163 AHG11: Updating NNPFC and NNPFA SEI messages for the NNPF [M. Santamaria, M. M. Hannuksela, F. Cricri, R. Yang (Nokia)]

JVET-AD0233 AHG9: Miscellaneous NNPF topics (set 2) [J. Xu, Y.-K. Wang, C. Lin, Y. Li, J. Li, L. Zhang, K. Zhang (Bytedance)]

JVET-AD0240 AHG9: Support more NNPF purposes [J. Xu, Y.-K. Wang, L. Zhang, J. Li, C. Lin (Bytedance)]

    - SEI processing order SEI message

JVET-AD0059AHG9: Indicating processing order of NNPFs and SEI messages [M. M. Hannuksela, F. Cricri (Nokia)]

(JVET-AD0059 also relates to the NNPF SEI messages)

JVET-AD0061 AHG9: On the SEI processing order SEI message [Y. He, M. Coban, M. Karczewicz (Qualcomm)]

JVET-AD0079 AHG9: On the SEI processing order SEI message [P. Yin, S. McCarthy, W. Husak, K. Konstantinides, T. Lu, F. Pu, A. Arora, T. Shao (Dolby)]

(JVET-AD0079 also relates to the NNPF SEI messages)

JVET-AD0087 AHG9: On cascading of post-processing filters [Y.-K. Wang, L. Zhang, J. Xu, C. Lin, Y. Li, J. Li, K. Zhang (Bytedance)

(JVET-AD0087 also relates to the NNPF SEI messages)

    - Post filter hint SEI message

JVET-AD0078 AHG9: On comments for the draft of VSEI [T. Chujoh, Y. Yasugi, T. Ikai (Sharp)]

(JVET-AD0078 also relates to the post filter hint SEI message)

    - Annotated regions SEI message

JVET-AD0042 [AHG8] Proposed text for optimization of encoders and receiving systems for machine analysis of coded video content [J. Chen (Alibaba), C. Hollmann (Ericsson), S. Liu (Tencent)]

(Clause 8 discusses the annotated region SEI message)

    - Film grain characteristics SEI message

JVET-AD0241 AHG13: On Film Grain Objective Metrics [X. Meng, W. Zhang, S. Labrozzi (Disney Streaming)]

  - Identify potential needs for additional SEI messages

JVET-AD0051 AHG9: Common SEI Message of Generative Face Video [B. Chen, J. Chen, Y. Ye (Alibaba), S. Wang (CityU)]

JVET-AD0060 AHG8/AHG9: Indications related to suboptimal user viewing [M. M. Hannuksela, F. Cricri, H. Zhang (Nokia)]

JVET-AD0121 AHG9: Attenuation Map Information SEI for reducing energy consumption of displays [O. Le Meur, C.H. Demarty, F. Aumont, E. François, L. Blonde, E. Reinhard (InterDigital)]

JVET-AD0137 [AHG8] SEI for encoder optimization information [C. Kim, Hendry, D. Gwak, J. Lim(LGE)]

JVET-AD0161 AHG9: Alternative Output Timing Hint SEI [H.-B. Teo, J. Gao, C.-S. Lim, K. Abe (Panasonic)]

JVET-AD0175 AHG9: On object mask auxiliary picture and object mask information SEI message for VSEI and HEVC [J. Chen, S. Wang, Y. Ye (Alibaba)]

- Activities

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de) with [AHG9] in message headers. There were no emails sent to the JVET reflector during the AHG period with [AHG9] in the message header.

- Recommendations

The AHG recommends to:

- Review all related contributions; and

- Continue SEI messages studies.

BoG (Y.-K. Wang. M. Hannuksela, S. Deshpande) to review the contributions on NNPF SEIs, and relationship with processing order SEI. The “additional SEI messages” and non-NNPF from the ongoing amendment are to be discussed in main track.

JVET-AD0010 JVET AHG report: Encoding algorithm optimization (AHG10) [P. de Lagrange, A. Duenas, R. Sjöberg, A. Tourapis (AHG chairs)]

- Related contributions

A total of 7 contributions, not including cross-checks, are identified relating to AHG10, and summarized in the following sections.

It is noted that some of the mandates of AHG8 are related to test conditions and encoder optimization, so some of the contribution relating to AHG8 can also be of interest for AHG10.

  - Visual optimization

    - JVET-AD0045 - AHG10: Encoder MV selections and DMVR revisited

This contribution describes an encoder method to decrease visual artefacts related to DMVR on sub-block boundaries, with lower BDRate impact than what was proposed in JVET-W0061. It consists in avoiding usage of DMVR when motion derivation is more likely to be unreliable and problems could propagate, e.g. in low temporal layers when reference pictures are far, and/or frame rate is low, and when artefacts are likely (spatial activity at boundaries). There are also conditions related to block size and QP. Compared to JVET-W0061, BDRate losses are reduced from 0.2% to around 0.04% (SDR CTCs) / 0.08% (HDR CTCs). In the context of GOP-based RPR and higher QPs, losses are around 0.5% compared to 2.3% for DMVR-off.

  - Bug fix

    - JVET-AD0129 - AHG10: Improvement of Input Video Padding in VTM

This contribution describes a bug fix in the VTM when the right side of the image is padded to reach a multiple of 8 or minimum CU size, before entering the encoder. This condition is not triggered by the CTCs. When tested with relevant picture sizes (cropped from the CTC sequences), impact on BDRate is significant (around -0.35% for RA).

  - Lambda adjustment (RDO)

    - JVET-AD0133 - AHG10: Lambda-QP Relationship Fix for Slice-level Multi-QP Optimization

This contribution proposes fixes to the frame lambda computation in case of slice-level QP RDO. 	The lambda is derived from m_vdRdPicLambda instead of a (potentially inconsistent) formula, and is made independent from the QP sweep. Also a conflict with CU results cache is solved by disabling the cache in that case. BDRate impact as high as -10% is reported in random access.

    - JVET-AD0136 - AhG10: Lagrange multiplier optimization for chroma ALF and CCALF

This contribution proposes to adjust the lambda for the RDO of chroma ALF when luma ALF coefficients are to be transmitted, to account for a reduced overhead of the combined signalling, and also encourage combined signalling (resulting in more chroma filter diversity). Chroma gains are reported.

  - Low-delay, GDR

    - JVET-AD0206 - AHG 12: CABAC Initialization for GDR Pictures

This contribution reports that inter-picture CABAC state inheritance as implemented in ECM8.0 is currently incompatible with GDR as it can cause mismatch at recovery point. It is proposed to enable that feature by always forcing CABAC initialization for GDR pictures. BDRate difference of around -0.5% is reported over TempCabacInit=0 (in low delay configuration with GDR enabled).

  - Local QP adaptation

    - JVET-AD0138 - [AHG8] QPA with low activity threshold for machine task

This contribution proposes a method to prevent the local QP adaptation to lower the QP too much on smooth area, by applying a low-activity threshold. This type of technique is generally useful, even outside of machine analysis tasks, for example to avoid catching noise in intra picture, causing intra pumping or “dirty window” effect.

  - Adaptive resolution

    - JVET-AD0169 - AHG12: RPR filters for scale factors below 1.5x

This contribution proposes to:

- Change non-normative downscaling filters for scaling ratios below 1.33x to larger bandwidth filters, providing sharper downscaled input picture (beneficial to upscale RPR)

- Replace the normative downscale RPR filters in a similar way (beneficial to downscale RPR)

Reported combined BD-Rate difference with ECM-8.0 is around -9% (for 1.25x) and -13% (for 1.33x), using RPR random-access test conditions, whereas non-normative changes alone result in around -8% (for 1.25x) and -10.5% (for 1.33x), which seem to indicate that sharp downscaled pictures are of primary importance for adaptive resolution performance.

The proposed downscale filters could be used in the general case, even for the VTM and scalability cases.

- Recommendation

The AHG recommends that the related input contributions are reviewed and to further continue the study of encoding algorithm optimizations in JVET.

JVET-AD0011 JVET AHG report: Neural network-based video coding (AHG11) [E. Alshina, S. Liu, A. Segall (co-chairs), F. Galpin, J. Li, R.-L. Liao, D. Rusanovskyy, T. Shao, M. Wien, P. Wu (vice-chairs)]

  - Common Test Conditions

    - Document

The AHG released revised common test conditions as decided at the 29th meeting.  The final version was uploaded as document JVET-AC2016 on February 15, 2023.

    - Anchor Encoding

Anchors for the NN-based video coding activity made available on the Git repository used for the AHG activity: https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master.

  - EE Coordination

The AHG finalized, conducted and discussed the EE on NN based video coding.  The final version of the EE description was uploaded to the document repository on February 6, 2023.

A summary report for the EE is available at this meeting as:

| JVET-AD0022 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang, Z. Xie (EE coordinators) |
| --- | --- | --- |

  - Performance Evaluation

The performance of the NNVC-4.0 anchor compared to NNVC-3.0 is reported below.

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 102% | 107% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 102% | 106% | 100% |
| Class B | 0.01% | 0.03% | 0.05% | 0.02% | 0.06% | 0.06% | 103% | 108% | 100% |
| Class C | 0.00% | 0.00% | -0.01% | 0.00% | 0.00% | 0.00% | 102% | 108% | 100% |
| Class E | - | - | - | - | - | - | - | - | - |
| Overall | 0.00% | 0.01% | 0.01% | 0.01% | 0.02% | 0.02% | 102% | 107% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 103% | 107% | 100% |
| Class F | 0.00% | 0.01% | 0.00% | 0.00% | 0.00% | 0.00% | 103% | 108% | 100% |
| Class H | - | - | - | - | - | - | - | - | - |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | - | - | - | - | - | - | - | - | - |
| Class A2 | - | - | - | - | - | - | - | - | - |
| Class B | 0.00% | 0.05% | -0.03% | 0.01% | 0.03% | -0.01% | 102% | 107% | 100% |
| Class C | 0.03% | -0.03% | -0.01% | 0.01% | -0.04% | 0.00% | 101% | 104% | 100% |
| Class E | 0.00% | 0.00% | -0.03% | 0.00% | -0.01% | -0.02% | 103% | 108% | 100% |
| Overall | 0.01% | 0.01% | -0.02% | 0.01% | 0.00% | -0.01% | 102% | 106% | 100% |
| Class D | 0.00% | -0.06% | 0.00% | 0.00% | -0.03% | -0.03% | 101% | 105% | 100% |
| Class F | 0.02% | -0.18% | -0.08% | -0.04% | -0.27% | 0.07% | 102% | 109% | 100% |
| Class H | - | - | - | - | - | - | - | - | - |
|  | All Intra Main10 |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 106% | 106% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 106% | 106% | 100% |
| Class B | 0.00% | 0.01% | 0.02% | 0.00% | 0.01% | 0.02% | 106% | 108% | 100% |
| Class C | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 106% | 103% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 105% | 102% | 100% |
| Overall | 0.00% | 0.01% | 0.01% | 0.00% | 0.00% | 0.01% | 106% | 105% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 106% | 105% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 103% | 109% | 100% |
| Class H | - | - | - | - | - | - | - | - | - |

  - Technical Evaluation

The AHG made meaningful progress on the mandate to evaluate and quantify potential NN based video coding technologies.  A summary of AHG11 related non-EE1 contributions provided as input to the 30th meeting is provided below:

|  | Title | Common Test Conditions | Results |  |  | Training Data |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RA | LDB | AI | CTC | Additional |
| Loop Filter |  |  |  |  |  |  |  |
| JVET-AD0050 | AHG11/AHG14: Fix input QP for NN filter set #1 | Yes | Yes | Yes | No | N/A | N/A |
| JVET-AD0109 | AHG11: Neural network loop filter | No | Partial | No | No | BVI-DVC | DIV2K |
| Post Filtering |  |  |  |  |  |  |  |
| JVET-AD0163 | AHG11: Updating NNPFC and NNPFA SEI messages for the NNPF | Yes | Yes | No | No | N/A | N/A |
| Intra-Prediction |  |  |  |  |  |  |  |
| JVET-AD0212 | AhG11 : neural network-based intra prediction with reduced complexity | Yes | Yes | Yes | No | BVI-DVC, TVD, UVG | ILSVCR2012, DIV2K |
| EE Related |  |  |  |  |  |  |  |
| JVET-AD0107 | EE1-related: Simplified parameter selection for filter set #1 | Yes | Yes | Yes | No | N/A | N/A |
| JVET-AD0157 | EE1-related: Neural-network loop filters in EE1-1.1.2 with further complexity reduction | Yes | Yes | No | Yes | BVI-DVC | DIV2K |
| JVET-AD0162 | EE1-2.1-related: DRF Model without QP Input | No | Yes | No | No | BVI-DVC, TVD | DIV2K |
| JVET-AD0170 | EE1 related: Performance Improvement of AC0052 filter for RPR-based SR in RA configuration | No | Yes | No | No | BVI-DVC, TVD | DIV2K |
| JVET-AD0189 | EE1-related: A simplified NN-based RDO model for filter set #1 | No | Yes | Yes | Yes | N/A | N/A |
| JVET-AD0207 | EE1-Related: Additional results for EE1-1.3.1 and EE1-1.3.5 | Yes | Yes | No | Yes | BVI-DVC | DIV2K |
| JVET-AD0211 | EE1-Related: Combination test of EE1-1.3.5 and multi-scale component of EE1-1.6 | Yes | Yes | No | Yes | BVI-DVC | DIV2K |
| JVET-AD0237 | EE1-related: Residue input for filter set #1 | No | Yes | Yes | Yes | BVI-DVC | DIV2K |

- Input contributions

There are 52 input contributions related to the AHG mandates. Twenty-three of the contributions are part of the EE activity, while the remaining 29 contributions are related to AHG11 but not part of the EE. The list of input contributions is provided below.

  - EE and Related Input Contributions

| Reporting |  |  |
| --- | --- | --- |
| JVET-AD0022 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang, Z. Xie (EE coordinators) |
| EE Technology |  |  |
| JVET-AD0106 | EE1-1.6: In-loop filter with wide activation and large receptive field | Y. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AD0156 | EE1-1.1: Complexity Reduction on Neural-Network Loop Filter | J. N. Shingala, A. Shyam, A. Suneja, S. Badya (Ittiam), T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy (Dolby) |
| JVET-AD0160 | EE1-2.1: Deep Reference Frame Generation for Inter Prediction Enhancement | J. Jia, Y. Zhang, H. Zhu, Z. Chen (Wuhan Univ.), Z. Liu, X. Xu, S. Liu (Tencent) |
| JVET-AD0166 | EE1-1.7: Optimization of training and network for NNVC filter set 0 | R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AD0167 | EE1-1.7-related: Combination test for NNVC based on filter set 0 | R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AD0168 | EE1-1.4 Channel redistribution for luma and chroma | P. Wennersten, J. Ström, D. Liu (Ericsson) |
| JVET-AD0205 | EE1-1.3: Reduced complexity CNN-based in-loop filtering | S. Eadie, Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm) |
| JVET-AD0226 | EE1-1.5: Ablation Study and Minor Improvements on RTNN | H. Zhang, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO) |
| EE Related |  |  |
| JVET-AD0107 | EE1-related: Simplified parameter selection for filter set #1 | Y. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AD0157 | EE1-related: Neural-network loop filters in EE1-1.1.2 with further complexity reduction | T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy (Dolby), J. N. Shingala, A. Shyam, A. Suneja, S. Badya (Ittiam) |
| JVET-AD0162 | EE1-2.1-related: DRF Model without QP Input | J. Jia, D. Ding, W. Meng, Z. Chen (Wuhan Univ.), Z. Liu, X. Xu, S. Liu (Tencent) |
| JVET-AD0170 | EE1 related: Performance Improvement of AC0052 filter for RPR-based SR in RA configuration | S. Huang, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO) |
| JVET-AD0189 | EE1-related: A simplified NN-based RDO model for filter set #1 | Y. Li, J. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AD0207 | EE1-Related: Additional results for EE1-1.3.1 and EE1-1.3.5 | Y. Li, S. Eadie, D. Rusanovskyy, M. Karczewicz (Qualcomm) |
| JVET-AD0211 | EE1-Related: Combination test of EE1-1.3.5 and multi-scale component of EE1-1.6 | Y.Li, S.Eadie, D.Rusanovskyy, M.Karczewicz (Qualcomm) |
| JVET-AD0237 | EE1-related: Residue input for filter set #1 | Y. Li, K. Zhang, L. Zhang (Bytedance) |
| Cross Checks |  |  |
| JVET-AD0248 | Cross-check of JVET-AD0226 (EE1-1.5: Ablation Study and Minor Improvements on RTNN) | M. Santamaria (Nokia) |
| JVET-AD0250 | Crosscheck of JVET-AD0156 (EE1-1.1: Complexity Reduction on Neural-Network Loop Filter) | J. Ström, M. Damghanian (Ericsson) |
| JVET-AD0253 | Crosscheck of JVET-AD0162 (EE1-2.1: Deep Reference Frame Generation for Inter Prediction Enhancement) | N. GIULIANI |
| JVET-AD0270 | Crosscheck of JVET-AD0106 (EE1-1.6: In-loop filter with wide activation and large receptive field) | C. Zhou (vivo) |
| JVET-AD0282 | Crosscheck of EE1-1.6: In-loop filter with wide activation and large receptive field (JVET-AD0106) | Y. Li (Qualcomm) |
| JVET-AD0284 | Crosscheck of JVET-AD0205 (EE1-1.3: Reduced complexity CNN-based in-loop filtering) | Y. Li (Bytedance) |

  - Non-EE Input Contributions

| Reporting |  |  |
| --- | --- | --- |
| JVET-AD0011 | JVET AHG report: Neural network-based video coding (AHG11) | E. Alshina, S. Liu, A. Segall (co chairs), F. Galpin, J. Li, R.-L. Liao, D. Rusanovskyy, T. Shao, M. Wien, P. Wu (vice chairs) |
| Loop Filtering |  |  |
| JVET-AD0050 | AHG11/AHG14: Fix input QP for NN filter set #1 | D. Liu, J. Ström, K. Andersson (Ericsson) |
| JVET-AD0109 | AHG11: Neural network loop filter | R. Yang, M. Santamaria, N. Zou, F. Cricri, R. G. Youvalari, J. Lainema, H. Zhang, M. M. Hannuksela (Nokia) |
| Post Filtering |  |  |
| JVET-AD0163 | AHG11: Updating NNPFC and NNPFA SEI messages for the NNPF | M. Santamaria, M. M. Hannuksela, F. Cricri, R. Yang (Nokia) |
| JVET-AD0052 | AHG9: NNPFC SEI extension mechanism to enable usages beyond user viewing | M. M. Hannuksela, F. Cricri, H. Zhang (Nokia) |
| JVET-AD0053 | AHG9: On NNPFC SEI for filtering input pictures and upsampling picture rate | M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia) |
| JVET-AD0054 | AHG9: On NNPFC SEI for picture rate upsampling | M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia) |
| JVET-AD0055 | AHG9: On buffering requirements for multi-picture NNPFC SEI | M. M. Hannuksela, F. Cricri (Nokia) |
| JVET-AD0059 | AHG9: Indicating processing order of NNPFs and SEI messages | M. M. Hannuksela, F. Cricri (Nokia) |
| JVET-AD0065 | AHG9: On NNPFC Auxiliary Input Information | S. Deshpande (Sharp) |
| JVET-AD0066 | AHG9: On NNPFC Description and Identification | S. Deshpande (Sharp) |
| JVET-AD0088 | AHG9: Overall post-processing filtering process using NNPFs | Y.-K. Wang, J. Xu, L. Zhang (Bytedance) |
| JVET-AD0089 | AHG9: NNPF activation parameters | Y.-K. Wang, Y. Li, J. Xu, C. Lin, J. Li, L. Zhang, K. Zhang (Bytedance) |
| JVET-AD0090 | AHG9: On input pictures to an NNPF | Y.-K. Wang, L. Zhang, C. Lin (Bytedance) |
| JVET-AD0091 | AHG9: Miscellaneous NNPF topics (set 1) | Y.-K. Wang, J. Xu, C. Lin, Y. Li, J. Li, L. Zhang, K. Zhang (Bytedance) |
| JVET-AD0094 | AHG9: On the NNPFC SEI message involving depth pictures | C. Lin, Y.-K. Wang, J. Xu, Y. Li, J. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AD0104 | AHG9: On NNPF Overlap | A. Sidiya, S. Deshpande (Sharp) |
| JVET-AD0141 | AHG9: On discardable and non-output pictures for NNPF SEI messages | Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE) |
| JVET-AD0143 | AHG9: On picture size for input and output pictures of NNPFC SEI message | Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE) |
| JVET-AD0145 | AHG9: On non-consecutive input pictures for NNPFC SEI message | Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE) |
| JVET-AD0149 | AHG9: On carriage of NNPF bitstream only in NNPFC SEI message | Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE) |
| JVET-AD0151 | AHG9: On output picture design in NNPFC SEI message | Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE) |
| JVET-AD0154 | AHG9: Additional purposes for the NNPFC SEI message | M. Pettersson, M. Damghanian, R. Sjöberg (Ericsson) |
| JVET-AD0159 | AHG9: Explicit identification of NNPF input pictures | Rickard Sjöberg, Martin Pettersson, Mitra Damghanian (Ericsson) |
| JVET-AD0233 | AHG9: Miscellaneous NNPF topics (set 2) | J. Xu, Y.-K. Wang, C. Lin, Y. Li, J. Li, L. Zhang, K. Zhang (Bytedance) |
| JVET-AD0240 | AHG9: Support more NNPF purposes | J. Xu, Y.-K. Wang, L. Zhang, J. Li, C. Lin (Bytedance) |
| JVET-AD0258 | AHG9: A summary of proposals on NNPF and SEI processing order SEI messages | Y.-K. Wang (Bytedance) |
| Intra Prediction |  |  |
| JVET-AD0212 | AhG11 : neural network-based intra prediction with reduced complexity | T. Dumas, F. Galpin, P. Bordes (Interdigital) |
| Cross Checks |  |  |
| JVET-AD0314 | Crosscheck of JVET-AD0163 (AHG11: Updating NNPFC and NNPFA SEI messages for the NNPF) | H. Zhang, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO) |

- Recommendations

The AHG recommends:

- Review all input contributions.

- Continue investigating neural network-based video coding tools, including coding performance and complexity.

JVET-AD0012 JVET AHG report: Enhanced compression beyond VVC capability (AHG12) [M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, H. Yang (vice-chairs)]

JVET-AD0146, "EE2-releated: EE2-1.15a improvement", S. Peng, X. Zhang, C. Fang, D. Jiang, J.-C. Lin, K.Fu, P.Zhang (Dahua)

JVET-AD0148, "Non-EE2: SGPM combined with multiple IntraTMP predictors", C. Fang, S. Peng, D. Jiang, J.-C. Lin, X. Zhang, H.Jin, X.-M.Shi, F.Ye (Dahua)

JVET-AD0150, "Non-EE2: combination of JVET-AD0146 and JVET-AD0148", C. Fang, S. Peng, X. Zhang, D. Jiang, J.-C. Lin, H.Jin, X.-M.Shi, K.Fu, P.Zhang, F.Ye (Dahua)

JVET-AD0155, "Non-EE2: High-Accuracy template matching", Y. Wang, K. Zhang, L. Zhang (Bytedance)

JVET-AD0178, "non EE2-4.3: IntraTMP Padding", K. Naser, T. Poirier, A. Robert, H. Guermoud (InterDigital)

JVET-AD0179, "[AHG12] Extending TIMD with IntraTMP", K. Naser, P. Bordes, K. Reuze, F. Galpin (InterDigital)

JVET-AD0184, "Non-EE2: Removal of Division Operations", W. Jia, K. Zhang, Z. Deng, L. Zhang (Bytedance)

JVET-AD0203, "EE2-related: Reconstruction-Reordered TMP based on lineal filter model for screen content coding (RRTMP-LFM)", D. Ruiz Coll, O. Patino, P. Andrivon, (Ofinno)

JVET-AD0214, "Non-EE2: On search region of intra template matching in ECM8.0", X. Xiu, N. Yan, C. Ma, H.-J. Jhu, C.-W. Kuo, W. Chen, X. Wang (Kwai)

JVET-AD0239, "Non-EE2: Fixes for IntraTMP template availability", G. Verba, Z. Zhang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AD0308, "Non-EE2: Extension of unified PDPC", Gagan Rath, Thierry Dumas, Fabrice Le Leannec, Kevin Reuze (InterDigital)

  - IntraBC (9)

JVET-AD0127, "Non-EE2: Harmonization between IBC HMVP and IBC-LIC", N. Zhang, K. Zhang, L. Zhang (Bytedance)

JVET-AD0134, "Non-EE2: Bi-predictive IBC for natural and screen content", Y. Kidani, H. Kato, K. Kawamura (KDDI)

JVET-AD0083, "Non-EE2: Improvement over IBC-LIC", Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AD0215, "Non-EE2: Extension of IBC-GPM", C. Ma, X. Xiu, W. Chen, H.-J. Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai)

JVET-AD0216, "Non-EE2: IBC with non-adjacent spatial candidates", C. Ma, X. Xiu, W. Chen, H.-J. Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai)

JVET-AD0217, "Non-EE2: Filtered Intra Block Copy (FIBC)", H.-J. Jhu, X. Xiu, C.-W. Kuo, W. Chen, N. Yan, C. Ma, X. Wang (Kwai)

JVET-AD0223, "Non-EE2: Filtering for IBC predicted block", B. Ray, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AD0224, "AHG12: IBC Merge list construction modification based on the candidates' Reconstruction-Reordered type", D. Ruiz Coll, O. Patino, (Ofinno)

JVET-AD0231, "Non-EE2: Non-adjacent spatial candidates for IBC", Y. Wang, K. Zhang, L. Zhang (Bytedance)

  - CCCM (4)

JVET-AD0048, "EE2-related: Cross-component merge mode with temporal candidates", H.-Y. Tseng, C.-M. Tsai, C.-Y. Chuang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

JVET-AD0080, "Non-EE2: Chroma-to-chroma convolutional cross-component model (C2C-CCCM) for intra prediction", Y. Huo, Y. Liu (Transsion)

JVET-AD0100, "AHG12: Block vector guided CCCM", R. G. Youvalari, D. Bugdayci Sansli, P. Astola, J. Lainema (Nokia)

JVET-AD0108, "AHG12: Cross-component residual model (CCRM) for inter prediction”, P. Astola, J. Lainema (Nokia)

  - Inter (8)

JVET-AD0064, "AHG12: Enhanced geometrical prediction mode", R. Yu, P. Wennersten, J. Enhorn, K. Andersson (Ericsson)

JVET-AD0114, "Non-EE2: Modification of MV redundancy/similarity check considering LIC during merge candidate list construction", K. Kim, D. Kim, J.-H. Son, J.-S. Kwak (WILUS)

JVET-AD0174, "Non-EE2: CIIP with more combination candidates",	C. Zhou, Z. Lv, J. Zhang (vivo)

JVET-AD0176, "Non-EE2: Improvements on multi-pass DMVR", J. Chen, R.-L. Liao, X. Li, Y. Ye (Alibaba)

JVET-AD0180, "Non-EE2: Affine AMVP mode with one MVD", H. Huang, V. Seregin, Z. Zhang, M. Karczewicz (Qualcomm)

JVET-AD0183, "Non-EE2: SIF flag and BCW weight usage in OBMC", A. Robert, F. Galpin, T. Poirier, Y. Chen (InterDigital)

JVET-AD0196, "EE2-related: On Iterative BDOF and EE2-2.6 Improvement", M. Salehifar, Y. He, K. Zhang, H. Liu, L. Zhang (Bytedance)

JVET-AD0235, "Non-EE2: Enhanced subblock-based motion compensation", L. Zhao, K. Zhang, L. Zhang (Bytedance)

  - RPR (1)

JVET-AD0169, "AHG12: RPR filters for scale factors below 1.5x", J. Samuelsson-Allendes, S. Deshpande (Sharp)

  - Partitioning (2)

JVET-AD0130, "AhG12: On ECM partitioning prediction", G. Laroche, P. Onno (Canon)

JVET-AD0185, "Non-EE2: ISP Extension for Chroma", W. Jia, K. Zhang, Y. Wang, L. Zhang (Bytedance)

  - In Loop Filters (4)

JVET-AD0218, "AHG12: CCSAO with extended edge classifiers and history offsets", C.-W. Kuo, X. Xiu, W. Chen, H.-J. Jhu, N. Yan, C. Ma, X. Wang (Kwai)

JVET-AD0220, "Non-EE2: Improved fixed filters for ALF", M. Karczewicz, N. Hu, V. Seregin (Qualcomm)

JVET-AD0234, "Non-EE2: Extensions of Residual-based Taps in ALF and CCALF", W. Yin, K. Zhang, L. Zhang (Bytedance)

JVET-AD0236, "Non-EE2: On Classification of In-Loop Filters", W. Yin, K. Zhang, L. Zhang (Bytedance)

  - Transform (3)

JVET-AD0172, " AHG12: Adaptation of LFNST/NSPT for low-delay configuration", C. Bonnineau, F. Le Léannec, K. Naser, T. Poirier, S. Puri (InterDigital)

JVET-AD0187, " Non-EE2: On large NSPT", M. Koo, J. Zhao, J. Lim, S. Kim (LGE)

JVET-AD0200, " EE2-related: InterMTS for IBC and IntraTMP", P. Garus, M. Coban, B. Ray, V. Seregin, M. Karczewicz (Qualcomm)

  - Coefficients and Entropy Coding (3)

JVET-AD0204, "AHG12: Context modeling for transform coefficients for LFNST/NSPT", P. Nikitin, M. Coban, B. Ray, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AD0251, "AhG12: Shifting Quantizer Center", M. Balcilar, K. Naser, F. Galpin, F. Le Léannec (InterDigital)

JVET-AD0206, "AHG 12: CABAC Initialization for GDR Pictures", S. Hong, L. Wang, K. Panusopone (Nokia)

  - Other (5)

JVET-AD0105, "Modifications of common test condition and ECM software", Y. Yasugi, T. Ikai (Sharp)

JVET-AD0128, "Ahg12: modified CTC proposal for low-delay configurations", F. Le Léannec, F. Galpin, K. Naser, T. Poirier, E. François (InterDigital)

JVET-AD0197, "EE2-related: Worst-Case Timing Reduction for ECM RA Simulations", M. Salehifar, Y. Wang, Y. He, K. Zhang, L. Zhang (Bytedance)

JVET-AD0229, "AHG4/AHG12/AHG7: Mixed-Content Sequences for ECM CTC or Tool Assessment", X. Li (Google), Z. Deng, K. Zhang, L. Zhang (Bytedance)

JVET-AD0242, "On Common Test Conditions document", R. Chernyak, L.-F. Chen, X. Xu, S. Liu (Tencent)

- Recommendations

The AHG recommends to:

- To review all the related contributions.

JVET-AD0013 JVET AHG report: Film grain technologies (AHG13) [W. Husak, M. Radosavljević, W. Wan (co-chairs), D. Grois, Y. He, P. de Lagrange, A. Segall, A. Tourapis, W. Zhang (vice-chairs)]

- Discussion

The group focused its efforts on preparing the technical report for the issuance of the CD ballot.  The editors met on a biweekly cadence throughout the period.  The editorial process consisted of resolving many editorial issues without any major changes in the structure of the document.

Additional discussions were related to development of multiple software packages for synthesis of film grain technologies by multiple sources.  Informal subjective testing results were also discussed.  Finally, content for film grain testing was discussed with efforts underway to select content and how to prepare content.  Several companies have been approaching this topic.

- Related contributions

Four contributions related to AHG13 were identified as of 04/21/2023.  One additional document related to film grain testing was uploaded for AHG4.

- One was the AHG report:

  - JVET-AD0013 JVET AHG report: Film grain technologies (AHG13)

- Three were related to film grain testing and software development:

  - JVET-AD0238 On Film Grain Synthesis Subjective Evaluation and Further Work

  - JVET-AD0241 AHG13: On Film Grain Objective Metrics

  - JVET-AD0332 AHG13/AHG3: software development related to FGC SEI message

- One was related to AHG4 and explore film grain testing:

  - JVET-AD0276 AHG4: experiments in preparation of film grain visual tests

  - Contributions

    - JVET-AD0238 On Film Grain Synthesis Subjective Evaluation and Further Work

This contribution would like to encourage the group to prioritize the use case that tries to preserve the artistic intent, this is considered of great potential, and it is also a harder goal to achieve.

In JVET-AC0199 [3], it was claimed by some of the authors of this contribution that after subjective evaluation of the current shared tools that the current algorithm may not be optimized, which leads to significant artifacts. It was noted that the MCTF-based denoising may also not be optimum and an alternative filtering process could be developed. It was also mentioned that for heavy grain sequences, the synthesized grain is rather coarse, and there are obvious repetitive patterns. Parameters are recommended to control the granularity of synthesized film grain, and the frequency-based FGS method and auto-regressive (AR) based method shows different results under different circumstances. Better implementations of the denoiser and film grain modeling are recommended to show the effectiveness of the current FGS SEI message and film grain synthesis method.

This contribution would like  to encourage the group to further develop and make available tools and methodologies that facilitate the usage of the FGS techniques under development in JVET.

    - JVET-AD0241 AHG13: On Film Grain Objective Metrics

The subjective evaluation of film grain synthesis is recommended in the last JVET meeting. However, there are no widely used metrics for the evaluation of film grain synthesis results, which highly limits the development of film grain related technologies. This document provides the evaluation of metrics, i.e., NSS (Natural Scene Statistics), for the film grain synthesis. The main observations and conclusions are as follows,

1.	For images with relatively simple content, the NSS is highly correlated with the subjective perception of the human eye.

2.	For images with complex content and film grain characteristics, the NSS has a poor relationship with subject results.

3.	For videos, the NSS shows even worse results.

4.	NSS is not suitable to be the film grain synthesis objective metric of videos.  An improved NSS or other methods are recommended to be studied for the film grain evaluation.

    - JVET-AD0332 AHG13/AHG3: software development related to FGC SEI message

This contribution describes the contents of the newly created JVET gitlab group called “JVET AHG Film Grain Technologies”, its connections with reference software, and the plans for future developments.

    - JVET-AD0276 AHG4: experiments in preparation of film grain visual tests

This document reports about some experiments in preparation of visual tests planned in JVET-AC2022 for FGC SEI visual tests on top of VVC, more specifically the first category which goal is to assess the visual improvement brought by film grain synthesis over a wide range of coding quality. Internal testing in InterDigital labs were performed and indicate that the test planned can show a measurable impact of film grain synthesis on visual quality, which can be translated to bitrate savings.

It is further recommended to add tests at a higher quality range using a direct comparison method, since film grain synthesis at high quality is expected to have a much larger impact on bitrate than for the quality range commonly used for DCR tests.

This document will be reviewed by AHG4.

  - Recommendations

The AHG recommends:

- the related input contributions are reviewed;

- testing of FGC be discussed; and

- continue the study of film grain technologies in JVET.

JVET-AD0014 JVET AHG report: NNVC software development (AHG14) [S. Eadie, F. Galpin, Y. Li, L. Wang, Z. Xie (AHG chairs)]

- Software development

  - Location

NNVC repository is located at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM

NNVC software is based on VTM-11.0 with enabled MCTF including the update from JVET-V0056, GOP32, and enabling deblocking in the RDO.

NNVC-4.0 anchor at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM is used for NNVC performance evaluation.

  - Software changes

The following changes were integrated:

- Integration of JVET-AC0057 – SADL v4

- Integration of JVET-AC0089 – Combined intra/inter model filterset1

- Integration of JVET-AC0116 – Intra Prediction NN

- Integration of JVET-AC0177 – Flip input+Temporal filtering filterset1

- Integration of JVET-AC0194 – improve NN model/training filterset0

- Integration of JVET-AC0328 – AC0194+encoder optimization filterset0

- Integration of JVET-AC0055 – content adaptive NN post-filter

- Integration of JVET-AC0196 – GOP adaptive NN super-resolution

The following fixes:

- Fix aggressive global floating-point optimization leading to slight encoding differences on some compiler/platforms

- Correct issue with EncDbOpt from VTM

- Various fixes (20 issues resolved)

Other changes:

- The repository now uses git LFS to store the model to lower the burden on the server. In order to add/update models, git lfs should be installed locally.

- Improvement of the data loader for training

- Added configuration files to test intra/post-filters/super-resolution

  - Software version

NNVC-4.0 was tagged March 8th, 2023.

NNVC-3.0 (a.k.a VTM-11.0_nnvc3.0) was tagged December 1st 2022.

NCS-1.0 (a.k.a NNVC-3.0wip2) was tagged September 4th 2022 (first release containing the FilterSets, using NNVC 2.0 as a base).

VTM-11.0_nnvc-2.0 was tagged August 4th 2022 (add deblocking in RDO).

VTM-11.0_nnvc-1.0 was tagged May 6th 2021 (VTM-11.0 base with MCTF enabled).

- CTC performance

See configurations section for naming convention.

  - NNVC-3.0-EE1 vs NNVC-4.0-EE1

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc3.0-allNnToolsOff |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 102% | 107% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 102% | 106% |
| Class B | 0.01% | 0.03% | 0.05% | 0.02% | 0.06% | 0.06% | 103% | 108% |
| Class C | 0.00% | 0.00% | -0.01% | 0.00% | 0.00% | 0.00% | 102% | 108% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.01% | 0.01% | 0.02% | 0.02% | 102% | 107% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 103% | 107% |
| Class F | 0.00% | 0.01% | 0.00% | 0.00% | 0.00% | 0.00% | 103% | 108% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc3.0-allNnToolsOff |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0.00% | 0.05% | -0.03% | 0.01% | 0.03% | -0.01% | 102% | 107% |
| Class C | 0.03% | -0.03% | -0.01% | 0.01% | -0.04% | 0.00% | 101% | 104% |
| Class E | 0.00% | 0.00% | -0.03% | 0.00% | -0.01% | -0.02% | 103% | 108% |
| Overall | 0.01% | 0.01% | -0.02% | 0.01% | 0.00% | -0.01% | 102% | 106% |
| Class D | 0.00% | -0.06% | 0.00% | 0.00% | -0.03% | -0.03% | 101% | 105% |
| Class F | 0.02% | -0.18% | -0.08% | -0.04% | -0.27% | 0.07% | 102% | 109% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc3.0-allNnToolsOff |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 106% | 106% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 106% | 106% |
| Class B | 0.00% | 0.01% | 0.02% | 0.00% | 0.01% | 0.02% | 106% | 108% |
| Class C | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 106% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 105% | 102% |
| Overall | 0.00% | 0.01% | 0.01% | 0.00% | 0.00% | 0.01% | 106% | 105% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 106% | 105% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 103% | 109% |

  - NNVC-4.0-EE1 vs set0

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.99% | -16.19% | -20.23% | -11.08% | -19.20% | -21.59% | 160% | 51849% |
| Class A2 | -10.19% | -19.85% | -15.94% | -9.89% | -19.43% | -13.27% | 151% | 49894% |
| Class B | -9.20% | -22.08% | -21.09% | -8.70% | -22.06% | -20.31% | 153% | 51772% |
| Class C | -9.92% | -24.44% | -23.53% | -9.60% | -21.79% | -20.97% | 132% | 42818% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -9.75% | -21.09% | -20.54% | -9.65% | -20.89% | -19.34% | 148% | 48868% |
| Class D | -10.97% | -23.77% | -23.47% | -8.43% | -21.17% | -19.82% | 131% | 37770% |
| Class F | -4.62% | -14.46% | -12.77% | -4.91% | -14.84% | -13.81% | 183% | 21741% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -8.40% | -17.94% | -19.07% | -8.04% | -18.48% | -21.04% | 148% | 49113% |
| Class C | -9.55% | -19.05% | -20.70% | -9.50% | -14.07% | -17.16% | 131% | 41520% |
| Class E | -9.01% | -15.76% | -17.24% | -8.12% | -16.36% | -17.92% | 220% | 42542% |
| Overall | -8.93% | -17.76% | -19.16% | -8.55% | -16.48% | -18.97% | 157% | 44801% |
| Class D | -10.96% | -20.88% | -20.72% | -9.17% | -16.74% | -18.41% | 128% | 37255% |
| Class F | -5.13% | -12.70% | -10.43% | -6.03% | -12.04% | -14.06% | 184% | 21762% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -6.99% | -15.27% | -17.94% | -8.51% | -17.44% | -18.76% | 189% | 36450% |
| Class A2 | -6.52% | -16.95% | -14.22% | -7.30% | -16.95% | -12.21% | 146% | 29599% |
| Class B | -6.67% | -16.85% | -17.02% | -6.62% | -18.35% | -18.66% | 137% | 27698% |
| Class C | -7.77% | -17.63% | -19.66% | -7.36% | -19.35% | -20.52% | 119% | 18695% |
| Class E | -10.11% | -17.09% | -17.84% | -10.31% | -16.57% | -18.87% | 144% | 31435% |
| Overall | -7.52% | -16.82% | -17.43% | -7.83% | -17.89% | -18.05% | 143% | 27438% |
| Class D | -7.69% | -16.96% | -19.44% | -6.80% | -18.98% | -19.87% | 114% | 17342% |
| Class F | -4.71% | -13.65% | -12.65% | -4.42% | -15.60% | -14.52% | 117% | 23319% |

  - NNVC-4.0-EE1 vs set1

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -8.73% | -16.46% | -20.68% | -9.13% | -20.29% | -23.16% | 183% | 34448% |
| Class A2 | -9.89% | -20.16% | -16.07% | -9.78% | -20.93% | -14.94% | 176% | 32708% |
| Class B | -9.10% | -22.24% | -22.94% | -8.61% | -23.41% | -24.26% | 178% | 33397% |
| Class C | -10.66% | -23.19% | -24.34% | -9.58% | -21.46% | -22.22% | 154% | 29864% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -9.60% | -20.92% | -21.49% | -9.21% | -21.77% | -21.63% | 172% | 32482% |
| Class D | -12.75% | -24.00% | -27.26% | -9.05% | -23.07% | -23.30% | 147% | 26666% |
| Class F | -5.49% | -14.31% | -12.50% | -6.15% | -16.03% | -12.72% | 229% | 14355% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -7.37% | -19.78% | -20.34% | -7.06% | -22.14% | -25.79% | 173% | 31774% |
| Class C | -9.22% | -21.73% | -21.89% | -8.66% | -19.88% | -18.88% | 153% | 29343% |
| Class E | -8.59% | -17.09% | -19.69% | -8.85% | -20.21% | -23.23% | 286% | 23513% |
| Overall | -8.29% | -19.76% | -20.69% | -8.04% | -20.90% | -22.85% | 188% | 28698% |
| Class D | -10.98% | -24.75% | -27.81% | -8.61% | -23.31% | -25.79% | 147% | 25684% |
| Class F | -5.36% | -14.16% | -10.75% | -6.21% | -17.49% | -11.46% | 231% | 15907% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -6.55% | -16.45% | -18.07% | -6.39% | -18.98% | -21.02% | 174% | 24110% |
| Class A2 | -6.70% | -19.07% | -14.98% | -7.13% | -20.20% | -14.41% | 146% | 19376% |
| Class B | -6.69% | -17.69% | -21.26% | -6.67% | -20.76% | -23.39% | 140% | 18375% |
| Class C | -7.43% | -18.91% | -21.75% | -7.41% | -21.28% | -23.44% | 127% | 12432% |
| Class E | -10.01% | -19.82% | -22.18% | -10.43% | -20.16% | -24.60% | 143% | 20712% |
| Overall | -7.39% | -18.34% | -19.94% | -7.49% | -20.39% | -21.71% | 143% | 18142% |
| Class D | -7.30% | -18.83% | -22.69% | -6.54% | -22.66% | -23.70% | 123% | 11234% |
| Class F | -5.17% | -15.17% | -13.89% | -5.28% | -17.91% | -16.35% | 120% | 15540% |

  - NNVC-4.0-EE1 vs intra

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -2.32% | -0.70% | -1.55% | -2.23% | -0.29% | -1.23% | 114% | 178% |
| Class A2 | -1.45% | -0.03% | -0.76% | -1.36% | 0.35% | 0.01% | 113% | 157% |
| Class B | -1.82% | -0.23% | -0.52% | -1.75% | 0.14% | 0.47% | 115% | 178% |
| Class C | -1.69% | -0.99% | -0.98% | -1.63% | -0.04% | 0.11% | 117% | 210% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -1.81% | -0.49% | -0.90% | -1.74% | 0.05% | -0.06% | 115% | 181% |
| Class D | -1.42% | -0.92% | -1.13% | -1.50% | -0.47% | -0.18% | 115% | 216% |
| Class F | -1.11% | -0.66% | -0.59% | -1.13% | -0.01% | 0.34% | 116% | 151% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.46% | 0.29% | -0.43% | -0.37% | 0.62% | -0.41% | 107% | 134% |
| Class C | -0.33% | -0.64% | -0.25% | -0.34% | -0.70% | 0.91% | 106% | 144% |
| Class E | -0.37% | 0.22% | 0.33% | -0.49% | 0.76% | 0.49% | 102% | 105% |
| Overall | -0.39% | -0.04% | -0.18% | -0.39% | 0.21% | 0.25% | 106% | 129% |
| Class D | -0.15% | -0.74% | -0.67% | -0.12% | -0.14% | -1.52% | 106% | 138% |
| Class F | -0.27% | 0.06% | -0.19% | -0.06% | 0.25% | -0.25% | 106% | 123% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -4.31% | -3.67% | -4.03% | -4.55% | -3.57% | -3.88% | 172% | 547% |
| Class A2 | -2.82% | -2.35% | -2.77% | -2.85% | -1.95% | -2.30% | 179% | 664% |
| Class B | -3.24% | -2.79% | -3.00% | -3.09% | -2.47% | -2.37% | 179% | 712% |
| Class C | -3.20% | -2.78% | -3.01% | -3.22% | -2.26% | -2.13% | 171% | 765% |
| Class E | -4.89% | -4.58% | -3.82% | -4.94% | -4.32% | -2.39% | 175% | 740% |
| Overall | -3.61% | -3.16% | -3.27% | -3.63% | -2.83% | -2.56% | 175% | 689% |
| Class D | -3.00% | -2.68% | -2.88% | -2.94% | -2.07% | -2.22% | 164% | 844% |
| Class F | -2.21% | -1.77% | -1.88% | -2.11% | -1.47% | -1.57% | 141% | 443% |

  - NNVC-4.0-EE1 vs sr

Note: Tool is activated only for A1/A2 classes. Overall shows results on classes A1/A2 only.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -7.45% | -8.36% | -8.06% | -42.44% | -29.45% | -28.71% | 121% | 11654% |
| Class A2 | -3.22% | 3.20% | 3.71% | -41.43% | -24.09% | #VALUE! | 117% | 1748% |
| Overall | -5.34% | -2.58% | -2.17% | -41.94% | -26.77% | #VALUE! | 119% | 4513% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -5.54% | -7.19% | -7.10% | -41.44% | -21.28% | -20.83% | 174% | 5620% |
| Class A2 | -1.26% | -9.31% | -3.10% | -65.10% | -56.66% | -51.26% | 126% | 11038% |
| Overall | -3.40% | -8.25% | -5.10% | -53.27% | -38.97% | -36.05% | 148% | 7876% |

  - NNVC-4.0-EE1 vs pf

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -6.40% | -13.70% | -20.30% | -8.34% | -16.26% | -21.23% | 115% | 9400% |
| Class A2 | -4.22% | -18.38% | -13.60% | -3.41% | -18.19% | -12.22% | 110% | 8846% |
| Class B | -5.13% | -20.59% | -16.95% | -3.22% | -20.82% | -16.41% | 131% | 11294% |
| Class C | -3.59% | -18.92% | -15.99% | -2.38% | -17.57% | -15.72% | 121% | 9864% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -4.79% | -18.32% | -16.69% | -4.06% | -18.52% | -16.35% | 121% | 10000% |
| Class D | -3.60% | -18.30% | -17.35% | 0.24% | -16.72% | -15.51% | 122% | 10503% |
| Class F | -1.42% | -13.15% | -10.64% | -1.09% | -14.48% | -12.55% | 146% | 6831% |

  - NNVC-4.0-EE1 vs set0+intra

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -11.84% | -17.13% | -21.39% | -12.77% | -20.16% | -22.57% | 174% | 51790% |
| Class A2 | -11.35% | -20.38% | -16.60% | -11.08% | -19.78% | -13.84% | 163% | 49848% |
| Class B | -10.65% | -22.70% | -21.81% | -10.11% | -22.66% | -20.84% | 168% | 51875% |
| Class C | -11.25% | -25.45% | -24.54% | -10.80% | -22.37% | -21.24% | 149% | 42909% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -11.19% | -21.86% | -21.41% | -11.02% | -21.51% | -19.90% | 163% | 48908% |
| Class D | -12.12% | -24.67% | -24.43% | -9.78% | -21.62% | -20.35% | 146% | 37837% |
| Class F | -5.53% | -15.02% | -13.11% | -5.87% | -15.49% | -14.04% | 198% | 21659% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -8.75% | -18.13% | -19.25% | -8.41% | -18.33% | -20.99% | 155% | 49074% |
| Class C | -9.80% | -19.62% | -20.38% | -9.61% | -14.37% | -16.18% | 136% | 41467% |
| Class E | -9.11% | -15.60% | -17.14% | -8.15% | -16.35% | -17.85% | 222% | 42892% |
| Overall | -9.19% | -17.99% | -19.10% | -8.74% | -16.51% | -18.60% | 162% | 44859% |
| Class D | -11.10% | -21.47% | -20.33% | -9.42% | -17.87% | -17.62% | 134% | 37315% |
| Class F | -5.34% | -12.58% | -10.62% | -5.85% | -10.90% | -15.06% | 189% | 21713% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -10.69% | -18.44% | -21.28% | -12.47% | -21.03% | -21.84% | 269% | 37178% |
| Class A2 | -9.02% | -19.19% | -16.09% | -9.94% | -18.93% | -13.98% | 226% | 30198% |
| Class B | -9.46% | -19.53% | -19.85% | -9.38% | -20.70% | -20.98% | 217% | 28415% |
| Class C | -10.44% | -20.77% | -22.48% | -10.17% | -21.93% | -22.83% | 190% | 19401% |
| Class E | -14.08% | -21.34% | -21.59% | -14.25% | -20.64% | -22.12% | 219% | 31993% |
| Overall | -10.58% | -19.87% | -20.34% | -10.97% | -20.72% | -20.55% | 220% | 28130% |
| Class D | -10.16% | -19.66% | -22.09% | -9.36% | -21.15% | -22.07% | 178% | 18126% |
| Class F | -6.65% | -15.30% | -14.32% | -6.39% | -16.84% | -16.28% | 159% | 23705% |

  - NNVC-4.0-EE1 vs set1+intra

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -10.53% | -17.71% | -22.20% | -10.80% | -21.37% | -24.58% | 199% | 34485% |
| Class A2 | -11.01% | -20.76% | -16.93% | -10.76% | -21.58% | -15.62% | 189% | 32691% |
| Class B | -10.54% | -23.00% | -23.69% | -9.97% | -24.04% | -24.87% | 194% | 33462% |
| Class C | -11.98% | -24.24% | -25.39% | -10.82% | -22.34% | -22.75% | 171% | 30036% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -11.02% | -21.82% | -22.49% | -10.52% | -22.56% | -22.40% | 187% | 32556% |
| Class D | -13.89% | -25.07% | -28.38% | -10.22% | -23.83% | -24.33% | 162% | 26827% |
| Class F | -6.41% | -15.05% | -13.28% | -6.98% | -16.66% | -12.63% | 245% | 14516% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -7.81% | -20.15% | -21.10% | -7.52% | -22.54% | -26.45% | 180% | 31857% |
| Class C | -9.52% | -22.02% | -22.29% | -8.83% | -20.25% | -19.26% | 158% | 29405% |
| Class E | -8.68% | -17.61% | -19.52% | -9.05% | -19.64% | -23.52% | 289% | 23176% |
| Overall | -8.60% | -20.14% | -21.10% | -8.34% | -21.05% | -23.32% | 194% | 28646% |
| Class D | -11.15% | -24.97% | -27.84% | -8.85% | -23.18% | -26.47% | 153% | 25745% |
| Class F | -5.58% | -14.05% | -11.31% | -6.58% | -17.56% | -11.84% | 236% | 15845% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -10.19% | -19.78% | -21.27% | -10.20% | -22.30% | -23.92% | 253% | 24688% |
| Class A2 | -9.17% | -21.15% | -17.07% | -9.65% | -22.09% | -16.24% | 228% | 20053% |
| Class B | -9.40% | -21.05% | -23.91% | -9.31% | -23.57% | -25.56% | 223% | 19164% |
| Class C | -10.10% | -21.71% | -24.47% | -10.15% | -23.89% | -25.90% | 200% | 13194% |
| Class E | -13.88% | -23.81% | -25.97% | -14.29% | -23.68% | -28.09% | 220% | 21282% |
| Overall | -10.40% | -21.46% | -22.80% | -10.53% | -23.20% | -24.23% | 223% | 18866% |
| Class D | -9.81% | -21.40% | -25.43% | -9.03% | -24.89% | -26.43% | 188% | 12048% |
| Class F | -7.14% | -16.85% | -15.75% | -7.17% | -19.19% | -17.86% | 162% | 15994% |

  - NNVC-4.0-EE1 vs set0+ rdo+intra

Note: additional test with RDO on (default setting is RDO off when using filterset0)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -11.90% | -17.13% | -21.45% | -12.87% | -20.35% | -22.72% | 180% | 51946% |
| Class A2 | -11.61% | -20.41% | -16.79% | -11.26% | -19.72% | -13.89% | 173% | 49928% |
| Class B | -10.86% | -22.79% | -21.94% | -10.22% | -22.56% | -20.87% | 177% | 51878% |
| Class C | -11.57% | -25.21% | -24.46% | -11.13% | -22.01% | -21.35% | 158% | 42997% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -11.41% | -21.83% | -21.48% | -11.20% | -21.40% | -19.97% | 172% | 48981% |
| Class D | -12.50% | -24.63% | -24.11% | -10.14% | -21.99% | -19.58% | 156% | 37876% |
| Class F | -5.77% | -15.20% | -13.30% | -6.16% | -16.03% | -14.11% | 206% | 21764% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -8.91% | -17.75% | -19.05% | -8.56% | -17.95% | -20.59% | 166% | 49224% |
| Class C | -9.92% | -19.57% | -20.67% | -9.45% | -14.59% | -16.75% | 147% | 41492% |
| Class E | -9.26% | -15.83% | -17.16% | -8.67% | -16.16% | -17.85% | 235% | 42613% |
| Overall | -9.33% | -17.88% | -19.12% | -8.89% | -16.38% | -18.62% | 173% | 44852% |
| Class D | -11.21% | -20.83% | -21.08% | -9.34% | -16.88% | -18.38% | 145% | 37402% |
| Class F | -5.45% | -12.29% | -11.01% | -6.25% | -11.53% | -14.86% | 196% | 21714% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -11.00% | -18.60% | -21.31% | -12.77% | -21.00% | -21.63% | 295% | 37356% |
| Class A2 | -9.45% | -19.27% | -16.44% | -10.26% | -18.82% | -14.22% | 252% | 30208% |
| Class B | -9.87% | -19.48% | -19.86% | -9.85% | -20.56% | -20.88% | 243% | 28413% |
| Class C | -10.88% | -20.26% | -22.32% | -10.62% | -21.86% | -22.82% | 214% | 19486% |
| Class E | -14.53% | -21.44% | -21.72% | -14.71% | -20.43% | -22.08% | 243% | 32181% |
| Overall | -10.99% | -19.80% | -20.39% | -11.39% | -20.61% | -20.53% | 246% | 28208% |
| Class D | -10.61% | -19.09% | -21.63% | -9.79% | -20.99% | -21.63% | 201% | 18100% |
| Class F | -6.97% | -15.34% | -14.38% | -6.90% | -17.17% | -16.31% | 168% | 23743% |

  - NNVC-4.0-EE1 vs set1+rdo+intra+temporal filter

Note: additional test with temporal filter and RDO on (default setting is RDO and temporal filter off when using filterset1)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -11.31% | -17.65% | -22.12% | -11.46% | -21.38% | -24.28% | 208% | 33077% |
| Class A2 | -12.13% | -20.90% | -16.85% | -11.64% | -21.47% | -15.44% | 204% | 31897% |
| Class B | -11.46% | -23.03% | -23.91% | -10.84% | -23.98% | -24.96% | 209% | 32792% |
| Class C | -12.87% | -24.28% | -25.62% | -11.92% | -22.28% | -23.00% | 186% | 29441% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -11.94% | -21.86% | -22.59% | -11.41% | -22.50% | -22.40% | 201% | 31742% |
| Class D | -14.76% | -25.00% | -28.30% | -11.35% | -23.91% | -24.30% | 179% | 26312% |
| Class F | -6.83% | -14.99% | -13.38% | -7.47% | -16.46% | -13.30% | 256% | 13084% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -8.24% | -19.74% | -20.64% | -7.85% | -22.00% | -25.57% | 200% | 31615% |
| Class C | -10.11% | -21.74% | -21.71% | -9.29% | -19.61% | -17.74% | 177% | 29301% |
| Class E | -9.49% | -17.15% | -20.33% | -9.34% | -19.38% | -23.08% | 312% | 23441% |
| Overall | -9.18% | -19.76% | -20.92% | -8.70% | -20.55% | -22.34% | 215% | 28603% |
| Class D | -11.71% | -25.05% | -27.55% | -9.38% | -23.31% | -26.25% | 172% | 25714% |
| Class F | -6.12% | -14.10% | -10.81% | -6.93% | -16.98% | -11.77% | 249% | 15810% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -10.31% | -19.86% | -21.35% | -10.40% | -22.21% | -23.81% | 300% | 24684% |
| Class A2 | -9.42% | -21.31% | -17.38% | -9.91% | -22.25% | -16.55% | 275% | 19996% |
| Class B | -9.62% | -21.13% | -24.24% | -9.59% | -23.64% | -25.70% | 272% | 19089% |
| Class C | -10.39% | -21.75% | -24.68% | -10.34% | -23.89% | -26.04% | 246% | 13173% |
| Class E | -14.14% | -23.95% | -26.05% | -14.50% | -24.03% | -27.80% | 265% | 21236% |
| Overall | -10.63% | -21.56% | -23.02% | -10.76% | -23.29% | -24.28% | 270% | 18822% |
| Class D | -10.10% | -21.46% | -25.39% | -9.25% | -24.99% | -26.26% | 232% | 11997% |
| Class F | -7.31% | -17.10% | -15.94% | -7.55% | -19.70% | -17.83% | 180% | 15986% |

  - NNVC-4.0-EE1 vs sr+intra

Note: Tool is activated only for A1/A2 classes. Overall shows results on classes A1/A2 only.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.34% | -9.50% | -9.44% | -43.57% | -29.57% | -29.09% | 267% | 123% |
| Class A2 | -4.69% | 2.81% | 2.71% | -42.38% | -23.78% | -33.28% | 189% | 144% |
| Overall | -7.01% | -3.34% | -3.36% | -42.97% | -26.68% | -31.19% | #NUM! | #NUM! |

|  | BD-rate Over NNVC-4.0 AI |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.09% | -10.92% | -10.51% | -43.69% | -24.44% | -23.28% | 480% | 665% |
| Class A2 | -3.93% | -12.03% | -5.96% | -65.93% | -57.39% | -52.44% | 356% | 723% |
| Overall | -6.51% | -11.47% | -8.23% | -54.81% | -40.92% | -37.86% | #NUM! | #NUM! |

  - NNVC-4.0-EE1 vs intra+pf

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -8.46% | -14.47% | -21.66% | -10.21% | -17.01% | -22.43% | 144% | 9988% |
| Class A2 | -5.55% | -18.71% | -14.37% | -4.75% | -18.09% | -12.29% | 140% | 9438% |
| Class B | -6.79% | -21.05% | -17.39% | -4.84% | -20.98% | -16.58% | 143% | 10573% |
| Class C | -5.15% | -19.98% | -16.90% | -3.92% | -18.24% | -16.05% | 139% | 9719% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -6.44% | -18.98% | -17.51% | -5.65% | -18.88% | -16.75% | 142% | 9992% |
| Class D | -4.90% | -19.33% | -18.31% | -1.15% | -17.62% | -15.98% | 140% | 10854% |
| Class F | -2.45% | -13.84% | -11.19% | -2.46% | -14.48% | -12.44% | 163% | 6819% |

  - Overview

The table below shows the sum-up results for RA configuration, PSNR BD-rates.

|  |  |  |  | + intra |  |  | +intra+rdo(+filter only for set1) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| set0 | -9.75% | -21.09% | -20.54% | -11.19% | -21.86% | -21.41% | -11.41% | -21.83% | -21.48% |
| set1 | -9.60% | -20.92% | -21.49% | -11.02% | -21.82% | -22.49% | -11.94% | -21.86% | -22.59% |
| intra | -1.81% | -0.49% | -0.90% |  |  |  |  |  |  |
| sr (A1/A2) | -5.34% | -2.58% | -2.17% | -7.01% | -3.34% | -3.36% |  |  |  |
| pf | -4.79% | -18.32% | -16.69% | -6.44% | -18.98% | -17.51% |  |  |  |

The table below shows the sum-up results for RA configuration, Ms-SSIM BD-rates.

|  |  |  |  | + intra |  |  | +intra+rdo(+filter only for set1) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| set0 | -9.65% | -20.89% | -19.34% | -11.02% | -21.51% | -19.90% | -11.20% | -21.40% | -19.97% |
| set1 | -9.21% | -21.77% | -21.63% | -10.52% | -22.56% | -22.40% | -11.41% | -22.50% | -22.40% |
| intra | -1.74% | 0.05% | -0.06% |  |  |  |  |  |  |
| sr (A1/A2) | -5.34% | -2.58% | -2.17% | -42.97% | -26.68% | -31.19% |  |  |  |
| pf | -4.79% | -18.32% | -16.69% | -5.65% | -18.88% | -16.75% |  |  |  |

- Discussions

  - Previous discussions

The following points were pending at last meeting:

- SOLVED using git lfs: The setting of a better solution for models’ storage in order to avoid repository excess size.

- SOLVED: manual update when needed. Upgrade the VTM version to benefit from bug corrections from AhG3.

- PARTIALLY ENFORCED: To enforce adopted contributions to merge into existing code, including for the training scripts if they exist (see issue with [MR 21/23]).

- SOLVED: Issue raised on the encoder invariance: likely due to an aggressive optimization compilation option affecting all float operations in the encoder.

  - New discussions

The following points were discussed or are still discussed in the group:

- Official configuration files for NNVC software and official configuration files for EE1 anchors:

  - Current EE1 anchors files are based on all tools off, i.e. anchor is essentially VTM-11.0

  - One official set of configuration files cannot be decided because some tools are mutually exclusive.

  - It raised some issues:

    - Progress of NNVC can be difficult to demonstrate without any official configuration.

    - With all tools off, new integrated tools do not take into account existing integrated tools and conflict might appear.

    - Synergy of tools are not easily shown when adopting a new tool.

    - It increases a lot the number of combinations to generate.

- To enforce adopted contributions to provide training scripts to train a model from scratch (issue with AC0055). It raised the following issues:

  - Fine-tuned models are not modifiable by other proponents (architecture, training policy etc.)

  - Training is not officially cross-check.

- A MR (MR68) was submitting after the software release deadline, modifying the results. The MR consists in one line change and improve the results, but the changes were not in the cross-checked software. No consensus has been reached to integrate the modification in the next software version.

- Configurations

The following configurations is used to generate the different NNVC results.

The column “tested” is read as follow:

- Y: the configuration has been tested using the new NNVC-4.0 software

- P: the results are the ones from the previous NNVC-3.0 software basis

- N: not tested.

The column “xcheck” is read as follow:

- Y: the test has been cross-checked

- P: no cross-checked performed but results are consistent with previous version on NNVC-3.0

- N: no cross-check available

| Name | Tools | Configuration | Tested | Xcheck |
| --- | --- | --- | --- | --- |
| EE1 | All NN tools off | encoder_xxx_vtm.cfg | Y | Y |
| set0 | Loop filter set #0 | encoder_xxx_vtm.cfg + nn-based/NnlfOption_1.cfg | Y | Y |
| set1 | Loop filter set #1 | encoder_xxx_vtm.cfg + nn-based/NnlfOption_2.cfg | Y | Y |
| intra | Intra prediction | nn-based/encoder_xxx_nnvc.cfg | Y | Y |
| set0+intra | Loop filter set #0+ Intra prediction | nn-based/encoder_xxx_nnvc.cfg + nn-based/NnlfOption_1.cfg | Y | Y |
| set1+intra | Loop filter set #1+ Intra prediction | nn-based/encoder_xxx_nnvc.cfg + nn-based/NnlfOption_2.cfg | Y | Y |
| set0+rdo+intra | Loop filter set #0 + Rdo + Intra prediction | nn-based/encoder_xxx_nnvc.cfg + nn-based/NnlfOption_1.cfg + --EncNnlfOpt=1 | Y | Y |
| set1+rdo+intra+temporal filter | Loop filter set #1 + Rdo + Intra prediction + Temporal filter | nn-based/encoder_xxx_nnvc.cfg + nn-based/NnlfOption_2.cfg + --EncNnlfOpt=1 + --NnlfSet1Multiframe=1 | Y | Y |
| sr | Super-resolution | encoder_xxx_vtm.cfg + nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg | Y | N |
| set0+sr | Loop filter set #0+Super-resolution | encoder_xxx_vtm.cfg + nn-based/NnlfOption_1.cfg + nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg | N | N |
| set1+sr | Loop filter set #1+Super-resolution | encoder_xxx_vtm.cfg + nn-based/NnlfOption_2.cfg + nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg | N | N |
| sr+intra | Super-resolution +Intra prediction | nn-based/encoder_xxx_nnvc.cfg + nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg | Y | Y |
| set0+sr+intra | Loop filter set #0+Super-resolution+Intra prediction | nn-based/encoder_xxx_nnvc.cfg + nn-based/NnlfOption_1.cfg + nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg | N | N |
| set1+sr+intra | Loop filter set #01+Super-resolution+Intra prediction | nn-based/encoder_xxx_nnvc.cfg + nn-based/NnlfOption_2.cfg + nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg | N | N |
| pf | Adaptive post-filters | encoder_xxx_vtm.cfg + nn-based/nnpf_int16.cfg | Y | P |
| Intra+pf | Intra prediction + adaptive post-filter | nn-based/encoder_xxx_nnvc.cfg + nn-based/nnpf_int16.cfg | Y | N |

Combination of loop filtering (or post filtering) with super resolution was not tested. It is verbally reported that gains found were not additive, but would need more investigation what reasons are.

It is noted that training scripts allowing re-training from scratch are not available for one of the models, (from JVET-AC0055) as sequence-adaptive training is starting from an initial pre-trained model. It would be desirable to get that provided. Revisit.

- Recommendations

The AHG recommends to:

- Continue to develop NNVC software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/issues

- Encourage people to submit merge requests fixing identified bugs.

# Project development (35)

## AHG1: Deployment and advertisement of standards (2)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0020 Deployment status of the HEVC standard [G. J. Sullivan (SC 29 Chair & Q6/16 Rapporteur)]

JVET-AD0021 Deployment status of the VVC standard [G. J. Sullivan (SC 29 Chair & Q6/16 Rapporteur)]

## AHG2: Text development and errata reporting (1)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0077 AHG2: On num_l0_weights and num_l1_weights for VVC specification [T. Aono, T. Chujoh, T. Ikai (Sharp)]

## AHG3: Test conditions (0)

This section is kept as a template for future use.

## AHG3: Software development (1)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0332 AHG13/AHG3: software development related to FGC SEI message [P. de Lagrange, F. Urban (InterDigital)] [late]

## AHG4: Subjective quality testing and verification testing (4)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0062 AHG4: Report on AHG meeting on subjective quality testing of FGC SEI message [M. Wien]

JVET-AD0063 AHG4: Report on AHG meeting on verification testing for VVC multilayer coding [M. Wien]

JVET-AD0102 Proposed verification test plan for content layering [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

JVET-AD0276 AHG4: experiments in preparation of film grain visual tests [P. de Lagrange, T. Filoche, F. Urban (InterDigital)] [late]

## AHG4: Test material (3)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0044 AHG4: Volumetric video-based coding (V3C) test content [S. Schwarz, M. M. Hannuksela (Nokia)]

JVET-AD0201 [AHG4] Availability and copyright status of 3DV test materials [K. Sühring (HHI)]

JVET-AD0229 AHG4/AHG12: Mixed-Content Sequences for Enhanced Compression Tool Testing [X. Li (Google), Z. Deng, K. Zhang, L. Zhang (Bytedance)]

## AHG4: Subjective quality assessment methodology (0)

This section is kept as a template for future use.

## AHG5: Conformance test development (1)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0101 Additional conformance tests of scaling window functionality for multilayer coding [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

## AHG7: ECM tool assessment (4)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0041 AHG7: Report on AHG meeting on ECM tool assessment [X. Li]

JVET-AD0082 [AHG7] A configuration option for TMRL mode [L. Xu, J. Gan, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AD0272 Crosscheck of JVET-AD0082 ([AHG7] A configuration option for TMRL mode) [C. Zhou (vivo)] [late] [miss]

JVET-AD0131 AHG 7: Coding efficiency/runtime tradeoff metrics [P. Onno, G. Laroche (Canon)]

JVET-AD0246 [AHG7] ECM tool assessment with HDR dataset [Z. Fan, Y. Yausig, T. Chujoh, T. Ikai (Sharp)] [late]

## AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (13)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0042 [AHG8] Proposed text for optimization of encoders and receiving systems for machine analysis of coded video content [J. Chen (Alibaba), C. Hollmann (Ericsson), S. Liu (Tencent)]

JVET-AD0043 [AHG8] Clean-ups for the reporting template [C. Hollmann (Ericsson)]

JVET-AD0047 AHG8: A preprocessing software implementation for OFM [B. Li, S. Wang, J. Chen, Y. Ye (Alibaba), S. Wang (CityU)]

JVET-AD0052 AHG9: NNPFC SEI extension mechanism to enable usages beyond user viewing [M. M. Hannuksela, F. Cricri, H. Zhang (Nokia)]

This contribution also fits under section 6.1.

JVET-AD0060 AHG8/AHG9: Indications related to suboptimal user viewing [M. M. Hannuksela, F. Cricri, H. Zhang (Nokia)]

This contribution also fits under section 6.2.

JVET-AD0110 [AHG8] Evaluating and Improving Coding Performance for General Image Classification Models [Q. Zhang, Z. Wang, S. Wang, S. Ma (Peking Univ.)]

JVET-AD0113 AHG8: Adaptive Mask Pre-Process algorithm for Video Coding for Machines [Z. Huang, Z. Wang, C. Jia, S. Wang, S. Ma (Peking Univ.)]

JVET-AD0122 [AHG8] Configuration changes for random access and low delay [C. Hollmann, R. Sjöberg, J. Ström (Ericsson)]

JVET-AD0307 Crosscheck of JVET-AD0122 ([AHG8] Configuration changes for random access and low delay) [Z. Liu (Tencent)] [late] [miss]

JVET-AD0135 [AHG8] Additional details on RoI-based adaptive QP for TR [C. Hollmann, R. Sjöberg, J. Ström (Ericsson)]

JVET-AD0137 [AHG8] SEI for encoder optimization information [C. Kim, Hendry, D. Gwak, J. Lim (LGE)]

This contribution also fits under section 6.2.

JVET-AD0138 [AHG8] QPA with low activity threshold for machine task [C. Kim, D. Gwak, J. Lim (LGE)]

JVET-AD0181 AHG8: Update on TVD object tracking dataset [Z. Liu, L. Wang, X. Xu, S. Liu (Tencent)]

JVET-AD0252 Cross-check of JVET-AD0181 (AHG8: Update on TVD object tracking dataset) [C. Hollmann (Ericsson)] [late]

JVET-AD0186 AHG8: VVC tool analysis for Machines Vision Tasks [Z. Liu, L. Wang, X. Xu, S. Liu (Tencent)]

## AHG10: Encoding algorithm optimization (4)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0045 AHG10: Encoder MV selections and DMVR revisited [K. Andersson, R. Yu (Ericsson)]

JVET-AD0343 Crosscheck of JVET-AD0045 (AHG10: Encoder MV selections and DMVR revisited) [J. Samuelsson-Allendes (Sharp)] [late] [miss]

JVET-AD0129 AHG10: Improvement of Input Video Padding in VTM [J. Liao, L. Li, D. Liu, H. Li, F. Wu (USTC)]

JVET-AD0300 Crosscheck of JVET-AD0129 (AHG10: Improvement of Input Video Padding in VTM) [N. Yan (Kwai)] [late] [miss]

JVET-AD0133 AHG10: Lambda-QP Relationship Fix for Slice-level Multi-QP Optimization [J. Liao, L. Li, D. Liu, H. Li, F. Wu (USTC)]

JVET-AD0302 Crosscheck of JVET-AD0133 (AHG10: Lambda-QP Relationship Fix for Slice-level Multi-QP Optimization) [N. Yan (Kwai)] [late] [miss]

JVET-AD0136 AHG10: Lagrange multiplier optimization for chroma ALF and CCALF [W.-J. Zou, Y. Zhou, C.-M. Gu, J.-W. Fan (Xidian Univ.), S.-W. Xie, G. Ying, M.-H. Jia, Y.-L. H, C. Huang (ZTE)]

JVET-AD0358 Crosscheck of JVET-AD0136 (AhG10: Lagrange multiplier optimization for chroma ALF and CCALF) [X. Li (Alibaba)] [late] [miss]

## AHG13: Film grain synthesis (2)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0238 On Film Grain Synthesis Subjective Evaluation and Further Work [X. Meng, W. Zhang (Disney Streaming), A. Norkin, J. Sole (Netflix), A. Duenas (Discovery)]

JVET-AD0241 AHG13: On Film Grain Objective Metrics [X. Meng, W. Zhang, S. Labrozzi (Disney Streaming)]

JVET-AD0276 AHG4: experiments in preparation of film grain visual tests [P. de Lagrange, T. Filoche, F. Urban (InterDigital)] [late]

JVET-AD0332 AHG13/AHG3: software development related to FGC SEI message [P. de Lagrange, F. Urban (InterDigital)] [late]

## Implementation studies (2)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0266 Update on open, optimized VVC implementations VVenC and VVdeC [A. Wieckowski, J. Brandenburg, C. Bartnik, V. George, J. Güther, G. Hege, C. Helmrich, A. Henkel, T. Hinz, C. Lehmann, C. Stoffers, B. Bross, H. Schwarz, D. Marpe, T. Schierl (HHI)] [late]

JVET-AD0361 Update on optimized VVC encoder implementation BVC for live-streaming [X. Gao, Y. He, X. Jiang, Y. Li, J. Wang, Y. Wu, L. Zhang, W. Zhang, W. Zhu (Bytedance)] [late] [miss]

## Profile/tier/level specification (1)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0232 HEVC Multiview Profiles: Implementation Status [S. Paluri, D. Podborksi, A.Tourapis (Apple)] [late] [miss]

## Proposed modification of system interface (0)

This section is kept as a template for future use.

## General aspects of standards development and applications of standards (0)

This section is kept as a template for future use.

# Low-level tool technology proposals

## AHG11/AHG14: Neural network-based video coding (25)

### Summary, BoG reports, and information documents

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0022 EE1: Summary report of exploration experiment on neural network-based video coding [E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang, Z. Xie (EE coordinators)]

### EE1 contributions: Neural network-based video coding (10)

Selected contributions in this area (as far as not covered in the EE summary report) were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

For actions decided to be taken, see section 5.2.1, unless otherwise noted.

JVET-AD0068 EE1-1.2: On adjustment of residual for NNLF [Z. Dai, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AD0333 Crosscheck of JVET-AD0068 (EE1-1.2: On adjustment of residual for NNLF) [R. Chang (Tencent)] [late] [miss]

JVET-AD0069 EE1-1.8: On flipping of input and output of model in NNVC filter set0 [Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AD0334 Crosscheck of JVET-AD0069 (EE1-1.8: On flipping of input and output of model in NNVC filter set0) [R. Chang (Tencent)] [late]

JVET-AD0070 EE1-1.9: Improvement over multi-frame model for NNVC filter set1 [Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AD0340 Crosscheck of JVET-AD0070 (EE1-1.9: Improvement over multi-frame model for NNVC filter set1) [R. Chang (Tencent)] [late]

JVET-AD0106 EE1-1.6: In-loop filter with wide activation and large receptive field [Y. Li, K. Zhang, L. Zhang (Bytedance), S. Eadie, Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)]

JVET-AD0270 Crosscheck of JVET-AD0106 (EE1-1.6: In-loop filter with wide activation and large receptive field) [C. Zhou (vivo)] [late] [miss]

JVET-AD0282 Crosscheck of EE1-1.6: In-loop filter with wide activation and large receptive field (JVET-AD0106) [Y. Li (Qualcomm)] [late] [miss]

JVET-AD0322 Crosscheck of JVET-AD0106 (EE1-1.6: In-loop filter with wide activation and large receptive field) [T. Shao (Dolby)] [late] [miss]

JVET-AD0156 EE1-1.1: Complexity Reduction on Neural-Network Loop Filter [J. N. Shingala, A. Shyam, A. Suneja, S. Badya (Ittiam), T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, Sean McCarthy (Dolby)]

JVET-AD0250 Crosscheck of JVET-AD0156 (EE1-1.1: Complexity Reduction on Neural-Network Loop Filter) [J. Ström, M. Damghanian (Ericsson)] [late] [miss

JVET-AD0160 EE1-2.1: Deep Reference Frame Generation for Inter Prediction Enhancement [J. Jia, Y. Zhang, H. Zhu, Z. Chen (Wuhan Univ.), Z. Liu, X. Xu, S. Liu (Tencent)]

JVET-AD0330 Crosscheck of JVET-AD0160 (EE1-2.1: Deep Reference Frame Generation for Inter Prediction Enhancement) [Z. Xie (OPPO)] [late] [miss]

JVET-AD0166 EE1-1.7: Optimization of training and network for NNVC filter set 0 [R. Chang, L. Wang, X. Xu, S. Liu (Tencent)]

JVET-AD0329 Crosscheck of JVET-AD0166 (EE1-1.7: Optimization of training and network for NNVC filter set 0) [Z. Xie (OPPO)] [late] [miss]

JVET-AD0168 EE1-1.4 Channel redistribution for luma and chroma [P. Wennersten, J. Ström, D. Liu (Ericsson)]

JVET-AD0321 Crosscheck of JVET-AD0168 (EE1-1.4 Channel redistribution for luma and chroma) [T. Shao (Dolby)] [late] [miss]

JVET-AD0205 EE1-1.3: Reduced complexity CNN-based in-loop filtering [S. Eadie, Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)]

JVET-AD0284 Crosscheck of JVET-AD0205 (EE1-1.3: Reduced complexity CNN-based in-loop filtering) [Y. Li (Bytedance)] [late] [miss]

JVET-AD0339 Crosscheck of JVET-AD0205 (EE1-1.3: Reduced complexity CNN-based in-loop filtering) [T. Shao (Dolby)] [late] [miss]

JVET-AD0226 EE1-1.5: Ablation Study and Minor Improvements on RTNN [H. Zhang, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO)]

JVET-AD0248 Cross-check of JVET-AD0226 (EE1-1.5: Ablation Study and Minor Improvements on RTNN) [M. Santamaria (Nokia)] [late] [miss]

### EE1 related contributions: Neural network-based video coding (9)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0107 EE1-related: Simplified parameter selection for filter set #1 [Y. Li, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0346 Crosscheck of JVET-AD0107 (EE1-related: Simplified parameter selection for filter set #1) [D. Liu (Ericsson)] [late] [miss]

JVET-AD0157 EE1-related: Neural-network loop filters in EE1-1.1.2 with further complexity reduction [T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, Sean McCarthy (Dolby), J. N. Shingala, A. Shyam, A. Suneja, S. Badya (Ittiam)]

JVET-AD0162 EE1-2.1-related: DRF Model without QP Input [J. Jia, D. Ding, W. Meng, Z. Chen (Wuhan Univ.), Z. Liu, X. Xu, S. Liu (Tencent)]

JVET-AD0253 Crosscheck of JVET-AD0162 (EE1-2.1: Deep Reference Frame Generation for Inter Prediction Enhancement) [N. Giuliani, E. Alshina (Huawei)] [late] [miss]

JVET-AD0167 EE1-1.7-related: Combination test for NNVC based on filter set 0 [R. Chang, L. Wang, X. Xu, S. Liu (Tencent)]

JVET-AD0170 EE1 related: Performance Improvement of AC0052 filter for RPR-based SR in RA configuration [S. Huang, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO)]

JVET-AD0189 EE1-related: A simplified NN-based RDO model for filter set #1 [Y. Li, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0207 EE1-Related: Additional results for EE1-1.3.1 and EE1-1.3.5 [Y. Li, S. Eadie, D. Rusanovskyy, M. Karczewicz (Qualcomm)]

JVET-AD0320 Crosscheck of JVET-AD0207 (EE1-Related: Additional results for EE1-1.3.1 and EE1-1.3.5) [T. Shao (Dolby)] [late] [miss]

JVET-AD0211 EE1-Related: Combination test of EE1-1.3.5 and multi-scale component of EE1-1.6 [Y. Li, S. Eadie, D. Rusanovskyy, M. Karczewicz (Qualcomm)]

JVET-AD0347 Crosscheck of JVET-AD0211 (EE1-Related: Combination test of EE1-1.3.5 and multi-scale component of EE1-1.6) [D. Liu (Ericsson)] [late] [miss]

JVET-AD0237 EE1-related: Residue input for filter set #1 [Y. Li, K. Zhang, L. Zhang (Bytedance)]

### Improvements of NNVC software beyond EE1 (6)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0050 AHG11/AHG14: Fix input QP for NN filter set #1 [D. Liu, J. Ström, K. Andersson (Ericsson)]

JVET-AD0109 AHG11: Neural network loop filter [R. Yang, M. Santamaria, N. Zou, F. Cricri, R. G. Youvalari, J. Lainema, H. Zhang, M. M. Hannuksela (Nokia)]

JVET-AD0111 AHG14: Guidelines for NNVC reference software development [F. Galpin, S. Eadie, L. Wang, Z. Xie, Y. Li (software coordinators)]

JVET-AD0163 AHG11: Updating NNPFC and NNPFA SEI messages for the NNPF [M. Santamaria, M. M. Hannuksela, F. Cricri, R. Yang (Nokia)]

JVET-AD0314 Crosscheck of JVET-AD0163 (AHG11: Updating NNPFC and NNPFA SEI messages for the NNPF) [H. Zhang, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO)] [late] [miss]

JVET-AD0164 AHG14: SADL v5 changes [F. Galpin, T. Dumas, P. Bordes, E. François (InterDigital)]

JVET-AD0212 AhG11: neural network-based intra prediction with reduced complexity [T. Dumas, F. Galpin, P. Bordes (InterDigital)] [late]

Initial version rejected as “placeholder”.

### Other aspects of neural network-based video coding (0)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

## AHG6/AHG12: Enhanced compression beyond VVC capability (103)

### Summary and BoG reports

Contributions in this area were discussed at 1410–1950 on Friday 21 April 2023 (chaired by JRO).

JVET-AD0023 EE2: Summary report of exploration experiment on enhanced compression beyond VVC capability [V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang (EE coordinators)]

This document provides a summary report of Exploration Experiment on Enhanced Compression beyond VVC capability. The tests are categorized as intra prediction, inter prediction, screen content coding, transform, and in-loop filtering.

The software basis for this EE is ECM-8.0, released at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-8.0. ECM-8.0 is used as an anchor in the tests.

Software for EE tests is released in the corresponding branches at https://vcgit.hhi.fraunhofer.de/ecm/jvet-ac-ee2/ECM/-/branches.

Test results can be found in input JVET contributions, cross-check results are uploaded to https://vcgit.hhi.fraunhofer.de/ecm/jvet-ac-ee2/simulation-results if cross-check reports are not submitted as they are optional for EE tests.

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Intra prediction |  |  |  |
| 1.1 | Extends the CCCM template selection to six | InterDigital P. Bordes JVET-AD0147 | Kwai H.-J. Jhu |
| 1.2 | CCCM using multiple downsampling filters | Qualcomm Y.-J. Chang JVET-AD0202 | Nokia J. Lainema |
| 1.3a | MM-CCLM/MM-CCCM filtering with neighbouring samples | Bytedance K. Zhang JVET-AD0120 | vivo Z. Lv |
| 1.3b | Cross-component prediction usage is determined by template cost | Bytedance K. Zhang JVET-AD0120 | vivo Z. Lv |
| 1.3c | Cross-component prediction signalling modification | Bytedance K. Zhang JVET-AD0120 | vivo Z. Lv |
| 1.3d | Test 1.3a + Test 1.3b + Test 1.3c | Bytedance K. Zhang JVET-AD0120 | vivo Z. Lv |
| 1.4a | Non-adjacent cross-component prediction | Bytedance K. Zhang | withdrawn |
| 1.4b | History-based cross-component prediction | Bytedance K. Zhang | withdrawn |
| 1.4c | Test 1.4a + Test 1.4b | Bytedance K. Zhang | withdrawn |
| 1.5 | Inherit cross-component prediction model from neighbours | MediaTek C.-M. Tsai | withdrawn |
| 1.6a | Cross-component prediction merge list includes spatial adjacent, non-adjacent, and default candidates | MediaTek C.-M. Tsai Bytedance K. Zhang JVET-AD0188 | Kwai C.-W. Kuo |
| 1.6b | Cross-component prediction merge list includes spatial adjacent, history-based, and default candidates | MediaTek C.-M. Tsai Bytedance K. Zhang JVET-AD0188 | Qualcomm Y.-J. Chang |
| 1.6c | Cross-component prediction merge list includes spatial adjacent, non-adjacent, history-based, and default candidates | MediaTek C.-M. Tsai Bytedance K. Zhang JVET-AD0188 | Alibaba J. Chen |
| 1.7 | TIMD using selectable template regions | Sharp Y. Yasugi JVET-AD0103 | Qualcomm H. Wang |
| 1.8a | HLS tool control with encoder optimization for IBC for camera captured content | Qualcomm B. Ray JVET-AD0208 | KDDI Y. Kidani |
| 1.8b | Test 1.8a + fractional pel BV | Qualcomm B. Ray | withdrawn |
| 1.8c | Test 1.8a + fractional pel IBC | Qualcomm C.-C. Chen Kwai W. Chen JVET-AD0208 | KDDI Y. Kidani |
| 1.9a | Fractional pel IBC | Kwai W. Chen | withdrawn |
| 1.9b | Test 1.9a (fractional pel IBC) + Test 1.8a | Kwai W. Chen Qualcomm B. Ray | withdrawn |
| 1.10a | Multi-candidate IntraTMP without search procedure change | OPPO F. Wang JVET-AD0073 | Qualcomm P.-H. Lin JVET-AD0327 |
| 1.10b | Multi-candidate IntraTMP with search procedure change | OPPO F. Wang JVET-AD0073 | Qualcomm P.-H. Lin JVET-AD0327 |
| 1.11a | Intra template-matching prediction fusion | OPPO L. Zhang JVET-AD0072 | Dolby F. Pu JVET-AD0309 |
| 1.11b | Intra template-matching prediction fusion with no change to the IntraTMP search procedure | OPPO L. Zhang JVET-AD0072 | Dolby F. Pu JVET-AD0309 |
| 1.11c | Intra template-matching prediction fusion with different number of candidates | OPPO L. Zhang JVET-AD0072 | Dolby F. Pu JVET-AD0309 |
| 1.12 | IntraTMP with sub-pel precision | Alibaba X. Li JVET-AD0125 | Dolby F. Pu |
| 1.13 | Fusion of intra template matching | Ittiam J. R. Arumugam Dolby F. Pu JVET-AD0158 | OPPO F. Wang Alibaba X. Li |
| 1.14a | Extended the adjacent search area of IntraTMP | Xidian Univ. Y. Ma H. Du JVET-AD0115 | Alibaba X. Li |
| 1.14b | Enlarged the search range for small blocks | OPPO F. Wang JVET-AD0074 |  |
| 1.14c | Test 1.14a + Test 1.14b | Xidian Univ. Y. Ma H. Du OPPO F. Wang JVET-AD0075 |  |
| 1.15a | Intra template matching based on linear filter model | Xidian Univ. J. Huo W. Qiao JVET-AD0112 | Alibaba X. Li |
| 1.15b | Filtered template matching based intra prediction | Nokia R. Youvalari JVET-AD0099 | Alibaba X. Li |
| 1.15c | Test 1.15a+Test 1.15b | Xidian Univ. J. Huo W. Qiao Nokia R. Youvalari | withdrawn |
| 1.16 | IntraTMP fusion with multiple reference blocks | Xidian Univ. J. Huo H. Du JVET-AD0116 | Alibaba X. Li |
| 1.17 | IntraTMP fusion with intra prediction | Bytedance Y. Wang JVET-AD0092 | Ericsson R. Yu JVET-AD0305 |
| 1.18 | CIIP extension with IntraTMP | InterDigital K. Naser JVET-AD0177 | OPPO F. Wang JVET-AD0259 |
| 1.19a | IntraTMP left and above template modes | Qualcomm P.-H. Lin JVET-AD0194 | OPPO F. Wang JVET-AD0263 |
| 1.19b | IntraTMP L-shape fusion mode | Qualcomm P.-H. Lin JVET-AD0194 | OPPO F. Wang JVET-AD0263 |
| 1.19c | Test 1.19a + Test 1.19b | Qualcomm P.-H. Lin JVET-AD0194 | OPPO F. Wang JVET-AD0263 |
| 1.20a | Test 1.10 + Test 1.11 | OPPO F. Wang JVET-AD0076 | Qualcomm P.-H. Lin JVET-AD0328 |
| 1.20b | Test 1.10 + Test 1.11 + Test 1.14 | OPPO F. Wang JVET-AD0076 | Qualcomm P.-H. Lin JVET-AD0328 |
| 1.20c | Test 1.14 + Test 1.19 | Qualcomm P.-H. Lin Xidian Univ. J. Huo OPPO F. Wang JVET-AD0198 | Kwai H.-J. Jhu JVET-AD0279 |
| 1.20d | Test 1.10 + Test 1.12 + Test 1.14 + Test 1.15 + Test 1.19 | Qualcomm P.-H. Lin Xidian Univ. J. Huo OPPO F. Wang Alibaba X. Li | withdrawn |
| 1.20e | Test 1.13 + Test 1.10 | Ittiam J. R. Arumugam OPPO F. Wang JVET-AD0191 | Alibaba X. Li |
| 1.20f | Test 1.13 + Test 1.12 | Dolby F. Pu Alibaba X. Li JVET-AD0192 | OPPO F. Wang |
| 1.20g | Test 1.13 + Test 1.17 | Dolby F. Pu Bytedance Y. Wang | withdrawn |
| 1.20h | Test 1.14 + Test 1.15 + Test 1.16 | Xidian Univ. Y. Ma H. Zhang JVET-AD0118 | Alibaba X. Li |
| 1.20i | Test 1.10 + Test 1.11 + Test 1.12 + Test 1.15 + Test 1.16 + Test 1.19 + Test 1.14a | OPPO F. Wang Xidian Univ. Y. Ma H. Zhang Qualcomm P.-H. Lin Alibaba X. Li JVET-AD0086 | Google X. Li JVET-AD0254 |
| 1.20j | Test 1.10 + Test 1.11 + Test 1.12 + Test 1.15 + Test 1.16 + Test 1.19 + Test 1.14c | OPPO F. Wang Xidian Univ. Y. Ma H. Zhang Qualcomm P.-H. Lin Alibaba X. Li JVET-AD0086 | Google X. Li JVET-AD0254 |
| 1.20k | Test 1.12 + Test 1.14c | Alibaba X. Li Xidian Univ. Y. Ma OPPO F. Wang JVET-AD0126 | Dolby F. Pu |
| 1.20m | Test 1.17 + Test 1.18 | Bytedance Y. Wang InterDigital K. Naser JVET-AD0119 | OPPO F. Wang JVET-AD0260 |
| 1.20n | Test 1.20f + Test 1.14c | Dolby F. Pu Alibaba X. Li JVET-AD0247 | Nokia R. Youvalari |
| 1.21a | Checking more neighbouring positions with intra prediction modes replacement by partitioning angles for SGPM/GPM | OPPO L.Xu JVET-AD0071 |  |
| 1.21b | Angular modes extension from 65 to 129 for TMRL | OPPO L. Xu JVET-AD0071 |  |
| 1.21c | Test 1.21a + Test 1.21b | OPPO L. Xu JVET-AD0071 |  |
| 1.22 | Template-based intra MPM list construction | vivo C. Zhou JVET-AD0173 | Alibaba J. Chen |
| 1.23 | Modification to MPM list derivation | Qualcomm H. Wang JVET-AD0190 | Sharp Y. Yasugi |
| 1.24a | Test 1.21 + Test 1.22 | OPPO L. Xu vivo C. Zhou JVET-AD0085 | Bytedance Z. Deng |
| 1.24b | Test 1.21 + Test 1.23 | OPPO L. Xu Qualcomm H. Wang JVET-AD0085 |  |
| 1.24c | Test 1.22 + Test 1.23 | vivo C. Zhou Qualcomm H. Wang JVET-AD0085 |  |
| 1.24d | Test 1.21 + Test 1.22 + Test 1.23 | OPPO L. Xu vivo C. Zhou Qualcomm H. Wang JVET-AD0085 | Sharp Y. Yasugi |
| 1.25 | Filtered template matching based intra prediction | Nokia R. Youvalari | duplicate of 1.15b |
| 2 Inter prediction |  |  |  |
| 2.1a | Step 1 of CPMV refinement with simplification | Xiaomi M. Blestel Qualcomm Y. Zhang JVET-AD0182 | InterDigital F. Le Léannec |
| 2.1b | Step 2 of CPMV refinement with simplification | Xiaomi M. Blestel Qualcomm Y. Zhang JVET-AD0182 | InterDigital F. Le Léannec |
| 2.1c | Adaptive DMVR for affine merge + affine DMVR for affine MMVD | Qualcomm Y. Zhang JVET-AD0182 | Kwai W. Chen |
| 2.1d | Affine parameter refinement | Alibaba J. Chen Xiaomi M. Blestel JVET-AD0182 | MediaTek C.-M. Tsai |
| 2.1e | Affine parameter refinement with simplification | Alibaba J. Chen Xiaomi M. Blestel JVET-AD0182 | MediaTek C.-M. Tsai |
| 2.1f | Test 2.1a + Test 2.1b | Xiaomi M. Blestel Qualcomm Y. Zhang | withdrawn |
| 2.1g | Test 2.1a + Test 2.1c | Qualcomm Y. Zhang Xiaomi M. Blestel | withdrawn |
| 2.1h | Test 2.1b+ Test 2.1c | Qualcomm Y. Zhang Xiaomi M. Blestel | withdrawn |
| 2.1i | Test 2.1c+ Test 2.1f | Qualcomm Y. Zhang Xiaomi M. Blestel | withdrawn |
| 2.1j | Test2.1d + Test 2.1a | Alibaba J. Chen Xiaomi M. Blestel Qualcomm Y. Zhang | withdrawn |
| 2.1k | Test 2.1d + Test 2.1f | Alibaba J. Chen Xiaomi M. Blestel Qualcomm Y. Zhang JVET-AD0182 | InterDigital F. Le Léannec |
| 2.1l | Test 2.1d + Test 2.1c | Alibaba J. Chen Qualcomm Y. Zhang Xiaomi M. Blestel JVET-AD0182 | ByteDance L. Zhao |
| 2.1m | Test 2.1d + Test 2.1i | Alibaba J. Chen Qualcomm Y. Zhang Xiaomi M. Blestel | withdrawn |
| 2.2 | AMVP with SbTMVP mode | Alibaba R.-L. Liao JVET-AD0152 | Ofinno V. Rufitskiy |
| 2.3a | SbTMVP with MMVD | Tencent L.-F. Chen JVET-AD0209 | Kwai X. Xiu |
| 2.3b | SbTMVP with MMVD by using TM-based reordering | Tencent L.-F. Chen JVET-AD0209 | Kwai X. Xiu |
| 2.4 | Template matching-based subblock motion refinement | Bytedance L. Zhao JVET-AD0165 | Alibaba R.-L. Liao |
| 2.5a | Test 2.2 + Test 2.4 | Alibaba R.-L. Liao Bytedance L. Zhao JVET-AD0153 | Canon G. Laroche |
| 2.5b | Test 2.2 + Test 2.3 | Alibaba R.-L. Liao Tencent L.-F. Chen JVET-AD0210 | Google X. Li |
| 2.6 | High precision MV refinement for BDOF | Bytedance M. Salehifar JVET-AD0195 |  |
| 2.7a | LIC for bi-prediction | Kwai X. Xiu JVET-AD0213 | Tencent L.-F. Chen |
| 2.7b | Test2.7a + OBMC with LIC | Kwai X.Xiu JVET-AD0213 | Tencent L-F. Chen |
| 2.8a | OBMC flag inheritance in merge mode | Qualcomm K. Cui | withdrawn |
| 2.8b | OBMC flag is signalled only for affine block without MHP | Qualcomm K. Cui | withdrawn |
| 2.9a | OBMC boundary level decision based on prediction samples, keeping the existing CU level OBMC on/off flag | Google X. Li | withdrawn |
| 2.9b | OBMC boundary level decision based on prediction samples, without the existing CU level OBMC on/off flag | Google X. Li | withdrawn |
| 2.9c | OBMC boundary level decision based on reference samples, keeping the existing CU level OBMC on/off flag | Google X. Li | withdrawn |
| 2.9d | OBMC boundary level decision based on reference samples, without the existing CU level OBMC on/off flag | Google X. Li | withdrawn |
| 2.10a | OBMC on/off decision based on prediction samples and neighbouring information | Bytedance Z. Deng | withdrawn |
| 2.10b | Using reference samples rather than prediction samples for OBMC on/off decision | Bytedance Z. Deng | withdrawn |
| 2.11a | OBMC on/off decision at CU level and boundary level based on prediction samples and neighbouring information, with existing CU level OBMC on/off flag | Google X. Li Bytedance Z. Deng | withdrawn |
| 2.11b | OBMC on/off decision at CU level and boundary level based on prediction samples and neighbouring information, without existing CU level OBMC on/off flag | Google X. Li Bytedance Z. Deng | withdrawn |
| 2.11c | OBMC on/off decision at CU level and boundary level based on reference samples, with existing CU level OBMC on/off flag | Google X. Li Bytedance Z. Deng | withdrawn |
| 2.11d | OBMC on/off decision at CU level and boundary level based on reference samples, without existing CU level OBMC on/off flag | Google X. Li Bytedance Z. Deng | withdrawn |
| 2.11e | Adaptive OBMC control | Qualcomm K. Cui Google X. Li Bytedance Z. Deng JVET-AD0193 | Kwai X. Xiu OPPO J. Gan |
| 2.12a | Prediction of MVD magnitude suffix bins (up to 6 bins for blocks with width and height larger than 4, and up to 2 bins otherwise) | Ofinno A. Filippov JVET-AD0140 | Xiaomi M. Blestel |
| 2.12b | Prediction of MVD magnitude suffix bins (up to 6 bins for blocks with width and height larger than 4, and up to 4 bins otherwise) | Ofinno A. Filippov | withdrawn |
| 2.12c | Prediction of MVD magnitude suffix bins (up to 6 bins per block) | Ofinno V. Rufitskiy | withdrawn |
| 2.12d | Prediction of MVD magnitude suffix bins (up to 8 bins for blocks with width and height larger than 4, and up to 4 bins otherwise) | Ofinno V. Rufitskiy | withdrawn |
| 2.12e | Prediction of MVD magnitude suffix bins (up to 8 bins for blocks with width and height larger than 4, and up to 6 bins otherwise) | Ofinno V. Rufitskiy | withdrawn |
| 2.12f | Tool-off test for MVD sign prediction as implemented in ECM | Ofinno V. Rufitskiy JVET-AD0140 | Xiaomi M. Blestel |
| 2.12g | Test 2.2 + Test 2.12 | Ofinno V. Rufitskiy Alibaba R.-L. Liao JVET-AD0256 | Xiaomi M. Blestel |
| 2.12h | Test 2.7 + Test 2.12 | Ofinno A. Filippov Kwai X. Xiu JVET-AD0257 | Xiaomi M. Blestel |
| 3 Screen content coding |  |  |  |
| 3.1 | IntraTMP using reconstruction-reordered for screen content coding | Ofinno D. Ruiz Coll JVET-AD0171 |  |
| 3.2a | IBC MBVD list derivation with max BVD offset 128-pel | Qualcomm Z. Zhang JVET-AD0199 | Ofinno D. Ruiz Coll |
| 3.2b | IBC MBVD list derivation with max BVD offset 256-pel | Qualcomm Z. Zhang JVET-AD0199 | Ofinno D. Ruiz Coll |
| 3.3 | Temporal block vector prediction | Bytedance N. Zhang JVET-AD0095 | Qualcomm Z. Zhang |
| 3.4a | Copy-padding for IBC | Bytedance N. Zhang JVET-AD0096 | Hanbat National University H. Han JVET-AD0139 Qualcomm Z. Zhang |
| 3.4b | Test 3.4a + Test 3.3 | Bytedance N. Zhang JVET-AD0096 | Qualcomm Z. Zhang |
| 3.5 | Template matching for IBC BVD suffix derivation | Qualcomm P. Nikitin | withdrawn |
| 3.6 | IBC-CIIP with adaptive weights | Kwai C. Ma | withdrawn |
| 3.7a | Test 3.2 + Test 3.4b | Bytedance N. Zhang Qualcomm Z. Zhang JVET-AD0097 | MediaTek C.-M. Tsai |
| 3.7b | Test 3.2b + Test 3.4b | Bytedance N. Zhang Qualcomm Z. Zhang JVET-AD0097 | MediaTek C.-M. Tsai |
| 4 In-loop filtering |  |  |  |
| 4.1 | Additional fixed and signalled filters for ALF | Bytedance W. Yin JVET-AD0221 | Qualcomm N. Hu |
| 4.2 | ALF classifier based on residual data | Qualcomm I. Jumakulyyev JVET-AD0219 | BytedanceW. Yin |
| 4.3 | ALF filter shape with more taps applied to fixed filter results | Qualcomm I. Jumakulyyev JVET-AD0219 | BytedanceW. Yin |
| 4.4a | Test 4.2 + Test 4.3 | Qualcomm I. Jumakulyyev JVET-AD0219 | BytedanceW. Yin |
| 4.4b | Test 4.1 + Test 4.2 + Test 4.3 | Qualcomm I. Jumakulyyev Bytedance W. Yin JVET-AD0222 | Ericsson K. Andersson |
| 4.4c | Test 4.1 + Test 4.2 | Qualcomm I. Jumakulyyev Bytedance W. Yin JVET-AD0222 | Ericsson K. Andersson |
| 5 GDR |  |  |  |
| 5.1 | Reference picture padding for GDR | InterDigital T. Poirier JVET-AD0123 | Nokia S. Hong |

Inter prediction

### Test 1.1 Extends the CCCM template selection to six (JVET-AD0147)

In this test, the number of CCCM templates is extended (in addition to top and left, only left, and only top) to 6 as shown in the below figure in the second row. When one of the 3 extended templates is selected, the model is derived selecting the samples of the template with the reconstructed luma value within the range of the reconstructed current CU luma values. Also, the CCCM parameters are applied with 32-bits precision. If CCCM mode is selected, similarly to GL-CCCM, a flag is coded to indicate whether regular or extended template is used.

### Test 1.2 CCCM using multiple downsampling filters (JVET-AD0202)

It the test, multiple downsampling filters are applied to a group of reconstructed luma samples in a CCCM. The linear combination of these downsampled reconstructed samples is multiplied by derived filter coefficients to form the final chroma predictor. The horizontal or vertical location of the center luma sample are also considered in the tested model. The cross-component models shown below are tested as additional CCCM modes with a mode index signalled in the bitstream:

- Model 1: predChroma = c0 * H(C) + c1 *  G1(C) + c2 *  G2(C) + c3 *  G3(C) + c4 *  P(H(C)) + c5 *  P(G1(C)) + c6 * P(G2(C)) + c7 * X + c8 * Y + c9 * B

- Model 2: predChroma = c0 * H(C) + c1 *  H(W) + c2 *  H(E) + c3 *  G1(C) + c4 *  G1(W) + c5 *  G1(E) + c6 *  P(H(C)) + c7 *  P(H(W)) + c8 *  P(H(E)) + c9 * X + c10 * B

- Model 3: predChroma = c0 * H(C) + c1 *  H(NE) + c2 *  H(SW) + c3 *  G3(C) + c4 *  G3(NE) + c5 *  G3(SW) + c6 *  P(H(C)) + c7 *  P(H(NE)) + c8 *  P(H(SW)) + c9 * Y + c10 * B

where H(·), G1(·), G2(·), G3(·) are various downsampling filters as indicated in Figure 1, C denotes the current chroma sample position, and N, S, W, E, NE, SW are the positions around C as indicated in Figure 2, ci are filter coefficients, P and B are nonlinear term and bias term, and X and Y are the horizontal and vertical locations of the center luma sample with respect to the top-left coordinates of the block.

Figure 1. Various downsampling filters used in the tested cross-component models

Figure 2. The positions of chroma samples

### Test 1.3: Local-boosting cross-component prediction (JVET-AD0120)

In this method, the local information including neighbouring and prediction samples are considered for cross-component prediction (CCCM and CCLM). The aspects are included.

Aspect #1: Prediction samples of MM-CCLM/MM-CCCM can be filtered with neighbouring samples with a 3×3 low-pass filter. At a top/left boundary, the filtering window involves neighbouring reconstructed samples. For inner samples, the filtering window only involves prediction samples, which are padded. A flag is signalled to indicate whether filtering is applied or not for a block coded with MM-CCLM/MM-CCCM.

Aspect #2: Template costs are calculated to implicitly determine the usage of the cross-component prediction. The template cost is derived by applying the candidate cross-component mode on the template and calculating the SAD between the prediction samples and reconstruction samples in the template. The mode with a lower template cost is selected as the final method for the current block.

Aspect #3: The binarization of the mode signalling is modified. Instead of signaling CCLM-L/CCLM-T/MM-CCLM-L/MM-CCLM-T (or CCCM-L/CCCM-T/MM-CCCM-L/MM-CCCM-T) with a truncated unary code, two syntax elements are coded: multi_mode_flag indicates whether multiple or single model is applied, and ccp_dir_flag indicates whether left or above direction is applied.

All aspects are tested individually and in a combination as follows.

Test 1.3a: Aspect #1 (MM-CCLM/MM-CCCM filtering with neighbouring samples).

Test 1.3b: Aspect #2 (cross-component prediction usage is determined by template cost).

Test 1.3c: Aspect #3 (cross-component prediction signalling modification).

Test 1.3e: Aspects 1 and 2.

Test 1.3d: Aspects 1, 2, and 3.

### Test 1.6: Non-local cross-component prediction and cross-component merge mode (JVET-AD0188)

In the test, a cross-component prediction merge candidate list is constructed from the spatial adjacent, spatial non-adjacent, or history-based candidates. After including these candidates, default models are further included to fill the remaining empty positions in the merge list. Pruning is applied to remove redundant candidates. After constructing the list, the models in the list are reordered depending on the SAD costs, which are obtained using the neighbouring template of the current block.

Spatial adjacent and non-adjacent candidates

The positions and inclusion order of the spatial adjacent and non-adjacent candidates are the same as those defined in ECM for regular inter merge prediction candidates.

History-based candidates

A history-based table is maintained to include the recently used cross-component models, and the table is reset at the beginning of each CTU row. If the current list is not full after including spatial adjacent and non-adjacent candidates, the models from the history-based table are added into the list.

Default candidates

CCLM candidates with default scaling parameters are considered, only when the list is not full after including the spatial adjacent, spatial non-adjacent, or history-based candidates. If the current list has no candidates with the single model CCLM mode, the default scaling parameters are {0, 1/8, -1/8, 2/8, -2/8, 3/8, -3/8, 4/8, -4/8, 5/8, -5/8, 6/8}. Otherwise, the default scaling parameters are {0, the scaling parameter of the first CCLM candidate + {1/8, -1/8, 2/8, -2/8, 3/8, -3/8, 4/8, -4/8, 5/8, -5/8, 6/8}}. The offset parameter is derived according to the default scaling parameter, average neighbouring reconstructed luma sample value, and average neighbouring reconstructed Cb/Cr sample value.

A flag is signalled to indicate whether the cross-component prediction merge mode is applied or not. If it is applied, an index is signalled to indicate which candidate model is used by the current block. The mode is not allowed when the current CU is coded by intra subpartitions (ISP) with single tree, or the current chroma coding block size is less than or equal to 16.

The following tests are performed:

Test 1.6a: the merge list includes spatial adjacent, non-adjacent, and default candidates.
Test 1.6b: the merge list includes spatial adjacent, history-based, and default candidates.
Test 1.6c: the merge list includes spatial adjacent, non-adjacent, history-based, and default candidates.

In Test 1.6a and Test 1.6b, the maximum allowed list size is 6. In Test 1.6c, the size is changed to 12 to accommodate more potential candidates. Besides, the maximum allowed history-based table size is 6 in Test 1.6b and Test 1.6c.

### Test 1.7: TIMD using selectable template regions (JVET-AD0103)

In ECM, the templates for TIMD mode are implicitly selected from the left, top, or top and left templates depending on the coordinates for the current block. If x=0, only top template is used, if y=0 only left template is used, otherwise left and top templates are applied.

In the test, a syntax element is signalled to explicitly indicate one of the three types of template regions, even for CUs that are not located at the top or left boundary of the frame.

### Test 1.8 IBC adaptation for camera captured content (JVET-AD0208)

In the tests, there are two main aspects: to provide encoder optimization for IBC and extending IBC to use fractional pel BVs.

Aspect #1. This aspect involves high level tool control of various IBC coding tools and encoder optimization, which is applied to camera captured content (non-SCC) classes:

- IBC merge modes are disabled by using an SPS level flag to indicate that. So, only IBC AMVP mode is activated.

- RR-IBC, TM-IBC, and IBC-CIIP are disabled. For RR-IBC and TM-IBC, corresponding SPS flags are implemented (as ECM-8.0 does not have SPS control for these tools).

- IBC is applied to intra slices only which is indicated by HLS.

- At encoder, IBC block vector search is optimized, and the RDO process may be skipped for an IBC AMVP mode if its SAD cost is much worse than the lowest SAD cost of all intra modes.

- IBC AMVP modes may not be evaluated when the best intra mode has less than 3 nonzero coefficients.

- In ECM-8.0 encoder, some partitioning depth in an inter slice is skipped depending on the POC distance between the current picture and its nearest reference picture. When IBC is enabled from SPS level, ECM-8.0 always sets this POC distance equal to 0, while in this test, for inter slices, the true POC distance is used instead of setting to 0.

Aspect #2. Fractional pel BVs for IBC as follows:

- The option of block vector resolutions is extended to include quarter-pel resolution in additional to full-pel and 4-pel. Like inter AMVR syntax, the first bin is signalled to indicate whether BV is in quarter-pel resolution, and the second bin is signalled to switch between full-pel and 4-pel resolutions.

- The interpolation filters applied to the luma (8-tap) and chroma (6-tap existed inter interpolation) components of an IBC block. For template-based IBC tools, a 2-tap bilinear interpolation filter is applied to generate template prediction blocks.

- Reference sample padding is performed when some of them are located outside IBC reference area. When needed, it performs in horizontal direction first and then vertical direction.

Two tests are performed:

Test 1.8a: HLS tool control with encoder optimization for IBC for camera captured content (aspect #1)

Test 1.8c: Test 1.8a with fractional pel BVs (aspect #2, applied for all classes)

### Test 1.10: Multi-candidate IntraTMP (JVET-AD0073)

In the test, a candidate list is constructed, and the candidate BVs are ranked in ascending order of their template matching costs. An index is signalled in the bit-stream to indicate which candidate BV is used for current block.

To build a BV candidate list of size N, in the sparse search, the sub-sampling factor is set to 3 instead of 2 used in ECM-8.0 and 2N BVs with the smallest template matching costs are maintained. Then, in the refinement search, each 3x3 block around each of the 2N BVs is checked and N BVs with the smallest cost are selected to form the candidate list.

N equal to 15 and 19 is tested.

Test 1.10a: Multi-candidate IntraTMP without search procedure change.

Test 1.10b: Multi-candidate IntraTMP with search procedure change.

### Test 1.11: Intra template matching prediction fusion (JVET-AD0072)

The tested fusion method includes the following aspects:

- Generate multiple matched blocks during the IntraTMP search.

During the subsampled IntraTMP search process with the step size 3 instead of the current 2, a candidate list is initially generated for 30 matched blocks with the smallest template SAD. For each candidate, a full 3x3 pixel refinement search around each of the 30 matched blocks is performed and the best 3 candidate matched blocks measured by template SAD are selected.

- Select candidate matched blocks for fusion.

For each of the best 3 candidate matched blocks, a threshold equal to 2 * SAD1 is used to select candidates for fusion, SAD1 is the smallest template SAD of the three candidate matched blocks. Candidates SAD <= Threshold are used for fusion.

- Calculate fusion weight for each of the selected matched blocks.

Once blocks to be fused are decided, they are fused with the weights calculated by their SAD as follows:

sum= i=1nSADi

wi= sum-SADin-1*sum

To reduce the implementation cost, the division operations are replaced by an integer look-up table.

- The final fused predictor is determined by

pfusion=i=1nwipi

where pi is the ith matched block, and n is the number of blocks selected for fusion. In a special case, when only one matched block remains after step b), the final predictor is calculated as

pfusion=w1pTMP+w2pplanar

where pTMP is the single matched block and pplanar is the planar intra predictor, where the weights are set as w1=7/8 and w2=1/8.

A CU level flag is added to signal whether an IntraTMP CU is predicted by the proposed fusion method or by the original method.

Test 1.11a: Intra template-matching prediction fusion with 3 candidates

Test 1.11b: Intra template-matching prediction fusion with no change to search procedure

Test 1.11c: Intra template-matching prediction fusion with 5 candidates

Test 1.12: IntraTMP with sub-pel precision (JVET-AD0125)

In the test, three sub-pel precisions, including half-pel, quarter-pel and three quarter-pel, with eight directions around the integer-pel position are tested as shown in Figure 3. Furthermore, the eight directions are sorted by template cost with half-pel precision for each CU, and only the first four directions can be used for each sub-pel precision.

Figure 3. The adjacent half-pel positions in 8 directions.

If Intra TMP mode is selected for the current block, a precision index is signalled to indicate which of the integer-pel and three sub-pel precisions is used. If one of the three sub-pel precisions is used, a direction index is signalled to indicate which of the four directions is used.

Four-tap DCT-IF interpolation filters in ECM are used for sub-pel interpolation in Intra TMP.

Test 1.13: Fusion of intra template matching (JVET-AD0158)

In the first search pass, the best matching block with the lowest TM cost is selected, specified its template as Tbest. Then in the second search pass, the template is modified as T'=2*T-Tbest. A separate flag is signalled to indicate whether the second search with the template update is used at decoder.

Test 1.13a: Adaptive IntraTMP fusion with up to two prediction blocks.

Test 1.13b: Test 1.13a + search process from Test 1.10

Test 1.14a: Extended the adjacent search area of IntraTMP (JVET-AD0115)

In this test, R3 and R4 regions are extended shown in Figure 4 as vertical upper side and horizontal left side of the current block that are also reconstructed and can be used for prediction.

Figure 4. IntraTMP search regions in ECM-8.0 (top) and extended regions (bottom)

Test 1.14b: Enlarged the search range for small blocks (JVET-AD0074)

In ECM, IntraTMP search range is based on the current block size:

SearchRangeWidth = a * BlkWidth

SearchRangeHight = a * BlkHeight,

where a is equal to 5.

In the test, IntraTMP search range for small blocks is enlarged as follows:

SearchRangeWidth = max(a * BlkWidth, minSearchRange)

SearchRangeHight = max(a * BlkHeight, minSearchRange),

where minSearchRange is set as 64.

Test 1.14c: Combination of Test 1.14a and Test 1.4b (JVET-AD0075)

Test 1.15a: Intra template matching based on linear filter model (JVET-AD0112)

In this method, a linear model is derived between the reference and current templates, the derived model is applied to the IntraTMP predictor. Similar to CCCM, a model is represented by 6-tap filter consisting of a 5-tap spatial and a bias terms.

predLumaVal = c0C + c1N + c2S + c3E + c4W + c5 * (1<<(bitdepth – 1) )

The template size is 4, and the method is applied to the best BV candidate. A flag is signalled to indicate the mode usage.

Test 1.15b: Filtered template matching based intra prediction (JVET-AD0099)

Similar to 1.15a, the same 6-tap filter model applied to IntraTMP is tested. The template size used for training is 4 lines above and to the left of the current block depending on their availability.

The filtered method uses the current IntraTMP’s SAD based search in ECM-8.0 for creating a list of four candidate reference blocks to be filtered. Entry to the candidate list is based on SAD cost of the unfiltered template.

The filter parameters are calculated for each candidate in the list and the best performing one in terms of template cost after filtering is selected and used as the final candidate. The final prediction of the block is then generated by applying the derived filter for the best candidate.

Test 1.16: IntraTMP fusion with multiple reference blocks (JVET-AD0116)

IntraTMP fusion is tested, N best BVs are derived during the subsampled search with step size of 2, then those N best predictors are fused with the weights as follows.

predSamples=n=0N-1wn*refBlockn+wN*(1≪bitdepth-1)

where predSamples represent the final predicted block, refBlockn is one of the N candidate reference blocks. The weights wn are derived from the candidate matching templates and current template using the decomposition method from CCCM.

A flag is signalled to indicate the mode usage.

Test 1.17: IntraTMP fusion with intra prediction (JVET-AD0092)

In IntraTMP fusion with intra prediction, the prediction block is the weighted sum of the two prediction signals generated by IntraTMP and intra prediction derived by TIMD. A CU flag is signalled to indicate whether the tested method is used.

Test 1.18: CIIP extension with IntraTMP (JVET-AD0177)

The tested method is similar to inter CIIP, where the inter part is replaced by IntraTMP prediction, while the intra prediction and weights calculation is kept unchanged. A flag is signalled to indicate the mode usage.

Test 1.18b: CIIP extension with IntraTMP

Test 1.18a: Test 1.18b + Test 1.14c (search range modification)

Test 1.19: IntraTMP with multiple modes (JVET-AD0194)

In Test-1.19a, 3 template types are defined. In addition to L-shape (left and above templates), using only left and only above templates as shown in the below figure is tested. For each template type, the best 4 candidates are derived in the template matching process and selected by HAD cost. To signal the selected candidate (template type and candidate index), a flag is signalled to indicate which template type is used followed by the candidate index.

In Test-1.19b, a fusion method is used to combine multiple candidates, the fusion weights are derived by template matching costs or using the derivation method of MSE minimization existed in CCCM. A flag is signalled to indicate which weight derivation method is used. The candidates to be fused are the best 2 L-shape candidates or the best 5 L-shape candidates. Another flag is signalled to indicate the number of candidates to be fused (2 or 5).

Test 1.19c is the combination of tests 1.19a and 1.19b.

Tests 1.20: Combination of IntraTMP related tests

In the tests, there are the following elements:

- Extended IntraTMP search range

- Enlarged search range for small blocks

- Modified IntraTMP search process

- Multiple candidates for IntraTMP

- Fusion candidates for IntraTMP (TM cost and MSE based weights derivation)

- Filtered IntraTMP mode

- Fractional pel IntraTMP

- Fusion of intra template matching

- Combination of IntraTMP and intra prediction

A combination Test 1.20j includes all aspects except the last two.

Test 1.20a (JVET-AD0076): Test 1.10b (19 candidates) + Test 1.11c (without fusion with intra)

Test 1.20b (JVET-AD0076): Test 1.10b (19 candidates) + Test 1.11c (without fusion with intra) + Test 1.14c

Test 1.20c (JVET-AD0198): Test 1.14c + Test 1.19c

Test 1.20e (JVET-AD0191): Test 1.13b + Test 1.10b (15 candidates)

Test 1.20f (JVET-AD0192): Test 1.13b + Test 1.12

Test 1.20h (JVET-AD0118): Test 1.14a + Test 1.15a + Test 1.16

Test 1.20i (JVET-AD0086): Test 1.10b(19 candidates) + Test 1.11c + Test 1.12 + Test 1.15a + Test 1.16 + Test 1.19c + Test 1.14a

Test 1.20j (JVET-AD0086): Test 1.10b(19 candidates) + Test 1.11c + Test 1.12 + Test 1.15a + Test 1.16 + Test 1.19c + Test 1.14c

Test 1.20k (JVET-AD0126): Test 1.12 + Test 1.14c

Test 1.20m (JVET-AD0119): Test 1.17 + Test 1.18b

Test 1.20m* (JVET-AD0119): Test 1.17 + Test 1.18a (includes search range modification of Test 1.14c)

Test 1.20n (JVET-AD0247): Test 1.20f + Test 1.14c

Test 1.21: Modifications on template-based multiple reference line intra prediction (JVET-AD0071)

Two aspects are tested modifying TMRL prediction.

In Test 1.21a, the construction of intra candidate list for MPM and TMRL is modified as follows:

- Non-adjacent positions are added as candidates in constructing the intra candidate list.

- If the neighbouring or non-adjacent blocks are coded with SGPM or GPM modes, the intra modes of the blocks are replaced by the partitioning angles.

In Test 1.21b, the precision of angular prediction is extended from 65 to 129 for TMRL.

Test 1.21c is a combination of Test 1.21a and Test 1.21b.

Test 1.22: Template-based intra MPM list construction (JVET-AD0173)

In ECM, the regular MPM list consists of primary MPMs, secondary MPMs, and DIMD modes. There is no ordering of the modes based on template cost.

In the test, the regular MPM list is constructed by the following three steps:

- The first entry in the general MPM list is always planar mode.

- The next entries are composed of the intra modes of neighboring blocks, and DIMD modes, which are sorted in ascending order of SAD cost. Up to 5 modes with the smallest SAD cost are added. The SAD cost is computed between the prediction and the reconstruction samples of the template.

- The sorted directional modes with added offset are added into the general MPM list, and then the default modes, until the general MPM list with 22 entries is constructed.

For the signaling part, MPM list is equally divided into four groups and the group index is parsed first. Then, a mode index is further parsed to indicate which mode in the selected group is used.

Test 1.23: Modification to MPM list derivation (JVET-AD0190)

In ECM, there are two methods to construct regular MPM and TMRL MPM lists. In the tests, one MPM list derivation method is introduced for both cases as follows:

- For the spatial neighbors of the current block, if a neighbor block is coded by MIP/IntraTMP/Planar mode, the first non-planar mode from the spatial MPM modes of the neighbour block is selected as the derived mode and the derived mode is tagged as ‘reserved’ for the partial MPM reordering performed in ECM-8.0 when generating MPM list for the current block.

- Secondary MPM modes are derived by adding offset to all primary angular modes, this process is the same as in ECM-8.0 for TMRL modes, where the offset is added to DIMD modes first.

- If MPM list is incomplete, the same default modes are added as in ECM-8.0.

Tests 1.24: Combinations of MPM related tests (JVET-AD0085)

Test 1.24a: Test 1.21c + Test 1.22

Test 1.24b: Test 1.21c + Test 1.23

Test 1.24c: Test 1.22 + Test 1.23

Test 1.24d: Test 1.21c + Test 1.22 + Test 1.23

Note: The “*” marking in the table above expresses that test are not finished yet, and rate gain is estimated by using anchor results. Likely, final results could be better (cannot be worse).

1.1: Small benefit in compression, more templates. No action.

1.2: Some complexity increase (in some cases, multiple downsampling filters). Cross-checker and other experts supported this as an interesting extension of CCCM. Decision: Adopt JVET-AD0202.

1.3: Gain is asserted to be independent of 1.2. The complexity increase (except for test b which does additional template matching) is small according to cross-checker. Several opinions were expressed about whether 1.3e (comb. of a and b) or 1.3a standalone would be more preferable, also as gain of 1.3b for RA appears negligible (however it has gain in AI). It was reported that test 1.3b in RA case was only applied to I slices, which might explain. Decision: Adopt JVET-AD0120 test 1.3e.

1.6: All three solutions have similar increase in encoder/decoder run times. The full combination of all additional candidates (adjacent, non-adjacent, history based & default) 1.6c has best compression benefit, both for AI and RA. Some concern was expressed if there might be a conflict with 1.3b, as both 1.6c and 1.3b rely on TM, where 1.3b is relying on template cost, and 1.6c uses it for list reordering. It was asserted that in worst case a combination could mean that they cannot be used together. Decision: Adopt JVET-AD0188 test 1.6c.

1.7: No good tradeoff, in AI 13% encoder run time increase. No action.

1.8: Significant gain in particular for 1.8c which uses fractional pel compensation in addition with the modified search and other encoder optimization of 1.8a, in particular for camera captured content, but with fractional pel also for screen content. Good tradeoff with slightly increased runtime increase. It was asked why 8-tap interpolation was used? It was reported that the gain by using 12-tap was extremely small, so they were not used considering complexity. Decision: Adopt JVET-AD0208 test 1.8c.

The contributors are asked to provide a similar software implementation with appropriate encoder optimization for VTM (in the context of AHG10) such that IBC could be also be enabled for camera captured content in VTM.

Tests 1.10ff. relate to modifications of IntraTMP

From the results, test 1.20j provides best gain (>1% in AI), with acceptable tradeoff (5% encoder, 6% decoder run time increase) Decision: Adopt-JVET-AD0086 test 1.20j

1.21ff relate to modifications of list construction which are also tested in combination. The best combination is 1.24a, which however comes with a decoder run time increase of 2% (most likely caused by 1.21a). It is reported by the proponents that the number of TM operations is not increased which makes it difficult to understand where the runtime increase comes from. Without this runtime increase, 1.24a would be attractive for adoption, but more explanation about the decoder runtime increase and possibility to reduce that is expected. Revisit.

Inter prediction

Tests 2.1: Affine DMVR (JVET-AD0182)

Several aspects related to affine DMVR are tested.

1. Affine DMVR extension.

The refinement of the translational of the affine motion existed in DMVR is further extended to affine MMVD and adaptive BM merge modes. For affine MMVD mode, the MMVD offset is added to the affine DMVR refined affine merge base candidate if the base candidate meets the affine DMVR refinement condition. For adaptive BM merge mode, an affine merge list that only contains affine merge candidates that meet the affine DMVR conditions are constructed and then refined.

2. CPMV search.

For each control-point, perform bilateral matching for a block that centered by the control-points coordinates to derive the refined CPMVs. Loop over combinations of initial and refined CPMVs to derive the best set of CPMVs that minimize the bilateral matching cost of the current block.

3. CPMV refinement.

For CPMV refinement, an iterative search process is applied to further refine the CPMVs to minimize the bilateral matching cost of the current block. In each iteration, only one CPMV is refined while the others are fixed.

4. Affine parameters refinement.

In this refinement process, wherein each of CPMVs is fixed as base MV in turn, and an offset is added to the non-translation parameter of affine model by minimizing the bilateral matching cost, and then the other two CPMVs are calculated according to the base MV and refined non-translation parameters. To reduce the complexity, different number of search rounds and search directions are set for different stages with cost based early termination.

5. Simplifications.

Early termination based on the best PU distortion is applied. If initial distortion before CPMV refinement is lower than a threshold, then the iterative CPMVs refinement is bypassed.

The affine CPMV refinement is disabled if one of the following conditions is true:

- If block is square and is large than 128x128

- If block is non-square and width or height is greater than 64

The following tests are performed.

In Test 2.1a, CPMV search with simplification is tested.

In Test 2.1b, CPMV refinement with simplification is tested.

In Test 2.1c, affine DMVR extension is tested. Affine DMVR extension includes applying affine DMVR and affine regression to affine MMVD mode and adaptive BM merge mode.

In Test 2.1d, affine parameters refinement is tested.

In Test 2.1e, Test 2.1d with simplification is tested.

In Test 2.1k, a combination of Test 2.1a, Test 2.1b, and Test 2.1d is tested.

In Test 2.1l, a combination of Test 2.1e and Test 2.1c is tested.

Test 2.2: AMVP with SbTMVP mode (JVET-AD0152)

In the test, SbTMVP is applied to AMVP mode. When the mode is applied, a CU is split into 4x4 subblocks, and each subblock derives its own motion from a corresponding subblock in the collocated picture. The corresponding subblock is obtained using MVP with a signalled MVD. The MVP is derived in the same way to that of MVP of normal AMVP mode. The number of MVDs is determined according to the following rules:

- If the POC distance between current picture and its collocated picture is larger than 2, the number of MVD is set to 16 which comprises 4 motion directions and 4 motion magnitudes (Figure 5);

- Otherwise, if the current picture is a low delay picture, the number of MVD is set to 32 which comprises 4 motion directions and 8 motion magnitudes (Figure 6);

- Otherwise, the number of MVD is set to 64 which comprises 8 motion directions and 8 motion magnitudes (Figure 7).

Figure 5. 16 candidates for picture whose POC distance between current picture and collocated picture is larger than 2

Figure 6. 32 candidates for low delay picture

Figure 7. 64 candidates for non-low delay picture whose POC distance between current picture and collocated picture is smaller than or equal to 2

The adaptive motion vector resolution (AMVR), local illumination compensation (LIC) and multi-hypothesis prediction (MHP) are not applied to the proposed mode. The OBMC is always applied to the proposed mode without signalling.

When a CU is coded using AMVP mode, a flag is signalled to indicate whether the CU is predicted with SbTMVP. If the SbTMVP is used, another flag is signalled to indicate whether MVD exists, and an MVD index is signalled to indicate the motion direction and motion magnitude if MVD exists. The inter prediction direction and reference picture index are directly set to the collocated picture instead of being signalled.

Test 2.3: SbTMVP with MMVD (JVET-AD0209)

In the test, MVD offsets are applied to SbTMVP candidate in subblock MMVD merge list, an index is signalled to indicate the MVD offset, and this offset is used to derive the offset value of the displacement vector (DV) for each SbTMVP-MMVD candidate. The final DV for each SbTMVP-MMVD candidate is calculated from the DV of SbTMVP merge candidate with the selected offset.

The spiral scanning order is used to scan all MMVD candidates and the available candidate, which has the valid center motion vector, will be added into the candidate list.

Picture level mode flag is used to indicate whether the SbTMVP-MMVD is enabled, it is determined based on the temporal ID and the mode is only enabled at the highest two temporal layers for RA.

In Test 2.3a, MMVD offset is the integer pel step size of {4, 8, 12, 16, 20}, the number of directions is kept as 8 which is used in affine MMVD as well. Like the affine MMVD, the total number of the SbTMVP-MMVD candidates is less than or equal to 16. The spiral scanning order is applied and up to 16 available candidates are added into the candidate list.

In Test 2.3b, all 40 MMVD candidates will be scanned in spiral order and available candidates are added to the candidate list. Then, the subblock-based template-matching is used to reorder all SbTMVP-MMVD candidates by using template-matching cost in ascending order and up to 16 candidates with the smallest template-matching cost will be kept. The constraint of the size of collocated position is extended from one CTU row to three CTU rows. This extension is applied for SbTMVP-MMVD only, and there is no size extension for TMVP and SbTMVP.

Test 2.4: Template matching-based subblock motion refinement (JVET-AD0165)

Two aspects are included in the test.

Firstly, after the candidate list is constructed for affine merge mode, uni-directional affine candidates will perform CPMV refinement based on TM, where the same MV offset is applied to all the CPMVs of an affine candidate, and the TM cost of the affine candidate is calculated accordingly. The optimal CPMV offset with the minimum TM cost is used to refine the corresponding affine candidate if CRefined<T×COrig, where CRefined  denotes the TM cost after CPMVs are refined by the optimal offset, COrig is the TM cost calculated by the original CPMVs, and T is a constant threshold.

Secondly, the initial motion shift to specify SbTMVP is further refined with TM.

Test 2.5: Combination of SbTMVP related tests

Test 2.5a (JVET-AD0153): Test 2.2 + Test 2.4 (enabled for larger than 720p at encoder).

Test 2.5b (JVET-AD0210): Test 2.2 + Test 2.3a

Test 2.5c (JVET-AD0210): Test 2.2 + Test 2.3b

Test 2.6: High precision MV refinement for BDOF (JVET-AD0195)

In the test, high-precision equations to derive the BDOF MV refinement parameters are used:

Gx.Gx * vx  + Gx.Gy * vy  = dI . Gx   s1 * vx + s2 * vy = s3

Gx.Gy * vx  + Gy.Gy * vy  = dI . Gy   s2 * vx + s5 * vy = s6

where Gx/Gy are the summation of the 2 horizontal/vertical gradients derived for each reference block.

Summations () are weighted sums, where weights depend on the position in the summation window, higher weights are given for center samples.

The subblock size of BDOF DMVR is adaptively selected depending on the width×height. For blocks smaller than 256, subblock size of 4×4, and otherwise 8×8 is used.

Test 2.7: LIC for bi-prediction (JVET-AD0213)

In the first aspect, LIC mode is extended to bi-predictive CUs, where two different linear models are applied to the two prediction blocks which are then combined to generate the bi-prediction samples of the current CU, i.e.,

P'x,y=1-ω∙p0'x,y+ω∙p1'x,y

and

p0'x,y=α0∙P0x,y+β0

p1'x,y=α1∙P1x,y+β1

where α0 and β0, and α1 and β1 indicate the scales and the offsets in L0 and L1, respectively; ω indicates the weight (as indicated by the CU-level BCW index) for the weighted combination of L0 and L1 predictions.

The method firstly derives the L0 parameters by minimizing difference between L0 template prediction T0 and the template T and the samples in T are updated by subtracting the corresponding samples in T0.  Then, the L1 parameters are calculated that minimizes the difference between L1 template prediction T1 and the updated template. Finally, the L0 parameter is refined again in the same way.

Following the current LIC design, one flag is signalled for AMVP bi-predicted CUs for the indication of the LIC mode while the flag is inherited for merge related inter CUs. Additionally, the LIC is disabled for DMVR and BDOF.

In the second aspect, the OBMC is enabled for the inter blocks that are coded with the LIC mode. And, to reduce the complexity, the OBMC is only applied to the top and left CU boundaries while being always disabled for the boundaries of the internal sub-blocks of one LIC CU. Additionally, when one neighboring block is coded with the LIC, its LIC parameters are applied to generate the corresponding prediction samples for the OBMC of one current block.

Test 2.7a: LIC for bi-prediction

Test 2.7b: Test 2.7a + OBMC with LIC

Test 2.11e: Adaptive OBMC control (JVET-AD0193)

In the test, in addition to the existed SPS flag, OBMC is adaptively controlled on a block level as follows:

- OBMC flag is inherited from a neighboring affine block for affine merge mode.

- OBMC is not applied to a block if there is a neighbor block coded with IBC, palette, or BDPCM modes.

- When applying OBMC to a block, block boundary check whether OBMC is applied to the boundary is further made based on the reference samples of the current block. If any absolute difference between the prediction sample and non-interpolated (integer pel) reference sample is greater than a threshold, the OBMC is not applied to that boundary.

In the test, addition results are provided for the mixed context sequences from JVET-AC1015 “Common test conditions for SCM screen content coding” denoted as class M.

Test 2.12: Prediction of MVD magnitude suffix bins (JVET-AD0140)

In ECM, MVD sign prediction is utilized. In the test, the most important difference between the MVD sign prediction and the tested method is in removing the restriction of predicting just MVD signs whereas the number of bins to be predicted is kept the same.

Most significant bins of MVD remainder suffixes are predicted using template matching and the correctness of prediction hypotheses are indicated by corresponding MVD suffix bins that are coded in the bitstream using regular CABAC mode. The proposed method is applied for MVDs of translational motion, including SMVD and affine modes. Candidates for template matching used to predict bin values are shown in Figure 8. The less significant bins of magnitude suffixes of horizontal and vertical MVD components may be coded in by-pass mode.

Figure 8. Prediction of MVD sign and magnitude suffix bins

Test 2.12a: Prediction of MVD magnitude suffix bins

Test 2.12f: ECM-8.0 with MVD sign prediction disabled

Test 2.12a*: Test 2.12 with MVD sign prediction disabled

Test 2.12h (JVET-AD0256): Test 2.12a + Test 2.7a

Test 2.12g (JVET-AD0257): Test 2.12a + Test 2.2

Results RA/LB

2.1: It was verbally reported that an original test 2.1f had been planned which combined 2.1a and 2.1b, and gave approv. 0.07% luma gain in RA without runtime increase. This was withdrawn.

From the current results, various experts expressed that test 2.1l gives the most attractive benefit in terms of compression vs. runtime tradeoff. Decision: Adopt JVET-AD0182 test 2.1l.

Tests 2.2 … 2.4 are all related to SbTMVP modifications (some may also have other aspects), Test 2.5 investigates combinations.

In 2.5a, test 2.2* is used (different from the table above)

It is asserted that tests 2.2 … 2.5 do not provide attractive benefit in terms of compression, compared to the increased encoder (and partially decoder run time). No significant support by other experts. No action.

Test 2.6 (modifications of BDOF) provides attractive tradeoff (0.2% gain in RA vs. 1% decoding time increase). Support by independent experts. Decision: Adopt JVET-AD0195.

Test 2.7: Both variants (and in particular 2.7b which has higher compression gain of > 0.35% in RA and LB) are attractive, though increasing encoder runtime by 2-3% (and decoder 1%). It was confirmed that LIC is invoked in LDB in combination with CIIP as well (not mentioned in the proposal). Supported by various independent experts. Decision: Adopt JVET-AD0213 test 2.7b.

Test 2.11: It is clarified that class “M” stands for mixed content (non-CTC class from old SCC test conditions). It was asked what the benefit of the three elements of the proposal (see description above) are. It was asked in particular how important the third element (block boundary gain) is. It was reported to provide roughly 0.5% out of 1.32% in LB for mixed content. Independent experts expressed that this is a good supplement to the high-level disabling of OBMC, in particular for mixed content (where also class F where also significant gain is reported, which also has some mixed-content sequences). Decision: Adopt JVET-AD0193 test 2.11e.

Test 2.12a is a straightforward extension of existing MVD sign prediction, giving similar gain as the latter, also in combination with it. The other sub-experiments are comninations with various other EE tests which show that the gain is retained. Several experts expressed support. Decision: Adopt JVET-AD0140 test 2.12a.

Screen content coding

Test 3.1: IntraTMP using reconstruction-reordered for screen content coding (JVET-AD0171)

In ECM, there is a reconstruction-reordered IBC mode. In the test, reconstruction-reordered IntraTMP mode is tested. When this mode is applied, the encoder and decoder search for the most similar template to the flipped current template in a predefined area of the search region. Only the current CTU (R1) and a portion of the left region (R4) are explored for the horizontal flip type. Otherwise, only the current CTU (R1) and a part of the above region (R3) are explored in the vertical flip mode.

The dimensions of search regions in R3 and R4 regions are five times of block dimension (CbHeight, CbWidth) as it is determined for the regular IntraTMP, as shown in Figure 9.

The flip type is selected among the regular TMP (non-flip), horizontally flipped, and vertical flipped TMP modes according to the lowest TM cost. Consequently, no additional signaling is required. The corresponding block is flipped according to flip type, horizontal or vertical flip, and used as a prediction block.

Figure 9. RR-IntraTMP search region for (a) horizontal and (b) vertical flip.

For horizontal flipping, in case the neighboring block is coded with a horizontal flip of RR-TMP, the horizontal component of BVcur is calculated by adding a motion shift to the horizontal component of BVnbr (denoted as BVhnbr), and the vertical component remains the same:

- BVhcur=2xnbr-xcur+BVhnbr

- BVvcur=BVvnbr

Similarly, in case the neighboring block is coded with a vertical flip, the vertical component of BVcur is calculated by adding a motion shift to the vertical component of BVnbr (denoted as BVvnbr), and the horizontal component remains invariant:

- BVhcur=BVhnbr

- BVvcur=2ynbr-ycur+BVvnbr

Where (xnbr,ynbr) and (xcur,ycur) represent the coordinates of the center sample of the neighboring block and the current block, respectively, BVnbr and BVcur denotes the BV of the neighboring block and the current block, respectively.

Test 3.2: IBC MBVD list derivation (JVET-AD0199)

In the test, MBVD offsets are searched starting from fixed offsets defined by the initial step size along the MBVD direction. At every search step, the best 8 candidates are identified based on TM SAD cost and the search process repeats with the MVBD offsets checked around the identified candidates with the reduced step size by half. The search process stops when the step size reaches 1-pel resulting in the best 8 MBVD offset candidates.

When a MBVD candidate has a left or above template outside of the BV search area, the MBVD candidate is set to invalid candidate. When two BVP candidates have the same BV component in horizontal or vertical direction, the BV difference in other direction has to be larger than a threshold T.

The amount of MBVD candidates and MBVD index signaling is kept the same as ECM-8.0.

Test 3.2a: IBC MBVD list derivation with max BVD offset 128-pel.

The max BVD offset is 128-pel, initial search step size is 4-pel, BVP difference threshold T = 6-pel.

Test 3.2b: IBC MBVD list derivation with max BVD offset 256-pel.

The max BVD offset is equal to 256-pel, initial search step size is 4-pel, BVP difference threshold T = 32-pel.

Test 3.3: Temporal block vector prediction (JVET-AD0095)

In ECM, IBC merge/AMVP candidates are derived from spatial, HMVP, pairwise average, and some predefined BV candidates. However, temporal BV candidates are not utilized.

In this test, a temporal BVP is introduced to mimic TMVP. Temporal BV candidates are added to IBC merge/AMVP candidate lists.

Temporal BV candidates are derived with the full pruning applied from the same temporal positions as TMVP, in the list they are added before the HMVP candidates.

Test 3.4: Copy-padding for IBC (JVET-AD0096)

In this test, copy-padding can be applied to non-reconstructed area. With copy-padding, an unreconstructed sample in the overlapped with the current block region is padded by copying its prediction samples, as shown in Figure 10, and is represented as follows.

P’(x,y) = P(x+BVx, y+BVy)

wherein P’(x,y) is a padded sample at position (x, y), P(x+BVx, y+BVy) is a prediction sample, and (BVx, BVy) is the BV of the current block.

Copy-padding is performed only if the horizontal BV component is smaller than or equal to 0 and the vertical BV component is smaller than or equal to 0.

Figure 10. Unreconstructed samples (shaded) in the reference block are padded by copying their prediction samples

Test 3.4a: Copy-padding for IBC.

Test 3.4b: Test 3.4a + Test 3.3

Test 3.7: Combination of Test 3.2 and Test 3.4b (JVET-AD0097)

The following tests are performed.

Test 3.7a: Test 3.2a + Test 3.4b

Test 3.7b: Test 3.2b + Test 3.4b

Results AI/RA/LB

Test 1.1 does not provide benefit (also loss for class F in AI) – no action.

Generally, with all proposals gains are relatively low (compared to what is usually observed in screen content), and concern was raised that all of them increase complexity somehow (multiple search steps in 3.2x, access to reference picture BV for 3.3). 3.4 seems to have least impact on complexity, but in particular for class F, 3.4a which is the standalone version gives almost no gain. Non consensus about taking action on any of the proposals, no significant support by non-proponents.

In-loop filtering

Test 4.1: Additional fixed-filter for ALF (JVET-AD0221)

In the test, one additional fixed filter with a diamond 7x7 shape and one signalled 7x7 cross filter shape are introduced to ALF shown in Figure 11 with turquoise colour, there is no classification for the newly added fixed filter.

The reconstructed samples before DBF are used as an input to the additional fixed filter to produce the filter output, then this filter output is used as an input to the introduced 7x7 cross signalled filter.

The coefficient signaling mechanism is the same as the current design in ECM. The proposed method is always enabled without any filter shape switching.

Figure 11. 7x7 cross signalled filter for ALF

Test 4.2: ALF classifier based on residual data (JVET-AD0219)

In the test, a third classifier based on luma residual sample values is introduced. For each 2x2 luma block, the sum of absolute values of the residual samples in a neighbouring 8x8 window is calculated, and the class index is derived as:

classIdx = sum >> (sample bit depth – 4).

The value of classIdx is in the range of 0 to 24, same as in ECM-8.0. The classifier usage is signalled for each luma filter set in APS.

Test 4.3: ALF filter shape with more taps applied to fixed filter results (JVET-AD0219)

In the test, the signalled ALF filter shape is modified, compared to the filter shape in ECM-8.0, the overall number of the filter coefficients for a signalled luma filter is increased by 2 as shown in Figure 12, the length of the filter that is applied to the neighboring reconstructed samples (taps #0-#9) is reduced and the length of the filter applied to the samples after the fixed-filter (taps #10-#29) is increased.

Figure 12. Modified filter shape of the signalled ALF filter.

Test 4.4: Combination of ALF related tests

Test 4.4a (JVET-AD0219): Test 4.2 + Test 4.3

Test 4.4b (JVET-AD0222): Test 4.1 + Test 4.2 + Test 4.3

Test 4.4c (JVET-AD0222): Test 4.1 + Test 4.2

Results AI/RA/LB

It was confirmed that no additional buffer for residual data is required for 4.2 (relative to the residual buffer introduced in the last meeting).

4.3 requires four additional line buffers.

It can be expected that the combination 4.4b will end up with approximately 0.2% compression gain in RA. Already the 0.18% estimated from preliminary results appear attractive gain for RA, considering that encoder/decoder runtime is practically kept constant. Decision: Adopt JVET-AD0222 test 4.4b.

GDR

Test 5.1: Reference picture padding for GDR (JVET-AD0123)

In ECM, CUs in the “clean area” use the reference pictures where all reconstructed samples in the “dirty area” are set to 2bitdepth-1.

In this test, the “dirty area” is padded using samples from the “clean area” of the reference as shown in Figure 13.

Figure 13. Padding of the "dirty area"

The test was conducted using LB configuration following GDR test conditions described in EE2 (JVET-AC2024).

Gain for LDB:

| -0,22% | -0,09% | -0,05% | 101% | 100% |
| --- | --- | --- | --- | --- |

This is asserted to be a reasonable improvement of GDR. Decision: Adopt JVET-AD0123.

### EE2 contributions: Enhanced compression beyond VVC capability (53)

Selected contributions in this area (as far as not covered in the EE summary report) were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

For actions decided to be taken, see section 5.3.1, unless otherwise noted.

JVET-AD0071 EE2-1.21: Modifications on template-based multiple reference line intra prediction [L. Xu, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AD0072 EE2-1.11: Intra template matching prediction fusion [L. Zhang, F. Wang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AD0309 Crosscheck of JVET-AD0072 (EE2-1.11: Intra template matching prediction fusion) [F. Pu (Dolby)] [late] [miss]

JVET-AD0073 EE2-1.10 multi-candidate IntraTMP [F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AD0327 Crosscheck of JVET-AD0073 (EE2-1.10 Multi-candidate IntraTMP) [P.-H. Lin (Qualcomm)] [late] [miss]

JVET-AD0074 EE2-1.14b Enlarge search range for small blocks in IntraTMP [F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AD0075 EE2-1.14c combination test of EE2-1.14a and EE2-1.14b [Y. Ma, J. Huo, H. Du, H. Zhang, Z. Zhang, W. Qiao, F. Yang (Xidian Univ.), F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AD0303 crosscheck of JVET-AD0075 (EE2-1.14c combination test of EE2-1.14a and EE2-1.14b) [K. Naser (InterDigital)] [late] [miss]

JVET-AD0076 EE2-1.20a/b: Combination of EE2-1.10, EE2-1.11 and EE2-1.14 [L. Zhang, F. Wang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AD0328 Crosscheck of JVET-AD0076 (EE2-1.20a/b: Combination of EE2-1.10, EE2-1.11 and EE2-1.14) [P.-H. Lin (Qualcomm)] [late] [miss]

JVET-AD0085 EE2-1.24: Combinations of MPM related tests [L. Xu, Y. Yu, H. Yu, D. Wang (OPPO), C. Zhou, Z. Lv, J. Zhang (vivo), H. Wang, Y. Chang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AD0265 Crosscheck of JVET-AD0085 EE2-1.24d: Combinations of MPM related tests [Y. Yasugi (Sharp)] [late] [miss]

JVET-AD0287 Crosscheck of JVET-AD0085 (EE2-1.24a: Combinations of MPM related tests) [Z. Deng (ByteDance)] [late] [miss]

JVET-AD0086 EE2-1.20i/j: Combination of IntraTMP tests [F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO), Y. Ma, H. Zhang, J. Huo, F. Yang (Xidian Univ.), P. Lin, J. Lin, V. Seregin, M. Karczewicz (Qualcomm), X. Li, R. Liao, J. Chen, Y. Ye (Alibaba group)]

JVET-AD0254 Cross-check of JVET-AD0086 EE2-1.20i/j: Combination of IntraTMP tests [X. Li (Google)] [late] [miss]

JVET-AD0092 EE2-1.17: IntraTMP fusion with intra prediction [Y. Wang, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0305 Crosscheck of JVET-AD0092 (EE2-1.17: IntraTMP fusion with intra prediction) [R. Yu (Ericsson)] [late] [miss]

JVET-AD0095 EE2-3.3: Temporal block vector prediction [N. Zhang, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0304 Crosscheck of JVET-AD0095 and JVET-AD0096 (EE2-3.3: Temporal block vector prediction and EE2-3.4: Copy-Padding for IBC) [Z. Zhang (Qualcomm)] [late]

JVET-AD0096 EE2-3.4: Copy-Padding for IBC [N. Zhang, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0304 Crosscheck of JVET-AD0095 and JVET-AD0096 (EE2-3.3: Temporal block vector prediction and EE2-3.4: Copy-Padding for IBC) [Z. Zhang (Qualcomm)] [late]

JVET-AD0139 Cross-check of JVET-AD0096 (EE2-3.4a: Copy-Padding for IBC) [H. Han, H. Choi (HNU)]

JVET-AD0097 EE2-3.7: Combination tests of EE2-3.2+EE2-3.4b [N. Zhang, K. Zhang, L. Zhang (Bytedance), Z. Zhang, P. Nikitin, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AD0354 Crosscheck of JVET-AD0097 (EE2-3.7: Combination tests of EE2-3.2+EE2-3.4b) [C.-M. Tsai (MediaTek)] [late] [miss]

JVET-AD0099 EE2-1.15b: Filtered template matching based intra prediction [R. G. Youvalari, D. Bugdayci Sansli, P. Astola, J. Lainema (Nokia)]

JVET-AD0294 Crosscheck of JVET-AD0099 (EE2-1.15b: Filtered template matching based intra prediction) [X. Li (Alibaba)] [late] [miss]

JVET-AD0103 EE2-1.7: TIMD using selectable template regions [Y. Yasugi, T. Ikai (Sharp)]

JVET-AD0312 Crosscheck of JVET-AD0103 (EE2-1.7: TIMD using selectable template regions) [H. Wang (Qualcomm)] [late] [miss]

JVET-AD0112 EE2-1.15a: Intra template matching (Intra TMP) based on linear filter model [J.-Y. Huo, W.-H. Qiao, X. Hao, Z.-Y. Zhang, H.-Q. Du, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.)]

JVET-AD0293 Crosscheck of JVET-AD0112 (EE2-1.15a: Intra template matching (Intra TMP) based on linear filter model) [X. Li (Alibaba)] [late] [miss]

JVET-AD0115 EE2-1.14a: Extend search area in Intra Template Matching Prediction (IntraTMP) [J.-Y. Huo, H.-Q. Du, H.-L. Zhang, Z.-Y. Zhang, W.-H. Qiao, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.)]

JVET-AD0292 Crosscheck of JVET-AD0115 (EE2-1.14a: Extend search area in Intra Template Matching Prediction (IntraTMP)) [X. Li (Alibaba)] [late] [miss]

JVET-AD0116 EE2-1.16: A Fusion method of Intra Template Matching Prediction (Intra TMP) [J.-Y. Huo, H.-Q. Du, H.-L. Zhang, W.-H. Qiao, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.)]

JVET-AD0295 Crosscheck of JVET-AD0116 (EE2-1.16: A Fusion method of Intra Template Matching Prediction (Intra TMP)) [X. Li (Alibaba)] [late] [miss]

JVET-AD0118 EE2-1.20h: Combination of EE2-1.14a, EE2-1.15a and EE2-1.16 for Intra TMP [J.-Y. Huo, N.-F. Qiu, H.-L. Zhang, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), H. Yuan (Shandong Univ.)]

JVET-AD0297 Crosscheck of JVET-AD0118 (EE2-1.20h: Combination of EE2-1.14a, EE2-1.15a and EE2-1.16 for Intra TMP) [X. Li (Alibaba)] [late] [miss]

JVET-AD0119 EE2-1.20m: Combination test of Test 1.17 and Test 1.18 [Y. Wang, K. Zhang, L. Zhang (Bytedance), K. Naser, P. Bordes, F. Galpin, K. Reuzé, A. Robert (InterDigital)]

JVET-AD0260 Crosscheck of JVET-AD0119 (EE2-1.20m: Combination test of Test 1.17 and Test 1.18) [F. Wang (OPPO)] [late] [miss]

JVET-AD0120 EE2-1.3: Local-Boosting Cross-Component Prediction [Z. Deng, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0243 Crosscheck of JVET-AD0120 EE2-1.3: Local-Boosting Cross-Component Prediction [Z. Lv (vivo)] [late] [miss]

JVET-AD0123 EE2-5.1: Reference frame padding for GDR [T. Poirier, S. Puri (InterDigital)]

JVET-AD0351 Cross Check of JVET-AD0123 (EE2-5.1: Reference frame padding for GDR) [S. Hong (Nokia)] [late]

JVET-AD0125 EE2-1.12: Intra TMP with sub-pel precision [X. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)]

JVET-AD0310 Crosscheck of JVET-AD0125 (EE2-1.12: Intra TMP with sub-pel precision) [F. Pu (Dolby)] [late] [miss]

JVET-AD0126 EE2-1.20k: Combination of Test 1.12 and Test 1.14c [X. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba), Y. Ma, J. Huo, H. Du, H. Zhang, Z. Zhang, W. Qiao, F. Yang (Xidian Univ.), F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AD0311 Crosscheck of JVET-AD0126 (EE2-1.20k: Combination of Test 1.12 and Test 1.14c) [F. Pu (Dolby)] [late] [miss]

JVET-AD0140 EE2-2.12: Prediction of MVD magnitude suffix bins [A. Filippov, V. Rufitskiy, K. Suverov (Ofinno)]

JVET-AD0267 Crosscheck of JVET-AD0140 (EE2-2.12: Prediction of MVD magnitude suffix bins) [M. Blestel (Xiaomi)] [late] [miss]

JVET-AD0147 EE2-1.1: Extends the CCCM template selection to six [P. Bordes, K. Naser, F. Galpin, E. François (InterDigital)]

JVET-AD0278 Crosscheck of JVET-AD0147 (EE2-1.1: Extends the CCCM template selection to six) [H.-J. Jhu (Kwai)] [late] [miss]

JVET-AD0152 EE2-2.2: AMVP with SbTMVP mode [R.-L. Liao, J. Chen, Y. Ye, X. Li (Alibaba)]

JVET-AD0277 Cross-check of JVET-AD0152 (EE2-2.2: AMVP with SbTMVP mode) [V. Rufitskiy, A. Filippov (Ofinno)] [late] [miss]

JVET-AD0153 EE2-2.5a: Combined test of Test 2.2 and Test 2.4 [R.-L. Liao, J. Chen, Y. Ye, X. Li (Alibaba), L. Zhao, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0326 Cross-check of JVET-AD0153 on EE2-2.5a [G. Laroche (Canon)] [late]

JVET-AD0158 EE2-1.13: Test on Fusion of Intra Template Matching [J. R. Arumugam, A. Natesan, V. Valvaiker, A. Sarate, J. N. Shingala (Ittiam), F. Pu, T. Lu, P. Yin, T. Shao, A. Arora, S. McCarthy (Dolby)]

JVET-AD0261 Crosscheck of JVET-AD0158 (EE2-1.13: Test on Fusion of Intra Template Matching) [F. Wang (OPPO)] [late] [miss]

JVET-AD0291 Crosscheck of JVET-AD0158 (EE2-1.13: Test on Fusion of Intra Template Matching) [X. Li (Alibaba)] [late] [miss]

JVET-AD0165 EE2-2.4: Template matching-based subblock motion refinement [L. Zhao, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0290 Crosscheck of JVET-AD0165 (EE2-2.4: Template matching-based subblock motion refinement) [R.-L. Liao (Alibaba)] [late] [miss]

JVET-AD0171 EE2-3.1: TMP using Reconstruction-Reordered for screen content coding (RR-TMP) [D. Ruiz Coll, O. Patino, P. Andrivon (Ofinno)]

JVET-AD0306 crosscheck of JVET-AD0171 (EE2-3.1: TMP using Reconstruction-Reordered for screen content coding (RR-TMP)) [K. Naser (InterDigital)] [late] [miss]

JVET-AD0173 EE2-1.22: Template-based intra MPM list construction [C. Zhou, Z. Lv, J. Zhang (vivo)]

JVET-AD0337 Crosscheck of JVET-AD0173 (EE2-1.22: Template-based intra MPM list construction) [J. Chen (Alibaba)] [late] [miss]

JVET-AD0177 EE2-1.18: CIIP Extension with IntraTMP [K. Naser, P. Bordes, F. Galpin, A. Robert (InterDigital)]

JVET-AD0259 Crosscheck of JVET-AD0177 (EE2-1.18: CIIP Extension with IntraTMP) [F. Wang (OPPO)] [late] [miss]

JVET-AD0182 EE2-2.1: On Affine DMVR [M. Blestel, M. Radosavljević (Xiaomi), Y. Zhang, H. Huang, V. Seregin, M. Karczewicz (Qualcomm), J. Chen, R. -L. Liao, X. Li, Y. Ye (Alibaba)]

JVET-AD0255 Cross-check of JVET-AD0182 ("EE2-2.1: On Affine DMVR", Test EE2-2.1g and Test EE2-2.1i) [V. Rufitskiy, A. Filippov (Ofinno)] [late] [miss]

JVET-AD0281 Cross-check of JVET-AD0182 ("EE2-2.1: On Affine DMVR", Test EE2-2.1c and Test EE2-2.1g) [W. Chen (Kwai)] [late] [miss]

JVET-AD0283 Cross-check of JVET-AD0182 ("EE2-2.1: On Affine DMVR", Test EE2-2.1l and Test EE2-2.1n) [L. Zhao (Bytedance)] [late] [miss]

JVET-AD0323 Cross-check of JVET-AD0182 (EE2-2.1: On Affine DMVR) tests 2.1a, 2.1b, 2.1f and 2.1k [F. Le Léannec (InterDigital)] [late] [miss]

JVET-AD0353 Crosscheck of JVET-AD0182 (EE2-2.1: On Affine DMVR) [C.-M. Tsai (MediaTek)] [late] [miss]

JVET-AD0188 EE2-1.6: Non-local cross-component prediction and cross-component merge mode [K. Zhang, L. Zhang, Z. Deng (Bytedance), C.-M. Tsai, H.-Y. Tseng, C.-Y. Chuang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Chen, Y.-W. Huang, S.-M. Lei (Mediatek)]

JVET-AD0324 Crosscheck of JVET-AD0188 (EE2-1.6b: Non-local cross-component prediction and cross-component merge mode) [Y.-J. Chang (Qualcomm) [late]

JVET-AD0336 Crosscheck of JVET-AD0188 (EE2-1.6: Non-local cross-component prediction and cross-component merge mode) [J. Chen (Alibaba)] [late] [miss]

JVET-AD0355 Crosscheck of JVET-AD0188 (EE2-1.6a: Non-local cross-component prediction and cross-component merge mode) [C.-W. Kuo (Kwai)] [late]

JVET-AD0190 EE2-1.23: Modification to MPM list derivation [H. Wang, Y.-J. Chang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AD0264 Crosscheck of JVET-AD0190 EE2-1.23: Modification to MPM list derivation [Y. Yasugi (Sharp)] [late] [miss]

JVET-AD0191 EE2-1.20e: Combination Test of Test 1.10 + Test 1.13 [J. R. Arumugam, A. Natesan, V. Valvaiker, A. Sarate, J. N. Shingala (Ittiam), F. Pu, T. Lu, P. Yin, S. McCarthy (Dolby), F. Wan (OPPO)]

JVET-AD0296 Crosscheck of JVET-AD0191 (EE2-1.20e: Combination Test of Test 1.10 + Test 1.13) [X. Li (Alibaba)] [late] [miss]

JVET-AD0192 EE2-1.20f: Combination Test of Test 1.12 + Test 1.13b [F. Pu, T. Lu, P. Yin, S. McCarthy (Dolby), J. R. Arumugam, A. Natesan, V. Valvaiker, A. Sarate, J. N. Shingala (Ittiam), X. Li, R.-L. Liao, J. Chen (Alibaba)]

JVET-AD0262 Crosscheck of JVET-AD0192 (EE2-1.20f: Combination Test of Test 1.12 + Test 1.13b) [F. Wang (OPPO)] [late] [miss]

JVET-AD0193 EE2-2.11e: Combination of EE2-2.8, EE2-2.9 and EE2-2.10 [K. Cui, Z. Zhang, H. Huang, V. Seregin, M. Karczewicz (Qualcomm), X. Li (Google), Z. Deng, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0289 Crosscheck of JVET-AD0193 (EE2-2.11e: Adaptive OBMC control) [X. Xiu (Kwai)] [late] [miss]

JVET-AD0360 Crosscheck of JVET-AD0193 on non-CTC mixed content sequences (EE2-2.11e: Adaptive OBMC control) [J. Gan (OPPO)] [late]

JVET-AD0194 EE2-1.19: IntraTMP with multiple modes [P.-H. Lin, J.-L- Lin, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AD0263 Crosscheck of JVET-AD0194 (EE2-1.19: IntraTMP with multiple modes) [F. Wang (OPPO)] [late] [miss]

JVET-AD0195 EE2-2.6: High-Precision MV Refinement for BDOF [M. Salehifar, Y. He, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0198 EE2-1.20c: Combination of Test1.14 and Test1.19 [P.-H. Lin, J.-L- Lin, V. Seregin, M. Karczewicz (Qualcomm), F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO), Y. Ma, J. Huo, F. Yang (Xidian Univ.)]

JVET-AD0279 Crosscheck of JVET-AD0198 (EE2-1.20c: Combination of Test1.14 and Test1.19) [H.-J. Jhu (Kwai)] [late] [miss]

JVET-AD0199 EE2-3.2: IBC MBVD list derivation [Z. Zhang, P. Nikitin, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AD0244 Crosscheck of JVET-AD0199 EE2-3.2: IBC MBVD list derivation [D. Ruiz Coll (Ofinno) [late] [miss]

JVET-AD0202 EE2-1.2: CCCM using multiple downsampling filters [Y.-J. Chang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AD0349 Crosscheck of JVET-AD0202 (EE2-1.2: CCCM using multiple downsampling filters) [J. Lainema (Nokia)] [late]

JVET-AD0208 EE2-1.8/1.9: IBC adaptation for coding of natural content [C.-C. Chen, B. Ray, M. Coban, V. Seregin, M. Karczewicz (Qualcomm), W. Chen, X. Xiu, C. Ma, H.-J. Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai)]

JVET-AD0299 Crosscheck of JVET-AD0208 (EE2-1.8/1.9: IBC adaptation for coding of natural content) [Y. Kidani, K. Kawamura (KDDI)] [late] [miss]

JVET-AD0209 EE2-2.3: SbTMVP with MMVD [L.-F. Chen, R. Chernyak, X. Zhao, X. Xu, S. Liu (Tencent)]

JVET-AD0280 Crosscheck of JVET-AD0209 (EE2-2.3: SbTMVP with MMVD) [X. Xiu (Kwai)] [late] [miss]

JVET-AD0210 EE2-2.5b: Combined test of Test 2.2 and Test 2.3 [L.-F. Chen, R. Chernyak, X. Zhao, X. Xu, S. Liu (Tencent), R.-L. Liao, J. Chen, Y. Ye, X. Li (Alibaba)]

JVET-AD0230 Crosscheck of JVET-AD0210 EE2-2.5b: Combined test of Test 2.2 and Test 2.3 [X. Li (Google)] [late] [miss]

JVET-AD0213 EE2-Test2.7: Improvements on local illumination compensation [X. Xiu, N. Yan, C. Ma, H.-J. Jhu, C.-W. Kuo, W. Chen, X. Wang (Kwai)]

JVET-AD0225 Crosscheck of JVET-AC0213 on EE2-Test2.7: Improvements on local illumination compensation [L.-F. Chen (Tencent)] [late] [miss]

JVET-AD0219 EE2-4.2/4.3/4.4a: Residual based classifier and modified filter shape for ALF [I. Jumakulyyev, N. Hu, Z. Zhang, V. Seregin, M. Karczewicz, H. Huang (Qualcomm)]

JVET-AD0227 Crosscheck of JVET-AD0219 (EE2-4.2/4.3/4.4a: Residual based classifier and modified filter shape for ALF) [W. Yin (Bytedance)] [late] [miss]

JVET-AD0221 EE2-4.1: Additional Fixed Filter for ALF [W. Yin, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0228 Crosscheck of JVET-AD0221 (EE2-4.1: Additional Fixed Filter for ALF) [N. Hu (Qualcomm)] [late] [miss]

JVET-AD0222 EE2-4.4b and 4.4c: Combined Tests for ALF [I. Jumakulyyev, N. Hu, Z. Zhi, V. Seregin, M. Karczewicz, H. Huang (Qualcomm), W. Yin, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0341 Cross-check of JVET-AD0222 (EE2-4.4b and 4.4c: Combined Tests for ALF) [K. Andersson (Ericsson)] [late] [miss]

JVET-AD0247 EE2-1.20n: Combination Test of Test 1.20f + Test 1.14c [F. Pu, T. Lu, P. Yin, S. McCarthy (Dolby), J. R. Arumugam, A. Natesan, V. Valvaiker, A. Sarate, J. N. Shingala (Ittiam), Xinwei Li, Ru-Ling Liao, Jie Chen (Alibaba), Y. Ma, J. Huo, H. Du, H. Zhang, Z. Zhang, W. Qiao, F. Yang (Xidian), F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)] [late]

JVET-AD0269 Crosscheck of JVET-AD0247 (EE2-1.20n: Combination Test of Test 1.20f + Test 1.14c) [R. G. Youvalari (Nokia)] [late] [miss]

JVET-AD0256 EE2-2.12h: Combined test of Test 2.7 and Test 2.12a [A. Filippov, V. Rufitskiy, K. Suverov (Ofinno), X. Xiu, N. Yan, C. Ma, H.-J. Jhu, C.-W. Kuo, W. Chen, X. Wang (Kwai)] [late]

JVET-AD0274 Crosscheck of JVET-AD0256 (EE2-2.12h: Combined test of Test 2.7 and Test 2.12a) [M. Blestel (Xiaomi)] [late] [miss]

JVET-AD0257 EE2-2.12g: Combined test of Test 2.2 and Test 2.12a [V. Rufitskiy, A. Filippov, K. Suverov (Ofinno), R.-L. Liao, J. Chen, Y. Ye, X. Li (Alibaba)] [late]

JVET-AD0275 Crosscheck of JVET-AD0257 (EE2-2.12g: Combined test of Test 2.2 and Test 2.12a) [M. Blestel (Xiaomi)] [late] [miss]

### EE2 related contributions (10)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0048 EE2-related: Cross-component merge mode with temporal candidates [H.-Y. Tseng, C.-M. Tsai, C.-Y. Chuang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-AD0084 EE2-related: On search range of intraTMP [Y. Yu, L. Zhang, F. Wang, J. Gan, H. Yu, L. Xu, Z. Xie, D. Wang (OPPO)]

JVET-AD0286 Crosscheck of JVET-AD0084 (EE2-related: On search range of intraTMP) [X. Xiu (Kwai)] [late] [miss]

JVET-AD0117 EE2-related: Search area extension of bottom-left and top-right regions for IntraTMP mode [Y.-Z. Ma, H.-L. Zhang, H.-Q. Du, W.-H. Qiao, J.-Y. Huo, F.-Z. Yang (Xidian Univ.)]

JVET-AD0144 EE2-related: FTMP multi-filter extend based on gradient and location information [X. Zhang, D. Jiang, J.-C. Lin, X.-H. Zhang, D.-Q. Cheng, C.-H. Zheng, Z.-Q. Wang, Y.-F. Lu (Dahua)]

JVET-AD0146 EE2-related: EE2-1.15a improvement [S. Peng, X. Zhang, C. Fang, D. Jiang, J.-C. Lin, K. Fu, P. Zhang (Dahua)]

JVET-AD0178 non EE2-4.3: IntraTMP Padding [K. Naser, T. Poirier, A. Robert, H. Guermoud (InterDigital)]

JVET-AD0196 EE2-related: On Iterative BDOF and EE2-2.6 Improvement [M. Salehifar, Y. He, K. Zhang, H. Liu, L. Zhang (Bytedance)]

JVET-AD0348 Crosscheck of JVET-AD0196 (EE2-related: Iterative BDOF and EE2-2.6 Improvement) [Z. Lv (vivo)] [late] [miss]

JVET-AD0200 EE2-related: InterMTS for IBC and IntraTMP [P. Garus, M. Coban, B. Ray, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AD0203 EE2-related: Reconstruction-Reordered TMP based on linear filter model for screen content coding (RRTMP-LFM) [D. Ruiz Coll, O. Patino, P. Andrivon (Ofinno)] [late] [miss]

JVET-AD0342 EE2-related: Combination of JVET-AD0084, JVET-AD0117 and JVET-AD0214 [Y. Yu, L. Zhang, F. Wang, J. Gan, H. Yu, L. Xu, Z. Xie, D. Wang (OPPO), Y. Ma, H. Zhang, H. Du, W. Qiao, J. Huo, (Xidian University), X. Xiu, N. Yan, C. Ma, H. Jhu, C. Kuo, W. Chen, (Kwai)] [late] [miss]

JVET-AD0359 Crosscheck of JVET-AD0342 (EE2-related: Combination of JVET-AD0084, JVET-AD0117 and JVET-AD0214) [X. Li (Alibaba)] [late] [miss]

### ECM modifications and software improvements beyond EE2 (36)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0064 AHG12: Enhanced geometrical prediction mode [R. Yu, P. Wennersten, J. Enhorn, K. Andersson (Ericsson)]

JVET-AD0318 Crosscheck of JVET-AD0064 (AHG12: Enhanced geometrical prediction mode) [Y. Ahn (LGE)] [late] [miss]

JVET-AD0080 Non-EE2: Chroma-to-chroma convolutional cross-component model (C2C-CCCM) for intra prediction [Y. Huo, Y. Liu (Transsion)]

JVET-AD0081 Non-EE2: An extrapolation filter-based intra prediction mode [L. Xu, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AD0271 Crosscheck of JVET-AD0081 (Non-EE2: An extrapolation filter-based intra prediction mode) [C. Zhou (vivo)] [late] [miss]

JVET-AD0357 Crosscheck of JVET-AD0081 (Non-EE2: An extrapolation filter-based intra prediction mode) [H.-J. Jhu, X. Xiu (Kwai)] [late] [miss]

JVET-AD0083 Non-EE2: Improvement over IBC-LIC [Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AD0100 AHG12: Block vector guided CCCM [R. G. Youvalari, D. Bugdayci Sansli, P. Astola, J. Lainema (Nokia)] [late]

JVET-AD0108 AHG12: Cross-component residual model (CCRM) for inter prediction [P. Astola, J. Lainema (Nokia)]

JVET-AD0288 Crosscheck of JVET-AD0108 (AHG12: Cross-component residual model (CCRM) for inter prediction) [Z. Deng (ByteDance)] [late] [miss]

JVET-AD0114 Non-EE2: Modification of MV redundancy/similarity check considering LIC during merge candidate list construction [K. Kim, D. Kim, J.-H. Son, J.-S. Kwak (WILUS)]

JVET-AD0124 AHG 12: Fix related to template type selection in Intra TMP [D. Kim, K. Kim, J.-H. Son, J.-S. Kwak (WILUS)]

JVET-AD0127 Non-EE2: Harmonization between IBC HMVP and IBC-LIC [N. Zhang, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0331 Crosscheck of JVET-AD0127(Non-EE2: Harmonization between IBC HMVP and IBC-LIC) [Z. Xie (OPPO)] [late] [miss]

JVET-AD0130 AHG12: On ECM partitioning prediction [G. Laroche, P. Onno (Canon)]

JVET-AD0134 Non-EE2: Bi-predictive IBC for natural and screen content [Y. Kidani, H. Kato, K. Kawamura (KDDI)]

JVET-AD0313 Crosscheck of JVET-AD0134 (Non-EE2: Bi-predictive IBC for natural and screen content) [C.-C. Chen (Qualcomm)] [late] [miss]

JVET-AD0142 Non-EE2: bugfix for intraTMP [X. Zhang, D. Jiang, J.-C. Lin, X.-H. Zhang, D.-Q. Cheng, C.-H. Zheng, Z.-Q. Wang, Y.-F. Lu (Dahua)]

JVET-AD0148 Non-EE2: SGPM combined with multiple IntraTMP predictors [C. Fang, S. Peng, D. Jiang, J.-C. Lin, X. Zhang, H. Jin, X.-M. Shi, F. Ye (Dahua)]

JVET-AD0150 Non-EE2: combination of JVET-AD0146 and JVET-AD0148 [C. Fang, S. Peng, X. Zhang, D. Jiang, J.-C. Lin, H. Jin, X.-M. Shi, K. Fu, P. Zhang, F. Ye (Dahua)]

JVET-AD0155 Non-EE2: High-Accuracy template matching [Y. Wang, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0298 Crosscheck of JVET-AD0155 (Non-EE2: High-Accuracy template matching) [X. Li (Alibaba)] [late] [miss]

JVET-AD0169 AHG12: RPR filters for scale factors below 1.5x [J. Samuelsson-Allendes, S. Deshpande (Sharp)]

JVET-AD0345 Cross-check of JVET-AD0169 (AHG12: RPR filters for scale factors below 1.5x) [K. Andersson (Ericsson)] [late] [miss]

JVET-AD0172 AHG12: Adaptation of LFNST/NSPT for low-delay configuration [C. Bonnineau, F. Le Léannec, K. Naser, T. Poirier, S. Puri (InterDigital)]

JVET-AD0325 Crosscheck of JVET-AD0172 (AHG12: Adaptation of LFNST/NSPT for low-delay configuration) [M. Radosavljević (Xiaomi)] [late] [miss]

JVET-AD0174 On CIIP improvement [C. Zhou, Z. Lv, J. Zhang (vivo)]

JVET-AD0249 Crosscheck of JVET-AD0174: Non-EE2: CIIP with more combination candidates [L. Xu (OPPO)] [late] [miss]

JVET-AD0317 Crosscheck of JVET-AD0174 (Non-EE2: CIIP with more combination candidates) [Z. Deng (Bytedance)] [late] [miss]

JVET-AD0176 Non-EE2: Improvements on multi-pass DMVR [J. Chen, R.-L. Liao, X. Li, Y. Ye (Alibaba)]

JVET-AD0179 [AHG12] Extending TIMD with IntraTMP [K. Naser, P. Bordes, K. Reuze, F. Galpin (InterDigital)]

JVET-AD0180 Non-EE2: Affine AMVP mode with one MVD [H. Huang, V. Seregin, Z. Zhang, M. Karczewicz (Qualcomm)]

JVET-AD0183 Non-EE2: SIF flag and BCW weight usage in OBMC [A. Robert, F. Galpin, T. Poirier, Y. Chen (InterDigital)]

JVET-AD0316 Crosscheck of JVET-AD0183 (Non-EE2: SIF flag and BCW weight usage in OBMC) [Z. Deng (Bytedance)] [late] [miss]

JVET-AD0184 Non-EE2: Removal of Division Operations [W. Jia, K. Zhang, Z. Deng, L. Zhang (Bytedance)]

JVET-AD0225 Crosscheck of JVET-AC0184 on EE2-Test2.7: Improvements on local illumination compensation [L.-F. Chen (Tencent)] [late] [miss]

JVET-AD0273 Crosscheck of JVET-AD0184 (Non-EE2: Removal of Division Operations) [C. Zhou (vivo)] [late] [miss]

JVET-AD0185 Non-EE2: ISP Extension for Chroma [W. Jia, K. Zhang, Y. Wang, L. Zhang (Bytedance)]

JVET-AD0187 Non-EE2: On large NSPT [M. Koo, J. Zhao, J. Lim, S. Kim (LGE)]

JVET-AD0204 AHG12: Context modeling for transform coefficients for LFNST/NSPT [P. Nikitin, M. Coban, B. Ray, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AD0206 AHG12: CABAC Initialization for GDR Pictures [S. Hong, L. Wang, K. Panusopone (Nokia)]

JVET-AD0352 Crosscheck of JVET-AD0206 (AHG 12: CABAC Initialization for GDR Pictures) [J. Enhorn (Ericsson)] [late] [miss]

JVET-AD0214 Non-EE2: On search region of intra template matching in ECM8.0 [X. Xiu, N. Yan, C. Ma, H.-J. Jhu, C.-W. Kuo, W. Chen, X. Wang (Kwai)]

JVET-AD0285 Crosscheck of JVET-AD0214 (Non-EE2: On search region of intra template matching in ECM8.0) [Y. Yu (OPPO)] [late] [miss]

JVET-AD0215 Non-EE2: Extension of IBC-GPM [C. Ma, X. Xiu, W. Chen, H.-J. Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai)]

JVET-AD0216 Non-EE2: IBC with non-adjacent spatial candidates [C. Ma, X. Xiu, W. Chen, H.-J. Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai)]

JVET-AD0344 Crosscheck of JVET-AD0216 (Non-EE2: IBC with non-adjacent spatial candidates) [Y. Wang (Bytedance)] [late] [miss]

JVET-AD0217 Non-EE2: Filtered Intra Block Copy (FIBC) [H.-J. Jhu, X. Xiu, C.-W. Kuo, W. Chen, N. Yan, C. Ma, X. Wang (Kwai)]

JVET-AD0356 Crosscheck of JVET-AD0217 (Non-EE2: Filtered Intra Block Copy (FIBC) [L. Zhang, Y. Yu (OPPO)] [late] [miss]

JVET-AD0218 AHG12: CCSAO with extended edge classifiers and history offsets [C.-W. Kuo, X. Xiu, W. Chen, H.-J. Jhu, N. Yan, C. Ma, X. Wang (Kwai)] [late]

Initial version rejected as “placeholder”.

JVET-AD0220 Non-EE2: Improved fixed filters for ALF [M. Karczewicz, N. Hu, V. Seregin (Qualcomm)]

JVET-AD0223 Non-EE2: Filtering for IBC predicted block [B. Ray, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AD0224 AHG12: IBC Merge list construction modification based on the candidates' Reconstruction-Reordered type [D. Ruiz Coll, O. Patino (Ofinno)] [late] [miss]

JVET-AD0231 Non-EE2: Non-adjacent spatial candidates for IBC [Y. Wang, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0350 Crosscheck of JVET-AD0231 (Non-EE2: Non-adjacent spatial candidates for IBC) [C. Ma (Kwai)] [late] [miss]

JVET-AD0234 Non-EE2: Extensions of Residual-based Taps in ALF and CCALF [W. Yin, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0235 Non-EE2: Enhanced subblock-based motion compensation [L. Zhao, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0236 Non-EE2: On Classification of In-Loop Filters [W. Yin, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0335 Crosscheck of JVET-AD0236 (Non-EE2: On Classification of In-Loop Filters) [V. Shchukin, J. Ström (Ericsson)] [late] [miss]

JVET-AD0239 Non-EE2: Fixes for IntraTMP template availability [G. Verba, Z. Zhang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AD0251 AhG12: Shifting Quantizer Center [M. Balcilar, K. Naser, F. Galpin, F. Le Léannec (InterDigital)] [late]

JVET-AD0308 Non-EE2: Extension of unified PDPC [G. Rath, T. Dumas, F. Le Léannec, K. Reuzé (InterDigital)] [late]

JVET-AD0315 Non-EE2: Directional DC modes [K. Kim, D. Kim, J.-H. Son, J.-S. Kwak (WILUS)] [late] [miss]

JVET-AD0338 Non-EE2: On MHP weight derivation [Y. Zhang, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)] [late]

### CTC for EE2/ECM (4)

JVET-AD0105 Modifications of common test condition and ECM software [Y. Yasugi, T. Ikai (Sharp)]

JVET-AD0128 AHG12: modified CTC proposal for low-delay configurations [F. Le Léannec, F. Galpin, K. Naser, T. Poirier, E. François (InterDigital)]

JVET-AD0268 Crosscheck of JVET-AD0128 (Ahg12: modified CTC proposal for low-delay configurations) [M. Blestel (Xiaomi)] [late] [miss]

JVET-AD0229 AHG4/AHG12: Mixed-Content Sequences for Enhanced Compression Tool Testing [X. Li (Google), Z. Deng, K. Zhang, L. Zhang (Bytedance)]

Also in 4.9

JVET-AD0197 EE2-related: Worst-Case Timing Reduction for ECM RA Simulations [M. Salehifar, Y. Wang, Y. He, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0242 On Common Test Conditions document [R. Chernyak, L.-F. Chen, X. Xu, S. Liu (Tencent)]

# High-level syntax (HLS) and related proposals (40)

## AHG9: SEI messages on neural-network post filter and SEI processing order (32)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0258 AHG9: A summary of proposals on NNPF and SEI processing order SEI messages [Y.-K. Wang (Bytedance)] [late]

JVET-AD0052 AHG9: NNPFC SEI extension mechanism to enable usages beyond user viewing [M. M. Hannuksela, F. Cricri, H. Zhang (Nokia)]

This contribution also fits under section 4.10.

JVET-AD0053 AHG9: On NNPFC SEI for filtering input pictures and upsampling picture rate [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

JVET-AD0054 AHG9: On NNPFC SEI for picture rate upsampling [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

JVET-AD0055 AHG9: On buffering requirements for multi-picture NNPFC SEI [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AD0056 AHG9: Miscellaneous VSEI changes on neural-network post-filter SEI messages [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

JVET-AD0057 AHG9: Miscellaneous VVC changes on SEI messages [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

JVET-AD0057 items 1, 2, 5 and 6 belong to this category.

JVET-AD0058 AHG9: Approaches for region-based neural-network post-filtering [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AD0059 AHG9: Indicating processing order of NNPFs and SEI messages [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AD0059 item 2 overlaps with JVET-AD0061 and JVET-AD0079.

JVET-AD0061 AHG9: On the SEI processing order SEI message [Y. He, M. Coban, M. Karczewicz (Qualcomm)]

JVET-AD0079 AHG9: On the SEI processing order SEI message [P. Yin, S. McCarthy, W. Husak, K. Konstantinides, T. Lu, F. Pu, A. Arora, T. Shao (Dolby)]

JVET-AD0065 AHG9: On NNPFC Auxiliary Input Information [S. Deshpande (Sharp)]

JVET-AD0066 AHG9: On NNPFC Description and Identification [S. Deshpande (Sharp)]

JVET-AD0067 AHG9: Comments on Neural-Network Post-Filter Characteristics Message [S. Deshpande (Sharp)]

JVET-AD0078 AHG9: On comments for the draft of VSEI [T. Chujoh, Y. Yasugi, T. Ikai (Sharp)]

JVET-AD0078 items 1 to 3 belong to this category.

JVET-AD0087 AHG9: On cascading of post-processing filters [Y.-K. Wang, L. Zhang, J. Xu, C. Lin, Y. Li, J. Li, K. Zhang (Bytedance)]

JVET-AD0088 AHG9: Overall post-processing filtering process using NNPFs [Y.-K. Wang, J. Xu, L. Zhang (Bytedance)]

JVET-AD0089 AHG9: NNPF activation parameters [Y.-K. Wang, Y. Li, J. Xu, C. Lin, J. Li, L. Zhang, K. Zhang (Bytedance)]

JVET-AD0090 AHG9: On input pictures to an NNPF [Y.-K. Wang, L. Zhang (Bytedance)]

JVET-AD0091 AHG9: Miscellaneous NNPF topics (set 1) [Y.-K. Wang, J. Xu, C. Lin, Y. Li, J. Li, L. Zhang, K. Zhang (Bytedance)]

JVET-AD0093 AHG9: Neural-network post-processing filter with downsampling capabilities [Y. Li, J. Xu, Y.-K. Wang, C. Lin, J. Li, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0094 AHG9: On the NNPFC SEI message involving depth pictures [C. Lin, Y.-K. Wang, J. Xu, Y. Li, J. Li, K. Zhang, L. Zhang (Bytedance)]

JVET-AD0104 AHG9: On NNPF Overlap [A. Sidiya, S. Deshpande (Sharp)]

JVET-AD0141 AHG9: On discardable and non-output pictures for NNPF SEI messages [Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE)]

JVET-AD0143 AHG9: On picture size for input and output pictures of NNPFC SEI message [Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE)]

JVET-AD0145 AHG9: On non-consecutive input pictures for NNPFC SEI message [Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE)]

JVET-AD0149 AHG9: On carriage of NNPF bitstream only in NNPFC SEI message [Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE)]

JVET-AD0151 AHG9: On output picture design in NNPFC SEI message [Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE)]

JVET-AD0154 AHG9: Additional purposes for the NNPFC SEI message [M. Pettersson, M. Damghanian, R. Sjöberg (Ericsson)]

JVET-AD0159 AHG9: Explicit identification of NNPF input pictures [R. Sjöberg, M. Pettersson, M. Damghanian (Ericsson)]

JVET-AD0233 AHG9: Miscellaneous NNPF topics (set 2) [J. Xu, Y.-K. Wang, C. Lin, Y. Li, J. Li, L. Zhang, K. Zhang (Bytedance)]

JVET-AD0240 AHG9: Support more NNPF purposes [J. Xu, Y.-K. Wang, L. Zhang, J. Li, C. Lin (Bytedance)]

## AHG9: SEI messages on topics other than NNPF and SEI processing order (8)

Contributions in this area were discussed at XXXX–XXXX on XXday 2X April 2023 (chaired by JRO).

JVET-AD0051 AHG9: Common SEI Message of Generative Face Video [B. Chen, J. Chen, Y. Ye (Alibaba), S. Wang (CityU)]

JVET-AD0057 AHG9: Miscellaneous VVC changes on SEI messages [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

JVET-AD0057 items 3 and 4 belong to this category.

JVET-AD0060 AHG8/AHG9: Indications related to suboptimal user viewing [M. M. Hannuksela, F. Cricri, H. Zhang (Nokia)]

This contribution also fits under section 4.10.

JVET-AD0078 AHG9: On comments for the draft of VSEI [T. Chujoh, Y. Yasugi, T. Ikai (Sharp)]

JVET-AD0078 items 4 and 5 belong to this category.

JVET-AD0121 AHG9: Attenuation Map Information SEI for reducing energy consumption of displays [O. Le Meur, C. H. Demarty, F. Aumont, E. François, L. Blonde, E. Reinhard (InterDigital)]

JVET-AD0137 [AHG8] SEI for encoder optimization information [C. Kim, Hendry, D. Gwak, J. Lim (LGE)]

This contribution also fits under section 4.10.

JVET-AD0161 AHG9: Alternative Output Timing Hint SEI [H.-B. Teo, J. Gao, C.-S. Lim, K. Abe (Panasonic)]

JVET-AD0175 AHG9: On object mask auxiliary picture and object mask information SEI message for VSEI and HEVC [J. Chen, S. Wang, Y. Ye (Alibaba)]

## Non-SEI HLS aspects (0)

Section kept as a template for future use.

# Plenary meetings, joint meetings, BoG reports, and liaison communications

## JVET plenaries

No intermediate plenaries were held, as document review and decisions were made in single-track mode at this meeting (with some BoG activity as noted). Further detail on scheduling is recorded in section 2.13.

Communication and coordination items were discussed on Monday April 24 at XXXX

- Joint meetings:

  - With XXX on XXX: XXday 2X April XXXX (1 hr.)

  - …

- Session planning (participants were encouraged to see the posted online calendar)

- Preparation of DoCRs: XXX (draft DoCR and preliminary text of new edition to be produced)

General plenary wrap-up discussions are recorded under sections 8, 9, and 10.

## Information sharing meetings

Information sharing sessions with other WGs and AGs of the MPEG community were held on Monday 24 April 0900–1130, Wednesday 26 April 0900–1000, and Friday 28 April 1400–XXXX. The status of the work in the MPEG WGs and AGs was reviewed at these information sharing sessions.

## Joint meetings with MPEG AG/WG X on XXXX

## BoGs (1)

The following break-out groups were established at this meeting to conduct discussion and develop recommendations on particular subjects

JVET-AD0362 BoG report on NNPF and SEI processing order SEI messages [Y.-K. Wang, M. M. Hannuksela, S. Deshpande (BoG coordinators)]

See section X for notes on this BoG report.

## Liaison communications

m62300 Liaison statement from SC 29/WG 1 to WG 2, WG 4, and WG 5 on JPEG AI [WG 1 via SC 29 Secretariat]

A reply was drafted by JVET as WG 5 N 191. This was reviewed in session 24.

# Project planning

## Software timeline

ECM8 software (including all adoptions) was planned to be available 3 weeks after the meeting.

The NNVC 4.0 codebase software was planned to be available 4 weeks after the meeting.

VTM20.0 software was planned to be available on 2022-02-17. (Note that further updates may be released later)

Updates on top of HM17.0 software were not planned, but might be released after merging pending requests, as appropriate.

## Core experiment and exploration experiment planning

An EE on neural network-based video coding was established, as recorded in output document JVET-AC2023.

An EE on enhanced compression technology beyond VVC capability using techniques other than neural-network technology was also established, as recorded in output document JVET-AC2024.

Initial versions of these documents were presented and approved in the first plenary session on Friday 20 January.

## Drafting of specification text, encoder algorithm descriptions, and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM/VTM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- VTM algorithm description text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for experiments

It was emphasized that those rules which had been set up or refined during the 12th JVET meeting should be observed. In particular, for some CEs of some previous meetings, results were available late, and some changes in the experimental setup had not been sufficiently discussed on the JVET reflector.

Group coordinated experiments have been planned as follows:

- “Core experiments” (CEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of a draft standard by the next meeting or in the near future.

- “Exploration experiments” (EEs) are also coordinated experiments. These are conducted on technology which is not foreseen to become part of a draft standard in the near future. The investigating methodology for assessment of such technology can also be an important part of an EE. (Further general rules for EEs, as far as deviating from the CE rules below, should be discussed in a future meeting. For the current meeting, procedures as described in the EE description document are deemed to be sufficient.)

- A CE is a test of a specific fully described technology in a specific agreed way. It is not a forum for thinking of new ideas (like an AHG). The CE coordinators are responsible for making sure that the CE description is complete and correct and has adequate detail. Reflector discussions about CE description clarity and other aspects of CE plans are encouraged.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. The experiment description document should provide the names of individual people, not just company names.

- Software for tools investigated in a CE will be provided in one or more separate branches of the software repository. Each CE will have a “fork” of the software, and within the CE there may be multiple branches established by the CE coordinator. The software coordinator will help coordinate the creation of these forks and branches and their naming. All JVET members will have read access to the CE software branches (using shared read-only credentials as described below).

- During the experiment, revisions of the experiment plans can be made, but not substantial changes to the proposed technology.

- The CE description must match the CE testing that is done. The CE description needs to be revised if there has been some change of plans.

- The CE summary report must describe any changes that were made in the process of finalizing the CE.

- By the next meeting it is expected that at least one independent cross-checker will report a detailed analysis of each proposed feature that has been tested and confirm that the implementation is correct. Commentary on the potential benefits and disadvantages of the proposed technology in cross-checking reports is highly encouraged. Having multiple cross-checking reports is also highly encouraged (especially if the cross-checking involves more than confirmation of correct test results). The reports of cross-checking activities may (and generally should) be integrated into the CE report rather than submitted as separate documents.

- It is mandatory to report encoder optimizations made for the benefit of a tool, and if an equivalent optimization could be applied on the anchor, a comparison against the improved anchor shall be provided.

- A new proposal can be included in a CE based on group decision, regardless if an independent party has already performed a cross-check in the meeting when it was first proposed.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the group test model software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments for SDR video are described in the prior output document JVET-T2010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a proponent perspective – e.g. not referring to methods as “improved”, “optimized”, “enhanced”, etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions contain a basic description of the technology under test, but should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in documents that are referenced in the CE description that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

The CE development workflow is described at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/Core-experiment-development-workflow

CE read access is available using shared accounts: One account exists for MPEG members, which uses the usual MPEG account data. A second account exists for VCEG members with account information available in the TIES informal ftp area (IFA) system at:

https://www.itu.int/ifa/t/2017/sg16/exchange/wp3/q06/vceg_account.txt

Some agreements relating to CE activities were established as follows:

- Only qualified JVET members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting. Participation is requested by contacting the CE coordinator.

- All software, results, and documents produced in the CE should be announced and made available to JVET in a timely manner.

- A JVET CE reflector will be established and announced on the main JVET reflector. Discussion of logistics arrangements, exchange of data, minor refinement of the test plans, and preparation of documents shall be conducted on the JVET CE reflector, with subject lines prefixed by “[CEx: ]”, where “x” is the number of the CE. All substantial communications about a CE other than such details shall take place on main JVET reflector. In the case that large amounts of data are to be distributed, it is recommended to send a link to the data rather than the data itself, or upload the data as an input contribution to the next meeting.

General timeline for CEs

T1= 3 weeks after the JVET meeting: To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to be marked as “withdrawn”.

T2 = Test model software release + 2 weeks: Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3.

- 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description shall be documented in an updated CE description by this time.

T4: Regular document deadline minus 1 week: CE contribution documents including specification text and complete test results shall be uploaded to the JVET document repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting).

The CE summary reports shall be available by the regular contribution deadline. This shall include documentation about crosscheck of software, matching of CE description and confirmation of the appropriateness of the text change, as well as sufficient crosscheck results to create evidence about correctness (crosscheckers must send this information to the CE coordinator at least 3 days ahead of the document deadline). Furthermore, any deviations from the timelines above shall be documented. The numbers used in the summary report shall not be changed relative to the description document.

CE reports may contain additional information about tests of straightforward combinations of the identified technologies. Such supplemental testing needs to be clearly identified in the report if it was not part of the CE plan.

New branches may be created which combine two or more tools included in the CE document or the VTM (as applicable).

It is not necessary to formally name cross-checkers in the initial version of the CE description document. To adopt a proposed feature at the next meeting, JVET would like to see comprehensive cross-checking done, with analysis of whether the description matches the software, and a recommendation of the value of the tool and given tradeoffs.

The establishment of a CE does not indicate that a proposed technology is mature for adoption or that the testing conducted in the CE is fully adequate for assessing the merits of the technology, and a favourable outcome of CE does not indicate a need for adoption of the technology into a standard or test model.

Availability of specification text is important to have a detailed understanding of the technology and also to judge what its impact on the complexity of the specification will be. There must also be sufficient time to study this in detail. CE contributions without sufficiently mature draft specification text in the CE input document should not be considered for adoption.

Lists of participants in CE documents should be pruned to include only the active participants. Read access to software will be available to all members.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JVET reflector (jvet@lists.rwth-aachen.de).

Review of AHG plans was conducted during the plenary on Friday 28 April 2023 at XXXX.

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Project Management (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate overall JVET interim efforts. - Supervise AHG and experiment studies. - Report on project status to JVET reflector. - Provide a report to the next meeting on project coordination status. - Supervise processing and delivery of output documents | J.-R. Ohm (chair), G. J. Sullivan (vice-chair) | N |
| Draft text and test model algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize draft text outputs of the meeting (JVET-AC1008, JVET-AC2002, JVET-AC2005, JVET-AC2027, and JVET-AC2032). - Collect reports of errata for the VVC, VSEI, HEVC, AVC, CICP, the codepoint usage TR specification and the published HDR-related technical reports and produce the JVET-AC1004 errata output collection. - Coordinate with the test model software development AhG to address issues relating to mismatches between software and text. - Collect and consider errata reports on the texts. | B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| Test model software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of test models (VTM, HM, SCM, SHM, HTM, MFC, MFCD, JM, JSVM, JMVM, 3DV-ATM, 360Lib, and HDRTools) software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Enable software support for recently standardized additional SEI messages. - Discuss and make recommendations on the software development process. - Propose improvements to the guideline documents for developments of the test model software. - Perform comparative tests of test model behaviour using common test conditions, including HDR, high bit depth and high bit rate. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG on Draft text and test model algorithm description editing (AHG2) to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate. - Prepare drafts of merged and updated CTC documents for HM and VTM, as applicable. | F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Consider plans for additional verification testing of VVC capability, particularly target establishing a test plan for VVC scalability features by the next meeting. - Maintain the video sequence test material database for testing the VVC and HEVC standards and potential future extensions, as well as exploration activities. - Study coding performance and characteristics of available and proposed video test material. - Identify and recommend appropriate test material for testing the VVC standard and potential future extensions, as well as exploration activities. - Identify and characterize missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material, in coordination with other AHGs, as appropriate. - Maintain and update the directory structure for the test sequence repository, as necessary. - Collect information about test sequences that have been made available by other organizations. - Prepare and conduct remote expert viewing for purposes of subjective quality evaluation. - Coordinate with AG 5 in studying and developing further methods of subjective quality evaluation, e.g. based on crowd sourcing. - Prepare availability of viewing equipment and facilities arrangements for future meetings. | V. Baroncini, T. Suzuki, M. Wien (co-chairs), S. Liu, S. Puri, A. Segall, P. Topiwala, S. Wenger, J. Xu, Y. Ye (vice-chairs) | Y (tel., 2 weeks notice) |
| Conformance testing (AHG5) (jvet@lists.rwth-aachen.de) - Produce and finalize the draft conformance testing for operation range extensions JVET-AC2026 and the additional conformance bitstreams for VVC multilayer configurations JVET-AC2028, and investigate the need for future improvements of conformance testing specifications. - Study the requirements of VVC, HEVC, and AVC conformance testing to ensure interoperability. - Maintain and update the conformance bitstream database, and contribute to report problems, and suggest actions to resolve these. - Study additional testing methodologies to fulfil the needs for VVC conformance testing. | D. Rusanovskyy, I. Moccagatta (co-chairs), F. Bossen, K. Kawamura, T. Ikai, H.-J. Jhu, K. Sühring, Y. Yu (vice-chairs) | N |
| ECM software development (AHG6) (jvet@lists.rwth-aachen.de) - Coordinate development of the ECM software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Prepare and deliver ECM-8.0 software version and the reference configuration encodings according to the ECMcommon test conditions. - Investigate encoder speedup and other encoder software optimization. - Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate. | V. Seregin (chair), J. Chen, F. Le Léannec, K. Zhang (vice-chairs) | Y (tel., 2 weeks notice) |
| ECM tool assessment (AHG7) (jvet@lists.rwth-aachen.de) - Discuss JVET-AC0354, and prepare an updated document as input to the next meeting on the methodology of tool assessment, tool grouping and a list of tools in each group to be tested. - Investigate missing tool controls in ECM, and suggest appropriate modifications to software in coordination with AHG6. - Prepare configuration files and generate bitstreams and results of tool-on/tool-off testing. - Prepare reporting of tool assessment results. | X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, H.-J. Jhu, R.-L. Liao, H. Wang (vice-chairs) | Y (tel., 2 weeks notice) |
| Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) (jvet@lists.rwth-aachen.de) - Solicit and study non-normative encoder and receiving systems technologies that enhance performance of machine analysis tasks on coded video content. - Identify and collect test materials that are suitable to be used by JVET for machine analysis tasks. - Develop common test conditions and generate anchors. - Develop an evaluation framework, including evaluation procedures and methodologies. - Coordinate software development, and investigate the possibility of migrating the software basis to newest VTM version. - Coordinate experiments on optimization of encoders and receiving systems for machine analysis of coded video content. - Evaluate proposed technologies and their suitability for machine analysis applications. - Prepare a draft technical report on optimization of encoders and receiving systems for machine analysis of coded video content. - Coordinate with WG 4 VCM AHG on common interests and activities such as common test conditions, test and training materials, and on studying characteristics and requirements of targeted machine analysis tasks, etc. | C. Hollmann, S. Liu, S. Wang, M. Zhou (AHG chairs) | N |
| SEI message studies (AHG9) (jvet@lists.rwth-aachen.de) - Study the SEI messages in VSEI, VVC, HEVC and AVC. - Collect software and showcase information for SEI messages, including encoder and decoder implementations and bitstreams for demonstration and testing. - Identify potential needs for additional SEI messages, including the study of SEI messages defined in HEVC and AVC for potential use in the VVC context. - Coordinate with AHG8 and WG 4 to study mechanisms for signalling metadata in the context of machine analysis of coded video content. - Coordinate with AHG3 for software support of SEI messages. | S. McCarthy, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, C. Fogg, Hendry, P. de Lagrange, G. J. Sullivan, A. Tourapis, S. Wenger (vice-chairs) | N |
| Encoding algorithm optimization (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using techniques such as tool adaptation and configuration, and perceptually optimized adaptive quantization for encoder optimization. - Study the impact of non-normative techniques of preprocessing for the benefit of encoder optimization. - Study encoding techniques of optimization for objective quality metrics and their relationship to subjective quality. - Study optimized encoding for reference picture resampling and scalability modes in VTM. - Study optimized encoding and tool combinations for low latency and low complexity. - Consider neural network-based encoding optimization technologies for video coding standards. - Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding. - Study methods of rate control and rate-distortion optimization and their impact on performance, subjective and objective quality. - Study the potential of defining default or alternate software configuration settings and test conditions optimized for either subjective quality, or higher objective quality, and coordinate such efforts with AHG3 and AHG6. - Study the effect of varying configuration parameters depending on temporal layer, such as those related to deblocking, partitioning, chroma QP. | P. de Lagrange, A. Duenas, R. Sjöberg, A. Tourapis (AHG chairs) | N |
| Neural network-based video coding (AHG11) (jvet@lists.rwth-aachen.de) - Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools and novel architectures. - Discuss potential refinements of the test conditions for NN-based video coding in JVET-AB2016. Generate and distribute anchor encoding, and develop supporting software as needed. - Study the impact of training (including the impact of loss functions) on the performance of candidate technologies, and identify suitable material for testing and training. - Analyse complexity characteristics, perform complexity analysis, and develop complexity reductions of candidate technology. - Finalize and discuss the EE on neural network-based video coding. - Coordinate with other relevant groups, including SC29/AG5 on the evaluation and assessment of visual quality, and AHG12 on the interaction with ECM coding tools. If possible, prepare encodings with combinations of tools included in the NNVC software for visual quality assessment at the next meeting. - Coordinate with AHG14 on items related to NNVC software development, and study the potential for unification of filter sets #0 and #1. | E. Alshina, S. Liu, A. Segall (co-chairs), F. Galpin, J. Li, R.-L. Liao, D. Rusanovskyy, T. Shao, M. Wien, P. Wu (vice-chairs) | Y (tel., 2 weeks notice) |
| Enhanced compression beyond VVC capability (AHG12) (jvet@lists.rwth-aachen.de) - Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC. - Discuss and propose refinements to the ECM8 algorithm description JVET-AC2025. - Coordinate with AHG7 to study the performance and complexity tradeoff of these video coding tools. - Coordinate with AHG6 on ECM software development. - Support AHG6 in generating anchors according to the test conditions in JVET-Y2017. - Analyse the results of exploration experiments described in JVET-AC2024 in coordination with the EE coordinators. - Coordinate with AHG11 to study the interaction with neural network-based coding tools. | M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, H. Yang (vice-chairs) | Y (tel., 2 weeks notice) |
| Film grain technologies (AHG13) (jvet@lists.rwth-aachen.de) - Study the benefits and characteristics of film grain technologies, including autoregressive and frequency-filtering technologies. - Produce and finalize the JVET-AC2020 draft of the Technical Report on Film grain synthesis technology for video applications and suggest future improvements as necessary. - Study alternative film grain models and their associated documentation. - Coordinate with AHG4 on soliciting additional test materials as needed for further investigations on film grain technologies. - Study preprocessing and encoder technologies for determining values for FGC (Film Grain Characteristics) SEI message syntax elements. - Identify potential need for additional film grain technology and signalling, if needed. - Coordinate development of film grain technology software and configuration files. - Coordinate with AG 5 on finalizing the draft plan for subjective quality testing of the FGC SEI message JVET-AC2022, and conduct preparations for such testing. - Coordinate with AHG3 for software support of the FGC SEI message. | W. Husak, M. Radosavljević, W. Wan (co-chairs), D. Grois, Y. He, P. de Lagrange, A. Segall, A. Tourapis, W. Zhang (vice-chairs) | Y (tel., 2 weeks notice) |
| NNVC software development (AHG14) (jvet@lists.rwth-aachen.de) - Coordinate development of the NNVC software and associated configuration files. - Prepare and deliver NNVC-4.0 software version and the reference configuration encodings according to the NNVC common test conditions JVET-AB2016. Prepare and release anchor data for all configurations of the software. - Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed. - Coordinate with NNVC algorithm and software description (JVET-AC2019) editors to identify any mismatches between software and description document, suggest further updates to the description document as appropriate. - Investigate combinations of tools included in the NNVC software and coordinate with AHG11 on defining default settings of such combinations. - Develop software guidelines for NNVC and SADL. - Coordinate with AHG11 on items related to NNVC activities. | S. Eadie, F. Galpin, Y. Li, L. Wang, Z. Xie (AHG chairs) |  |

It was confirmed that the rules which can be found in document ISO/IEC JTC 1/‌SC 29/‌AG 2 N 046 “Ad hoc group rules for MPEG AGs and WGs” (available at https://www.mpegstandards.org/adhoc/), are consistent with the operation mode of JVET AHGs. It is pointed out that JVET does not maintain separate AHG reflectors, such that any JVET member is implicitly a member of any AHG. This shall be mentioned in the related WG Recommendations. The list above was also issued as a separate WG 5 document (ISO/IEC JTC 1/‌SC 29/‌WG 5 N XXX) in order to make it easy to reference.

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. Where applicable, dates of planned finalization and corresponding parent-body document numbers are also noted.

It was reminded that in cases where the JVET document is also made available as a WG 5 output document, a separate version under the WG 5 document header should be generated. This version should be sent to GJS and JRO for upload.

The list of JVET ad hoc groups was also issued as a WG 5 output document WG 5 N XXX, as noted in section 9.

JVET-AC1000 Meeting Report of the 30th JVET Meeting [J.-R. Ohm] [WG 5 N XXX] (2023-05-26)

Initial versions of the meeting notes (d0 … d9) were made available on a daily basis during the meeting.

JVET-AC1001 Guidelines for HM-based software development [K. Sühring, F. Bossen, X. Li (software coordinators)] (2023-02-03)

Developed from JVET-AC0204

Remains valid – not updated: JVET-Y1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 16 [C. Rosewarne (primary editor), K. Sharman, R. Sjöberg, G. J. Sullivan (co-editors)] [WG 5 N 103]

Remains valid – not updated: JVET-Z1003 Coding-independent code points for video signal type identification (Draft 1 of 3rd edition) [WG 5 DIS N 162] [G. J. Sullivan, A. Tourapis]

JVET-AC1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP [Y.-K. Wang, B. Bross, I. Moccagatta, C. Rosewarne, G. J. Sullivan] (2023-03-17, near next meeting)

New aspects included the issues pointed out in JVET-AC0311 and JVET-AC0346.

Remains valid – not updated: JVET-Z1005 New levels for HEVC (Draft 3) [T. Suzuki, A. Tourapis, Y.-K. Wang]

The content of this document will be included in a new edition of HEVC. For additional corrigenda items included, see documents JVET-AB1004, as well as notes under JVET-AC0311 and JVET-AC0346. A DoCR on ISO/IEC DIS23008-2 was submitted as WG 5 N178 (reviewed during session 24), and the FDIS was submitted as WG 5 N 179.

(JVET-Z1005 can be removed after publication of the new edition of ISO/IEC 23008-2.)

No output: JVET-Axx1006

Remains valid – not updated: JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye] [WG 11 N 15778]

JVET-AC1008 Additional colour type identifiers for AVC, HEVC and Video CICP (Draft 3) [G. J. Sullivan, W. Husak, A. Tourapis] [WG 5 Preliminary WD N186] (2023-02-24)

Modified from JVET-AB1008 in response to JVET-AC0352.

JCTVC-AC1009 Common test conditions for SHVC [K. Sühring] (2023-02-17)

This requires an update, as the previous version referred to an outdated location of test sequences.

Remains valid – not updated JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan]

Remains valid – not updated: JVET-AA1011 HEVC multiview profiles supporting extended bit depth (draft 1) [A. Tourapis, W. Husak] [WG 5 preliminary WD N 143] (2022-08-26)

Proponents of the new profiles reported that they are currently developing software and conformance bitstreams.

Remains valid – not updated: JVET-AB1012 Overview of IT systems used in JVET [J.-R. Ohm, I. Moccagatta, K. Sühring, M. Wien]

Remains valid – not updated: JCT3V-G1003 3D-AVC Test Model 9 [	D. Rusanovskyy, F. C. Chen, L. Zhang, T. Suzuki ] [WG 11 N 14239]

Remains valid – not updated: JCT3V-K1003 Test Model 11 of 3D-HEVC and MV-HEVC [Y. Chen, G. Tech, K. Wegner, S. Yea] [WG 11 N 15141]

JVET-AC1013 Common test conditions of 3DV experiments [K. Sühring] (2023-02-17)

This requires an update, as the previous version referred to an outdated location of test sequences.

Remains valid – not updated JCTVC-V1014 Screen Content Coding Test Model 7 Encoder Description (SCM 7) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye] [WG 11 N 16049]

JCTVC-AC1015 Common test conditions for SCM-based screen content coding [K. Sühring] (2023-02-17)

This requires an update, as the previous version referred to an outdated location of test sequences.

No output: JVET-Axx1016 through JVET-Axx1099

Remains valid – not updated JVET-AA1100 Common Test Conditions for HM Video Coding Experiments [K. Sühring, K. Sharman]

This specifies only the CTC for non-4:2:0 colour formats. The corresponding document for VVC is JVET-T2013, with no unification yet.

No output: JVET-Axx2001

JVET-AC2002 Algorithm description for Versatile Video Coding and Test Model 19 (VTM 19) [A. Browne, Y. Ye, S. Kim] [WG 5 N 183] (2023-04-14, near next meeting)

Updates from JVET-AC0096 (description of RPR switching control).

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AC2003 Guidelines for VTM-based software development [F. Bossen, X. Li, K. Sühring] (2023-02-03)

Developed from JVET-AC0166.

Remains valid – not updated: JVET-T2004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 12) [Y. Ye, J. Boyce]

JVET-AC2005 New level and systems-related supplemental enhancement information for VVC (Draft 4) [E. François, B. Bross, M. M. Hannuksela, A. Tourapis, Y.-K. Wang] (2023-03-31)

JVET-AC2005 is an update of JVET-AA2005 (previously submitted as WG 5 DAM 1 N 145). For elements included, see documents JVET-AB1004, as well as notes under JVET-AC0311 and JVET-AC0346.

The content of this document was included in a new edition of VVC. A draft DoCR on ISO/IEC 23090-3/DAM1 was submitted as WG 5 N180 (reviewed during session 24), and the preliminary FDIS text was submitted as WG 5 N 181.

Remains valid – not updated: JVET-AB2006 Additional SEI messages for VSEI (Draft 3) [S. McCarthy, T. Chujoh, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang] [WG 5 DAM N 158]

No output: JVET-Axx2007

Remains valid – not updated: JVET-X2008 Conformance testing for versatile video coding (Draft 7) [J. Boyce, F. Bossen, K. Kawamura, I. Moccagatta, W. Wan]

Remains valid – not updated: JVET-Y2009 Reference software for versatile video coding (Draft 3) [F. Bossen, K. Sühring, X. Li]

Remains valid – not updated JVET-AB2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video [F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring]

JVET-AC2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy] (2023-02-03)

Remains valid – not updated: JVET-U2012 JVET common test conditions and evaluation procedures for 360° video [Y. He, J. Boyce, K. Choi, J.-L. Lin]

Remains valid – not updated: JVET-T2013 VTM common test conditions and software reference configurations for non-4:2:0 colour formats [Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu]

Remains valid – not updated: JVET-Q2014 JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding [T.-C. Ma, A. Nalci, T. Nguyen]

Remains valid – not updated: JVET-Q2015 JVET functionality confirmation test conditions for reference picture resampling [J. Luo, V. Seregin]

JVET-AC2016 Common test conditions and evaluation procedures for neural network-based video coding technology [E. Alshina, R.-L. Liao, S. Liu, A. Segall] (2023-02-03)

Template has an error to be fixed.

Remains valid – not updated: JVET-Y2017 Common test conditions and evaluation procedures for enhanced compression tool testing [M. Karczewicz and Y. Ye]

Remains valid – not updated: JVET-AA2018 Common test conditions for high bit depth and high bit rate video coding [A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu, Y. Yu]

JVET-AC2019 Description of algorithms and software in neural network-based video coding (NNVC) version 2 [Y. Li, H. Wang, L. Wang, F. Galpin, J. Ström] [WG 5 N 188] (2023-03-10)

New elements from notes elsewhere in this report:

- Decision: Adopt JVET-AC0194 method 1.1-2

- Decision: Adopt JVET-AC0196 into NNVC 4.0 (EE1-2.2)

- Decision: Adopt JVET-AC0055

- Decision: Adopt JVET-AC0116

- Decision: Adopt JVET-AC0310, but make 1.5.3 the default configuration, and allow disabling 1.7.1 component. Further, the method shall be changed to use the actual reference frame (not the unfiltered decoded frame)

- Decision: Adopt JVET-AC0328, but disable the RDO network in the encoder by default (unless such an approach is generally enabled for both filter sets #0 and #1)

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AC2020 Film grain synthesis technology for video applications (Draft 4) [D. Grois, Y. He, W. Husak, P. de Lagrange, A. Norkin, M. Radosavljević, A. Tourapis, W. Wan] [WG 5 CDTR N 176] (2023-03-31)

Developed from JVET-AC0151.

JVET-AC2021 Draft verification test plan for VVC multilayer coding [S. Iwamura, P. de Lagrange, M. Wien] (2023-03-03)

See notes under section 4.4 for updates.

JVET-AC2022 Draft plan for subjective quality testing of FGC SEI message [P. de Lagrange, W. Husak, M. Radosavljević, M. Wien] (2023-03-03)

See notes under section 4.4 for updates.

JVET-AC2023 Exploration experiment on neural network-based video coding (EE1) [E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang, Z. Xie] [WG 5 N 187] (2023-02-03)

An initial draft of this document was reviewed and approved at 1515-1535 on Friday 20 January.

Categories are enhancement loop filters and inter coding.

JVET-AC2024 Exploration experiment on enhanced compression beyond VVC capability (EE2) [V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang] [WG 5 N 189] (2023-03-17)

An initial draft of this document was reviewed and approved at 1535-1550 on Friday 20 January.

Categories are intra prediction, inter prediction, screen content coding, in-loop filters, and GDR.

It was suggested moving test 1.2 after 1.7 to better group the CCCM related proposals.

The number of tests in the intra category is rather large, and will require significant time for review at the next meeting. Proponents are asked to consider withdrawing those tests where they find that the benefit is insignificant.

JVET-AC2025 Algorithm description of Enhanced Compression Model 8 (ECM 8) [M. Coban, F. Le Léannec, R.-L. Liao, K. Naser, J. Ström, L. Zhang] [WG 5 N 190] (2023-03-24)

New elements from notes elsewhere in this report:

- Decision: Adopt JVET-AC0105 (test 1.2)

- Decision: Adopt JVET-AC0119 (test 1.6)

- Decision: Adopt JVET-AC0098 (test 1.8)

- Decision: Adopt JVET-AC0094 (test 1.10)

- Decision: Adopt JVET-AC0054 (test 1.12a)

- Decision: Adopt JVET-AC0147 (test 1.13)

- Decision: Adopt JVET-AC0144 (test 2.2)

- Decision: Adopt JVET-AC0158 (test 2.5a) with the following modification: It was agreed to adopt a solution with pixel-based affine as in test 2.5a (not using pixel based affine whenever OBMC is used in the given block), but without the modification of OBMC flag signalling

- Decision: Adopt JVET-AC0185 test 2.7a

- Decision: Adopt JVET-AC0113 (version 3.5a)

- Decision: Adopt JVET-AC0112 test 3.6d

- Decision: Adopt JVET-AC0115 test 4.1a

- Decision: Adopt JVET-AC0130 (test 4.2b)

- Decision: Adopt JVET-AC0162 (5.1b)

- Decision: Adopt JVET-AC0053

- Decision: Adopt JVET-AC0189 (in CTC, set the PPS flag to 1 for class F and class TGM)

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AC2026 Conformance testing for VVC operation range extensions (Draft 4) [D. Rusanovskyy, T. Hashimoto, H.-J. Jhu, I. Moccagatta, Y. Yu] (2023-04-14)

JVET-AC2026 is an update of JVET-Y2026 (previously submitted as WG 5 DAM N 110).

The content of this document was included in a new edition of VVC conformance. A DoCR on ISO/IEC 23090-15/DAM1 was submitted as WG 5 N184 (reviewed during session 24), and the FDIS text was submitted as WG 5 N 185.

JVET-AC2027 SEI processing order SEI message in VVC (draft 3) [S. McCarthy, M. M. Hannuksela, Y.-K. Wang] [WG 5 preliminary WD 3 N 182] (2023-03-31)

Updated from JVET-AC0058.

JVET-AC2028 Additional conformance bitstreams for VVC multilayer configurations [S. Iwamura, I. Moccagatta] (2023-04-14)

Removing bitstreams that are put into the FDIS, and some other wording changes.

Remains valid – not updated: JVET-AB2029 Visual quality comparison of ECM/VTM encoding [V. Baroncini, J.-R. Ohm, M. Wien] [AG 5 N75]

JVET-AC2030 Optimization of encoders and receiving systems for machine analysis of coded video content (draft 1) [J. Chen, C. Hollmann, S. Liu] [WG 5 N177)] (2023-03-31)

JVET-AC2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content [S. Liu, C. Hollmann] (2023-02-17)

JVET-AC2032 Improvements under consideration for neural network post filter SEI messages [S. McCarthy, S. Deshpande, M. M. Hannuksela, Hendry, G. J. Sullivan, Y.-K. Wang] [WG 5 N175] (2023-03-03)

For elements included, see the notes on the BoG report JVET-AC0324 in section 6.1.

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines (assuming face-to-face meetings):

- Meeting under ITU-T SG16 auspices when it meets (ordinarily starting meetings on the Wednesday of the first week and closing it on the Wednesday of the second week of the SG16 meeting – a total of 8 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/‌SC 29 auspices when its MPEG WGs meet (ordinarily starting meetings on the Friday prior to the main week of such meetings and closing it on the same day as other MPEG WGs – a total of 8 meeting days).

In cases where an exceptionally high workload is expected for a meeting, an earlier starting date may be defined. In cases of online meetings, no sessions should be held on weekend days, such that meetings would typically start two days earlier.

Some specific future meeting plans (to be confirmed) were established as follows:

- During Tue. 11 – Wed. 19 July 2023, 31st meeting under ITU-T SG16 auspices in Geneva, CH

- During Fri. 13 – Fri. 20 October 2023, 32nd meeting under ISO/IEC JTC 1/‌SC 29 auspices in Hannover, DE.

- During Wed. 17 – Fri. 19 and Mon. 22 – Fri. 26 January 2024, 33rd meeting under ISO/IEC JTC 1/‌SC 29 auspices, to be held as teleconference meeting.

- During April 2024, 34th meeting under ITU-T SG16 auspices, date and location t.b.d.

- During Fri. 12 – Fri. 19 July 2024, 35th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Sapporo, JP

- During October 2024, 36th meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

- During January 2025, 37th meeting under ITU-T SG16 auspices, date and location t.b.d.

- During April 2025, 38th meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

- During Fri. 11 – Fri. 18 July 2025, 39th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Daejeon, KR

- During October 2025, 40th meeting under ITU-T SG16 auspices, date and location t.b.d.

The agreed document deadline for the 31st JVET meeting was planned to be Tuesday 4 July 2023.

It was suggested that it would be interesting to perform subjective quality investigation with the elements adopted to NNVC software in a future meeting (provided that close enough rate/quality matching with VTM anchor is achieved).

The following companies were thanked for preparing new bitstreams to be included in the second edition of VVC conformance specification: Alibaba, InterDigital, KDDI, Kwai, OPPO, Qualcomm, Sharp, Sony.

Marius Preda was thanked for his support in maintaining the document site jvet-experts.org, as well as the document sites of JCT-VC and JCT-3V. Institut Mines-Télécom is thanked for hosting the sites.

The 30th JVET meeting was closed at approximately XXXX hours on Friday 28 April 2023.

# Annex A to JVET report:
List of documents

# Annex B1 to JVET report:
List of meeting participants attending in person

The participants who were personally present at the meeting site of the thirtieth meeting of the JVET, according to an attendance sheet circulated during the JVET meeting sessions (approximately XXX people in total), were as follows:

# Annex B2 to JVET report:
List of meeting participants attending remotely

The remote participants of the thirtieth meeting of the JVET, according to the participation records from the Zoom teleconferencing tool used for the meeting sessions (approximately XXX people in total, not including those who had attended the meeting in person at least part-time (see annex B1), and not including those who attended only the joint sessions with other groups), were as follows:

# Annex C to JVET report:
Recommendations of the 11th meeting of
ISO/IEC JTC 1/SC 29/WG 5 MPEG Joint Video Coding Team(s) with ITU-T SG 16

ISO/IEC JTC 1/SC 29/WG 5 N XXX