| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0006-v1 |
| --- | --- |

| Title: | JVET AHG report: ECM software development (AHG6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Fabrice Le Leannec Kai Zhang | Tel: Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com Fabrice.Leleannec@InterDigital.com zhangkai.video@bytedance.com |
| Source: | AHG6 on ECM software development |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AHG6 on ECM software development that has taken place between the 29th and 30th JVET meetings.

# Introduction

The mandates for the AHG are as follows:

- Coordinate development of the ECM software and associated configuration files.

- Produce documentation of software usage for distribution with the software.

- Prepare and deliver ECM-8.0 software version and the reference configuration encodings according to the ECM common test conditions.

- Investigate encoder speedup and other encoder software optimization.

- Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate.

# Software development

ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

VTM-11.0ecm anchor https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC is used for ECM performance evaluation.

The following adopted aspects were integrated into ECM-7.0:

JVET-AB0171: Asymmetric deblocking (MR 303)

JVET-AC0105: Directional planar (MR 306)

JVET-AC0095: fix padding at picture boundary (MR 312)

JVET-AC0189: SGPM without blending (MR 313)

JVET-AC0144: affine DMVR regression (MR 310)

JVET-AC0147: Subsampling is not applied to CCCM (MR 314)

JVET-AC0112: IBC-CIIP, IBC-GPM, IBC-LIC (MR 315)

JVET-AC0053: Gaussian solver for filter coefficients (MR 318)

JVET-AC0071: Direct block vector mode for chroma prediction (MR 325) (part of AC0113 3.5a)

JVET-AC0130: NSPT replacing DCT-II + LFNST for certain block shapes (MR 319)

JVET-AC0123: MTT correction for class B QP22 (MR 322)

JVET-AC0158: Pixel based affine motion compensation (MR 327)

JVET-AC0094: Optimizing the use of reference samples (MR 311)

JVET-AC0185: Enhanced temporal motion information derivation (MR 317)

JVET-AC0162: ALF using residual samples as additional inputs (MR 323)

JVET-AC0335: Content adaptive OBMC enabling (MR 324)

JVET-AC0119: On Chroma Fusion improvement (MR 331)

JVET-AC0098: Location-dependent DIMD (MR 329)

JVET-AC0054: Gradient and location-based CCCM (MR 332)

JVET-AC0115: Intra mode derivation for IntraTMP transform (Test 4.1a) (MR 320)

JVET-AC0104: Block vector difference prediction for IBC blocks (part of AC0113 Test 3.5a) (MR 333)

JVET-AC0060: IBC BVP candidates clustering and BVD sign derivation for reconstruction-reordered IBC mode (test EE2-3.4, as part of test EE2-3.5a) (MR 335)

JVET-AC0096: RPR functionality testing (MR 338)

Bug fixes:

Fix VB handling when SAO is EDGE 135 (MR 304)

Fix the wrong stride for TIMD (MR 305)

The following adopted aspects were integrated into VTM-11ecm anchor:

JVET-AB0081: Upscaling filter for display 12-tap luma and 6-tap chroma (MR 341)

The integrated changes do not change VTM-11 anchor performance under CTC.

JVET-AC0123: MTT correction for class B QP22 (MR 328)

JVET-AC0096: RPR functionality testing (MR 339)

ECM-8.0 and VTM-11ecm8.0 were tagged on February 24, 2023.

## CTC Performance

In this section, ECM test results following ECM CTC configuration descried in JVET-Y2017 are summarized.

Next tables show ECM-8.0 performance over ECM-7.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.40% | -1.62% | -1.41% | 113% | 108% |
| Class A2 | -0.75% | -1.83% | -1.71% | 119% | 110% |
| Class B | -0.74% | -1.59% | -1.62% | 117% | 113% |
| Class C | -1.22% | -1.79% | -1.72% | 119% | 111% |
| Class E | -0.86% | -1.15% | -1.01% | 115% | 109% |
| Overall | -0.81% | -1.61% | -1.52% | 117% | 111% |
| Class D | -0.88% | -1.65% | -1.70% | 118% | 108% |
| Class F | -4.77% | -5.65% | -5.71% | 122% | 113% |
| Class TGM | -6.14% | -7.04% | -7.77% | 118% | 109% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.53% | -1.23% | -1.18% | 109% | 112% |
| Class A2 | -1.18% | -1.93% | -2.42% | 109% | 111% |
| Class B | -0.72% | -1.49% | -1.53% | 100% | 108% |
| Class C | -0.86% | -1.40% | -1.44% | 108% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.81% | -1.50% | -1.61% | 106% | 109% |
| Class D | -0.63% | -1.55% | -1.42% | 109% | 108% |
| Class F | -3.87% | -5.14% | -5.27% | 111% | 106% |
| Class TGM | -4.71% | -4.90% | -5.03% | 112% | 114% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.23% | -0.62% | -0.81% | 104% | 112% |
| Class C | -0.32% | -0.78% | -0.71% | 104% | 110% |
| Class E | -0.11% | -2.86% | -0.72% | 103% | 108% |
| Overall | -0.23% | -1.23% | -0.75% | 104% | 110% |
| Class D | -0.22% | -0.70% | -1.28% | 106% | 109% |
| Class F | -2.03% | -3.11% | -3.96% | 110% | 110% |
| Class TGM | -2.15% | -2.53% | -2.65% | 107% | 115% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | -0.66% | -0.90% | 109% | 113% |
| Class C | -0.30% | -0.91% | -0.72% | 109% | 111% |
| Class E | 0.13% | -1.06% | -0.26% | 107% | 110% |
| Overall | -0.12% | -0.84% | -0.68% | 109% | 112% |
| Class D | -0.19% | -1.24% | -2.08% | 109% | 108% |
| Class F | -2.23% | -3.42% | -3.41% | 111% | 111% |
| Class TGM | -2.37% | -2.74% | -3.06% | 111% | 109% |

The below tables show ECM-8.0 performance comparing to VTM-11.0ecm8.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm8 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.03% | -18.78% | -25.18% | 703% | 380% |
| Class A2 | -13.53% | -22.78% | -25.89% | 695% | 382% |
| Class B | -8.30% | -21.80% | -20.07% | 692% | 421% |
| Class C | -9.22% | -14.00% | -14.34% | 677% | 419% |
| Class E | -10.49% | -18.86% | -17.70% | 648% | 423% |
| Overall | -9.86% | -19.24% | -20.22% | 684% | 407% |
| Class D | -7.50% | -11.48% | -11.53% | 672% | 418% |
| Class F | -22.22% | -30.58% | -30.24% | 460% | 365% |
| Class TGM | -35.58% | -42.92% | -42.31% | 387% | 329% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm8 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -19.88% | -22.82% | -29.84% | 687% | 742% |
| Class A2 | -22.38% | -29.61% | -32.91% | 632% | 887% |
| Class B | -18.26% | -29.12% | -27.08% | 558% | 758% |
| Class C | -19.95% | -23.64% | -23.52% | 574% | 757% |
| Class E |  |  |  |  |  |
| Overall | -19.86% | -26.50% | -27.85% | 601% | 779% |
| Class D | -20.75% | -25.02% | -25.17% | 594% | 876% |
| Class F | -24.95% | -32.44% | -32.74% | 438% | 465% |
| Class TGM | -33.27% | -38.66% | -38.88% | 489% | 417% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm8 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -16.36% | -30.93% | -29.37% | 543% | 665% |
| Class C | -17.70% | -26.18% | -26.20% | 559% | 671% |
| Class E | -14.69% | -24.67% | -23.73% | 545% | 419% |
| Overall | -16.39% | -27.78% | -26.90% | 549% | 594% |
| Class D | -19.81% | -26.86% | -27.28% | 545% | 768% |
| Class F | -22.06% | -31.32% | -30.64% | 448% | 474% |
| Class TGM | -30.41% | -36.98% | -36.69% | 458% | 388% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm8 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -15.52% | -30.67% | -28.56% | 501% | 648% |
| Class C | -17.16% | -25.51% | -24.94% | 481% | 736% |
| Class E | -14.20% | -22.86% | -23.10% | 514% | 481% |
| Overall | -15.74% | -26.99% | -25.99% | 497% | 627% |
| Class D | -19.95% | -26.98% | -27.49% | 462% | 750% |
| Class F | -21.24% | -31.27% | -29.98% | 436% | 436% |
| Class TGM | -27.60% | -35.05% | -34.98% | 490% | 361% |

The Excel files with the complete test results are attached to this report.

# Recommendations

The AHG recommends to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.