| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0008-v1 |
| --- | --- |

| Title: | JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Christopher Hollmann Shan Liu Shurun Wang Minhua Zhou | Email: | christopher.hollmann@ericsson.com shanl@tencent.com shurun.wsr@alibaba-inc.com minhua.zhou@broadcom.com |
| Source: | AHG 8 coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities of AHG 8: Optimization of encoders and receiving systems for machine analysis of coded video content between the 29th meeting (11 – 20 January 2023) held by teleconference and the 30th meeting (21 – 28 April 2023) held in Antalya, TR.

# Introduction

The AHG was first established in the 28th meeting and was re-established in the 29th meeting with refined mandates as follows:

- Solicit and study non-normative encoder and receiving systems technologies that enhance performance of machine analysis tasks on coded video content.

- Identify and collect test materials that are suitable to be used by JVET for machine analysis tasks.

- Develop common test conditions and generate anchors.

- Develop an evaluation framework, including evaluation procedures and methodologies.

- Coordinate software development, and investigate the possibility of migrating the software basis to newest VTM version.

- Coordinate experiments on optimization of encoders and receiving systems for machine analysis of coded video content.

- Evaluate proposed technologies and their suitability for machine analysis applications.

- Prepare a draft technical report on optimization of encoders and receiving systems for machine analysis of coded video content.

- Coordinate with WG 4 VCM AHG on common interests and activities such as common test conditions, test and training materials, and on studying characteristics and requirements of targeted machine analysis tasks, etc.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, for email discussion. There were a few emails exchanged on the reflector uner the context of AHG 8 during this period. There were active email exchanges (50+) and discussions among co-chairs, editors and interested experts related to output document prepration on common test conditions and technical report, test datasets, anchor generation process and results, and setup of an AHG git repository for software development, experiments and document management. There are 13 input contriubtions related to AHG 8 mandates submitted to this meeting. They are listed in Section 3.

## Common Test Conditions

Common test conditions (CTC) for optimization of encoders and receiving systems for machine analysis of coded video content, are summarized in output document JVET-AC2031. This document includes detailed descriptions of test datasets, anchor software and configurations, anchor generation processes, machine task networks used, test and training conditions, evaluation methodologies and metrics. A reporting template in Excel format is enclosed in the document package. The first version of this output document was uploaded on 2023-02-02, followed by a couple of revisions, the final verison includes crosschecked CTC results.

### Test datasets

Two video datasets, referred as SFU-HW and TVD, are included in the CTC and results on both are expected to be reported. In addition, three image datasets, referred as TVD (image), OpenImageV6 and FLIR, are included in the CTC and can be tested using intra-only configuration. The results on the image datasets are optional.

It was decided in the last JVET meeting to remove four sequences from the originally proposed SFU-HW (SFU-HW-objects-v1) dataset, including one class B sequence (Kimono) and three class E sequences, and thus the SFU-HW dataset now consist of 13 test video sequences, as shown in Table 1. The sequences can be found on ftp://hevc@mpeg.tnt.uni-hannover.de. The annotations are available at https://data.mendeley.com/datasets/hwm673bv4m/1.

Table 1. Test sequences in SFU-HW dataset

| Class | Sequence name | Frame count | Frame rate | Bit depth | Frames skipped | Frames coded | Intra | RA | LD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Traffic | 150 | 30 | 8 | 117 | 33 | M | M | M |
| B | ParkScene | 240 | 24 | 8 | 207 | 33 | M | M | M |
| B | Cactus | 500 | 50 | 8 | 403 | 97 | M | M | M |
| B | BasketballDrive | 500 | 50 | 8 | 403 | 97 | M | M | M |
| B | BQTerrace | 600 | 60 | 8 | 471 | 129 | M | M | M |
| C | RaceHorses | 300 | 30 | 8 | 235 | 65 | M | M | M |
| C | BQMall | 600 | 60 | 8 | 471 | 129 | M | M | M |
| C | PartyScene | 500 | 50 | 8 | 403 | 97 | M | M | M |
| C | BasketballDrill | 500 | 50 | 8 | 403 | 97 | M | M | M |
| D | RaceHorses | 300 | 30 | 8 | 235 | 65 | M | M | M |
| D | BQSquare | 600 | 60 | 8 | 471 | 129 | M | M | M |
| D | BlowingBubbles | 500 | 50 | 8 | 403 | 97 | M | M | M |
| D | BasketballPass | 500 | 50 | 8 | 403 | 97 | M | M | M |

Note: M – mandatory; O – optional.

It was decided in the last JVET meeting to split the originally proposed long sequences in Tencent Video Dataset (TVD) into multiple short clips. With some investigations, experiments and validations, seven clips are extracted from the originally proposed TVD sequences, as shown in Table 2. The dataset with corresponding annotations is available at https://multimedia.tencent.com/resources/tvd.

Table 2. Test sequences in TVD

| Sequence name | Frame count | Frame rate | Bit depth | Frames skipped | Frames coded | Intra | Random access | Low delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TVD-01_1 | 3000 | 50 | 8 | 1500 | 500 | M | M | M |
| TVD-01_2 | 3000 | 50 | 8 | 2000 | 500 | M | M | M |
| TVD-01_3 | 3000 | 50 | 8 | 2500 | 500 | M | M | M |
| TVD-02_1 | 636 | 50 | 8 | 0 | 636 | M | M | M |
| TVD-03_1 | 2334 | 50 | 8 | 0 | 500 | M | M | M |
| TVD-03_2 | 2334 | 50 | 8 | 500 | 500 | M | M | M |
| TVD-03_3 | 2334 | 50 | 8 | 1000 | 500 | M | M | M |

The original sequences are available in .mp4 format and shall be converted to YUV420 using FFmpeg:

ffmpeg -i {input.mp4} -f rawvideo -pix_fmt yuv420p -dst_range 1 {output.yuv}

The TVD (image) dataset is an image dataset of 166 images of 1920x1080 resolution that have annotations for object detection and instance segmentation. The dataset with corresponding annotations is available at https://multimedia.tencent.com/resources/tvd.

The OpenImages dataset consists of around 9 million images. A subset of the validation set of its version 6 containing 5000 images are selected for testing object detection in this activity. The dataset with corresponding annotations is available at https://storage.googleapis.com/openimages/web/index.html.

The FLIR dataset used in the WG 4 VCM activities is a dataset consisting of 300 infrared images. The images, annotations and the fine-tuned model for thermal images can be found on the MPEG content repository (https://content.mpeg.expert/data/).

More detailed information about these test datasets can be found in JVET-AC2031.

### Anchor software and configuration

Version 12.0 of the VTM software is used for generating anchor results. The VTM software is available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/.

In additiona, version 4.2.2 of the FFmpeg is used for the format conversion. The FFmpeg software is available at https://ffmpeg.org/releases.

Three test conditions are used for experimenting encoder and receiving system optimizations on video coding for machine consumptions, i.e., random-access, low-delay and all-intra. The default configuration files provided with the VTM software are used for anchor generation.

- “Random access” (RA): encoder_randomaccess_vtm.cfg

- “Low delay” (LD): encoder_lowdelay_vtm.cfg

- “All Intra” (AI): encoder_intra_vtm.cfg

It was decided in the last JVET meeting that temporal subsampling is disabled for “All Intra” configuration for this activity, thus the following line

TemporalSubsampleRatio : 8

in “encoder_intra_vtm.cfg” needs to be modified to

TemporalSubsampleRatio : 1

A subset of these configurations might be used for a particular experiment. However, as these test conditions are for video coding experiments, results for at least one of the random access or low delay configurations are expected to be provided.

### Anchor generation

The anchor generation process is illustrated in Figure 1. The task network Faster R-CNN X101-FPN which is part of Detectron2 (https://github.com/facebookresearch/detectron2) is used to evaluate object detection (SFU-HW dataset and image datasets), the model is available here. The task network JDE-1088x608 which is part of the Towards Realtime MOT framework (https://github.com/Zhongdao/Towards-Realtime-MOT) is used to evaluate object tracking (TVD), the model is available here. More details about anchor generation process and these machine task networks are referred to JVET-AC2031.

Figure 1. Anchor generation pipeline

Expriements were conducted after the last JVET meeting to carefully select the QP values for future tool development, testing and evalution. The updated QP values are listed in Table 3, Table 4 and Table 5.

Table 3. QPs for sequences in SFU-HW dataset

| Class | Sequence name | Configuration | QP values |
| --- | --- | --- | --- |
| A | Traffic | RA, LD, AI | {39, 45, 48, 51, 54, 58} |
| B | ParkScene | RA, LD, AI | {32, 36, 40, 44, 48, 52} |
| B | Cactus | RA, LD, AI | {43, 46, 49, 52, 55, 58} |
| B | BasketballDrive | RA, LD, AI | {40, 43, 46, 49, 52, 55} |
| B | BQTerrace | RA, LD, AI | {40, 43, 46, 49, 52, 55} |
| C | RaceHorses | RA, LD, AI | {27, 31, 35, 39, 43, 47} |
| C | BQMall | RA, LD, AI | {27, 32, 37, 42, 47, 52} |
| C | PartyScene | RA, LD, AI | {31, 35, 39, 43, 47, 51} |
| C | BasketballDrill | RA, LD, AI | {27, 31, 35, 39, 43, 47} |
| D | RaceHorses | RA, LD, AI | {22, 26, 30, 34, 38, 42} |
| D | BQSquare | RA, LD, AI | {24, 28, 31, 34, 38, 41} |
| D | BlowingBubbles | RA, LD, AI | {27, 31, 34, 37, 40, 43} |
| D | BasketballPass | RA, LD, AI | {22, 26, 30, 34, 38, 42} |

Table 4. QPs for sequences in TVD

| Sequence name | Configuration | QP values |
| --- | --- | --- |
| TVD-01_1 | RA, LD, AI | {20, 23, 26, 29, 32, 35} |
| TVD-01_2 | RA, LD, AI | {21, 24, 28, 31, 34, 37} |
| TVD-01_3 | RA, LD, AI | {23, 26, 30, 34, 38, 42} |
| TVD-02_1 | RA, LD, AI | {26, 32, 37, 42, 48, 53} |
| TVD-03_1 | RA, LD, AI | {34, 39, 42, 46, 50, 54} |
| TVD-03_2 | RA, LD, AI | {28, 32, 36, 40, 43, 49} |
| TVD-03_3 | RA, LD, AI | {22, 27, 32, 37, 42, 47} |

Table 5. QPs for image datasets

| Dataset | Configuration | QP values |
| --- | --- | --- |
| OpenImageV6 | AI | {22, 27, 32, 37, 42, 47} |
| FLIR (IR) | AI | {22, 27, 32, 37, 42, 47} |
| TVD-I | AI | {22, 27, 32, 37, 42, 47} |

Anchor results were generated following the CTC described in JVET-AC2031 and crosschecked by Ericsson and Tencent.

### Evaluation and reporting

Proposed technologies are evaluated based on their compression performance, measured by bitrate, PSNR, mAP and MOTA, as well as encoding and decoding runtime to reflect the complexity of the proposed technology, to some extent. Definitions and detailed descriptions of these metrics can be found in JVET-AC2031.

Figure 2. Reporting template summary

It is noted that the mean Average Precision (mAP) is used to measure object detection performance, and Multiple Object Tracking Accuracy (MOTA) is used to measure object tracking performance.

For the purpose of reporting encoding and decoding running times, the anchor and proposal should be simulated on the same platform, e.g. similar CPU and GPU configuration, to have reliable time comparison. Parallel encoding and decoding as described in JVET-B0036 may be applied for RA configurations.

In addition, relevant inference and training information should be reported if the proposed technology consists of learning-based components, such as network structure, number of network parameters, precision of network parameters, number of multiply–accumulate operations (MAC) per pixel, patch size, batch size, epoch, training time, training datasets, lost function, number of iterations, optimizer, and any pre- and post-processings used. More details can be found in JVET-AC2031.

A reporting template in Excel file format has been prepared to illustrate results following CTC and evaluation methodology described in JVET-AC2031. It is enclosed in JVET-AC2031 package. Some cleanups of the reporting template are suggested in input contribution JVET-AD0043.

## Technical Report

The draft 1 of the technical report (TR) has been prepared and uploaded as JVET-AC2030 based on discussions in the last JVET meeting on 2023-03-20. An input contribution JVET-AD0042 was submitted to suggest additional content to be discussed in this meeting. Another input contribution JVET-AD0135 was submitted to provide additional details on RoI-based adaptive QP for the TR.

## Git Management

Following the decision in the last JVET meeting, a git repository was set up for software development, experiments, document management and other activities that are related to this AHG, at https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm. This repository contains two projects, one containing instrucitons and information such as scripts for conducting experiements and evaluation, together with anchor results, while the other one containing implementation examples.

Two software example packages have been uploaded to the git, which can be found in the two branches under project https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/vtm-ofm:

- JVET-AB0275: a region of interest-based method that uses adaptive QP to reduce the quality in background areas

- JVET-AC0086: a method that uses a pre-analysis to perform content adaptive machine vision oriented preprocessing

## AHG Coordination

Intensive and collabrative discussions were held between this AHG and the VCM AHG in WG4 to synchronize common test conditions including test datasets, anchor QP points, encoder configurations, etc. By now the common test conditions of this AHG and the VTM branch of WG4 VCM AHG are largely aligned.

# Input contributions

There are 13 input contriubtions related to AHG 8 mandates. They are listed below.

| JVET-AD0008 | JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) | C. Hollmann, S. Liu, S. Wang, M. Zhou (AHG chairs) |
| --- | --- | --- |
| JVET-AD0042 | [AHG8] Proposed text for optimization of encoders and receiving systems for machine analysis of coded video content | J. Chen (Alibaba), C. Hollmann (Ericsson), S. Liu (Tencent) |
| JVET-AD0043 | [AHG8] Clean-ups for the reporting template | C. Hollmann (Ericsson) |
| JVET-AD0047 | AHG8: A preprocessing software implementation for OFM | B. Li, S. Wang, J. Chen, Y. Ye (Alibaba), S. Wang (CityU) |
| JVET-AD0060 | AHG8/AHG9: Indications related to suboptimal user viewing | M. M. Hannuksela, F. Cricri, H. Zhang (Nokia) |
| JVET-AD0110 | [AHG15] Evaluating and Improving Coding Performance for General Image Classification Models | Qi Zhang, Zhao Wang, Shanshe Wang, Siwei Ma |
| JVET-AD0113 | AHG8: Adaptive Mask Pre-Process algorithm for Video Coding for Machines | Z. Huang, Z. Wang, C. Jia, S. Wang, S. Ma |
| JVET-AD0122 | [AHG8] Configuration changes for random access and low delay | C. Hollmann, R. Sjöberg, J. Ström (Ericsson) |
| JVET-AD0135 | [AHG8] Additional details on RoI-based adaptive QP for TR | C. Hollmann, R. Sjöberg, J. Ström (Ericsson) |
| JVET-AD0137 | [AHG8] SEI for encoder optimization information | C. Kim, Hendry, D. Gwak, J. Lim(LGE) |
| JVET-AD0138 | [AHG8] QPA with low activity threshold for machine task | C. Kim, D. Gwak, J. Lim (LGE) |
| JVET-AD0181 | AHG8: Update on TVD object tracking dataset | Z. Liu, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AD0186 | AHG8: VVC tool analysis for Machines Vision Tasks | Z. Liu, L. Wang, X. Xu, S. Liu (Tencent) |

# Recommendations

The AHG recommends:

- Review all input contributions.

- Discuss and refine test conditions, evalution and reporting procedures.

- Discuss and refine anchor generation processes and results.

- Discuss existing and continue collecting new test materials.

- Continue investigating non-normative technologies and their suitability for machine analysis applications.

- Continue developing draft technical report on optimization of encoders and receiving systems for machine analysis of coded video content.