| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0011-v1 |
| --- | --- |

| Title: | JVET AHG report: Neural network-based video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Shan Liu Andrew Segall Franck Galpin Junru Li Ru-Ling Liao Dmytro Rusanovskyy Tong Shao Mathias Wien Ping Wu | Email: | elena.alshina@huawei.com shanl@tencent.com asegall@amazon.com Franck.Galpin@InterDigital.com lijunru@bytedance.com ruling.lrl@alibaba-inc.com dmytror@qti.qualcomm.com tshao@dolby.com wien@lfb.rwth-aachen.de pwuzte@gmail.com |
| Source: | AHG11 coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities of AHG11: Neural network-based video coding between the 29th meeting (11 – 20 January 2023) held by teleconference and the 39th meeting (21 – 28 April 2023) held in Antalya, TR.

# Introduction

The AHG was established with the following mandates:

- Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools and novel architectures.

- Discuss potential refinements of the test conditions for NN-based video coding in JVET-AB2016. Generate and distribute anchor encoding, and develop supporting software as needed.

- Study the impact of training (including the impact of loss functions) on the performance of candidate technologies, and identify suitable material for testing and training.

- Analyse complexity characteristics, perform complexity analysis, and develop complexity reductions of candidate technology.

- Finalize and discuss the EE on neural network-based video coding.

- Coordinate with other relevant groups, including SC29/AG5 on the evaluation and assessment of visual quality, and AHG12 on the interaction with ECM coding tools. If possible, prepare encodings with combinations of tools included in the NNVC software for visual quality assessment at the next meeting.

- Coordinate with AHG14 on items related to NNVC software development, and study the potential for unification of filter sets #0 and #1.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, for email. No emails were exchanged on the reflector related to the AHG mandates.

## Common Test Conditions

### Document

The AHG released revised common test conditions as decided at the 29th meeting.  The final version was uploaded as document JVET-AC2016 on February 15, 2023.

### Anchor Encoding

Anchors for the NN-based video coding activity made available on the Git repository used for the AHG activity: https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master.

## EE Coordination

The AHG finalized, conducted and discussed the EE on NN based video coding.  The final version of the EE description was uploaded to the document repository on February 6, 2023.

A summary report for the EE is available at this meeting as:

| JVET-AD0022 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang, Z. Xie (EE coordinators) |
| --- | --- | --- |

## Performance Evaluation

The performance of the NNVC-4.0 anchor compared to NNVC-3.0 is reported below.

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 102% | 107% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 102% | 106% | 100% |
| Class B | 0.01% | 0.03% | 0.05% | 0.02% | 0.06% | 0.06% | 103% | 108% | 100% |
| Class C | 0.00% | 0.00% | -0.01% | 0.00% | 0.00% | 0.00% | 102% | 108% | 100% |
| Class E | - | - | - | - | - | - | - | - | - |
| Overall | 0.00% | 0.01% | 0.01% | 0.01% | 0.02% | 0.02% | 102% | 107% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 103% | 107% | 100% |
| Class F | 0.00% | 0.01% | 0.00% | 0.00% | 0.00% | 0.00% | 103% | 108% | 100% |
| Class H | - | - | - | - | - | - | - | - | - |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | - | - | - | - | - | - | - | - | - |
| Class A2 | - | - | - | - | - | - | - | - | - |
| Class B | 0.00% | 0.05% | -0.03% | 0.01% | 0.03% | -0.01% | 102% | 107% | 100% |
| Class C | 0.03% | -0.03% | -0.01% | 0.01% | -0.04% | 0.00% | 101% | 104% | 100% |
| Class E | 0.00% | 0.00% | -0.03% | 0.00% | -0.01% | -0.02% | 103% | 108% | 100% |
| Overall | 0.01% | 0.01% | -0.02% | 0.01% | 0.00% | -0.01% | 102% | 106% | 100% |
| Class D | 0.00% | -0.06% | 0.00% | 0.00% | -0.03% | -0.03% | 101% | 105% | 100% |
| Class F | 0.02% | -0.18% | -0.08% | -0.04% | -0.27% | 0.07% | 102% | 109% | 100% |
| Class H | - | - | - | - | - | - | - | - | - |
|  | All Intra Main10 |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 106% | 106% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 106% | 106% | 100% |
| Class B | 0.00% | 0.01% | 0.02% | 0.00% | 0.01% | 0.02% | 106% | 108% | 100% |
| Class C | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 106% | 103% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 105% | 102% | 100% |
| Overall | 0.00% | 0.01% | 0.01% | 0.00% | 0.00% | 0.01% | 106% | 105% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 106% | 105% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 103% | 109% | 100% |
| Class H | - | - | - | - | - | - | - | - | - |

## Technical Evaluation

The AHG made meaningful progress on the mandate to evaluate and quantify potential NN based video coding technologies.  A summary of AHG11 related non-EE1 contributions provided as input to the 30th meeting is provided below:

|  | Title | Common Test Conditions | Results |  |  | Training Data |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RA | LDB | AI | CTC | Additional |
| Loop Filter |  |  |  |  |  |  |  |
| JVET-AD0050 | AHG11/AHG14: Fix input QP for NN filter set #1 | Yes | Yes | Yes | No | N/A | N/A |
| JVET-AD0109 | AHG11: Neural network loop filter | No | Partial | No | No | BVI-DVC | DIV2K |
| Post Filtering |  |  |  |  |  |  |  |
| JVET-AD0163 | AHG11: Updating NNPFC and NNPFA SEI messages for the NNPF | Yes | Yes | No | No | N/A | N/A |
| Intra-Prediction |  |  |  |  |  |  |  |
| JVET-AD0212 | AhG11 : neural network-based intra prediction with reduced complexity | Yes | Yes | Yes | No | BVI-DVC, TVD, UVG | ILSVCR2012, DIV2K |
| EE Related |  |  |  |  |  |  |  |
| JVET-AD0107 | EE1-related: Simplified parameter selection for filter set #1 | Yes | Yes | Yes | No | N/A | N/A |
| JVET-AD0157 | EE1-related: Neural-network loop filters in EE1-1.1.2 with further complexity reduction | Yes | Yes | No | Yes | BVI-DVC | DIV2K |
| JVET-AD0162 | EE1-2.1-related: DRF Model without QP Input | No | Yes | No | No | BVI-DVC, TVD | DIV2K |
| JVET-AD0170 | EE1 related: Performance Improvement of AC0052 filter for RPR-based SR in RA configuration | No | Yes | No | No | BVI-DVC, TVD | DIV2K |
| JVET-AD0189 | EE1-related: A simplified NN-based RDO model for filter set #1 | No | Yes | Yes | Yes | N/A | N/A |
| JVET-AD0207 | EE1-Related: Additional results for EE1-1.3.1 and EE1-1.3.5 | Yes | Yes | No | Yes | BVI-DVC | DIV2K |
| JVET-AD0211 | EE1-Related: Combination test of EE1-1.3.5 and multi-scale component of EE1-1.6 | Yes | Yes | No | Yes | BVI-DVC | DIV2K |
| JVET-AD0237 | EE1-related: Residue input for filter set #1 | No | Yes | Yes | Yes | BVI-DVC | DIV2K |

# Input contributions

There are 52 input contributions related to the AHG mandates. Twenty-three of the contributions are part of the EE activity, while the remaining 29 contributions are related to AHG11 but not part of the EE. The list of input contributions is provided below.

## EE and Related Input Contributions

| Reporting |  |  |
| --- | --- | --- |
| JVET-AD0022 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang, Z. Xie (EE coordinators) |
| EE Technology |  |  |
| JVET-AD0106 | EE1-1.6: In-loop filter with wide activation and large receptive field | Y. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AD0156 | EE1-1.1: Complexity Reduction on Neural-Network Loop Filter | J. N. Shingala, A. Shyam, A. Suneja, S. Badya (Ittiam), T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy (Dolby) |
| JVET-AD0160 | EE1-2.1: Deep Reference Frame Generation for Inter Prediction Enhancement | J. Jia, Y. Zhang, H. Zhu, Z. Chen (Wuhan Univ.), Z. Liu, X. Xu, S. Liu (Tencent) |
| JVET-AD0166 | EE1-1.7: Optimization of training and network for NNVC filter set 0 | R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AD0167 | EE1-1.7-related: Combination test for NNVC based on filter set 0 | R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AD0168 | EE1-1.4 Channel redistribution for luma and chroma | P. Wennersten, J. Ström, D. Liu (Ericsson) |
| JVET-AD0205 | EE1-1.3: Reduced complexity CNN-based in-loop filtering | S. Eadie, Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm) |
| JVET-AD0226 | EE1-1.5: Ablation Study and Minor Improvements on RTNN | H. Zhang, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO) |
| EE Related |  |  |
| JVET-AD0107 | EE1-related: Simplified parameter selection for filter set #1 | Y. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AD0157 | EE1-related: Neural-network loop filters in EE1-1.1.2 with further complexity reduction | T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy (Dolby), J. N. Shingala, A. Shyam, A. Suneja, S. Badya (Ittiam) |
| JVET-AD0162 | EE1-2.1-related: DRF Model without QP Input | J. Jia, D. Ding, W. Meng, Z. Chen (Wuhan Univ.), Z. Liu, X. Xu, S. Liu (Tencent) |
| JVET-AD0170 | EE1 related: Performance Improvement of AC0052 filter for RPR-based SR in RA configuration | S. Huang, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO) |
| JVET-AD0189 | EE1-related: A simplified NN-based RDO model for filter set #1 | Y. Li, J. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AD0207 | EE1-Related: Additional results for EE1-1.3.1 and EE1-1.3.5 | Y. Li, S. Eadie, D. Rusanovskyy, M. Karczewicz (Qualcomm) |
| JVET-AD0211 | EE1-Related: Combination test of EE1-1.3.5 and multi-scale component of EE1-1.6 | Y.Li, S.Eadie, D.Rusanovskyy, M.Karczewicz (Qualcomm) |
| JVET-AD0237 | EE1-related: Residue input for filter set #1 | Y. Li, K. Zhang, L. Zhang (Bytedance) |
| Cross Checks |  |  |
| JVET-AD0248 | Cross-check of JVET-AD0226 (EE1-1.5: Ablation Study and Minor Improvements on RTNN) | M. Santamaria (Nokia) |
| JVET-AD0250 | Crosscheck of JVET-AD0156 (EE1-1.1: Complexity Reduction on Neural-Network Loop Filter) | J. Ström, M. Damghanian (Ericsson) |
| JVET-AD0253 | Crosscheck of JVET-AD0162 (EE1-2.1: Deep Reference Frame Generation for Inter Prediction Enhancement) | N. GIULIANI |
| JVET-AD0270 | Crosscheck of JVET-AD0106 (EE1-1.6: In-loop filter with wide activation and large receptive field) | C. Zhou (vivo) |
| JVET-AD0282 | Crosscheck of EE1-1.6: In-loop filter with wide activation and large receptive field (JVET-AD0106) | Y. Li (Qualcomm) |
| JVET-AD0284 | Crosscheck of JVET-AD0205 (EE1-1.3: Reduced complexity CNN-based in-loop filtering) | Y. Li (Bytedance) |

## Non-EE Input Contributions

| Reporting |  |  |
| --- | --- | --- |
| JVET-AD0011 | JVET AHG report: Neural network-based video coding (AHG11) | E. Alshina, S. Liu, A. Segall (co chairs), F. Galpin, J. Li, R.-L. Liao, D. Rusanovskyy, T. Shao, M. Wien, P. Wu (vice chairs) |
| Loop Filtering |  |  |
| JVET-AD0050 | AHG11/AHG14: Fix input QP for NN filter set #1 | D. Liu, J. Ström, K. Andersson (Ericsson) |
| JVET-AD0109 | AHG11: Neural network loop filter | R. Yang, M. Santamaria, N. Zou, F. Cricri, R. G. Youvalari, J. Lainema, H. Zhang, M. M. Hannuksela (Nokia) |
| Post Filtering |  |  |
| JVET-AD0163 | AHG11: Updating NNPFC and NNPFA SEI messages for the NNPF | M. Santamaria, M. M. Hannuksela, F. Cricri, R. Yang (Nokia) |
| JVET-AD0052 | AHG9: NNPFC SEI extension mechanism to enable usages beyond user viewing | M. M. Hannuksela, F. Cricri, H. Zhang (Nokia) |
| JVET-AD0053 | AHG9: On NNPFC SEI for filtering input pictures and upsampling picture rate | M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia) |
| JVET-AD0054 | AHG9: On NNPFC SEI for picture rate upsampling | M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia) |
| JVET-AD0055 | AHG9: On buffering requirements for multi-picture NNPFC SEI | M. M. Hannuksela, F. Cricri (Nokia) |
| JVET-AD0059 | AHG9: Indicating processing order of NNPFs and SEI messages | M. M. Hannuksela, F. Cricri (Nokia) |
| JVET-AD0065 | AHG9: On NNPFC Auxiliary Input Information | S. Deshpande (Sharp) |
| JVET-AD0066 | AHG9: On NNPFC Description and Identification | S. Deshpande (Sharp) |
| JVET-AD0088 | AHG9: Overall post-processing filtering process using NNPFs | Y.-K. Wang, J. Xu, L. Zhang (Bytedance) |
| JVET-AD0089 | AHG9: NNPF activation parameters | Y.-K. Wang, Y. Li, J. Xu, C. Lin, J. Li, L. Zhang, K. Zhang (Bytedance) |
| JVET-AD0090 | AHG9: On input pictures to an NNPF | Y.-K. Wang, L. Zhang, C. Lin (Bytedance) |
| JVET-AD0091 | AHG9: Miscellaneous NNPF topics (set 1) | Y.-K. Wang, J. Xu, C. Lin, Y. Li, J. Li, L. Zhang, K. Zhang (Bytedance) |
| JVET-AD0094 | AHG9: On the NNPFC SEI message involving depth pictures | C. Lin, Y.-K. Wang, J. Xu, Y. Li, J. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AD0104 | AHG9: On NNPF Overlap | A. Sidiya, S. Deshpande (Sharp) |
| JVET-AD0141 | AHG9: On discardable and non-output pictures for NNPF SEI messages | Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE) |
| JVET-AD0143 | AHG9: On picture size for input and output pictures of NNPFC SEI message | Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE) |
| JVET-AD0145 | AHG9: On non-consecutive input pictures for NNPFC SEI message | Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE) |
| JVET-AD0149 | AHG9: On carriage of NNPF bitstream only in NNPFC SEI message | Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE) |
| JVET-AD0151 | AHG9: On output picture design in NNPFC SEI message | Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE) |
| JVET-AD0154 | AHG9: Additional purposes for the NNPFC SEI message | M. Pettersson, M. Damghanian, R. Sjöberg (Ericsson) |
| JVET-AD0159 | AHG9: Explicit identification of NNPF input pictures | Rickard Sjöberg, Martin Pettersson, Mitra Damghanian (Ericsson) |
| JVET-AD0233 | AHG9: Miscellaneous NNPF topics (set 2) | J. Xu, Y.-K. Wang, C. Lin, Y. Li, J. Li, L. Zhang, K. Zhang (Bytedance) |
| JVET-AD0240 | AHG9: Support more NNPF purposes | J. Xu, Y.-K. Wang, L. Zhang, J. Li, C. Lin (Bytedance) |
| JVET-AD0258 | AHG9: A summary of proposals on NNPF and SEI processing order SEI messages | Y.-K. Wang (Bytedance) |
| Intra Prediction |  |  |
| JVET-AD0212 | AhG11 : neural network-based intra prediction with reduced complexity | T. Dumas, F. Galpin, P. Bordes (Interdigital) |
| Cross Checks |  |  |
| JVET-AD0314 | Crosscheck of JVET-AD0163 (AHG11: Updating NNPFC and NNPFA SEI messages for the NNPF) | H. Zhang, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO) |

# Recommendations

The AHG recommends:

- Review all input contributions.

- Continue investigating neural network-based video coding tools, including coding performance and complexity.