| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, 21–28 April 2023 | Document: JVET-AD0047-v3 |
| --- | --- |

| Title: | AHG 8: A preprocessing software implementation for OFM |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Binzhe Li Shurun Wang Jie Chen Yan Ye (Alibaba) Shiqi Wang (City University of Hong Kong) | Email: | libinzhe.lbz@alibaba-inc.com srwang3-c@my.cityu.edu.hk jiechen.cj@alibaba-inc.com yan.ye@alibaba-inc.com shiqwang@cityu.edu.hk |
| Source: | Alibaba, City University of Hong Kong |  |  |

_____________________________

- Abstract

At the January meeting, it was agreed to create a software repository for AHG8 and to include preprocessing software based on JVET-AC0086 in this code repository. A software repository has been created since then and is referred to as optimization for machines (OFM). This contribution provides a description of the preprocessing part of the OFM software package, including the code architecture, instruction for installation, configuration and encoding/decoding scripts, and performance compared to VVC. This contribution suggests to add sections on the OFM software implementation to the technical report for the optimization of encoders and receiving systems for machine analysis of coded video content.

In version 3 of this contribution, it is further suggested to add some technical details of the pre-processing implementation to section 6.1 (Region of Interest-based methods) and section 6.2 (Pre-processing methods of foreground and background) in this contribution.

Preprocessing Software

Architecture

The architecture of the preprocessing software implementation is shown in Figure 1. To be specific, the input video is analyzed to extract the information for machine consumption. Based on the extracted information, block-based processing and feature-based blurring are performed on the input video for compression efficiency improvement, which is proposed in JVET-AC0086 [1]. Specifically, block-based processing eliminates redundancy based on the characteristics of the input video and the results from the analysis module. The representation efficiency is further improved with feature-based blurring, which achieves content adaptive processing on the basis of machine vision-oriented deep learning features. The pre-processed video is compressed and reconstructed with the VVC encoder and decoder. And then, the reconstructed video is consumed by machine vision models for ultimate results. This preprocessing software has been uploaded to GitLab repository for AHG8, with the repository link https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/vtm-ofm/-/tree/JVET-AC0086?ref_type=heads. In v3 of this contribution, section 6.1 (Region of Interest-based methods) and section 6.2 (Pre-processing methods of foreground and background) of the draft technical report are updated with technical details of the pre-processing implementation in JVET-AC0086 [1]. The exact proposed changes can be found in the attachment.

Figure 1 Workflow of the preprocessing software for OFM.

Installation

An installation file, install.sh, is prepared for the virtual environment setup and the software installation. The environment and software installation can be conducted automatically by “bash install.sh” under the OFM directory. After that, Detectron2 is also required in the virtual environment, which can be installed following the instructions at https://detectron2.readthedocs.io/en/latest/tutorials/install.html.

Source Code and Configuration

The source code directory of the preprocessing software for OFM is structured as follows:

- README.txt: contains introduction for the preprocessing part of OFM

- OFM_preprocessing: contains source code for the preprocessing part of OFM

  - Scripts: contains scripts to encode and decode SFU and TVD video datasets

  - VTM: contains VTM 12.0 source code

  - ROI_preprocess.py: source code of the preprocessing part for OFM

  - VTM_ffmpeg_JVET_codec.py: source code to merge the preprocessing and VTM codec

  - install.sh: scripts for installing the conda environment

The input and output of the preprocessing software for OFM are video sequences in YUV420 format. VTM is used as the inner codec. The input argument “--CompressMode” shows the configuration of VTM and can be set to “RA, LD, and AI” for random access, low delay, and all intra configurations, respectively.

The input argument “--Preprocess” can be specified to invoke the pre-processing method. The default pre-processing method is “Process_Blur”. If the argument is “None”, the reference software will bypass the pre-processing module. The intermediate results of compression and pre-processing are stored in the “OFM/Scripts/Temp_working”. If you need to keep the intermediate data, specify “--debug”. The implementation of the pre-processing module is available at “OFM/ROI_preprocess.py”.

Encoding/Decoding Scripts

Encoding and decoding scripts for the VCM video datasets are in the Scripts directory. The configurations of the encoding scripts are shown in the “batch_run_JVET_(SFU_yuv/track).py”, for SFU-HW and TVD video datasets, respectively. When pre-processing is performed, both CPU and GPU are used for video pre-processing; and just CPU is used for video coding. Since the encoding process may use both CPUs and GPUs, the following items should be adjusted according to the resources available in the user’s own computing platform:

cuda_devices: specifies the list of GPU devices to be used in the encoding process.

processes_per_gpu: specifies the number of processes running on each GPU. This number shall be specified according to the memory and processing capacity of the system.

cpu_per_gpu_process: specifies the number of CPU processes launched for each process. The product of cuda_devices, processes_per_gpu and cpu_per_gpu_process shall be less than the total number CPU cores available in the system.

Detailed instructions and explanations of scripts in the reference software for OFM are available in README.txt.

Performance

The BD-rate savings of the preprocessing software for OFM over the VVC anchors are calculated according to the reporting template of the Common Test Conditions (CTC) for optimization of encoders and receiving systems for machine analysis of coded video content [2], and shown in Table 1 and Table 2 for SFU-HW and TVD video datasets respectively. The overall Pareto BD-rate savings of the preprocessing software for OFM over VVC on SFU-HW dataset for object detection task are 14.61%, 11.54% and 21.91% under random access, low delay and all intra configurations, respectively. Moreover, for the TVD video dataset for object tracking task, the preprocessing software for OFM achieves 57.68%, 42.86% and 63.45% Pareto BD-rate savings under random access, low delay and all intra configurations, respectively. The overall results show that the preprocessing software is more effective at high resolution.

Table 1. Performance of the preprocessing software for OFM on SFU-HW Dataset for object detection task. (The ‘/’ means non-monotonous performance.)

|  | RA |  |  | LD |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate | BD-Rate Pareto | BD-mAP | BD-Rate | BD-Rate Pareto | BD-mAP | BD-Rate | BD-Rate Pareto | BD-mAP |
| Class A | / | -52.04% | 3.35 | -13.83% | -13.83% | 2.14 | / | -36.98% | 6.56 |
| Class B | / | -29.55% | 4.71 | -24.95% | -24.95% | 3.84 | -33.96% | -34.25% | 6.74 |
| Class C | -4.95% | -4.95% | 0.76 | / | -6.49% | 0.19 | -16.47% | -16.81% | 2.49 |
| Class D | 0.03% | 0.03% | -0.55 | -2.60% | -2.60% | -0.22 | -13.78% | -10.91% | 2.33 |
| Average | / | -14.61% | 1.77 | / | -11.54% | 1.34 | / | -21.91% | 4.06 |

Table 2. Performance of the preprocessing software for OFM on TVD video dataset for object tracking task. (The ‘/’ means non-monotonous performance.)

|  | RA |  |  | LD |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate | BD-Rate Pareto | BD-MOTA | BD-Rate | BD-Rate Pareto | BD-MOTA | BD-Rate | BD-Rate Pareto | BD-MOTA |
| TVD-01_1 | / | -82.98% | 12.78 | / | -65.38% | 14.33 | -87.72% | -87.57% | 24.79 |
| TVD-01_2 | -76.08% | -76.83% | 9.06 | / | -71.77% | 8.54 | -85.77% | -86.03% | 22.67 |
| TVD-01_3 | -56.02% | -56.80% | 8.25 | / | -25.57% | 4.10 | -74.59% | -74.06% | 14.26 |
| TVD-02_1 | 0.00% | 0.00% | 0.00 | 0.00% | 0.00% | 0.00 | 0.00% | 0.00% | 0.00 |
| TVD-03_1 | -51.21% | -48.61% | 7.58 | -29.31% | -23.89% | 5.31 | -56.68% | -54.29% | 12.42 |
| TVD-03_2 | -68.08% | -65.60% | 6.70 | / | -45.67% | 5.82 | -65.84% | -66.14% | 10.77 |
| TVD-03_3 | -68.89% | 0.00% | 7.96 | -65.14% | -62.43% | 6.14 | -76.10% | -76.08% | 14.58 |
| Average | / | -57.68% | 7.48 | / | -42.86% | 6.32 | -63.82% | -63.45% |  |

Conclusion

This preprocessing software implementation for OFM is described in this contribution. Simulation shows that preprocessing is effective and achieves superior performance to VVC on various machine tasks, test datasets and coding configurations. Specifically, this preprocessing software implementation for OFM achieves 14.61%, 11.54% and 21.91% Pareto BD-rate savings under random access, low delay and all intra configurations on SFU-HW dataset for object detection task. Considering TVD video dataset for object tracking task, 57.68%, 42.86% and 63.45% Pareto BD-rate savings are achieved under random access, low delay and all intra configurations, respectively. It is suggested to include information about the OFM software in the technical report for optimization of encoders and receiving systems for machine analysis of coded video content.

References

- B. Li, S. Wang, Y. Yan, S. Wang, “AHG15: Feature based Encoder-only algorithms for the Video Coding for Machines,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 input document JVET-AC0086, 29th JVET meeting, by teleconference, January 2023 (available online at https://jvet-experts.org/)

- ISO/IEC JTC 1/SC 29/WG 5, “Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 output document JVET-AC2031, 29th JVET meeting, by teleconference, January 2023 (available online at https://jvet-experts.org/).

Patent rights declaration(s)

Alibaba group and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

___________________