| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0051-v1 |
| --- | --- |

| Title: | AHG9: Common SEI Message of Generative Face Video |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bolin Chen, Jie Chen, Yan Ye, Shiqi Wang | Email: | chenbolin.chenboli@alibaba-inc.com jiechen.cj@alibaba-inc.com yan.ye@alibaba-inc.com shiqwang@cityu.edu.hk |
| Source: | Alibaba and City University of Hong Kong |  |  |

_____________________________

- Abstract

At the 29th JVET meeting, JVET-AC0088 proposed a generative face video SEI message to allow VVC-coded pictures to be used as source images to enable ultra low rate face video compression. In this updated proposal, we expand the proposed generative face video SEI message to include different facial representations, such as 2D keypoints, 2D landmarks, 3D keypoints, facial semantics and other formats. We demonstrate the effectiveness of the proposed SEI message in its capability to reconstruct high-quality talking face video at ultra-low bitrate. We also provide further information on rate allocation between VVC-coded pictures and SEI messasges and model complexity of a few typical generative systems. The proposed generative face video SEI message can be applicable to video conferencing, live entertainment and face animation applications.

Introduction

Recently, deep generative models, especially generative adversarial network (GAN) [1], have shown great potentials in face video compression [2]-[9]. Following the philosophy of early model-based coding [9]-[12], the generative compression codecs can economically model the input face video into a small number of the transmitted symbols (e.g., semantic parameters, facial landmarks, motion features, etc.) based on the analysis model, and subsequently the decoded symbols can facilitate the reconstruction of face images via the synthesis model. Compared to the early work on model-based video coding which had to rely on much simpler synthesis models, nowadays deep generative networks provide signifantly higher capability to infer and synthesize the reconstructed video, resulting in better reconstruction quality. It should be mentioned that a traditional video codec such as the VVC or HEVC can be employed to compress the first picture (or a set of key pictures) of a talking face video sequence as the base picture to provide essential texture reference. Then, the subsequent face pictures can be effectively reconstructed based on such base pictures and motion parameters from the GAN models. Figure 1 illustrates the general block diagram of generative compression codec.

Figure1: General block diagram of generative compression codec.

As shown in Table 1, we provide a summary of a few typical facial representations for generative face video compression algorithms. In particular, the face images exhibit strong statistical regularities, which can be economically characterized with 2D landmarks, 2D keypoints, region matrix, 3D keypoints, compact feature matrix or facial semantics. Such facial description strategies can lead to reduced coding bit-rate and improved coding efficiency. Thus, these are generally applicable to video conferencing and live entertainment.

Table 1 - Summary of facial representations for generative face video compression algorithms

| Facial Representation | Interpretation |
| --- | --- |
| 2D landamrks | VSBNet [4] is the representative model which can ulitize 98 groups of 2D facial landmarks R2×98 to depict the key structure information of human face, where the total number of encoding parameters for each inter frame is 196. |
| 2D keypoints+affine transformation matrix | FOMM [5] is the representative model which adopts 10 groups of learned 2D keypoints R2×10 along with their local affine transformations R2×2×10 to characterize complex motions. The total number of encoding parameters for each inter frame is 60. |
| region matrix | MRAA [6] is the representative model which extracts consistent regions of talking face to describe locations, shape, and pose, mainly represented with shift matrix R2×10, covariance matrix R2×2×10 and affine matrix R2×2×10. As such, the total number of encoding parameters for each inter frame is 100. |
| 3D keypoints | Face_vid2vid [7] is the representative model which can estimate 12-dimension head parameters (i.e., rotation matrix R3×3 and translation parameters R3×1 ) and 15 groups of learned 3D keypoint perturbations R3×15 due to facial expressions, where the total number of encoding parameters for each inter frame is 57. |
| compact feature martix | CFTE [8] is the representative model which can model the temporal evolution of faces into learned compact feature representation with the matrix R4×4, where the total number of encoding parameters for each inter frame is 16. |
| facial semantics | IFVC [9] is the representative model which adopts a collection of transmitted facial semantics to represent the face frame, including mouth parameters R6, eye parameter R1, rotation parameters R3, translation parameters R3 and location parameter R1. Totally, the number of encoding parameters for each inter frame is 14. |

Proposal

In this section, we describe the proposed SEI message which is designed according to these categories. This SEI message carries facial parameters for different feature representations listed in Table 1, such as 2D keypoints, 2D landmarks, 3D keypoints, facial semantics and other fomarts, which may be used for face generative compression. For these facial representations, they can be categorized into three types, including coordinate parameters, matrixs parameters and semantic paramters. It should be mentioned that every SEI message has a base picture that is the picture contained in the same PU as the SEI message. This base picture could be a dummy picture used to carry these facial parameters. Based on the base picture, the facial parameters in the SEI message can be used to reconstruct face picture, and each set of the parameters in the SEI message is used to generate one face picture.

Proposed SEI message scope

Table 2 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| … | … |
| Generative_face_video | The PU contains the SEI message, and the PU can be a dummy picture |

Syntax of generative face video SEI message

| Generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| coordinate_present_flag | u(1) |
| if(coordinate_present_flag){ |  |
| coordinate_quantization_factor | ue(v) |
| is_3D_coordinate_flag | u(1) |
| coordinate_point_num | ue(v) |
| for(i=0; i< coordinate_point_num;i++){ |  |
| x[i]; | ue(v) |
| y[i]; | ue(v) |
| if(is_3D_coordinate_flag){ |  |
| z[i]; | ue(v) |
| } |  |
| } |  |
| } |  |
| matrix_present_flag | u(1) |
| if(matrix_present_flag){ |  |
| matrix_quantization_factor | ue(v) |
| matrix_type_num | ue(v) |
| for(j=0; j<matrix_type_num;j++){ |  |
| matrix_idx[j] | ue(v) |
| if(coordinate_present_flag){ |  |
| matrix_num_equal_to_coordinate_point _flag[j] | u(1) |
| if(! coordinate_present_flag ¦¦ ! matrix_num_equal_to_coordinate_flag[j]){ |  |
| matrix_num[j] | ue(v) |
| MatrixNum[j]=matrix_num[j] |  |
| } |  |
| else{ |  |
| MatrixNum[j]= coordinate_point_num |  |
| } |  |
| } |  |
| matrix_width[j] | ue(v) |
| matrix_height[j] | ue(v) |
| for(k=0; k<MatrixNum[j]; k++){ |  |
| for(m=0; m< matrix_height[j];m++){ |  |
| for(n=0;n<matrix_width[j];n++){ |  |
| matrix_element[j][k][m][n]; | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| semantic_present_flag | u(1) |
| if(semantic_present_flag){ |  |
| semantic_quantization_factor | ue(v) |
| semantic_type_num | ue(v) |
| for(l=0; l<semantic_type_num;l++){ |  |
| semantic_idx[l] | ue(v) |
| semantic_width[l] | ue(v) |
| semantic_height[l] | ue(v) |
| for(y=0; y< semantic_width[l];y++){ |  |
| for(z=0; z< semantic_height[l];z++){ |  |
| semantic_element[l][y][z]; | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

Semantics of generative face video SEI message

gfv_id contains an identifying number that may be used to identify a generative face video filter. The value of gfv_id shall be in the range of 0 to 232 − 2, inclusive.

coordinate_present_flag equal to 1 indicates the SEI message carries the coordinate parameters. coordinate_present_flag equal to 0 indicates the SEI message dose not carry the coordinate parameters.

coordinate_quantization_factor specifies the quantization factor to process the facial coordinate parameters (i.e., x[i], y[i] and z[i]). The values of parameters used for face generation are equal to the values of corresponding syntax elements divided by coodrinate_quantization_factor.

is_3D_coordinate_flag equal to 1 indicates the coordinate parameters belong to three dimensional space, which can be representated with (x[i], y[i], z[i]). is_3D_coordinate_flag equal to 0 indicates the coordinate parameters belong to two dimensional space, which can be representated with (x[i], y[i]).

coordinate_point_num specifies the number of facial coordinate parameter set in the SEI message. One set of parameters represents one point in the coordinate. The value of coordinate_point_num shall be in the range of 0 to 210 , inclusive.

x[i] specifies the quantized x-axis value for i_th keypoint.

y[i] specifies the quantized y-axis value for i_th keypoint.

z[i] specifies the quantized z-axis value for i_th keypoint.

matrix_present_flag equal to 1 indicates the SEI message carries the matrix parameters. matrix_present_flag equal to 0 indicates the SEI message dose not carry the matrix parameters.

matrix_quantization_factor specifies the quantization factor to process the facial matrix paramters (i.e., matrix_element[j][k][m][n]). The values of parameters used for face generation are equal to the values of corresponding syntax elements divided by matrix_quantization_factor.

matrix_type_num specifies the number of facial matrix type in the SEI message. The value of matrix_type_num shall be in the range of 0 to 26 , inclusive. It should be mentioned that in our proposed SEI message, the facial matrix type can be categized into affine translation matrix, covariance matrix, rotation matrix, translation matrix and compact feature matrix. And these matrixs can be further distinguished whether is related with the points in the coordinate.

matrix_idx contains an identifying number regarding which facial matrix may be used in the SEI message.

Table 3 – Definition of matrix_idx

| Value | Interpretation |
| --- | --- |
| 0 | Affine translation matrix corresponding to the coordinate_point_num. Every keypoint in the coordinate may have a group of 2*2 affine translation matrix. |
| 1 | Covariance matrix corresponding to the coordinate_point_num. Every keypoint in the coordinate may have a group of 2*2 covariance matrix. |
| 2 | Rotation matrix with 3*3 dimension to represent the head rotation |
| 3 | Tranlation matrix with 1*3 dimension to represent the head translation |
| 4 | Compact feature matrix with 4*4 dimension |

Note: If there are other facial matrix representations that are different from the given matrix types, the matrix_idx can be further expanded.

matrix_num_equal_to_coordinate_point _flag[j] equal to 1 indicates the number of facial matrix is equal to coordinate_point_num.

matrix_num[j] specifies the number of facial matrix when coordinate_present_flag or matrix_num_equal_to_coordinate_point _flag[j] do not exist. Otherwise, if these two flags both exist, the number of facial matrix is equal to coordinate_point_num.

matrix_width[j] specifies the width (the number of row) of the j_th facial matrix and matrix_width[j] must be equal to or larger than 1.

matrix_height[j] specifies the height (the number of column) of the j_th facial matrix and matrix_height[j] must be equal to or larger than 1.

matrix_element[j][k][m][n] specifies the quantized element value from the corresponding facial matrix.

semantic_present_flag equal to 1 indicates the SEI message carries the semantic parameters. semantic_present_flag equal to 0 indicates the SEI message dose not carry the semantic parameters.

semantic_quantization_factor specifies quantization factor to process the facial semantic parameters (i.e., semantic_element[l][y][z]). The values of parameters used for face generation are equal to the values of corresponding syntax elements divided by semantic_quantization_factor.

semantic_type_num specifies the number of facial semantic type in the SEI message. The value of semantic_type_num shall be in the range of 0 to 26 , inclusive. It should be mentioned that in our proposed SEI message, the facial semantic type can be categized into mouth parameters, eye parameters, head rotation parameters, head translation parameters and head location parameters.

semantic_idx contains an identifying number regarding which facial semantic may be used in the SEI message.

Table 4 – Definition of semantic_idx

| Value | Interpretation |
| --- | --- |
| 0 | Mouth paramters with 1*6 dimension |
| 1 | Eye paramters with 1*1 dimension, representing the open-close status. |
| 2 | Head rotation paramters with 1*3 dimension, including yaw, roll and pitch |
| 3 | Head translation paramters with 1*3 dimension, including x-axis, y-axis and z-axis |
| 4 | Head location index with 1*1 dimension used for locate the face from the input image |

Note: If there are other facial semantic representations that are different from the given semantic types, the semantic_idx can be further expanded.

semantic_width[l] specifies the width (the number of row) of the l_th semantic type and semantic_width[l] must be equal to or larger than 1.

semantic_height[l] specifies the height (the number of column) of the l_th semantic type and semantic_height[l] must be equal to or larger than 1.

semantic_element[l][y][z] specifies the quantized element value from the corresponding facial semantic.

Supplementary Information

We also provide further information on rate allocation between VVC-coded pictures and SEI messasges and model complexity of a few typical generative systems.

Rate Allocation

In this section, we calculate the approximate bit percentage for different compression systems employing different SEI messages. It should be mentioned that the total bit consumption includes the SEI bits, the dummy picture bits and the VVC-coded first frame bits. The specific bit consumption can be seen in Table 5.

Table 5 –Bit consumption of generative face video SEI message for different compression systems, dummy picture and VVC-coded first frame

| Compression System |  | FOMM[5] | MRAA[6] | Face_vid2vid[7] | CFTE[8] | IFVC[9] |
| --- | --- | --- | --- | --- | --- | --- |
| Facial Representation |  | 2D keypoints | region matrix | 3D keypoints | compact feature martix | facial semantics |
| SEI bits |  | 242 | 239 | 192 | 154 | 146 |
| Dummy picture bits |  | 40 |  |  |  |  |
| VVC-coded first frame bits | QP: 27 | 48871 |  |  |  |  |
|  | QP: 32 | 30400 |  |  |  |  |
|  | QP: 37 | 18767 |  |  |  |  |
|  | QP: 42 | 11441 |  |  |  |  |

Herein, we provide two situations to calculate the approximate bit percentage.

1) we choose 10 sequences with 100 frames and execute the defined syntax for generative face video coding. The approximate bit percentage can be calculated as follows:

Bit_percentage=(Bit_dummy+Bit_SEI)×99Bit_VVC+(Bit_dummy+Bit_SEI)×99.

Table 6 – Bit percentage of generative face video SEI message for different compression systems

| Compression System | FOMM[5] | MRAA[6] | Face_vid2vid[7] | CFTE[8] | IFVC[9] |
| --- | --- | --- | --- | --- | --- |
| Facial Representation | 2D keypoints | region matrix | 3D keypoints | compact feature martix | facial semantics |
| Iframe_QP: 27 | 31.20% | 30.93% | 26.46% | 22.39% | 21.48% |
| Iframe_QP: 32 | 41.08% | 40.78% | 35.61% | 30.73% | 29.61% |
| Iframe_QP: 37 | 51.32% | 51.01% | 45.54% | 40.15% | 38.87% |
| Iframe_QP: 42 | 60.87% | 60.57% | 55.24% | 49.75% | 48.41% |

2) we choose 10 sequences with 250 frames and execute the defined syntax for generative face video coding. The approximate bit percentage can be calculated as follows:

Bit_percentage=(Bit_dummy+Bit_SEI)×249Bit_VVC+(Bit_dummy+Bit_SEI)×249.

Table 7 – Bit percentage of generative face video SEI message for different compression systems

| Compression System | FOMM[5] | MRAA[6] | Face_vid2vid[7] | CFTE[8] | IFVC[9] |
| --- | --- | --- | --- | --- | --- |
| Facial Representation | 2D keypoints | region matrix | 3D keypoints | compact feature martix | facial semantics |
| Iframe_QP: 27 | 50.60% | 50.29% | 44.83% | 39.46% | 38.19% |
| Iframe_QP: 32 | 59.89% | 59.59% | 54.22% | 48.72% | 47.39% |
| Iframe_QP: 37 | 67.72% | 67.44% | 62.47% | 57.17% | 55.86% |
| Iframe_QP: 42 | 73.79% | 73.55% | 69.08% | 64.18% | 62.95% |

Model Complexity

Complexity is considered more critical in the inference stage for many applications. In this section, we provide the detailed model parameters and computional complexity for a few typical generative systems.

Params : Total numbers of parameters for all models in the neural network.

MAC: Number of multiply-accumulate operations for the inference stage.

Table 7 – Model complexity of different compression systems with different facial representations.

| Compression System |  | FOMM[5] | MRAA[6] | Face_vid2vid[7] | CFTE[8] | IFVC[9] |
| --- | --- | --- | --- | --- | --- | --- |
| Facial Representation |  | 2D keypoints | region matrix | 3D keypoints | compact feature martix | facial semantics |
| Encoder | Params (M) | 14.22 | 14.15 | 72.20 | 14.13 | 40.29 |
|  | MAC (G) | 1.28 | 1.059 | 0.32 | 0.99 | 22.52 |
| Decoder | Params (M) | 45.58 | 45.58 | 53.11 | 43.89 | 44.59 |
|  | MAC (G) | 53.65 | 53.65 | 178.28 | 54.82 | 117.49 |

Coding Results

Figure 2 illustrates that generative face video SEI messages can operate at ultra-low bit rate range that are difficult to reach for traditional codecs.

- Rate-DISTS                                 (b) Rate-LPIPS

Figure2: Rate-distortion performance comparisons with the traditional codec (VVC) and generative compression systems with SEI messages (FOMM, MRAA, Face_vid2vid, CFTE and IFVC) in terms of Rate-DISTS and Rate-LPIPS.

Conclusion

A generative face video SEI message that supports a series of facial representations is proposed in this contribution. This proposed SEI message can be used to animate the base pictures coded using VVC or HEVC, and to reconstruct high quality face video at extremely low bitrate. Therefore, the proposed SEI can be applicable to video conferencing, live entertainment, face animation and metaverse-related functionalities. It is suggested to consider the proposed SEI message in future version of VSEI.

References

- Goodfellow I, Pouget-Abadie J, Mirza M, et al. “Generative adversarial nets,” Advances in neural information processing systems, vol. 27, 2014.

- Oquab M, Stock P, Haziza D, et al. Low bandwidth video-chat compression using deep generative models[C]//Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition. 2021: 2388-2397.

- Konuko G, Valenzise G, Lathuilière S. Ultra-low bitrate video conferencing using deep image animation[C]//ICASSP 2021-2021 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP). IEEE, 2021: 4210-4214.

- Feng D, Huang Y, Zhang Y, et al. A generative compression framework for low bandwidth video conference[C]//2021 IEEE International Conference on Multimedia & Expo Workshops (ICMEW). IEEE, 2021: 1-6.

- Siarohin A, Lathuilière S, Tulyakov S, et al. First order motion model for image animation[J]. Advances in Neural Information Processing Systems, 2019, 32.

- Siarohin A, Woodford O J, Ren J, et al. Motion representations for articulated animation[C]//Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition. 2021: 13653-13662.

- Wang T C, Mallya A, Liu M Y. One-shot free-view neural talking-head synthesis for video conferencing[C]//Proceedings of the IEEE/CVF conference on computer vision and pattern recognition. 2021: 10039-10049.

- Chen B, Wang Z, Li B, et al. Beyond keypoint coding: Temporal evolution inference with compact feature representation for talking face video compression[C]//2022 Data Compression Conference (DCC). IEEE, 2022: 13-22.

- Chen B, Wang Z, Li B, et al. Interactive Face Video Coding: A Generative Compression Framework[J]. arXiv preprint arXiv:2302.09919, 2023.

- Pearson D E, Robinson J A. Visual communication at very low data rates[J]. Proceedings of the IEEE, 1985, 73(4): 795-812.

- Musmann H G, Hötter M, Ostermann J. Object-oriented analysis-synthesis coding of moving images[J]. Signal processing: Image communication, 1989, 1(2): 117-138.

- Nakaya Y, Chuah Y C, Harashima H. Model-based/waveform hybrid coding for videotelephone images[C]//Acoustics, Speech, and Signal Processing, IEEE International Conference on. IEEE Computer Society, 1991: 2741, 2742, 2743, 2744-2741, 2742, 2743, 2744.

- Chung J S, Nagrani A, Zisserman A. , “Voxceleb2: Deep speaker recognition,” in Interspeech, 2018

- Chen B, Chen J, et al. “AHG9: Generative Face Video SEI Message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AC0088, January 2023.

Patent rights declaration(s)

Alibaba and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).