| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0053-v2 |
| --- | --- |

| Title: | AHG9: On NNPFC SEI for filtering input pictures and upsampling picture rate |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Francesco Cricri Maria Santamaria Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This input document asserts that:

- The constraint in JVET-AC2032 regarding the output order of pictures resulting from neural-network post-filter (NNPF) activations is unclear.

- Filtering the same input picture more than once is reasonable in an NNPF that combines picture rate upsampling and another purpose, such as visual quality enhancement. However, this is presumably disallowed in JVET-AC2032.

Version 2 of this input document provides an update in response to the NNPF BOG recommendations: "For a particular NNPF, disallow it to output a picture multiple times for any particular output order or output time instance" and "For all NNPFs activated for a bitstream, disallow the NNPFs (when alternative, only one is used) to output a picture multiple times for any particular output order or output time instance". The following items are proposed in version 2 of this input document:

- Remove the reportedly unclear constraint regarding the output order of pictures resulting from applying the NNPF in output order.

- Constrain the sequence of output pictures formed by concatenating the output pictures in the order that output tensors are generated by applying the NNPF as activated by the NNPFA SEI messages:

  - to contain the output pictures in output order, and

  - to contain at most one output picture representing the same input picture.

# Introduction

## Background

The semantics of the NNPFC SEI message in JVET-AC2032 contain the following regarding the output order of pictures generated by applying the NNPF:

The order of the pictures in the stored output tensor is in output order, and the output order generated by applying the NNPF in output order is interpreted to be in output order (and not conflicting with the output order of the input pictures).

In our understanding, the latter part of the sentence (after the comma) attempts to disallow filtering of the same input picture more than once and states that the activation of the NNPF results into output tensors that would, when concatenated into a sequence, contain filtered pictures in output order.

## Problem statement 1

In our opinion, the sentence cited in Section 1.1 is ambiguous, because of the following reasons:

- The phrase "by applying the NNPF in output order" supposedly means successive application of the NNPF as activated by the NNPFA SEI message(s), but it does not refer to activation controlled by NNPFA SEI message(s) explicitly.

- If the constraint refers to applying NNPF as activated by the NNPFA SEI message(s), it is misplaced, since it not only concerns the NNPFC SEI message but also the NNPFA SEI message(s) and is hence a bitstream conformance constraint relating to the appearance of both NNPFC and NNPFA SEI messages in the bitstream.

- The phrase "is interpreted to be in output order" is a statement and not a requirement or constraint.

- What does "not conflicting with the output order of the input pictures" mean?

## Problem statement 2

It is asserted that filtering the same input picture more than once is reasonable in an NNPF that combines picture rate upsampling and visual quality enhancement. This case is illustrated through the example below.

Figure 1 presents an example of a picture rate upsampling NNPF that takes two pictures as input, interpolates one output picture between the input pictures, and also enhances the visual quality of all the input pictures. Values indicate a relative output order of pictures.

| Input pictures | 0 |  | 2 |
| --- | --- | --- | --- |
|  |  | ↓ |  |
|  | NNPF |  |  |
|  |  | ↓ |  |
| Output pictures | 0 | 1 | 2 |

Figure 1 – An example of an NNPF for picture rate upsampling and visual quality enhancement

Figure 2 presents an example of two activations of the example NNPF, first at picture 2 and the second time at picture 4. It can be observed that picture 2 gets filtered in both activations, and hence we believe that this NNPF or activating this NNPF for each pair of pictures does not comply with the statement of JVET-AC2032 that is cited in Section 1.1.

| 1st activation | Input pictures | 0 |  | 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Output pictures | 0 | 1 | 2 |  |  |
| 2nd activation | Input pictures |  |  | 2 |  | 4 |
|  | Output pictures |  |  | 2 | 3 | 4 |

Figure 2 – An example of a picture rate upsampling NNPF

In our opinion, it is reasonable to allow filtering the same input picture more than once for a filter with both picture rate upsampling and visual quality enhancement.

# Proposal

Remove the text with strikethrough:

The order of the pictures in the stored output tensor is in output order, and the output order generated by applying the NNPF in output order is interpreted to be in output order (and not conflicting with the output order of the input pictures).

Add the following, and if JVET-AD0388 is recommended for adoption, also add the yellow-highlighted text:

It is a requirement of bitstream conformance that when the pictures that are present in the output tensors and are either interpolated pictures or determined to be output as specified in the semantics of the NNPFA SEI message are concatenated into a sequence in the order that the output tensors are generated by applying the NNPF having any particular nnpfc_id value as activated by the NNPFA SEI messages with nnpfa_target_id equal to that particular nnpfc_id value, the sequence shall contain the pictures in an order that does not conflict with the output order of the input pictures and the sequence shall contain at most one picture representing the same input picture.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).