| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0054-v1 |
| --- | --- |

| Title: | AHG9: On NNPFC SEI for picture rate upsampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Francesco Cricri Maria Santamaria Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This input document asserts that the VVC text for selecting input pictures for an NNPF that uses multiple input pictures has the following problems:

- When an NNPF uses multiple input pictures, the VVC text of selecting input pictures does not check if the preceding picture exists in the bitstream.

- The current VVC text can use input pictures of the previous CLVS. It is asserted that to follow the design principle of limiting the scope of an SEI message to be within a single CLVS and to allow splicing a CLVS into another bitstream, the input pictures of an NNPF should all originate from the same CLVS.

If the input pictures of an NNPF are constrained so that they all originate from the same CLVS and a picture rate upsampling NNPF does not interpolate pictures between the first pair of input pictures or the last pair of input pictures, it is asserted that the NNPF cannot be applied to upsample the picture rate at the start or at the end of a CLVS, respectively. For example, this happens when the NNPF takes 4 pictures as input and interpolates pictures between the midmost pair of input pictures only.

The following items are proposed in this input document:

- A limitation that input pictures of an NNPF originate from the current CLVS only.

- An indication nnpfc_absent_input_pic_zero_flag in the NNPFC SEI message that indicates how pictures that would not originate from the current CLVS are expected to be replaced in the input tensor.

  - nnpfc_absent_input_pic_zero_flag equal to 1 indicates that the NNPF expects an input picture that is not present in the current CLVS to be represented sample arrays with sample values equal to 0.

  - nnpfc_absent_input_pic_flag equal to 0 indicates that the NNPF expects an input picture that is not present in the current CLVS to be represented by the closest input picture in output order within the current CLVS.

- A constraint that NNPF shall not be activated so that it creates an interpolated picture before the CLVS start in output order.

- At the end of a CLVS, repeated inference of NNPF for creating interpolated pictures.

When an NNPF is active until the end of the CLVS, the NNPF is applied up to but excluding a set of input pictures that would cause creation of any interpolated picture after the last picture of the CLVS in output order.

# Introduction

## Existence of input pictures within the bitstream and current CLVS

The VVC text for selecting input pictures for an NNPF that uses multiple input pictures has the following asserted problems:

- When an NNPF uses multiple input pictures, the VVC text of selecting input pictures does not check if the preceding picture exists in the bitstream.

- The current VVC text can use input pictures of the previous CLVS. It is asserted that to follow the design principle of limiting the scope of an SEI message to be within a single CLVS and to allow splicing a CLVS into another bitstream, the input pictures of an NNPF should all originate from the same CLVS.

For convenience, the relevant VVC text from JVET-AC2005 is copied below:

The array inputPicPoc[ i ] for all values of i in the range of 0 to numInputPics − 1, inclusive, specifying the picture order count values of the input pictures for the NNPF, is derived as follows:

–	inputPicPoc[ 0 ] is set equal to PicOrderCntVal of currCodedPic.

–	When numInputPics is greater than 1, the following applies for each value of i in the range of 1 to numInputPics − 1, inclusive, in increasing order of i:

–	If currCodedPic is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and a particular value of fp_current_frame_is_frame0_flag, inputPicPoc[ i ] is set equal to PicOrderCntVal of the picture that precedes, in output order, the picture associated with index i − 1 and is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

–	Otherwise (currCodedPic is not associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5), inputPicPoc[ i ] is set equal to PicOrderCntVal of the picture that precedes, in output order, the picture associated with index i − 1.

## Picture rate upsampling at the start and at the end of CLVS

This subsection discusses using picture rate upsampling NNPF at the start and at the end of a CLVS, assuming that the input pictures of an NNPF are constrained so that they all originate from the same CLVS.

Figure 1 presents an example of a picture rate upsampling NNPF that takes four pictures as input and interpolates one output picture between the middle pair of the input pictures. Values indicate a relative output order of pictures.

| Input pictures | 0 |  | 2 |  | 4 |  | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ↓ |  |  |  |  |  |  |
|  | NNPF |  |  |  |  |  |  |
|  | ↓ |  |  |  |  |  |  |
| Output pictures |  |  |  | 3 |  |  |  |

Figure 1 – An example of a picture rate upsampling NNPF

To produce a constant picture rate throughout the CLVS with the same example NNPF, the use of the example NNPF in the following two special cases is proposed to be enabled:

- Interpolate an output picture between the first two pictures of a CLVS (Figure 2)

- Interpolate an output picture between the second last picture and the last picture of a CLVS (Figure 3)

| Input pictures | 0 | CLVS boundary | 2 |  | 4 |  | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↓ |  |  |  |
|  |  |  | NNPF |  |  |  |  |
|  |  |  |  | ↓ |  |  |  |
| Output pictures |  |  |  | 3 |  |  |  |

Figure 2 – The example picture rate upsampling NNPF at the start of a CLVS.

| Input pictures | 0 |  | 2 |  | 4 | CLVS boundary | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↓ |  |  |  |
|  |  |  | NNPF |  |  |  |  |
|  |  |  |  | ↓ |  |  |  |
| Output pictures |  |  |  | 3 |  |  |  |

Figure 3 – The example picture rate upsampling NNPF at the end of a CLVS.

If the proposal were not enabled, it would not be possible to generate the interpolated picture between the first two pictures of a CLVS or the last two pictures of a CLVS with the example NNPF. It is remarked that the greater the number of input pictures, the longer gap for the interpolated pictures at the start of a CLVS and at the end of a CLVS would be.

# Proposal

## Overview of the proposal

The proposal has the following ingredients:

- A limitation that input pictures of an NNPF originate from the current CLVS only.

The changes related to item 2 are indicated in pink highlight below.

- An indication of how input pictures that would not originate from the current CLVS are replaced.

In our experience, input pictures that would not originate from the current CLVS can be replaced using either of the following strategies:

- An NNPF can be trained to omit an input picture that has certain pre-defined sample values, such as all sample values equal to 0.

- If an NNPF is not trained to omit an input picture that has certain pre-defined sample values, it is asserted that a subtle impact would be likely achieved if the input pictures not originating from the current CLVS are copies of the closest input picture within the CLVS.

It is proposed to indicate with a new syntax element nnpfc_absent_input_pic_zero_flag in the NNPFC SEI message which strategy has been used in the NNPF.

The changes related to item 1 are indicated in yellow highlight below.

- A constraint that NNPF shall not be activated so that it creates an interpolated picture before the CLVS start in output order.

The changes related to item 3 are indicated in cyan highlight below.

- At the end of a CLVS, repeated inference of NNPF for creating interpolated pictures.

When an NNPF is active until the end of the CLVS, the NNPF is applied up to but excluding a set of input pictures that would cause creation of any interpolated picture after the last picture of the CLVS in output order.

The changes related to item 4 are indicated in green highlight below.

Gray highlights below indicate changes common to all proposal items.

## VSEI addition

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( nnpfc_num_input_pics_minus1 > 0 ) |  |
| nnpfc_absent_input_pic_zero_flag | u(1) |
| … |  |

nnpfc_absent_input_pic_zero_flag equal to 1 indicates that the NNPF expects an input picture that is not present in the current CLVS to be represented by sample arrays with sample values equal to 0. nnpfc_absent_input_pic_flag equal to 0 indicates that the NNPF expects an input picture that is not present in the current CLVS to be represented by the closest input picture in output order within the current CLVS.

## VVC changes

The variable numInferences is derived as follows:

–	If pictureRateUpsamplingFlag is equal to 1, the NNPFA SEI message that activated this NNPF has nnpa_persistence_flag equal to 1, nnpfc_interpolated_pics[ i ] is greater than 0 only for a single value of i that is greater than 0, and currCodedPic is the last picture in the current CLVS, the variable numPostRoll is set equal to the value of i such that nnpfc_interpolated_pics[ i ] is greater than 0 and the variable numInferences is set equal to 1 + numPostRoll.

–	Otherwise, the variable numInferences is set equal to 1.

For each value of j in the range of 0 to numInferences − 1, inclusive, the following applies:

–	The arrays inputPicPoc[ i ] and inputPicPresentFlag[ i ] for all values of i in the range of 0 to numInputPics − 1, inclusive, specifying the picture order count values of the input pictures for the NNPF and the presence of input pictures within the current CLVS, respectively, are derived as follows:

–	When j is greater than 0, for each value of k in the range of 0 to j − 1, inclusive, inputPicPoc[ k ] is set equal to PicOrderCntVal of currCodedPic and inputPicPresentFlag[ k ] is set equal to 0.

–	inputPicPoc[ 0 ] is set equal to PicOrderCntVal of currCodedPic.

–	inputPicPoc[ j ] is set equal to PicOrderCntVal of currCodedPic and inputPicPresentFlag[ j ] is set equal to 1.

–	When numInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numInputPics − j − 1, inclusive, in increasing order of i:

–	If currCodedPic is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and a particular value of fp_current_frame_is_frame0_flag, inputPicPoc[ i ] is set equal to PicOrderCntVal of the picture that precedes, in output order, the picture associated with index i − 1 and is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

–	If currCodedPic is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and a particular value of fp_current_frame_is_frame0_flag, the following applies:

–	If the current CLVS contains a picture prevPic that precedes, in output order, the picture associated with index i − 1 and is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag, inputPicPoc[ i ] is set equal to PicOrderCntVal of prevPic and inputPicPresentFlag[ i ] is set equal to 1.

–	Otherwise, the following applies:

–	inputPicPoc[ i ] is set equal to inputPicPoc[ i − 1 ] and inputPicPresentFlag[ i ] is set equal to 0.

–	It is a requirement of bitstream conformance that num_interpolated_pics[ i − 1 ] shall not be greater than 0.

–	Otherwise (currCodedPic is not associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5), inputPicPoc[ i ] is set equal to PicOrderCntVal of the picture that precedes, in output order, the picture associated with index i − 1.

–	Otherwise (currCodedPic is not associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5), the following applies:

–	If the current CLVS contains a picture prevPic that precedes, in output order, the picture associated with index i – 1, inputPicPoc[ i ] is set equal to PicOrderCntVal of prevPic and inputPicPresentFlag[ i ] is set equal to 1.

–	Otherwise, the following applies:

–	inputPicPoc[ i ] is set equal to inputPicPoc[ i − 1 ] and inputPicPresentFlag[ i ] is set equal to 0.

–	It is a requirement of bitstream conformance that num_interpolated_pics[ i − 1 ] shall not be greater than 0.

…

–	The luma sample arrays CroppedYPic[ i ] and the chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are derived as follows for each value of i in the range of 0 to numInputPics − 1, inclusive:

–	If inputPicPresentFlag[ i ] is equal to 1 or nnpfc_absent_input_pic_zero_flag is equal to 0, the following applies:

–	…

–	Otherwise (inputPicPresentFlag[ i ] is equal to 0 and nnpfc_absent_input_pic_zero_flag is equal to 1), the following applies:

–	The luma sample array CroppedYPic[ i ], when present, is a 2-dimensional array of CroppedWidth × CroppedHeight samples equal to 0.

–	The chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are 2-dimensional arrays ( CroppedWidth / SubWidthC ) × ( CroppedHeight / SubHeightC ) samples equal to 0.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).