| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0055-v1 |
| --- | --- |

| Title: | AHG9: On buffering requirements for multi-picture NNPFC SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Francesco Cricri Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This input document asserts that a neural-network post-filter (NNPF) that takes multiple pictures as input may increase the number of buffered decoded pictures and displaying latency when compared to the buffering and latency required for the output order conformance. To indicate the buffering and latency needs, the following syntax elements are proposed for the neural-network post-filter characteristics (NNPFC) SEI message when the number of input pictures is greater than 1:

- nnpfc_latency_info_present_flag specifying the presence of nnpfc_max_num_reorder_pics_increase and nnpfc_max_latency_increase.

- nnpfc_max_num_reorder_pics_increase specifying the maximum additional number of picture storage buffers needed for storing input pictures for post-filtering on top of what would be needed for picture reordering from decoding order to output order.

- nnpfc_max_latency_increase specifying the maximum additional number of pictures that need to be decoded to form an input picture set for post-filtering on top of the number of input pictures.

# Introduction

## Example of the buffering and delay impact of NNPF alignment with the GOP structure

Figure 1 presents an example of an NNPF that takes 4 pictures as input and filters all of them. The alphabetical order indicates a relative output order of pictures.

| Input pictures | A |  | B |  | C |  | D |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ↓ |  |  |  |  |  |  |
|  | NNPF |  |  |  |  |  |  |
|  | ↓ |  |  |  |  |  |  |
| Output pictures | A |  | B |  | C |  | D |

Figure 1 – An example of an NNPF filtering multiple picture in one inference.

Figure 2 shows an example of a CLVS with GOP size of 4. The values indicate the decoding order, and the output order is from left to right. Figure 2 also presents a sequence of NNPF activations that are aligned with GOPs. An NNPFA SEI message for the example NNPF is present in picture unit 1 to cause NNPF activation with input pictures 3, 2, 4, and 1 and in picture unit 5 to cause NNPF activation with input pictures 7, 6, 8, and 5. Since all of the input pictures in NNPF activation 1 precede, in decoding order, all of the pictures of NNPF activation 2, it is asserted that the NNPF activation causes no additional buffering delay or picture buffer memory usage on top of those needed for picture reordering from decoding order to output order.

Figure 2 – An example of NNPF activations that are aligned with GOPs.

Figure 3 presents a sequence of NNPF activations that are not aligned with GOPs. An NNPFA SEI message for the example NNPF is present in picture unit 4 to cause NNPF activation with input pictures 0, 3, 2, and 4 and in picture unit 8 to cause NNPF activation with input pictures 1, 7, 6, and 8. Decoded picture 1 needs to be buffered until picture 8 is decoded, causing the buffer memory usage to increase by 1 compared to what would be needed for picture reordering from decoding order to output order. In addition, the maximum latency to produce filtered output pictures is 7 pictures in decoding order, since 7 additional pictures need to be decoded after picture 1 has been decoded to filter picture 1 with the example NNPF.

Figure 3 – An example of NNPF activations that are not aligned with GOPs.

# Proposal

## Overview

The following syntax elements are proposed for NNPFC SEI message where the number of input pictures is greater than 1:

- nnpfc_latency_info_present_flag specifying the presence of nnpfc_max_num_reorder_pics_increase and nnpfc_max_latency_increase.

- nnpfc_max_num_reorder_pics_increase specifying the maximum additional number of picture storage buffers needed for storing input pictures for post-filtering on top of what would be needed for picture reordering from decoding order to output order.

- nnpfc_max_latency_increase specifying the maximum additional number of pictures that need to be decoded to form an input picture set for post-filtering on top of the number of input pictures.

It is remarked that the semantics of nnpfc_max_num_reorder_pics_increase and nnpfc_max_latency_increase is phrased casually above, and the exact proposed specification text is provided below.

## Proposed specification text

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| if( nnpfc_num_input_pics_minus1 > 0 ) { |  |
| nnpfc_latency_info_present_flag | u(1) |
| if( nnpfc_latency_info_present_flag ) { |  |
| nnpfc_max_num_ reorder_pics_increase | ue(v) |
| nnpfc_max_latency_increase | ue(v) |
| } |  |
| } |  |
| nnpfc_complexity_info_present_flag | u(1) |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

nnpfc_latency_info_present_flag equal to 1 specifies that nnpfc_max_num_reorder_pics_increase and nnpfc_max_latency_increase are present. nnpfc_latency_info_present_flag equal to 0 specifies that nnpfc_max_num_reorder_pics and nnpfc_max_latency_increase are not present.

nnpfc_max_num_reorder_pics_increase indicates the maximum allowed number of pictures that can precede, in decoding order within the CLVS, any picture in any set of the input pictures, hereafter denoted inputPicSet, of the NNPF defined by this NNPFC SEI message and follow inputPicSet in output order within the CLVS and are input pictures of the NNPF but not among inputPicSet. The value of nnpfc_max_num_reorder_pics_increase, when present, shall be less than or equal to 16 + nnpfc_num_input_pics_minus1.

nnpfc_max_latency_increase is used to compute the value of nnpfcMaxLatencyPictures, which specifies, among all sets of input pictures of the NNPF defined by this NNPFC SEI message, the maximum number of pictures within the CLVS that can follow the earliest picture of an input picture set in decoding order up to and including the latest picture of the input picture set in decoding order. The value of nnpfcMaxLatencyPictures is derived as follows:

nnpfcMaxLatencyPictures = nnpfc_num_input_pics_minus1 + nnpfc_max_latency_increase	(xxx)

The value of nnpfc_max_latency_increase shall be in the range of 0 to 232 − 1 − nnpfc_num_input_pics_minus1, inclusive.

## Examples

In the example of Figure 2:

- nnpfc_max_num_reorder_pics_increase is equal to 0.

- nnpfc_max_latency_increase is equal to 0.

- nnpfcMaxLatencyPictures is equal to 3. (For example, considering NNPF activation 1, pictures 2, 3, and 4 follow the earliest picture (i.e., picture 1) of the input picture set in decoding order up to and including the latest picture (i.e., picture 4) of the input picture set in decoding order.

In the example of Figure 3:

- nnpf_max_num_reorder_pics_increase is equal to 1.

- nnpf_latency_increase is equal to 4.

- nnpfcMaxLatencyPictures is equal to 7. (For example, considering NNPF activation 2, pictures 2, 3, 4, 5, 6, 7, and 8 follow the earliest picture (i.e., picture 1) of the input picture set in decoding order up to and including the latest picture (i.e., picture 8) of the input picture set in decoding order.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).