| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0056-v1 |
| --- | --- |

| Title: | AHG9: Miscellaneous VSEI changes on neural-network post-filter SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Francesco Cricri, Maria Santamaria Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This input document proposes 14 items for the VSEI text on the neural-network post-filter characteristics (NNPFC) and activation (NNPFA) SEI messages.

# Summary of changes

This section summarizes the proposed changes, while a more detailed justification and description of changes are provided in subsequent sections. The following items are proposed:

- It is proposed to move the nnpfc_base_flag next to nnpfc_id in the NNPFC SEI message syntax. This change is asserted to simplify decoder operation to conclude if the NNPFC SEI message contains a base NNPF or an NNPF update.

- It is asserted that a repetition of NNPFC SEI message defining the base NNPF affects the persistence of an NNPF update, which is asserted not to be the design intent. A fix to avoid a repetition of an NNPFC SEI message to affect the persistence of an NNPF update is proposed.

- It is asserted that the specified inference of upsampled resolution, colour primaries, transfer characteristics, and matrix coefficients syntax elements is incorrect when nnpfc_property_present_flag is equal to 0.

If moving nnpfc_base_flag earlier in the syntax as proposed in item 1 is adopted, the following changes are proposed (where removal is indicated with italics and addition is indicated with underline):

When nnpfc_property_present_flag is equal to 0, the values of all syntax elements that may be present only when nnpfc_property_present_flag is equal to 1 and for which inference values for each of them is not specified are inferred to be equal to their corresponding syntax elements, respectively, in the NNPFC SEI message that contains the base NNPF for which this SEI message provides an update.

Otherwise, the following changes are proposed:

When nnpfc_property_present_flag is equal to 0, the values of all syntax elements that may be present only when nnpfc_property_present_flag is equal to 1 and follow nnpfc_base_flag for which inference values for each of them is not specified are inferred to be equal to their corresponding syntax elements, respectively, in the NNPFC SEI message that contains the base NNPF for which this SEI message provides an update.

- For the resolution update purpose, it is proposed that:

  - It is a requirement of bitstream conformance that the value of nnpfc_pic_width_in_luma_samples % outSubWidthC shall be equal to 0.

  - It is a requirement of bitstream conformance that the value of nnpfc_pic_height_in_luma_samples % outSubHeightC shall be equal to 0.

- It is proposed to include nnpfc_input_pic_output_flag[ i ] in the NNPFC SEI message when nnpfc_num_input_pics_minus1 is greater than 0 regardless of the nnpfc_purpose value.

- When the output bit depth(s) are present, the following is proposed:

  - Move nnpfc_inp_order_idc and nnpfc_out_order_idc before the bit depth syntax elements.

  - Gate the bit depth syntax elements so that the luma bit depth is present only when the input or output tensor contains luma and the chroma bit depth is present only when the input or output tensor contains chroma.

  - Condition the constraints related to bit depth upsampling on the presence of the respective luma or chroma output bit depth.

- The following constraints for chroma upsampling and colourization are proposed (where additions are indicated with underline):

  - When nnpfc_purpose & 0x02 is not equal to 0, nnpfc_inp_order_idc shall not be equal to 0.

  - When nnpfc_purpose & 0x20 is not equal to 0, nnpfc_inp_order_idc shall be equal to 0.

  - When nnpfc_purpose & 0x02 is not equal to 0, nnpfc_out_order_idc shall not be equal to 0 or 3.

  - When nnpfc_purpose & 0x20 is not equal to 0, nnpfc_out_order_idc shall not be equal to 0.

- When colour primaries, transfer characteristics, and matrix coefficients are indicated for the output tensor, the following changes are proposed:

  - Move nnpfc_separate_colour_description_present_flag, nnpfc_colour_primaries, nnpfc_transfer_characteristics, and nnpfc_matrix_coeffs below nnpfc_out_format_idc, nnpfc_out_tensor_luma_bitdepth_minus8, nnpfc_out_tensor_chroma_bitdepth_minus8, and nnpfc_out_order_idc.

  - Include the nnpfc_matrix_coeffs syntax element in the syntax under the constraint that nnpfc_out_format_idc is equal to 1 (indicating integer sample values in the output tensor).

  - Include constraints on nnpfc_matrix_coeffs that correspond to the constraints of vui_matrix_coeffs but depend on the chroma format, chroma bit depth, and luma bit depth of the output tensor.

- It is proposed that nnpfc_luma_padding_val is present only when the input tensor contains a luma sample array and nnpfc_cb_padding_val and nnpfc_cr_padding_val are present only when the input tensor contains chroma sample arrays.

- It is asserted that for fixed padding, the InpSampleVal function incorrectly returns an array whereas a scalar value is expected. Therefore, it is proposed to:

  - Change the definition of the function InpSampleVal to take the component index (0 for luma, 1 for Cb, 2 for Cr) as an additional input argument.

  - For nnpfc_padding_type equal to 4, change the function InpSampleVal to return the fixed luma, Cb, or Cr padding value corresponding to the component index given as input argument.

  - For each invocation of the function InpSampleVal, add the component index among the input arguments.

- A metadata extension mechanism is proposed in the NNPFC SEI message, which is asserted to:

- allow new metadata syntax elements to be added in the NNPFC SEI message in future VSEI amendments or editions, which could for example be additional means to describe the neural network complexity, and

- allow legacy decoders (with the support of the NNPFC SEI message as defined in this VSEI amendment) to skip and ignore such additional metadata syntax elements.

- It is proposed to remove the following constraints on nnpfa_target_id values, which are asserted to be enforced by other constraints and are therefore asserted to be redundant:

Values of nnpfa_target_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering an NNPFA SEI message with nnpfa_target_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

- It is proposed to enable the activation of the base NNPF with a particular nnpfc_id value after an NNPF update for the same nnpfc_id is available. The activation of the base NNPF could be advantageous, when an update is derived from the first GOP of a segment but the picture content of the segment changes later considerably. The following changes are proposed:

  - Add nnpfa_base_flag syntax element in the NNPFA SEI message under the condition that nnpfa_cancel_flag is equal to 0.

  - nnpfa_base_flag equal to 1 specifies that the target NNPF is the base NNPF with nnpfc_id equal to nnpfa_target_id. nnpfa_base_flag equal to 0 specifies the target NNPF as currently specified in JVET-AC2032.

- It is proposed that any NNPFA SEI message with a particular nnpfa_target_id value ends the persistence of the previous NNPFA SEI message with the same nnpfa_target_id value. The following changes are proposed (where removal is indicated with italics):

nnpfa_persistence_flag equal to 1 specifies that the target NNPF may be used for post-processing filtering for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with a NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 1 is output that follows the current picture in output order.

NOTE 2 – The target NNPF is not applied for this subsequent picture in the current layer associated with a NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 1.

# Moving nnpfc_base_flag earlier in the syntax

## Problem statement

The semantics of nnpfc_mode_idc refer to a base NNPF or an update relative to the base NNPF, and also specifically refer to a repetition of the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS.

To determine whether an NNPFC SEI message is a repetition of the NNPFC SEI message defining the base NNPF or defines an NNPF update, a decoder needs to parse until nnpfc_property_present_flag (which, when equal to 0, indicates an NNPF update) or until nnpfc_base_flag.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| nnpfc_base_flag | u(1) |
| … |  |

It is asserted that the order of syntax elements would be more logical if nnpfc_base_flag preceded nnpfc_mode_idc, since then the nnpfc_mode_idc semantics can use nnpfc_base_flag to determine whether the SEI message defines a base NNPF or an update relative to the base NNPF.

## Proposed changes

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_base_flag | u(1) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| nnpfc_base_flag | u(1) |
| … |  |

nnpfc_base_flag equal to 1 specifies that the SEI message specifies the base NNPF. nnpf_base_flag equal to 0 specifies that the SEI message specifies an update relative to the base NNPF. When not present, the value of nnpfc_base_flag is inferred to be equal to 0.

The following constraints apply to the value of nnpfc_base_flag:

–	When an NNPFC SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, the value of nnpfc_base_flag shall be equal to 1.

–	When an NNPFC SEI message nnpfcB is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS and the value nnpfc_base_flag is equal to 1, the NNPFC SEI message shall be a repetition of the first NNPFC SEI message nnpfcA with the same nnpfc_id, in decoding order, i.e., the payload paylaod content of nnpfcB shall be the same as that of nnpfcA.

When nnpfc_base_flag is equal to 0an NNPFC SEI message is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS and not a repetition of the first NNPFC SEI message with that particular nnpfc_id, the following applies:

–	This SEI message defines an update relative to the preceding base NNPF in decoding order with the same nnpfc_id value.

–	This SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or up to but excluding the decoded picture that follows the current decoded picture in output order within the current CLVS and is associated with a subsequent NNPFC SEI message, in decoding order, having that particular nnpfc_id value within the current CLVS, whichever is earlier.

nnpfc_mode_idc equal to 0 indicates that this SEI message contains an ISO/IEC 15938-17 bitstream that specifies a base NNPF (when nnpfc_base_flag is equal to 1) or is an update relative to the base NNPF with the same nnpfc_id value (when nnpfc_base_flag is equal to 0).

When nnpfc_base_flag is equal to 1an NNPFC SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, nnpfc_mode_idc equal to 1 specifies that the base NNPF associated with the nnpfc_id value is a neural network identified by the URI indicated by nnpfc_uri with the format identified by the tag URI nnpfc_tag_uri.

When nnpfc_base_flag is equal to 0an NNPFC SEI message is neither the first NNPFC SEI message, in decoding order, nor a repetition of the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, nnpfc_mode_idc equal to 1 specifies that an update relative to the base NNPF with the same nnpfc_id value is defined by the URI indicated by nnpfc_uri with the format identified by the tag URI nnpfc_tag_uri.

The value of nnpfc_mode_idc shall be in the range of 0 to 1, inclusive, in bitstreams conforming to this edition of this document. Values of 2 to 255, inclusive, for nnpfc_mode_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_mode_idc in the range of 2 to 255, inclusive. Values of nnpfc_mode_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When this SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, the NNPF PostProcessingFilter( ) is assigned to be the same as the base NNPF.

When nnpfc_base_flag is equal to 0this SEI message is neither the first NNPFC SEI message, in decoding order, nor a repetition of the irst NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, an NNPF PostProcessingFilter( ) is obtained by applying the update defined by this SEI message to the base NNPF.

Updates are not cumulative but rather each update is applied on the base NNPF, which is the NNPF specified by the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS.

…

nnpfc_property_present_flag equal to 1 specifies that syntax elements related to the filter purpose, input formatting, output formatting, and complexity are present. nnpfc_property_present_flag equal to 0 specifies that no syntax elements related to the filter purpose, input formatting, output formatting, and complexity are present.

When nnpfc_base_flag is equal to 1this SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, nnpfc_property_present_flag shall be equal to 1.

# Avoiding repetitions of NNPFC SEI messages defining the base NNPF to affect the persistence of an NNPF update

## Problem statement

If an NNPFC SEI message with nnpfc_base_flag equal to 1 follows, in decoding order, an NNPFC SEI message with nnpfc_base_flag equal to 0 and the same nnpfc_id value, the persistence of the NNPFC SEI message with nnpfc_base_flag equal to 0 ends, as specified in the second bullet in the quoted text below. However, since the NNPFC SEI message with nnpfc_base_flag equal to 1 is just a repetition of the first NNPFC SEI message with that nnpfc_id value in decoding order in the CLVS, it should not affect the persistence.

When an NNPFC SEI message is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS and not a repetition of the first NNPFC SEI message with that particular nnpfc_id, the following applies:

–	This SEI message defines an update relative to the preceding base NNPF in decoding order with the same nnpfc_id value.

–	This SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or up to but excluding the decoded picture that follows the current decoded picture in output order within the current CLVS and is associated with a subsequent NNPFC SEI message, in decoding order, having that particular nnpfc_id value within the current CLVS, whichever is earlier.

## Proposed changes

When an NNPFC SEI message is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS and not a repetition of the first NNPFC SEI message with that particular nnpfc_id, the following applies:

–	This SEI message defines an update relative to the preceding base NNPF in decoding order with the same nnpfc_id value.

–	This SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or up to but excluding the decoded picture that follows the current decoded picture in output order within the current CLVS and is associated with a subsequent NNPFC SEI message, in decoding order, having nnpfc_base_flag equal to 0 and that particular nnpfc_id value within the current CLVS, whichever is earlier.

# Inferring syntax element values correctly when nnpf_property_present_flag is equal to 0

## Problem statement

The following paragraph specifies the inference of syntax element values when nnpf_property_present_flag is equal to 0:

When nnpfc_property_present_flag is equal to 0, the values of all syntax elements that may be present only when nnpfc_property_present_flag is equal to 1 and for which inference values for each of them is not specified are inferred to be equal to their corresponding syntax elements, respectively, in the NNPFC SEI message that contains the base NNPF for which this SEI provides an update.

The condition "and for which inference values for each of them is not specified" causes the exclusion of the following syntax elements from the inference rule:

- When not present, the value of nnpfc_base_flag is inferred to be equal to 0.

- When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively.

- When nnpfc_colour_primaries is not present in the NNPFC SEI message, the value of nnpfc_colour_primaries is inferred to be equal to vui_colour_primaries.

- When nnpfc_transfer_characteristics is not present in the NNPFC SEI message, the value of nnpfc_transfer_characteristics is inferred to be equal to vui_transfer_characteristics.

- When nnpfc_matrix_coeffs is not present in the NNPFC SEI message, the value of nnpfc_matrix_coeffs is inferred to be equal to vui_matrix_coeffs.

Case 2 above is problematic, since an NNPFC SEI message defining an update to an upsampling NNPF would be inferred to have the width and height of the input picture rather than the width and height of the output picture defined in the NNPFC SEI message for the base NNPF.

Cases 3 to 5 above are problematic, since an NNPFC SEI message defining an update would be inferred to have colour primaries, transfer characteristics, and matrix coefficients of the input picture, but the respective base NNPF could have different colour primaries, transfer characteristics or matrix coefficients compared to those of the input picture.

## Proposed changes

If moving nnpfc_base_flag earlier in the syntax as proposed in Section 1 is adopted, the following changes are proposed:

When nnpfc_property_present_flag is equal to 0, the values of all syntax elements that may be present only when nnpfc_property_present_flag is equal to 1 and for which inference values for each of them is not specified are inferred to be equal to their corresponding syntax elements, respectively, in the NNPFC SEI message that contains the base NNPF for which this SEI message provides an update.

Otherwise, the following changes are proposed:

When nnpfc_property_present_flag is equal to 0, the values of all syntax elements that may be present only when nnpfc_property_present_flag is equal to 1 and follow nnpfc_base_flag for which inference values for each of them is not specified are inferred to be equal to their corresponding syntax elements, respectively, in the NNPFC SEI message that contains the base NNPF for which this SEI message provides an update.

# Additional nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples constraints

## Problem statement

It is asserted that depending on the output chroma format, nnpfc_pic_width_in_luma_samples and/or nnpfc_pic_height_in_luma_samples should be required to be even values.

## Proposed changes

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth to CroppedWidth * 16 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight to CroppedHeight * 16 − 1, inclusive. It is a requirement of bitstream conformance that the value of nnpfc_pic_width_in_luma_samples % outSubWidthC shall be equal to 0. It is a requirement of bitstream conformance that the value of nnpfc_pic_height_in_luma_samples % outSubHeightC shall be equal to 0.

# Indicating which input pictures are filtered regardless of the filtering purpose

## Problem statement

In the JVET-AC meeting, the support of several input pictures for an NNPF was added to any filtering purpose. If the filtering purpose includes picture rate upsampling (i.e., if nnpfc_purpose & 0x08 is not equal to 0), nnpfc_input_pic_output_flag[ i ] indicates which input pictures are filtered for visual quality enhancement or other purpose.

It is asserted that indicating which pictures are filtered would be useful also when the filtering purpose does not include picture rate upsampling. This would enable, for example, a post-filter with three input pictures that provides visual quality improvement of the midmost input picture, where the two other input pictures have higher quality and can be used as implicit source for the quality improvement of the midmost input picture.

## Proposed changes (VSEI text)

It is proposed to indicate the loop of nnpfc_input_pic_output_flag[ i ] whenever the number of input pictures is greater than 1 regardless of the filtering purpose.

The following detailed specification changes are proposed:

| if( ( nnpfc_purpose & 0x08 ) != 0 ) { |  |
| --- | --- |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| if( nnpfc_num_input_pics_minus1 > 0 ) |  |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_output_flag[ i ] | u(1) |
| } |  |

nnpfc_input_pic_output_flag[ i ] equal to 1 indicates that for the i-th input picture the NNPF generates a corresponding output picture. nnpfc_input_pic_output_flag[ i ] equal to 0 indicates that for the i-th input picture the NNPF does not generate a corresponding output picture. When nnpfc_num_input_pics_minus1 is equal to 0, nnpfc_input_pic_output_flag[ 0 ] is inferred to be equal to 1.

The variables numInputPics, specifying the number of pictures used as input for the NNPF, and numOutputPics, specifying the total number of pictures resulting from the NNPF, are derived as follows:

numInputPics = nnpfc_num_input_pics_minus1 + 1
if( ( nnpfc_purpose & 0x08 )  !=  0 ) {
	for( i = 0, numOutputPics = 0; i < numInputPics; i++ )
		if( nnpfc_input_pic_output_flag[ i ] )
			numOutputPics++
if( ( nnpfc_purpose & 0x08 )  !=  0 )
	for( i = 0; i  <=  numInputPics − 2; i++ )		(76)
		numOutputPics  +=  nnpfc_interpolated_pics[ i ]
} else
	numOutputPics = 1

# Avoiding redundant bit depth syntax elements

## Problem statement

When no luma sample array is present in the input, nnpfc_inp_tensor_luma_bitdepth_minus8 is still redundantly present.

When no luma sample array is present in the output, nnpfc_out_tensor_luma_bitdepth_minus8 is still redundantly present.

When no chroma sample arrays are present in the input, nnpfc_inp_tensor_chroma_bitdepth_minus8 is still redundantly present.

When no chroma sample arrays are present in the output, nnpfc_out_tensor_chroma_bitdepth_minus8 is still redundantly present.

## Proposed changes (VSEI text)

It is proposed to

- Move nnpfc_inp_order_idc and nnpfc_out_order_idc before the bit depth syntax elements.

- Gate the bit depth syntax elements so that the luma bit depth is present only when the input or output tensor contains luma and the chroma bit depth is present only when the input or output tensor contains chroma.

- Condition the constraints related to bit depth upsampling on the presence of the respective luma or chroma output bit depth.

The following detailed changes are proposed:

| … |  |
| --- | --- |
| nnpfc_inp_order_idc | ue(v) |
| nnpfc_inp_format_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| if( nnpfc_inp_order_idc != 1 ) |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| if( nnpfc_inp_order_idc > 0 ) |  |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_inp_order_idc | ue(v) |
| … |  |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_out_format_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| if( nnpfc_out_order_idc != 1 ) |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| if( nnpfc_out_order_idc > 0 ) |  |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_order_idc | ue(v) |
| … |  |

When nnpfc_purpose & 0x10 is not equal to 0, the value of nnpfc_out_format_idc shall be equal to 1 and at least one of the following conditions shall be true:

–	nnpfc_out_tensor_luma_bitdepth_minus8 is present and nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is greater than BitDepthY.

–	nnpfc_out_tensor_chroma_bitdepth_minus8 is present and nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is greater than BitDepthC.

# Additional nnpfc_inp_order_idc and nnpfc_out_order_idc constraints for chroma upsampling and colourization

## Problem statement

The colourization and chroma upsampling purposes do not impose constraints on nnpfc_inp_order_idc values. However, for colourization, it only makes sense that the input tensor contains only the luma sample array (i.e., nnpfc_inp_order_idc is equal to 0). Similarly, for chroma upsampling, it is not meaningful that the input tensor contains only the luma sample array.

For nnpfc_out_order_idc, there is a constraint that disallows 4:2:0 chroma format output resulting from a NNPF for chroma upsampling. Additionally, outputting only the luma sample array should likewise be disallowed for the chroma upsampling and colourization purposes.

## Proposed changes

Add the following to the semantics of nnpfc_inp_order_idc:

When nnpfc_purpose & 0x02 is not equal to 0, nnpfc_inp_order_idc shall not be equal to 0.

When nnpfc_purpose & 0x20 is not equal to 0, nnpfc_inp_order_idc shall be equal to 0.

Add the following highlighted text to the semantics of nnpfc_out_order_idc:

When nnpfc_purpose & 0x02 is not equal to 0, nnpfc_out_order_idc shall not be equal to 0 or 3.

When nnpfc_purpose & 0x20 is not equal to 0, nnpfc_out_order_idc shall not be equal to 0.

# Additional nnpfc_matrix_coeffs constraints

## Problem statement

The equations for vui_matrix_coeffs are specified for to output integer values of a certain bit depth. Thus, nnpfc_matrix_coeffs should only be present when nnpfc_out_format_idc indicates that sample values of the output tensors are integer, i.e., when nnpfc_out_format_idc is equal to 1.

vui_matrix_coeffs is constrained as follows:

vui_matrix_coeffs shall not be equal to 0 unless both of the following conditions are true:

–	BitDepthC is equal to BitDepthY.

–	ChromaFormatIdc is equal to 3 (the 4:4:4 chroma format).

vui_matrix_coeffs shall not be equal to 8 unless one of the following conditions is true:

–	BitDepthC is equal to BitDepthY,

–	BitDepthC is equal to BitDepthY + 1 and ChromaFormatIdc is equal to 3 (the 4:4:4 chroma format).

These constraints are explicitly not inherited to nnpfc_matrix_coeffs, as specified by the following:

–	The values allowed for nnpfc_matrix_coeffs are not constrained by the chroma format of the decoded video pictures that is indicated by the value of ChromaFormatIdc for the semantics of the VUI parameters.

However, in our opinion, the constraints that correspond to those of vui_matrix_coeffs should be specified for nnpfc_matrix_coeffs depending on the bit depth and chroma format of the output pictures resulting from the NNPF.

The following constraint appears under the semantics of nnpfc_matrix_coeffs, which might have been intended for this purpose but is asserted to be wrongly phrased as a constraint to nnpfc_out_order_idc.

–	When nnpfc_matrix_coeffs is equal to 0, nnpfc_out_order_idc shall not be equal to 1 or 3.

## Proposed changes

It is proposed to

- Move nnpfc_separate_colour_description_present_flag, nnpfc_colour_primaries, nnpfc_transfer_characteristics, and nnpfc_matrix_coeffs below nnpfc_out_format_idc, nnpfc_out_tensor_luma_bitdepth_minus8, nnpfc_out_tensor_chroma_bitdepth_minus8, and nnpfc_out_order_idc.

- Include the nnpfc_matrix_coeffs syntax element in the syntax under the constraint that nnpfc_out_format_idc is equal to 1 (indicating integer sample values in the output tensor).

- Include constraints on nnpfc_matrix_coeffs that correspond to the constraints of vui_matrix_coeffs but depend on the chroma format, chroma bit depth, and luma bit depth of the output tensor.

The following detailed changes are proposed:

| … |  |
| --- | --- |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| nnpfc_matrix_coeffs | u(8) |
| } |  |
| nnpfc_out_format_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| if( nnpfc_out_format_idc = = 1 ) |  |
| nnpfc_matrix_coeffs | u(8) |
| } |  |
| … |  |

nnpfc_matrix_coeffs has the same semantics as specified in subclause 7.3 for the vui_matrix_coeffs syntax element, except as follows:

–	nnpfc_matrix_coeffs specifies the matrix coefficients of the picture resulting from applying the NNPF specified in the SEI message, rather than the matrix coefficients used for the CLVS.

–	When nnpfc_matrix_coeffs is not present in the NNPFC SEI message, the value of nnpfc_matrix_coeffs is inferred to be equal to vui_matrix_coeffs.

–	The values allowed for nnpfc_matrix_coeffs are not constrained by the chroma format of the decoded video input pictures, chroma bit depth and luma bit depth that is are indicated by the values of ChromaFormatIdc, BitDepthC, and BitDepthY, respectively, for the semantics of the VUI parameters.

–	When nnpfc_matrix_coeffs is equal to 0, nnpfc_out_order_idc shall not be equal to 1 or 3.

–	nnpfc_matrix_coeffs shall not be equal to 0 unless both of the following conditions are true:

–	nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8.

–	nnpfc_out_order_idc is equal to 2, outSubHeightC is equal to 1, and outSubWidthC is equal to 1.

–	nnpfc_matrix_coeffs shall not be equal to 8 unless one of the following conditions is true:

–	nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8.

–	nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8 + 1, nnpfc_out_order_idc is equal to 2, outSubHeightC is equal to 1, and outSubWidthC is equal to 1.

# Avoiding redundant padding value syntax elements

## Problem statement

When no luma sample array is present in the input and nnpfc_padding_type is equal to 4, nnpfc_luma_padding_val is redundantly present.

When no chroma sample arrays are present in the input and nnpfc_padding_type is equal to 4, nnpfc_cb_padding_val and nnpfc_cr_padding_val are redundantly present.

## Proposed changes (VSEI text)

It is proposed that nnpfc_luma_padding_val is present only when the input tensor contains a luma sample array and nnpfc_cb_padding_val and nnpfc_cr_padding_val are present only when the input tensor contains chroma sample arrays.

The following detailed syntax changes are proposed:

| … |  |
| --- | --- |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| if( nnpfc_inp_order_idc != 1 ) |  |
| nnpfc_luma_padding_val | ue(v) |
| if( nnpfc_inp_order_idc > 0 ) { |  |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| } |  |
| … |  |

# Correcting the InpSampleVal function for fixed padding

## Problem statement

As shown in the specification text excerpt below, when nnpfc_padding_type is equal to 4 (fixed padding), the function InpSampleVal returns an array, which is incorrect, since InpSampleVal is expected to return a scalar value. The syntax is also lacking curly brackets for the case of nnpfc_padding_type equal to 4.

The function InpSampleVal( y, x, picHeight, picWidth, croppedPic ) with inputs being a vertical sample location y, a horizontal sample location x, a picture height picHeight, a picture width picWidth, and sample array croppedPic returns the value of sampleVal derived as follows:

NOTE 6 – For the inputs to the function InpSampleVal( ), the vertical location is listed before the horizontal location for compatibility with input tensor conventions of some inference engines.

if( nnpfc_padding_type  = =  0 )
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth )
		sampleVal = 0
	else
		sampleVal = croppedPic[ x ][ y ]		(92)
else if( nnpfc_padding_type  = =  1 )
	sampleVal = croppedPic[ Clip3( 0, picWidth − 1, x ) ][ Clip3( 0, picHeight − 1, y ) ]
else if( nnpfc_padding_type  = =  2 ) 
	sampleVal = croppedPic[ Reflect( picWidth − 1, x ) ][ Reflect( picHeight − 1, y ) ]
else if( nnpfc_padding_type  = =  3 ) 
	if( y >= 0  &&  y < picHeight )
		sampleVal = croppedPic[ Wrap( picWidth − 1, x ) ][ y ] 
else if( nnpfc_padding_type  = =  4 ) 
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth )
		sampleVal[ 0 ]  = nnpfc_luma_padding_val			sampleVal[ 1 ]  = nnpfc_cb_padding_val
		sampleVal[ 2 ] = nnpfc_cr_padding_val
	else
		sampleVal = croppedPic[ x ][ y ]

## Proposed changes

It is proposed to

- Change the definition of the function InpSampleVal to take the component index (0 for luma, 1 for Cb, 2 for Cr) as an additional input argument.

- For nnpfc_padding_type equal to 4, change the function InpSampleVal to return the fixed luma, Cb, or Cr padding value corresponding to the component index given as input argument.

- For each invocation of the function InpSampleVal, add the component index among the input arguments.

The detailed changes for item 1 and 2 are presented below.

The function InpSampleVal( y, x, picHeight, picWidth, croppedPic, cIdx ) with inputs being a vertical sample location y, a horizontal sample location x, a picture height picHeight, a picture width picWidth, and sample array croppedPic, and component index cIdx (equal to 0 for luma, 1 for Cb, and 2 for Cr) returns the value of sampleVal derived as follows:

NOTE 6 – For the inputs to the function InpSampleVal( ), the vertical location is listed before the horizontal location for compatibility with input tensor conventions of some inference engines.

if( nnpfc_padding_type  = =  0 )
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth )
		sampleVal = 0
	else
		sampleVal = croppedPic[ x ][ y ]		(92)
else if( nnpfc_padding_type  = =  1 )
	sampleVal = croppedPic[ Clip3( 0, picWidth − 1, x ) ][ Clip3( 0, picHeight − 1, y ) ]
else if( nnpfc_padding_type  = =  2 ) 
	sampleVal = croppedPic[ Reflect( picWidth − 1, x ) ][ Reflect( picHeight − 1, y ) ]
else if( nnpfc_padding_type  = =  3 ) 
	if( y >= 0  &&  y < picHeight )
		sampleVal = croppedPic[ Wrap( picWidth − 1, x ) ][ y ] 
else if( nnpfc_padding_type  = =  4 ) 
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth )
		sampleVal = ( cIdx  = =  0 ? nnpfc_luma_padding_val : 
			( cIdx  = = 1 ? nnpfc_cb_padding_val : nnpfc_cr_padding_val ) )
		sampleVal[ 0 ]  = nnpfc_luma_padding_val			sampleVal[ 1 ]  = nnpfc_cb_padding_val
		sampleVal[ 2 ] = nnpfc_cr_padding_val
	else
		sampleVal = croppedPic[ x ][ y ]

# Enabling additional future metadata in the NNPFC SEI message without breaking legacy decoders

## Problem statement

It is asserted that it would be desirable to allow new metadata syntax elements to be added in the NNPFC SEI message in future VSEI amendments or editions, which could for example be additional means to describe the neural network complexity. Furthermore, the extension mechanism should allow the legacy decoders (with the support of the NNPFC SEI message as defined in this VSEI amendment) to skip and ignore such additional syntax elements that are solely metadata (i.e., do not affect the input tensor format or output tensor interpretation).

## Proposed changes

| … |  |
| --- | --- |
| nnpfc_complexity_info_present_flag | u(1) |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| nnpfc_metadata_extension_num_bits | ue(v) |
| if( nnpfc_metadata_extension_num_bits > 0 ) |  |
| nnpfc_reserved_metadata_extension | u(v) |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

nnpfc_metadata_extension_num_bits equal to 0 specifies that nnpfc_reserved_metadata_extension is not present. nnpfc_metadata_extension_num_bits greater than 0 specifies the length, in bits, of nnpfc_reserved_metadata_extension. nnpfc_metadata_extension_num_bits shall be equal to 0 in this edition of this document. Values in the range of 1 to 2048, inclusive, for nnpfc_metadata_extension_num_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall allow any value of nnpfc_metadata_extension_num_bits in the range of 0 to 2048, inclusive. Values of nnpfc_metadata_extension_num_bits greater than 2048 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

nnpfc_reserved_metadata_extension shall not be present in bitstreams conforming to this edition of this document. However, decoders conforming to this edition of this document shall ignore the presence and value of nnpfc_reserved_metadata_extension. When present, the length, in bits, of nnpfc_reserved_metadata_extension is equal to nnpfc_metadata_extension_num_bits.

NOTE X – Syntax elements that do not affect the input tensor generation or output tensor interpretation can take bits of nnpfc_reserved_metadata_extension into use in the future.

# Removing redundant constraints on nnpfa_target_id

## Problem statement

It is asserted that the constraints in the following paragraph are implied by other constraints and are hence unnecessarily expressed twice. Moreover, it is asserted that the following paragraph might give an incorrect impression that the syntax of NNPFA SEI message would change, if the reserved nnpfc_id values would be taken into use in a future VSEI amendment or edition.

Excerpt 1:

The value of nnpfa_target_id shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfa_target_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering an NNPFA SEI message with nnpfa_target_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

The following constraint implies that nnpfa_target_id shall have a value that has been used for nnpfc_id in the same CLVS:

Excerpt 2:

An NNPFA SEI message with a particular value of nnpfa_target_id shall not be present in a current PU unless one or both of the following conditions are true:

–	Within the current CLVS there is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_target_id present in a PU preceding the current PU in decoding order.

–	There is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_target_id in the current PU.

Certain value ranges of nnpfc_id have been reserved for future use.

Excerpt 3:

nnpfc_id contains an identifying number that may be used to identify an NNPF. The value of nnpfc_id shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfc_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering an NNPFC SEI message with nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

It is asserted that the constraints in Excerpts 1 and 2 imply all the constraints in Excerpt 3. The complete value range for nnpfa_target_id, i.e., of 0 to 232 − 2, inclusive, may still be good to express explicitly for readability.

## Proposed changes

nnpfa_target_id indicates the target NNPF, which is specified by one or more NNPFC SEI messages that pertain to the current picture and have nnpfc_id equal to nnpfa_target_id.

The value of nnpfa_target_id shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfa_target_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering an NNPFA SEI message with nnpfa_target_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

An NNPFA SEI message with a particular value of nnpfa_target_id shall not be present in a current PU unless one or both of the following conditions are true:

–	Within the current CLVS there is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_target_id present in a PU preceding the current PU in decoding order.

–	There is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_target_id in the current PU.

# Enabling activation of the base filter

## Problem statement

The NNPFA SEI message does not provide the indication whether it activates the base filter or the latest updated filter having nnpfc_id equal to nnpfa_target_id. Yet, the base filter is available for the decoder, since it is kept to serve as the basis for potential filter updates. The activation of the base filter could be advantageous, when an update is derived from the first GOP of a segment but the picture content of the segment changes later considerably. It is therefore proposed to enable activation of the base filter in the NNPFA SEI message.

## Proposed changes (VSEI text)

In 8.29.1:

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_base_flag | u(1) |
| nnpfa_persistence_flag | u(1) |
| } |  |
| } |  |

In 8.28.2:

When this SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS or when an NNPFA message activates the base NNPF, the NNPF PostProcessingFilter( ) is assigned to be the same as the base NNPF.

When this SEI message is neither the first NNPFC SEI message, in decoding order, nor a repetition of the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, an NNPF PostProcessingFilter( ) is obtained by applying the update defined by this SEI message to the base NNPF.

In 8.29.2:

The neural-network post-filter activation (NNPFA) SEI message activates or de-activates the possible use of the target neural-network post-processing filter (NNPF), identified by nnpfa_target_id, for post-processing filtering of a set of pictures. For a particular picture for which the NNPF is activated, the target NNPF is derived as follows:

–	If nnpfa_base_flag is equal to 1, the target NNPF is the base NNPF with nnpfc_id equal to nnpfa_target_id.

–	Otherwise (nnpfa_base_flag is equal to 0), the target NNPF is the NNPF specified by the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id, that precedes the first VCL NAL unit of the current picture in decoding order that is not a repetition of the NNPFC SEI message that contains the base NNPF.

…

nnpfa_base_flag equal to 1 specifies that the target NNPF is the base NNPF with nnpfc_id equal to nnpfa_target_id. nnpfa_base_flag equal to 0 specifies that the target NNPF is the NNPF specified by the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id, that precedes the first VCL NAL unit of the current picture in decoding order that is not a repetition of the NNPFC SEI message that contains the base NNPF.

# Cancelling the persistence of an NNPFA SEI message with any subsequent NNPFA SEI message with the same nnpfa_target_id

## Problem statement

The persistence of an NNPFA SEI message with nnpfa_persistence_flag equal to 1 requires a NNPFA SEI message with nnpfc_cancel_flag equal to 1 and the same nnpfa_target_id value, which is asserted to be redundant for example in the case where the next NNPFA SEI message has nnpfa_persistence_flag equal to 0 and the same nnpfa_target_id value and is contained in a CRA or GDR picture unit. The example is illustrated in the following bitstream structure with decoding order from the top towards the bottom:

- IDR PU: NNPFA SEI message with nnpfa_persistence_flag equal to 1

- Other PUs in the same CLVS

- CRA or GDR PU (in the same CLVS):

  - NNPFA SEI message with nnpfc_cancel_flag equal to 1, and

  - NNPFA SEI message with nnpfa_persistence_flag equal to 0

- Other PUs in the same CLVS

If the proposal in Section 12 (enabling activation of the base filter) is adopted, the activation of the base filter after  a persistent activation of an updated filter of the same nnpfc_id value and the activation of an updated filter after a persistent activation of the base filter of the same nnpfc_id value are other examples where the current text would require a specific NNPFA SEI message to cancel the persistence.

## Proposed changes

nnpfa_persistence_flag equal to 1 specifies that the target NNPF may be used for post-processing filtering for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with a NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 1 is output that follows the current picture in output order.

NOTE 2 – The target NNPF is not applied for this subsequent picture in the current layer associated with a NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 1.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).