| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0058-v1 |
| --- | --- |

| Title: | AHG9: Approaches for region-based neural-network post-filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Discussion |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Francesco Cricri Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Further study on region-based filtering toward a potential use beyond the timeline of VSEI version 3 (i.e., the amendment or the integrated edition to be technically finalized in April 2023) was encouraged in response to JVET-AC0132. This input document outlines and analyzes the following potential technical approaches for region-based neural-network post-filtering:

- Regional nesting SEI message that contains a region-nested neural-network post-filter activation (NNPFA) SEI message:

- The regional nesting SEI message is copied from HEVC to VSEI.

- Each rectangular region defined in the regional nesting SEI message is treated as an input picture for neural-network post-filter (NNPF) inference.

- In VSEI v4, an extension of the NNPFA SEI message with a region-based filtering gating flag and regions defining the input to the NNPF

- The syntax in VSEI v3 would need to allow such extension and decoders conforming to VSEI v3 shall ignore an NNPFA SEI message when an extension is present.

- NNPF regional activation SEI message

- The NNPF regional activation SEI message is similar to the NNPFA SEI message but also indicates the region for which the NNPF is activated.

This input document presents use cases for region-based filtering and concludes that the use cases can be handled when both of the following are included in the design:

- Either the NNPFA SEI message extension for indicating regions (approach 2 above) or the NNPF regional activation SEI message (approach 3 above)

- In the neural-network post-filter characteristics (NNPFC) SEI message, a new nnpfc_auxiliary_inp_idc value for providing indications about regions as input to the NNPF

It is proposed to adopt both A and B as the high-level design decisions based on which detailed syntax and semantics can be presented in the upcoming JVET meetings. The proponents do not have a preference whether the NNPFA SEI message extension for indicating regions or the NNPF regional activation SEI message is adopted as a design decision.

If approach 2 above is selected as a high-level design decision, it is proposed to add an NNPFA SEI message extension mechanism in VSEI v3, where the NNPFA SEI message syntax contains any number of nnpfa_extension_bit[ i ] until the end of the payload. nnpfa_extension_bit[ i ] is reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this VSEI version. Decoders conforming to this VSEI version encountering an NNPFA SEI message with any nnpfa_extension_bit[ i ] present shall ignore the SEI message.

# Introduction

JVET-AC0132 [1] proposed a design for region-based neural-network post-filter SEI message. Further study on region-based filtering toward a potential use beyond the timeline of VSEI v3 was encouraged. Questions to be studied include the boundary between filtered and non-filtered regions, the interaction and alignment between patches and regions, potential use of multiple regions with different filtering, the signalling of the granularity of the regions, and regional nesting.

A regional nesting SEI message is specified in HEVC. It associates the nested SEI messages with regions of the picture. For each region-nested SEI message in a regional nesting SEI message, one or more regions are specified in the regional nesting SEI message, and the semantics of the region-nested SEI message are to be interpreted as applying to each of these regions. HEVC lists SEI message types that may be region-nested, including the post filter hint SEI message.

This contribution requests feedback on the presented potential technical approaches and use cases. It is proposed that JVET would attempt to reach high-level design decisions so that more detailed technical contributions can be presented in the upcoming JVET meetings.

# Potential technical approaches for region-based filtering

The following potential technical approaches are outlined in this input document:

- Regional nesting SEI message that contains a region-nested NNPFA SEI message:

- The regional nesting SEI message is copied from HEVC to VSEI.

- Each rectangular region defined in the regional nesting SEI message is treated as an input picture for NNPF inference.

- In VSEI v4, an extension of the NNPFA SEI message with a region-based filtering gating flag and regions defining the input to the NNPF

- The syntax in VSEI v3 would need to allow such extension and decoders conforming to VSEI v3 shall ignore an NNPFA SEI message when an extension is present.

- Section 4 provides the proposed specification text changes for VSEI v3 and a skeleton how the NNPFA SEI message could be extended in VSEI v4.

- NNPF regional activation SEI message

- The NNPF regional activation SEI message is similar to the NNPFA SEI message but also indicates the region for which the NNPF is activated.

The NNPF inference for options 2 and 3 can be realized with either of the following:

- The rectangular area of samples corresponding to the region defined in the NNPFA SEI extension or the NNPF region activation SEI message is treated as an input picture for NNPF inference.

- In the NNPFC SEI message, a new nnpfc_auxiliary_inp_idc value is specified for providing indications about regions as input to the NNPF.

# Discussion

The following potential use cases were identified for region-based filtering:

- Rate-distortion-optimized region-based filtering: Several NNPFs for the same filtering purpose, such as visual quality enhancement, are available for an encoder. The encoder selects regions and region-based NNPFs in a rate-distortion-optimized manner.

- Constituent-frame-based filtering:

  - Stereoscopic frame packing where constituent frames of different views are packed into the same frame. The same NNPF is activated separately for each constituent frame to avoid filtering across the boundaries of the constituent frames.

  - Different component types, such as texture and depth, are packed into the same frame. Different NNPFs are activated for different constituent frames.

- Region-of-interest-based filtering: Only ROIs are post-filtered, or only the background is post-filtered, or the filter emphasizes ROIs differently than the background. The intended usage may be user viewing or machine analysis.

The following table provides our summary of the relation of the use cases to the study questions and technical approaches.

|  | Boundary between filtered and non-filtered regions | Alignment between patches and regions | Use of multiple regions with different filtering | Suitable technical approaches |
| --- | --- | --- | --- | --- |
| RD-optimized region-based filtering | Expected that the entire picture is filtered (potentially with different NNPFs) | Picture is divided into a grid of patches. An NNPF indicated or inferred for each grid cell | Expected that the entire picture is filtered (potentially with different NNPFs) | NNPFA SEI extension, regional nesting, or NNPF regional activation are likely to be impractical. |
| Constituent-frame-based filtering | Not expected to affect subjective quality. | A constituent frame is treated like an input picture for NNPF. | The maximum number of regions is equal to the number of constituent frames. | Regional nesting. NNPFA SEI extension or NNPF regional activation with regions defining the input. |
| Region-of-interest-based filtering | When the entire picture is filtered in a ROI-aware manner, the filter can smooth out ROI boundaries. | Not necessarily aligned. | Expected that a single NNPF only is used | Either NNPFA SEI extension or NNPF regional activation, and regions as auxiliary NNPF input. Additionally, if the number of ROIs is moderate, the other approaches are suitable too. |

It can be observed that the identified use cases can be handled when both of the following are included in the design:

- Either the NNPFA SEI message extension for indicating regions or the NNPF regional activation SEI message

- In the NNPFC SEI message, a new nnpfc_auxiliary_inp_idc value for providing indications about regions as input to the NNPF

It is proposed to adopt both as the high-level design decisions based on which detailed syntax and semantics can be presented in the upcoming JVET meetings. The proponents do not have a preference whether the NNPFA SEI message extension for indicating regions or the NNPF regional activation SEI message is adopted as a design decision.

# Specification text for NNPFA SEI message extension

## Proposed specification text for VSEI v3

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) |  |
| nnpfa_persistence_flag | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfa_extension_bit[ i ] | u(1) |
| } |  |

nnpfa_extension_bit[ i ] is reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document encountering an NNPFA SEI message with any nnpfa_extension_bit[ i ] present shall ignore the SEI message.

## Specification text skeleton for VSEI v4

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_persistence_flag | u(1) |
| if( more_data_in_payload( ) ) { |  |
| nnpfa_region_flag | u(1) |
| if( nnpfa_region_flag ) { |  |
| // syntax elements to define region(s) |  |
| } |  |
| } |  |
| } |  |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfa_extension_bit[ i ] | u(1) |
| } |  |

# References

Hendry, J. Nam, H. Jang, S. Kim, J. Lim, "AHG9: On design for region-based neural-network post-filter SEI message", JVET-AC0132.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).