| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0060-v1 |
| --- | --- |

| Title: | AHG8/AHG9: Indications related to suboptimal user viewing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Francesco Cricri Honglei Zhang Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution reviews some earlier contributions on preprocessing and encoding methods targeted at machine analysis and concludes that spatial and temporal quality fluctuations resulting from such methods may result into decoded video that looks subjectively unpleasant. To avoid accidental displaying of video that is not intended for user viewing, such as machine-targeted bitstreams, the following items are proposed in this input document:

- A VUI indication to indicate that there shall not be any annotated regions SEI messages with ar_not_optimized_for_viewing_flag equal to 1 or suboptimal user viewing SEI messages present. The following options for the VUI indication are proposed, from which the proponent prefers option 3:

  - Overloading vui_non_packed_constraint_flag.

  - Overlading vui_non_projected_constraint_flag.

  - Adding vui_suboptimal_user_viewing_constraint_flag.

- A suboptimal user viewing indication SEI message, as proposed earlier in JVET-AC0077, which indicates that the video resulting by decoding the current CLVS is not optimized for user viewing and may therefore look incoherent.

It is proposed to add the proposed items to a VSEI amendment working draft.

# Introduction

## Background

Contributions on optimizing preprocessing and encoding to be suitable for machine analysis of coded video content were presented in the 28th JVET meeting and are summarized below.

- JVET-AB0274 [1] presents a VVC-based system where video is preprocessed prior to VVC-compliant encoding by i) blacking out or blurring blocks that are not covering detected regions of interest (ROIs), ii) downsampling pictures adaptively, and iii) downsampling picture rate adaptively. Upsampling of both pictures and picture rate are performed as postprocessing subsequent to VVC decoding.

- JVET-AB0275 [2] presents an encoder-only method that uses a lower QP for CTUs covering detected ROIs compared to the remaining CTUs, where the QP difference is up to 4. Furthermore, JVET-AB0275 increases the QP offset by 5 for frames with TemporalId greater than or equal to 2 compared to the QP settings of the random access common test conditions.

It is asserted that the spatial and temporal quality fluctuations resulting from JVET-AB0274 and JVET-AB0275 are subjectively unpleasant. When no other preprocessing or encoding tool targeted for machines than the steeper QP cascading for temporal sublayers is in use, the entire bitstream has perceivable temporal quality fluctuations, whereas a sub-bitstream containing only certain lowest sublayers has a stable quality.

A suboptimal user viewing indication SEI message was proposed in JVET-AC0077 [3] and can be summarized as follows:

- The suboptimal user viewing indication SEI message indicates that the video resulting by decoding the current CLVS is not optimized for user viewing and may therefore look incoherent.

- A suboptimal user viewing indication SEI message shall be present in the first picture unit, in decoding order, within a CLVS that has a particular temporal sublayer identifier lowestTidInScope. For example, if a bitstream is encoded with an additional QP offset equal to 5 for frames with TemporalId greater than or equal to 2, a suboptimal user viewing indication SEI message is included in the first picture unit, in decoding order, within a CLVS that has TemporalId equal to 2 (thus, in this example, lowestTidInScope is equal to 2). Consequently, a sub-bitstream formed by picture units of TemporalId equal to 0 and 1 only would not contain a suboptimal user viewing indication SEI message and would be considered subjectively coherent.

The annotated regions SEI message carries parameters that identify annotated regions using bounding boxes representing the size and location of identified objects. In addition, the annotated regions SEI message contains the following indication, supposedly intended to be used when specific objects or background have been processed so that the subjective quality difference between objects and background could make the video suboptimal for user viewing.

ar_not_optimized_for_viewing_flag equal to 1 indicates that the decoded pictures that the annotated regions SEI message applies to are not optimized for user viewing, but rather are optimized for some other purpose such as algorithmic object classification performance. ar_not_optimized_for_viewing_flag equal to 0 indicates that the decoded pictures that the annotated regions SEI message applies to may or may not be optimized for user viewing.

## Discussion

In response to JVET-AC0077, it was noted in the meeting minutes that a concept of a VUI indication exists to indicate that a bitstream has some special meaning. It was suggested to study whether it would be more consistent to align signaling for a bitstream that is potentially suboptimal for user viewing with the VUI indication. In our understanding, this remark relates to vui_non_packed_constraint_flag and vui_non_projected_constraint_flag, specified as follows:

vui_non_packed_constraint_flag equal to 1 specifies that there shall not be any frame packing arrangement SEI messages present in the bitstream that apply to the CLVS. vui_non_packed_constraint_flag equal to 0 does not impose such a constraint.

vui_non_projected_constraint_flag equal to 1 specifies that there shall not be any equirectangular projection SEI messages or generalized cubemap projection SEI messages present in the bitstream that apply to the CLVS. vui_non_projected_constraint_flag equal to 0 does not impose such a constraint.

We have the following remarks:

- The semantics of neither vui_non_packed_constraint_flag nor vui_non_projected_constraint_flag seem currently suitable for indicating video that might not be optimized for user viewing. It is possible to extend the semantics of either flag to cover suboptimal user viewing, but such extension would have the following implications:

  - Overloading vui_non_packed_constraint_flag (see Section 2.1). The decoder would not be able to deduce from VUI whether this video is frame-packed or suboptimal for user viewing.

  - Overloading vui_non_projected_constraint_flag (see Section 2.2). The decoder would not be able to deduce from VUI whether this video is omnidirectional or suboptimal for user viewing.

- The semantics of vui_non_packed_constraint_flag and vui_non_projected_constraint_flag, when equal to 1, guarantee that the bitstream is constrained not to contain specific SEI messages. In a similar design, both the suboptimal user viewing indication SEI message and the VUI indication referring to the absence of suboptimal user viewing indication SEI message would be introduced.

- The annotated regions SEI message with ar_not_optimized_for_viewing_flag equal to 1 is suggested to be treated similarly to suboptimal user viewing indication SEI message when it comes to the proposed VUI indication.

- The annotated regions SEI message carries parameters that identify annotated regions using bounding boxes representing the size and location of identified objects and is therefore not suitable for all types of preprocessing or encoding for machine analysis, such as methods in JVET-AB0274 and JVET-AB0275. Thus, the suboptimal user viewing indication SEI message is re-proposed in this input document.

## Overview of the proposal

The following items are proposed in this input document:

- A VUI indication to indicate that there shall not be any annotated regions SEI messages with ar_not_optimized_for_viewing_flag equal to 1 or suboptimal user viewing SEI messages present. The following options for the VUI indication are proposed, from which the proponent prefers option 3:

  - Overloading vui_non_packed_constraint_flag.

  - Overlading vui_non_projected_constraint_flag.

  - Adding vui_suboptimal_user_viewing_constraint_flag.

- A suboptimal user viewing indication SEI message, like in JVET-AC0077

It is proposed to add the proposed items to a VSEI amendment working draft.

# Proposed VUI indication

## Option 1: overloading vui_non_packed_constraint_flag

vui_non_packed_constraint_flag equal to 1 specifies that there shall not be any frame packing arrangement SEI messages, annotated regions SEI messages with ar_not_optimized_for_viewing_flag equal to 1, or suboptimal user viewing SEI messages present in the bitstream that apply to the CLVS. vui_non_packed_constraint_flag equal to 0 does not impose such a constraint.

## Option 2: overloading vui_non_projected_constraint_flag

vui_non_projected_constraint_flag equal to 1 specifies that there shall not be any equirectangular projection SEI messages, or generalized cubemap projection SEI messages, annotated regions SEI messages with ar_not_optimized_for_viewing_flag equal to 1, or suboptimal user viewing SEI messages present in the bitstream that apply to the CLVS. vui_non_projected_constraint_flag equal to 0 does not impose such a constraint.

## Option 3: adding vui_suboptimal_user_viewing_constraint_flag

| vui_parameters( payloadSize ) { | Descriptor |
| --- | --- |
| vui_progressive_source_flag | u(1) |
| vui_interlaced_source_flag | u(1) |
| vui_non_packed_constraint_flag | u(1) |
| vui_non_projected_constraint_flag | u(1) |
| vui_aspect_ratio_info_present_flag | u(1) |
| if( vui_aspect_ratio_info_present_flag ) { |  |
| vui_aspect_ratio_constant_flag | u(1) |
| vui_aspect_ratio_idc | u(8) |
| if( vui_aspect_ratio_idc = = 255 ) { |  |
| vui_sar_width | u(16) |
| vui_sar_height | u(16) |
| } |  |
| } |  |
| vui_overscan_info_present_flag | u(1) |
| if( vui_overscan_info_present_flag ) |  |
| vui_overscan_appropriate_flag | u(1) |
| vui_colour_description_present_flag | u(1) |
| if( vui_colour_description_present_flag ) { |  |
| vui_colour_primaries | u(8) |
| vui_transfer_characteristics | u(8) |
| vui_matrix_coeffs | u(8) |
| vui_full_range_flag | u(1) |
| } |  |
| vui_chroma_loc_info_present_flag | u(1) |
| if( vui_chroma_loc_info_present_flag ) { |  |
| if( vui_progressive_source_flag && !vui_interlaced_source_flag ) |  |
| vui_chroma_sample_loc_type_frame | ue(v) |
| else { |  |
| vui_chroma_sample_loc_type_top_field | ue(v) |
| vui_chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| } |  |
| if( more_data_in_payload( ) ) |  |
| vui_suboptimal_user_viewing_constraint_flag | u(1) |
| } |  |

vui_suboptimal_user_viewing_constraint_flag equal to 1 specifies that there shall not be any annotated regions SEI messages with ar_not_optimized_for_viewing_flag equal to 1 or suboptimal user viewing SEI messages present in the bitstream that apply to the CLVS. vui_suboptimal_user_viewing_constraint_flag equal to 0 does not impose such a constraint. When not present, vui_suboptimal_user_viewing_constraint_flag is inferred to be equal to 0.

# Proposed suboptimal user viewing indication SEI message (copy of the proposal in JVET-AC0077)

## Persistence scope

| Table 4 – Persistence scope of SEI messages (informative) |  |
| --- | --- |
| SEI message | Persistence scope |
| … | … |
| Suboptimal user viewing indication | The CLVS containing the SEI message |

## Syntax

| suboptimal_user_viewing_indication( payloadSize ) { | Descriptor |
| --- | --- |
| } |  |

## Semantics

The suboptimal user viewing indication SEI message indicates that the video resulting by decoding the current CLVS is not optimized for user viewing and may therefore look incoherent.

NOTE 1 – The video is optimized for some other purpose than user viewing, such as a machine analysis task.

When a suboptimal user viewing indication SEI message is present for any picture of a CLVS, a suboptimal user viewing indication SEI message shall be present in the first picture unit, in decoding order, within a CLVS that has a particular temporal sublayer identifier equal to tId and shall not be present in any other picture unit of the CLVS. When a suboptimal user viewing indication SEI message is present in a picture unit with tId greater than 0, the sequence of cropped decoded output pictures decoded only from the picture units with temporal sublayer identifier less than tId is suitable for user viewing.

# References

JVET-AB0274, S. Wang, B. Li, Z. Wang, Y. Ye, S. Wang, “Encoder-only algorithms in Alibaba and City University of Hong Kong’s response to the Video Coding for Machines CfP”.

JVET-AB0275, C. Hollmann, J. Ström, P. Wennersten, L. Litwic, R. Sjöberg, M. Damghanian, “Information about Ericsson’s response to the CfP for Video Coding for Machines”.

JVET-AC0077, M. M. Hannuksela, F. Cricri, H. Zhang, “AHG9/AHG15: On bitstreams that are potentially suboptimal for user viewing”.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).