| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0064-v1 |
| --- | --- |

| Title: | AHG12: Enhanced geometrical prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu Per Wennersten Jack Enhorn Kenneth Andersson | Email: | ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The existing GPM design in ECM relies on uni-predictive motion vectors to generate motion compensated prediction samples for each inter partition. In this contribution, it is proposed to extend the design to allow usage of bi-predictive motion vectors. Furthermore, the current ECM tools GPM-MMVD and GPM-TM are modified to incorporate such a modification. The proposed method is reported to be implemented on top of ECM-8.0. The BD-rate PSNR impact is reported to be -0.21% (estimated number with QP 22 for one sequence still running, final number will be updated) and -0.27% under RA and LDB common test configurations, respectively. The encoding run time number for class B, C and D under RA is reported to be 101%. The decoding run time number for class B, C and D under RA is reported to be 100%. It is asserted that the proposed modifications are straightforward and provide a reasonable trade-off between BD-rate gains and run time impacts. It is proposed to include the proposed modifications into the next version of ECM.

# Introduction

The existing GPM design in ECM relies on uni-predictive motion vectors to generate motion compensated prediction samples for each inter partition. The current generation process of GPM merge candidate list consists of two steps. The first step is to generate an initial list where the list generation process is the same as the normal merge list generation process, and the second step is an extraction process to extract uni-predictive motion vectors from the initial list into the GPM merge candidate list.

# Proposal

It is proposed to extend the current GPM design to allow usage of bi-predictive motion vectors. The proposed modifications contain following elements.

The first element conditionally invokes the extraction process that extracts uni-predictive motion vectors from the initial list. The extraction process is invoked only for small blocks 8x8, 16x8 and 8x16. For other larger blocks, the extraction process is bypassed. The generation of the initial list is the same as before (i.e., the normal merge list generation without any candidate reordering) except that when generating the initial list for larger blocks (i.e., blocks with the extraction process bypassed), the motion vector difference threshold for controlling whether a candidate can be added into the initial list is increased to be one full sample distance.

The second element modifies GPM-MMVD to support bi-predictive motion vector as the base vector. For low-delay pictures, the signaled MVD is applied on top of the L0 and L1 motion vector as in the existing merge MMVD design. For non-low-delay pictures, the bi-predictive motion vector is converted into a uni-predictive motion vector first and then the MVD is applied on top.

The third element modifies GPM-TM to also support bi-predictive motion vectors. When the picture is a non-low-delay picture and the best template cost from using bi-prediction exceeds 75% of the best template cost from using uni-prediction, the refined uni-predictive motion vector is then determined to be the final refined motion vector. Otherwise, the refined bi-predictive motion vectors are determined to be the final refined motion vectors.

The last element is to enable 8x8 BDOF (i.e., as in the existing design of multi-pass DMVR) on top of the associated bi-predictive motion vectors for each inter partition.

# Results

The proposal has been implemented in ECM-8.0 [1] and evaluated under the ECM RA and LDB common test configurations [2].

The following table shows the objective results. For RA, the encoding and decoding run time numbers for class B, C and D are asserted to be accurate. For LDB, the encoding run time numbers for class C, E and D are asserted to be accurate. The decoding run time numbers for all classes are asserted to be accurate. For other run time numbers marked with * are not accurate.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | -0,28% | -0,24% | -0,17% | 104%* | 99%* |
| Class B | -0,18% | -0,11% | -0,21% | 101% | 99% |
| Class C | -0,22% | -0,22% | -0,06% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0,15% | 0,08% | -0,05% | 101% | 100% |
| Class F | -0,13% | -0,20% | -0,18% | 100%* | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,23% | -0,55% | -0,27% | 106%* | 100% |
| Class C | -0,30% | -0,08% | -0,02% | 102% | 102% |
| Class E | -0,30% | 0,31% | -0,01% | 102% | 101% |
| Overall | -0,27% | -0,18% | -0,12% | 104%* | 101% |
| Class D | -0,08% | -0,47% | -0,50% | 101% | 102% |
| Class F | 0,11% | -0,33% | -0,03% | 103%* | 102% |

# Conclusion

The proposal proposes to allow usage of bi-predictive motion vectors for GPM, furthermore, the GPM related tools GPM-MMVD and GPM-TM are modified to incorporate such changes. It is asserted that the proposed modifications are straightforward and the trade-off between BD-rate gains and run time impacts seem reasonable. It is proposed to include the proposed modifications into the next version of ECM.

# References

[1] ECM-8.0, available at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-8.0

[2] M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).