| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0067-v1 |
| --- | --- |

| Title: | AHG9: Comments on Neural-Network Post-Filter Characteristics Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Some modifications are proposed to the Neural-Network Post-Filter Characteristics SEI Message. Following is proposed:

- Proposal 1: It is proposed to signal nnpfc_inp_tensor_chroma_bitdepth_minus8 only when nnpfc_inp_order_idc is not equal to 0. For supporting this conditional signalling it is proposed to move the signalling of nnpfc_inp_order_idc syntax element before the signalling of input tensor luma and chroma bit depth syntax elements.

Similarly, it is proposed to signal nnpfc_out_tensor_chroma_bitdepth_minus8 only when nnpfc_out_order_idc is not equal to 0. For supporting this conditional signalling it is proposed to move the signalling of nnpfc_out_order_idc syntax element before the signalling of output tensor luma and chroma bit depth syntax elements.

- Proposal 2: It is proposed to signal the syntax element nnpfc_auxiliary_inp_idc before the syntax element nnpfc_inp_order_idc. It is asserted that the semantics of nnpfc_inp_order_idc syntax element use the value of nnpfc_auxiliary_inp_idc. As these two syntax elements are signalled one after another, it is proposed to signal in the order nnpfc_auxiliary_inp_idc followed by nnpfc_inp_order_idc.

- Proposal 3: It is proposed to code the syntax elements nnpfc_pic_width_in_luma_samples with an offset of CroppedWidth and nnpfc_pic_height_in_luma_samples with an offset of CroppedHeight.

- Proposal 4: A sanity check constraint is proposed which prevents signalling where the NNPF purpose (nnpfc_purpose) signals resolution upsampling, but the cropped decoded output picture resulting from applying the NNPF has the exact same resolution as the input picture.

- Proposal 5: Instead of signalling syntax elements nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples, it is proposed to signal the horizontal and vertical upsampling ratio/ scale factor. It is asserted that this could result in fewer NNPFC SEI messages. If proposal 5 is agreed, proposals 3 and 4 are not relevant.

- Proposal 6: It is proposed to move the nnpfc_base_flag outside the current conditional signalling inside nnpfc_property_present_flag and to move it earlier in the syntax.

- Proposal 7: It is proposed to include a syntax element nnpfc_full_range_flag in the NNPFC message when nnpfc_separate_colour_description_present_flag is equal to 1.

# Introduction

Additional SEI messages for VSEI [1] specifies the syntax and semantics for Neural network post-filter characteristics (NNPFC) SEI message and for neural-network post-filter activation SEI message. Neural network post-filter characteristics SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

Some modifications are proposed to the neural-network post-filter characteristics SEI Message related to the auxiliary input information.

# Proposals

Some modifications are proposed to the Neural-network post-filter characteristics SEI Message. Following is proposed:

## Proposal 1

It is proposed to signal nnpfc_inp_tensor_chroma_bitdepth_minus8 only when nnpfc_inp_order_idc is not equal to 0. It is asserted that when nnpfc_inp_order_idc is equal to 0, no chroma matrix is present in the input tensor (as there is only one luma matrix and possibly an auxiliary input matrix in the input tensor) so signalling chroma input tensor bit depth does not make sense and may be confusing. For supporting this conditional signalling it is proposed to move the signalling of nnpfc_inp_order_idc syntax element before the signalling of input tensor luma and chroma bit depth syntax elements.

Similarly, it is proposed to signal nnpfc_out_tensor_chroma_bitdepth_minus8 only when nnpfc_out_order_idc is not equal to 0. It is asserted that when nnpfc_out_order_idc is equal to 0, no chroma matrix is present in the output tensor (as there is only one luma matrix in the output tensor) so signalling chroma output tensor bit depth does not make sense and may be confusing. For supporting this conditional signalling it is proposed to move the signalling of nnpfc_out_order_idc syntax element before the signalling of output tensor luma and chroma bit depth syntax elements.

The proposed modifications are shown below compared with JVET-AC2032-v2.

8.28 Neural-network post-filter characteristics SEI message

8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| … | ue(v) |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| nnpfc_base_flag | u(1) |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| … | u(1) |
| } |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| nnpfc_inp_order_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| if( nnpfc_inp_order_idc ! = 0 ) |  |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_inp_order_idc | ue(v) |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| nnpfc_matrix_coeffs | u(8) |
| } |  |
| nnpfc_out_format_idc | ue(v) |
| nnpfc_out_order_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| if( nnpfc_out_order_idc ! = 0 ) |  |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| } |  |
| } |  |
| … |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

## Proposal 2

It is proposed to signal the syntax element nnpfc_auxiliary_inp_idc before the syntax element nnpfc_inp_order_idc. It is asserted that the semantics of nnpfc_inp_order_idc syntax element use the value of nnpfc_auxiliary_inp_idc. As these two syntax elements are signalled one after another, it is proposed to signal in the order nnpfc_auxiliary_inp_idc followed by nnpfc_inp_order_idc.

The proposed modifications are shown below compared with JVET-AC2032-v2.

  - Neural-network post-filter characteristics SEI message

### 8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| nnpfc_base_flag | u(1) |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( ( nnpfc_purpose & 0x02 ) != 0 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( ( nnpfc_purpose & 0x20 ) != 0 ) |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( ( nnpfc_purpose & 0x04 ) != 0 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| if( ( nnpfc_purpose & 0x08 ) != 0 ) { |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_output_flag[ i ] | u(1) |
| } |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_inp_order_idc nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_auxiliary_inp_idc nnpfc_inp_order_idc | ue(v) |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| nnpfc_matrix_coeffs | u(8) |
| } |  |
| … |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

## Proposal 3

In JVET-AC2032, two syntax elements nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are signalled when the NNPF has the purpose of resolution upsampling. nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture.

It is asserted that the minimum value of the syntax element nnpfc_pic_width_in_luma_samples is CroppedWidth and the minimum value of the syntax element nnpfc_pic_height_in_luma_samples is CroppedHeight. The syntax elements nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are coded as ue(v). It is thus proposed to signal each of these two syntax elements using an offset (or delta) which equals the minimum value those syntax elements are allowed to have. It is asserted that this saves bits and also allows signalling only valid lower range values for these two syntax elements.

Thus it is proposed to:

- Signal syntax element nnpfc_pic_width_in_luma_samples instead as a syntax element nnpfc_pic_width_in_luma_samples_delta by subtracting CroppedWidth

- Signal syntax element nnpfc_pic_height_in_luma_samples instead as a syntax element nnpfc_pic_height_in_luma_samples_delta by subtracting CroppedHeight

The proposed changes are shown highlighted below compared to JVET-AC2032-v2.

nnpfc_pic_width_in_luma_samples_delta plus CroppedWidth and nnpfc_pic_height_in_luma_samples_delta plus CroppedHeight specify the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples_delta and nnpfc_pic_height_in_luma_samples_delta are not present, they are inferred to be equal to 0 CroppedWidth and 0 CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples_delta shall be in the range of 0 CroppedWidth to CroppedWidth * 1516 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of 0 CroppedHeight to CroppedHeight * 1516 − 1, inclusive.

The variables nnpfcPicWidthInLumaSamples and nnpfcPicHeightInLumaSamples, specifying the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture, are derived as follows:

nnpfcPicWidthInLumaSamples = nnpfc_pic_width_in_luma_samples_delta + CroppedWidth

nnpfcPicHeightInLumaSamples = nnpfc_pic_height_in_luma_samples_delta + CroppedHeight

nnpfc_out_order_idc indicates the output order of samples resulting from the NNPF.

The value of nnpfc_out_order_idc shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfc_out_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_out_order_idc in the range of 4 to 255, inclusive. Values of nnpfc_out_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

…

The process StoreOutputTensors( ), for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensor outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numOutputPics; i++ ) {
	if( nnpfc_out_order_idc  = =  0 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				if ( yY < nnpfc_pic_height_in_luma_samples nnpfcPicHeightInLumaSamples  &&  xY < nnpfc_pic_width_in_luma_samples nnpfcPicWidthInLumaSamples )
					if( !nnpfc_component_last_flag )
						FilteredYPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
					else
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]			}
	else if( nnpfc_out_order_idc  = =  1 )		(85)
		for( yP = 0; yP < outPatchCHeight; yP++)
			for( xP = 0; xP < outPatchCWidth; xP++ ) {
				xSrc = cLeft * horCScaling + xP
				ySrc = cTop * verCScaling + yP
				if ( ySrc < nnpfc_pic_height_in_luma_samples nnpfcPicHeightInLumaSamples / outSubHeightC  &&
						xSrc < nnpfc_pic_width_in_luma_samples nnpfcPicWidthInLumaSamples / outSubWidthC )
					if( !nnpfc_component_last_flag ) {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
					} else {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  2 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				yC = yY / outSubHeightC 
				xC = xY / outSubWidthC 
				yPc = ( yP / outSubHeightC ) * outSubHeightC
				xPc = ( xP / outSubWidthC ) * outSubWidthC
				if ( yY < nnpfc_pic_height_in_luma_samples nnpfcPicHeightInLumaSamples && xY < nnpfc_pic_width_in_luma_samples nnpfcPicWidthInLumaSamples)
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 1 ][ yPc ][ xPc ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 2 ][ yPc ][ xPc ]
					} else {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 1 ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 2 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  3 )
		for( yP = 0; yP < outPatchHeight; yP++ )
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP
				xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP
				if ( ySrc < nnpfc_pic_height_in_luma_samples nnpfcPicHeightInLumaSamples / 2  &&
						xSrc < nnpfc_pic_width_in_luma_samples nnpfcPicWidthInLumaSamples / 2 )
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 2 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 3 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 4 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 5 ][ yP ][ xP ]
					} else {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 2 ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 3 ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 4 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 5 ]
					}
			}
}

nnpfc_extended_patch_height_cd_delta_minus1 plus 1 plus 2 * nnpfc_overlap, when nnpfc_constant_patch_size_flag equal to 0, indicates a common divisor of all allowed values of the height of an extended patch required for the input to the NNPF. The value of nnpfc_extended_patch_height_cd_delta_minus1 shall be in the range of 0 to Min( 32 766, CroppedHeight − 1 ), inclusive.

Let the variables inpPatchWidth and inpPatchHeight be the patch size width and the patch size height, respectively.

If nnpfc_constant_patch_size_flag is equal to 0, the following applies:

–	The values of inpPatchWidth and inpPatchHeight are either provided by external means not specified in this document or set by the post-processor itself.

–	The value of inpPatchWidth + 2 * nnpfc_overlap shall be a positive integer multiple ofnnpfc_extended_patch_width_cd_delta_minus1 + 1 + 2 * nnpfc_overlap and inpPatchWidth shall be less than or equal to CroppedWidth. The value of inpPatchHeight + 2 * nnpfc_overlap shall be a positive integer multiple ofnnpfc_extended_patch_height_cd_delta_minus1 + 1 + 2 * nnpfc_overlap and inpPatchHeight shall be less than or equal to CroppedHeight.

Otherwise (nnpfc_constant_patch_size_flag is equal to 1), the value of inpPatchWidth is set equal to nnpfc_patch_width_minus1 + 1 and the value of inpPatchHeight is set equal to nnpfc_patch_height_minus1 + 1.

The variables outPatchWidth, outPatchHeight, horCScaling, verCScaling, outPatchCWidth, and outPatchCHeight are derived as follows:

outPatchWidth = ( nnpfc_pic_width_in_luma_samples nnpfcPicWidthInLumaSamples * inpPatchWidth ) / CroppedWidth	(86)

outPatchHeight = ( nnpfc_pic_height_in_luma_samples nnpfcPicHeightInLumaSamples * inpPatchHeight ) / CroppedHeight	(87)

horCScaling = SubWidthC / outSubWidthC		(88)

verCScaling = SubHeightC / outSubHeightC		(89)

outPatchCWidth = outPatchWidth * horCScaling		(90)

outPatchCHeight = outPatchHeight * verCScaling		(91)

It is a requirement of bitstream conformance that outPatchWidth * CroppedWidth shall be equal to nnpfc_pic_width_in_luma_samples nnpfcPicWidthInLumaSamples * inpPatchWidth and outPatchHeight * CroppedHeight shall be equal to nnpfc_pic_height_in_luma_samples nnpfcPicHeightInLumaSamples * inpPatchHeight.

## Proposal 4

Currently it is allowed to signal nnpfc_pic_width_in_luma_samples value equal to CroppedWidth and nnpfc_pic_height_in_luma_samples CroppedHeight when ((nnpfc_purpose & 0x04) !=  0, which specifies resolution upsampling.

A sanity check constraint is proposed which prevents signalling where the NNPF purpose (nnpfc_purpose) signals resolution upsampling, but the cropped decoded output picture resulting from applying the NNPF has the exact same resolution as the input picture.

The proposed modifications are shown below compared with JVET-AC2032-v2.

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth to CroppedWidth * 16 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight to CroppedHeight * 16 − 1, inclusive.

When ((nnpfc_purpose & 0x04) !=  0), at least one of the following conditions shall be true:

–	nnpfc_pic_width_in_luma_samples shall be greater than CroppedWidth

–	nnpfc_pic_height_in_luma_samples shall be greater than CroppedHeight

The constraint for proposal 4 together with proposal 3 is as follows:

nnpfc_pic_width_in_luma_samples_delta plus CroppedWidth and nnpfc_pic_height_in_luma_samples_delta plus CroppedHeight specify the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples_delta and nnpfc_pic_height_in_luma_samples_delta are not present, they are inferred to be equal to 0 CroppedWidth and 0 CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples_delta shall be in the range of 0 CroppedWidth to CroppedWidth * 1516 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of 0 CroppedHeight to CroppedHeight * 1516 − 1, inclusive.

When ((nnpfc_purpose & 0x04) !=  0), at least one of the following conditions shall be true:

–	nnpfc_pic_width_in_luma_samples_delta shall be greater than 0

–	nnpfc_pic_height_in_luma_samples_delta shall be greater than 0

## Proposal 5

Instead of signalling syntax elements nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples, it is proposed to signal the horizontal and vertical upsampling ratio/ or scale factor. It is asserted that this could result in fewer NNPFC SEI messages in case the CLVS has different cropped decoded output picture resolutions, but same upsampling scale factors. If proposal 5 is agreed proposals 3 and 4 are not relevant.

Two alternative options are proposed:

- Option 1: The numerator is signalled as a delta compared to denominator (for upsampling)

- Option 2: The numerator is signalled directly (not as a delta compared to the denominator)

Option 1: The numerator is signalled as a delta compared to denominator (for upsampling)

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| nnpfc_base_flag | u(1) |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( ( nnpfc_purpose & 0x02 ) != 0 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( ( nnpfc_purpose & 0x20 ) != 0 ) |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( ( nnpfc_purpose & 0x04 ) != 0 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| nnpfc_out_pic_width_scale_factor_denominator_minus1 | u(20) |
| nnpfc_out_pic_width_scale_factor_numerator_delta | u(16) |
| nnpfc_out_pic_height_scale_factor_denominator_minus1 | u(20) |
| nnpfc_out_pic_height_scale_factor_numerator_delta | u(16) |
| } |  |
| if( ( nnpfc_purpose & 0x08 ) != 0 ) { |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_output_flag[ i ] | u(1) |
| } |  |
| … |  |
| } |  |

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth to CroppedWidth * 16 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight to CroppedHeight * 16 − 1, inclusive.

nnpfc_out_pic_width_scale_factor_denominator_minus1 plus 1 specifies the denominator of the scale factor  used for multiplying CroppedWidth to determine the width of the luma sample array of the picture resulting from applying the NNPF identified by nnpf_id to a cropped decoded output picture.

nnpfc_out_pic_width_scale_factor_numerator_delta plus (nnpfc_out_pic_width_scale_factor_denominator_minus1+1) specifies the numerator of the scale factor  used for multiplying CroppedWidth to determine the width of the luma sample array of the picture resulting from applying the NNPF identified by nnpf_id to a cropped decoded output picture.

nnpfc_out_pic_height_scale_factor_denominator_minus1 plus 1 specifies the denominator of the scale factor  used for multiplying CroppedHeight to determine the height of the luma sample array of the picture resulting from applying the NNPF identified by nnpf_id to a cropped decoded output picture.

nnpfc_out_pic_height_scale_factor_numerator_delta plus (nnpfc_out_pic_height_scale_factor_denominator_minus1+ 1) specifies the numerator of the scale factor  used for multiplying CroppedHeight to determine the height of the luma sample array of the picture resulting from applying the NNPF identified by nnpf_id to a cropped decoded output picture.

The variables nnpfcPicWidthInLumaSamples and nnpfcPicHeightInLumaSamples, specifying the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture, are derived as follows:

nnpfcPicWidthInLumaSamples = CroppedWidth * (nnpfc_out_pic_width_scale_factor_numerator_delta+ nnpfc_out_pic_width_scale_factor_denominator_minus1+1) ÷ (nnpfc_out_pic_width_scale_factor_denominator_minus1+1)

nnpfcPicHeightInLumaSamples = CroppedHeight * (nnpfc_out_pic_height_scale_factor_numerator_delta+ nnpfc_out_pic_height_scale_factor_denominator_minus1+1) ÷ (nnpfc_out_pic_height_scale_factor_denominator_minus1+1)

Option 2: The numerator is signalled directly (not as a delta compared to the denominator)

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| nnpfc_base_flag | u(1) |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( ( nnpfc_purpose & 0x02 ) != 0 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( ( nnpfc_purpose & 0x20 ) != 0 ) |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( ( nnpfc_purpose & 0x04 ) != 0 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| nnpfc_out_pic_width_scale_factor_denominator_minus1 | u(20) |
| nnpfc_out_pic_width_scale_factor_numerator_minus1 | u(20) |
| nnpfc_out_pic_height_scale_factor_denominator_minus1 | u(20) |
| nnpfc_out_pic_height_scale_factor_numerator_minus1 | u(20) |
| } |  |
| if( ( nnpfc_purpose & 0x08 ) != 0 ) { |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_output_flag[ i ] | u(1) |
| } |  |
| … |  |
| } |  |

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth to CroppedWidth * 16 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight to CroppedHeight * 16 − 1, inclusive.

nnpfc_out_pic_width_scale_factor_denominator_minus1 plus 1 specifies the denominator of the scale factor  used for multiplying CroppedWidth to determine the width of the luma sample array of the picture resulting from applying the NNPF identified by nnpf_id to a cropped decoded output picture.

nnpfc_out_pic_width_scale_factor_numerator_minus1 plus 1 specifies the numerator of the scale factor  used for multiplying CroppedWidth to determine the width of the luma sample array of the picture resulting from applying the NNPF identified by nnpf_id to a cropped decoded output picture.

nnpfc_out_pic_height_scale_factor_denominator_minus1 plus 1 specifies the denominator of the scale factor  used for multiplying CroppedHeight to determine the height of the luma sample array of the picture resulting from applying the NNPF identified by nnpf_id to a cropped decoded output picture.

nnpfc_out_pic_height_scale_factor_numerator_minus1 plus 1 specifies the numerator of the scale factor used for multiplying CroppedHeight to determine the height of the luma sample array of the picture resulting from applying the NNPF identified by nnpf_id to a cropped decoded output picture.

The variables nnpfcPicWidthInLumaSamples and nnpfcPicHeightInLumaSamples, specifying the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture, are derived as follows:

nnpfcPicWidthInLumaSamples = CroppedWidth * (nnpfc_out_pic_width_scale_factor_numerator) ÷ (nnpfc_out_pic_width_scale_factor_denominator_minus1+1)

nnpfcPicHeightInLumaSamples = CroppedHeight * (nnpfc_out_pic_height_scale_factor_numerator) ÷ (nnpfc_out_pic_height_scale_factor_denominator_minus1+1)

The equation changes described in proposal 3 which uses nnpfcPicWidthInLumaSamples instead of nnpfc_pic_width_in_luma_samples, and nnpfcPicHeightInLumaSamples instead of nnpfc_pic_height_in_luma_samples also apply.

The constraint of proposal 4 also applies and is specified as follows:

When ((nnpfc_purpose & 0x04) !=  0), at least one of the following conditions shall be true:

–	(nnpfc_out_pic_width_scale_factor_numerator_minus1+1) ÷ (nnpfc_out_pic_width_scale_factor_denominator_minus1+1) shall be greater than 1.

–	(nnpfc_out_pic_height_scale_factor_numerator_minus1+1) ÷ (nnpfc_out_pic_height_scale_factor_denominator_minus1+1) shall be greater than 1.

## Proposal 6

The nnpfc_base_flag equal to 1 specifies that the SEI message specifies the base NNPF. nnpf_base_flag equal to 0 specifies that the SEI message specifies an update relative to the base NNPF. It is proposed to move the nnpfc_base_flag outside the current conditional signalling inside nnpfc_property_present_flag and to move it at an earlier position in the syntax.

It is asserted that currently nnpfc_base_flag syntax element is located after several other syntax elements including some syntax elements that are conditionally present based on if conditions.

It is asserted that at least in one scenario where nnpfc_base flag is set equal to 0, indicating that the NNPFC message includes an update relative to a base NNPF message, placing the nnpfc_base_flag at an earlier position as proposed can allow the receiving entity to abandon the parsing of the message sooner - for the case when it discovers that nnpfc_base_flag is equal to 0 and that it has not received a previous NNPFC message with the same nnpfc_id value as the nnpfc_id value in this message (which indicates that the base NNPFC message was lost).

The proposed modifications are shown below compared with JVET-AC2032-v2.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_base_flag | u(1) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| nnpfc_base_flag | u(1) |
| /* input and output formatting */ |  |
| … | ue(v) |
| } |  |
| … |  |
| } |  |

nnpfc_base_flag equal to 1 specifies that the SEI message specifies the base NNPF. nnpf_base_flag equal to 0 specifies that the SEI message specifies an update relative to the base NNPF. When not present, the value of nnpfc_base_flag is inferred to be equal to 0.

## Proposal 7

Currently NNPFC SEI message includes ability to signal colour description information. This includes the syntax elements nnpfc_colour_primaries, nnpfc_transfer_characteristics and nnpfc_matrix_coeffs.

It is asserted that ability to also signal information about full range flag will be useful and is currently missing.

It is proposed to include a syntax element nnpfc_full_range_flag in the NNPFC message when nnpfc_separate_colour_description_present_flag is equal to 1.

The proposed modifications are shown below compared with JVET-AC2032-v2.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| … |  |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| nnpfc_matrix_coeffs | u(8) |
| nnpfc_full_range_flag | u(1) |
| } |  |
| nnpfc_out_format_idc | ue(v) |
| … |  |
| } |  |

nnpfc_full_range_flag indicates the scaling and offset values applied in association with the matrix coefficients as specified by nnpfc_matrix_coeff. Its semantics are as specified for the VideoFullRangeFlag parameter in Rec. ITU-T H.273 | ISO/IEC 23091-2. When not present, the value of nnpfc_full_range_flag is inferred to be equal to 0.

# Conclusion

This document proposes modifications to neural network post processing characteristics SEI. It is proposed to adopt the proposals in the next version of “Additional SEI messages for VSEI” text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-AC2032, Improvements under consideration for neural network post filter SEI messages, Online, January 2023

Acknowledgement
These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.