| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0069-v1 |
| --- | --- |

| Title: | EE1-1.8: On flipping of input and output of model in NNVC filter set0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the results of EE1-1.8, which is JVET-AC0065. In JVET-AC0065, flipping operations applied to the inputs and output of the NNVC filter set#0 neural network are proposed to further improve the coding performance. This EE1-1.8 studies encoder optimizations for the proposed method, and several ratification subtests have been conducted during this round of EE, which shows the following results:

Test 1.8.1: Test results with EncNnlfOpt=1

Compared with NNVC-4.0 filter set#0,

RA: {-0.16%, -0.36%, -0.39%} EncT 102%, DecT 100%;

LDB: {-0.44%, -1.08%, -1.36%} EncT 103%, DecT 102%.

Compared with NNVC-4.0 EE anchor,

RA: {-10.13%	, -21.31%	, -20.79%} EncT 149%, DecT 52943%;

LDB: {-9.48%, -18.25%, -19.92%} EncT 161%, DecT 40446%.

Test 1.8.2: Test results with EncNnlfOpt=0

Compared with NNVC-4.0 filter set#0,

RA: {-0.16%, -0.34%, -0.39%} EncT 102%, DecT 100%;

LDB: {-0.43%, -0.94%, -1.39%} EncT 104%, DecT 102%.

Compared with NNVC-4.0 EE anchor,

RA: {-9.89%	, -21.34%	, -20.83%} EncT 148%, DecT 55847%;

LDB: {-9.32%, -18.50%, -20.27%} EncT 165%, DecT 45369%.

Introduction

In JVET-AC0065[1], the rec_yuv and pred_yuv inputs of the filter set0[2] network may be flipped horizontally or vertically before injection into the network. When flipping is performed on the inputs, the output of the network is flipped back to restore the correct order of the filtered samples.

These flipping operations are only allowed in B slices. A slice-level flag is signalled to indicate the usage of this method, and if it is applied, one additional slice-level flag is signalled to indicate the flipping type. Then all CTUs in the slice do the same flipping operation before and after network inference.

Proposed

This EE further studies the proposed flipping method. Several encoder-side optimizations are carried out to speed up the encoder running time.

Encoder-side speed up

First of all, an early termination method on the encoder side is proposed to skip some CTUs’ inferences during the determination of qp offset[3] and flipping type.

In the previous design, the encoder selects the best qp offset and flipping type by applying them to the inputs and executing the inference, then compares the distortion between the filtered pictures and original pictures. Finally, the candidate (qp offset or flipping type) with the smallest distortion will be selected and applied to all CTUs in the current slice.

It is proposed to only compare partial CTUs’ distortion for each candidate (different qp offsets and flipping types) and if the partial distortion of the current candidate is larger than a threshold T, then the filtering process of this candidate will be early terminated and skipped. Otherwise, the encoder continues to do the inference of those remaining CTUs in the current slice and then compares the distortions. In this contribution, T is set to 1.001 * smallest partial distortion. In Fig.1, shaded areas represent the CTUs filtered by filter set#0 and used for comparison and determination of qp-offset or flipping type.

CTU

- CTUs used for comparison in the previous design

CTU

- CTUs used for comparison in this contribution

Fig.1 Encoder-side method

Secondly, an early skip is applied to this proposed flipping method. When horizontal flipping is better than non-flipping or the original picture for the current slice, the vertical flipping will not be tested. Otherwise, the vertical flipping will be tested then.

Finally, it is proposed to filter only one for every three CTUs in a row and one for every two CTUs in the column for the current slice on the encoder side during the selection of qp offset and flipping type candidates.

Inference

The inference information is identical to NNVC-4.0 filter set#0.

Table. 1 Inference Information

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.9M |
|  | Total Number of Parameters (All Models) | 1.9M |
|  | Parameter Precision (Bits) | 16 |
|  | Memory Parameter (MB) | 3.8MB/model, 1 model in all |
|  | Multiply Accumulate (kMAC/pixel) | 485K (w/o block extension) 615K (w/ block extension) |
|  | Calculation Method | block basis |
| Optional |  |  |
|  | Total Conv. Layers | 101 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

No training is conducted in this contribution.

Experimental results

The proposed method is implemented on top of NNVC-4.0 filter set0 and all of the tests are conducted under the common test condition[4]. Only filter set0 is enabled, which means configuration NnlfOption is set to 1.

As discussed in the last meeting, the proposed method is tested in combination with RDO decision[5] enabled (3.1) and disabled (3.2), respectively.

EncNnlfOpt=1

Compared with NNVC-4.0 filter set0 (NnlfOption=1)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.11% | -0.13% | 101% | 100% |
| Class A2 | -0.18% | -0.55% | -0.44% | 103% | 100% |
| Class B | -0.15% | -0.26% | -0.52% | 102% | 101% |
| Class C | -0.20% | -0.52% | -0.38% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.36% | -0.39% | 102% | 100% |
| Class D | -0.19% | -0.58% | -0.55% | 103% | 101% |
| Class F | -0.08% | -0.35% | -0.36% | 106% | 108% |

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.38% | -0.88% | -1.64% | 102% | 101% |
| Class C | -0.49% | -1.81% | -1.40% | 102% | 101% |
| Class E | -0.48% | -0.42% | -0.59% | 109% | 106% |
| Overall | -0.44% | -1.08% | -1.30% | 103% | 102% |
| Class D | -0.45% | -2.69% | -1.97% | 104% | 100% |
| Class F | -0.52% | -1.74% | -2.02% | 106% | 106% |

Compared with NNVC-4.0 EE anchor (NnlfOption=0)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -10.18% | -16.29% | -20.35% | 156% | 55739% |
| Class A2 | -10.62% | -20.22% | -16.25% | 149% | 54636% |
| Class B | -9.58% | -22.20% | -21.39% | 153% | 55257% |
| Class C | -10.42% | -24.78% | -23.80% | 140% | 47158% |
| Class E |  |  |  |  |  |
| Overall | -10.13% | -21.31% | -20.79% | 149% | 52943% |
| Class D | -11.51% | -24.14% | -23.83% | 140% | 44222% |
| Class F | -4.83% | -14.91% | -12.87% | 213% | 30290% |

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -8.85% | -17.97% | -19.94% | 148% | 43919% |
| Class C | -10.15% | -20.71% | -21.26% | 134% | 37246% |
| Class E | -9.63% | -15.42% | -18.09% | 236% | 39352% |
| Overall | -9.48% | -18.25% | -19.92% | 161% | 40446% |
| Class D | -11.47% | -22.91% | -21.74% | 133% | 33695% |
| Class F | -5.68% | -13.76% | -12.25% | 188% | 19107% |

EncNnlfOpt=0

Compared with NNVC-4.0 filter set0 (NnlfOption=1)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.09% | -0.13% | 101% | 99% |
| Class A2 | -0.18% | -0.50% | -0.39% | 103% | 100% |
| Class B | -0.17% | -0.29% | -0.52% | 102% | 101% |
| Class C | -0.18% | -0.47% | -0.42% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.34% | -0.39% | 102% | 100% |
| Class D | -0.21% | -0.74% | -0.58% | 104% | 101% |
| Class F | -0.13% | -0.38% | -0.37% | 106% | 106% |

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | -0.73% | -1.43% | 103% | 102% |
| Class C | -0.50% | -1.88% | -1.25% | 103% | 101% |
| Class E | -0.50% | -0.02% | -1.53% | 109% | 105% |
| Overall | -0.43% | -0.94% | -1.39% | 104% | 102% |
| Class D | -0.29% | -2.25% | -1.54% | 103% | 101% |
| Class F | -0.50% | -1.31% | -1.51% | 106% | 106% |

Compared with NNVC-4.0 EE anchor (NnlfOption=0)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -10.06% | -16.26% | -20.33% | 147% | 54875% |
| Class A2 | -10.36% | -20.22% | -16.25% | 139% | 53373% |
| Class B | -9.35% | -22.31% | -21.49% | 161% | 61729% |
| Class C | -10.08% | -24.78% | -23.83% | 142% | 51656% |
| Class E |  |  |  |  |  |
| Overall | -9.89% | -21.34% | -20.83% | 148% | 55847% |
| Class D | -11.15% | -24.32% | -23.92% | 142% | 48555% |
| Class F | -4.73% | -14.75% | -13.07% | 194% | 26770% |

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -8.70% | -18.51% | -20.21% | 156% | 48448% |
| Class C | -10.00% | -20.56% | -21.68% | 137% | 43401% |
| Class E | -9.46% | -15.74% | -18.48% | 235% | 43142% |
| Overall | -9.32% | -18.50% | -20.27% | 165% | 45369% |
| Class D | -11.22% | -22.73% | -21.96% | 138% | 39078% |
| Class F | -5.59% | -13.80% | -11.71% | 197% | 20428% |

Conclusion

This contribution reports the results of EE1-1.8, which proposed applying flipping operations to both the input and output of the filter set0 to improve the coding performance. This proposed method can provide -0.16% coding gain for RA and -0.43% coding gain for LDB with a minor running time increase. Considering its good trade-off performance, it is suggested to adopt this method into NNVC common software.

Reference

[1] Z. Xie, Y. Yu, H. Yu, D. Wang. Non-EE1: On flipping of input and output of model in NNVC filter set 0. JVET-AC0065, 29th Meeting, by teleconference, 11-20 Jan.

[2] L. Wang, S. Lin, X. Xu, S. Liu, F. Galpin. EE1-1.5: Neural network based in-loop filter with a single model. JVET-AA0088, 27th Meeting, by teleconference, 13–22 July 2022.

[3] Z. Xie, Y. Yu, H. Yu, D. Wang. EE1-1.3: On BaseQP adjustment in CNNLF. JVET-AA0122, 27th Meeting, by teleconference, 13-22 July 2022.

[4] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AC2016, 29th Meeting, by teleconference, 11-20 Jan.

[5] R. Chang, L. Wang, X. Xu, S. Liu. EE1-related: Combination of EE1-1.1 and EE1-1.2	. JVET-AC0328, 29th Meeting, by teleconference, 11-20 Jan.

Appendix

This EE also conducted the following two sub tests to explore a better trade-off between coding performance and encoder running time.

Test 1.8.3 Test results of new mode of both horizontal and vertical flipping with EncNnlfOpt=1

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.06% | 0.01% | 100% | 100% |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.13% | -0.29% | 100% | 101% |
| Class C | -0.10% | -0.36% | -0.26% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.14% | -0.41% | -0.12% | 101% | 101% |
| Class F | -0.02% | -0.06% | 0.01% | 100% | 100% |

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.40% | -1.45% | -1.11% | 102% | 101% |
| Class E | -0.20% | -0.01% | -0.55% | 105% | 104% |
| Overall |  |  |  |  |  |
| Class D | -0.23% | -1.35% | -0.57% | 101% | 101% |
| Class F |  |  |  |  |  |

Test 1.8.4 Test results of new mode of both horizontal and vertical flipping with EncNnlfOpt=0

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Considering the trade-offs of both 1.8.3 and 1.8.4 are not as good as that of 1.8.1 and 1.8.2 respectively, these results are only attached as an appendix for reference.

Test results in JVET-AC0065 are attached as following for reference.

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.03% | -0.23% | 114% | 99% |
| Class A2 | -0.24% | -0.69% | -0.39% | 114% | 99% |
| Class B | -0.15% | -0.38% | -0.42% | 114% | 99% |
| Class C | -0.23% | -0.37% | -0.65% | 111% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.18% | -0.37% | -0.44% | 113% | 100% |
| Class D | -0.24% | -0.40% | -0.56% | 112% | 102% |
| Class F | -0.14% | -0.44% | -0.43% | 118% | 106% |

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.38% | -1.43% | -1.95% | 113% | 101% |
| Class C | -0.63% | -1.50% | -2.13% | 111% | 101% |
| Class E | -0.82% | -1.50% | -1.14% | 122% | 107% |
| Overall | -0.57% | -1.46% | -2.03% | 114% | 102% |
| Class D | -0.47% | -1.59% | -2.04% | 111% | 101% |
| Class F | -0.40% | -1.59% | -1.45% | 118% | 110% |

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).