| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0074-v1 |
| --- | --- |

| Title: | EE2-1.14b Enlarge search range for small blocks in IntraTMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Lai Zhang Yue Yu Haoping Yu Dong Wang | Tel: Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2-1.14b. In this test, enlarging search range for small blocks in IntraTMP is investigated.

On top of ECM-8.0, the simulation results are reported as follows:

EE2-1.14b

AI {-0.03% Y, -0.07% U, -0.03% V, 100% EncT, 101% DecT }

RA {xx% Y, xx% U, xx% V, xx% EncT, xx% DecT }

Introduction

In ECM-8.0[1], the search range width and height of IntraTMP are set proportional to the width and height of the current block, respectively. That is:

SearchRangeWidth = a * BlkWidth

SearchRangeHight = a * BlkHeight

Where ‘a’ is set as 5 in ECM-8.0.

Proposed method

Enlarging search range for small blocks in IntraTMP was proposed in JVET-AC0068[2]. the proposed method modifies the IntraTMP search range as follows:

SearchRangeWidth = max(a * BlkWidth, minSearchRange)

SearchRangeHight = max(a * BlkHeight, minSearchRange)

In EE2-1.14b the minSearchRange is set as 64 instead of 128 at JVET-AC0068.

Test results

EE2-1.14b was implemented on top of the ECM-8.0 software and tested based on the common test condition as specified in the Exploration Experiments 2 document (JVET-AC2024 [3]). The test results are summarized below.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.10% | -0.01% | 99% | 100% |
| Class A2 | -0.02% | -0.05% | -0.02% | 100% | 101% |
| Class B | -0.05% | -0.04% | -0.02% | 101% | 102% |
| Class C | -0.02% | -0.03% | -0.09% | 100% | 101% |
| Class E | -0.02% | -0.14% | 0.01% | 100% | 100% |
| Overall | -0.03% | -0.07% | -0.03% | 100% | 101% |
| Class D | -0.01% | -0.08% | -0.04% | 100% | 99% |
| Class F | 0.04% | -0.02% | 0.03% | 100% | 101% |
| Class TGM |  |  |  |  |  |

Reference

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0

- F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang, “Non-EE2: multi-candidate IntraTMP”, JVET-AC0068, January 2023

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AC2024, January 2023

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).