| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0078-v2 |
| --- | --- |

| Title: | AHG9: On comments for the draft of VSEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh, Yukinobu Yasugi, Tomohiro Ikai | Email: | chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution reports two editorial problems and three technical problems in JVET-AC2032, the latest draft of the versatile supplemental enhancement information messages for coded video bitstreams (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7) and proposes solutions. One technical suggestion regarding Neural-network post-filter characteristics SEI is that when only chroma samples are processed, the integer variable, strengthControlScaledVal, should be derived from the chroma variable, inpTensorBitDepthC. The others technical suggestions are about Post-filter hint SEI. The equation to derive the syntax elements filter_hint_value should be improved because the normalization value may be too large, and the pixel out of the picture may be indicated. The latter problems concern not only VSEI but also HEVC and AVC.

Introduction

The revision of the versatile supplemental enhancement information messages for coded video bitstreams (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7) is underway. Four newly introduced SEI, Shutter interval information, Neural-network post-filter characteristics, Neural-network post-filter activation and Phase indication, and Post-filter hint, are added.

This contribution reports two editorial problems and three technical problems in JVET-AC2032 [1], the latest draft document, and proposes solutions. Those are included in JNB comments.

Problems and solutions

Editorial problem 1 in subsection 8.28.2

Problem

There is an editorial problem in the semantics of syntax element, nnpfc_base_flag.

Solution

“nnpfc_parameter_parameter_type_idc” should be changed to “nnpfc_parameter_type_idc”.

Editorial problem 2 in subsection 8.28.2

Problem

There is an editorial problem in the semantics of syntax element, nnpfc_out_colour_format_idc.

Solution

“4:2:4 format” should be changed to “4:4:4 format”.

Technical problem in equation (83)

Problem

There is a technical problem in the semantics of syntax element, nnpfc_auxiliary_inp_idc. When nnpfc_inp_order_idc is equal to 1, only chroma samples are processed. Therefore, when nnpfc_inp_format_idc is equal to 1, the variable strengthControlScaledVal should be derived form the variable inpTensorBitDepthC.

Solution

The equation (83) should be changed to the following.

From,

if( nnpfc_inp_format_idc  = =  1 )

strengthControlScaledVal = Floor ( StrengthControlVal * ( ( 1  <<  inpTensorBitDepthY ) − 1 ) )		(83)

else

strengthControlScaledVal = StrengthControlVal

to

if( nnpfc_inp_format_idc  = =  1 ) {

if( nnpfc_inp_order_idc  = =  0  | |  nnpfc_inp_order_idc  = =  2  | |  nnpfc_inp_order_idc  = =  3 )

strengthControlScaledVal = Floor ( StrengthControlVal * ( ( 1  <<  inpTensorBitDepthY ) − 1 ) )

else if(nnpfc_inp_order_idc  = =  1 )									(83)

strengthControlScaledVal = Floor ( StrengthControlVal * ( ( 1  <<  inpTensorBitDepthC ) − 1 ) )

} else

strengthControlScaledVal = StrengthControlVal

Technical problem 1 in equation (95)

Problem

There is a technical problem in the equation (95) of subsection 8.30.4. The value of normalization, (28+bitDepth-1)2 in the following equation (95) may not be correct.

(95)

The syntax element, filter_hint_value, will be less than one because the value of (28+bitDepth-1) is greater than the maximum value of the pixels, indicated by s and s’. However, filter_hint_value is specified as an integer.

Solution

In equation (95), (28+bitDepth-1)2 should be replaced to 4bitdepth-8.

Technical problem 2 in equation (95)

Problem

There is a technical problem in the equation (95) of subsection 8.30.4. The decoded signal s’ indexes may indicate out of the picture.

Solution

In the equation (95), s’(m+cy-OffsetY,n+cx-OffsetX) should be replaced to s’(clip3(0,h-1,m+cy-OffsetY),clip3(0,v-1,n+cx-OffsetX)). As a result, the equation (95) should be changed to the following.

filterhintvaluecIdx,cy,cx=14bitDepth-8*h*wm=0h-1n=0w-1sm,n*s'(clip3(0,h-1,m+cy-OffsetY),clip3(0,w-1,n+cx-OffsetX))	(95)

Regarding equation (95), there are the same problems of Post-filter hint SEI message semantics in AVC and HEVC. It would be better to fix the formula similarly at an appropriate time.

Acknowledgements

These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.

References

[1]	S. McCarthy, S. Deshpande, M. M. Hannuksela, Hendry, G. J. Sullivan and Y.-K. Wang, Improvements under consideration for neural network post filter SEI messages, JVET-AC2023-v2 , Jan. 2023.

[2]	Text of ISO/IEC 23002-7:202x (2nd Ed.) DAM1 Additional SEI messages, MDS21947_WG05_N00158, Oct. 2022.

Patent rights declaration

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).