| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0084-v1 |
| --- | --- |

| Title: | EE2-related: On search range of intraTMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu, Lai Zhang, Fan Wang Jonathan Gan, Haoping Yu, Luhang Xu Zhihuang Xie, Dong Wang | Email: | yue.yu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

IntraTMP employs a pre-defined template of the current block to search for a candidate template of the same shape that best matches the template of the current block within a pre-determined search area. The corresponding reference block with the best matched template is selected as the prediction block for the current block. This contribution proposes to include the bottom-left and top-right areas of the current block as additional search areas for intraTMP. In addition, this contribution proposes to restrict the intraTMP search with a specified search range for all pre-determined search areas. On top of ECM-8.0, the simulation results are reported as follows:

AI(Y/U/V): -0.21%/-0.28%/-0.27%, EncT 103%, DecT 102%

RA(Y/U/V): -0.12%/-0.08%/-0.05%, EncT 101%, DecT 100%

On top of EE2-Test1.20j, the simulation results are reported as follows:

AI(Y/U/V): -xx%/-xx%/-xx%, EncT 10x%, DecT 10x%

RA(Y/U/V): -0.xx%/-0.xx%/-0.xx%, EncT xx%, DecT xx%

# Introduction

IntraTMP employs a pre-defined template of the current block to search for a candidate template of the same shape that best matches the template of the current block within a pre-determined search area. In order to search for the best template, a cost function, e.g., SAD is defined and both the encoder and decoder use the same search strategy to obtain the best matched block. Once the best matched block is found, the corresponding reference block will be selected as the prediction block for the current block.

The search area used in ECM-8.0 is shown on the left of Figure 1. EE2-Test1.14a adds two more search areas which are directly above and directly left of the current block. These two areas are marked with crosshatch shading shown on the right of Figure 1.

Fig.1. Left: the search areas used in ECM-8.0; Right: the search areas used in EE2-Test1.14a

# Proposed method

The bottom-left and top-right areas relative to the current block may also be available and they are very close to the current block, this contribution proposes to include these two available areas as additional search areas for intraTMP as shown on the left of Figure 2 with crosshatch shading. In addition, the current ECM-8.0 always searches all available positions within the current CTU while it may be beyond the specified search range or width/height. This contribution further proposes to restrict all search areas with the specified search range or width/height. The search areas with proposed search range restrictions are illustrated on the right of Figure 2.

Fig.2. Left: bottom-left and top-right added as additional search areas for intraTMP

Right: all available search areas restricted by specified search width/height range

# Simulation results

The proposed bottom-left and top-right areas are implemented, as additional search areas on top of the ECM-8.0 software [1], and tested based on the common test condition [2]. The test results are summarized in Table 1.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.09% | -0.08% | 106% | 100% |
| Class A2 | -0.40% | -0.57% | -0.53% | 106% | 106% |
| Class B | -0.30% | -0.29% | -0.27% | 102% | 102% |
| Class C | -0.07% | -0.14% | -0.15% | 102% | 101% |
| Class E | -0.21% | -0.35% | -0.36% | 102% | 101% |
| Overall | -0.21% | -0.28% | -0.27% | 103% | 102% |
| Class D | -0.08% | -0.35% | -0.26% | 101% | 101% |
| Class F | 0.02% | -0.03% | 0.07% | 103% | 100% |
| Class TGM | -0.18% | -0.17% | -0.13% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.07% | 101% | 100% |
| Class A2 | -0.21% | -0.16% | -0.09% | 101% | 100% |
| Class B | -0.22% | -0.09% | -0.08% | 100% | 100% |
| Class C | -0.02% | -0.05% | 0.01% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.08% | -0.05% | 101% | 100% |
| Class D | -0.02% | -0.07% | -0.10% | 100% | 100% |
| Class F | -0.02% | -0.57% | -0.44% | 100% | 100% |
| Class TGM | -0.07% | -0.09% | -0.07% | 101% | 102% |

Table 1. The proposed additional search areas over ECM-8.0

Furthermore, the proposed additional search areas and the search range restrictions are both implemented on top of the EE2-Test1.20j. The test results are shown in Table 2.

Table 2. The proposed additional search areas and search range restrictions on top of the EE2-Test1.20j over ECM-8.0

In addition, Table 3 shows the test results of EE2-Test1.20j with the additional search areas and the search range restrictions, compared with EE2-Test1.20j.

Table 3. The proposed additional search areas and search range restrictions on top of the EE2-Test1.20j over EE2-Test1.20j

# Conclusions

This contribution proposes to include two additional search areas for IntraTMP. The proposed method brings 0.21% coding gain for AI and 0.12% coding gain for RA over ECM-8.0. Furthermore, the proposed method combined with search range restrictions brings 0.xx% coding gain for AI and 0.xx% coding gain for RA on top of EE2-Test1.20j with x% and x% encoding and decoding increase, respectively. It is suggested to adopt the proposed method on top of EE2-Test1.20j.

# References

- ECM8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, July, 2022.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).