| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0085-v2 |
| --- | --- |

| Title: | EE2-1.24: Combinations of MPM related tests |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu Yue Yu Haoping Yu Dong Wang (OPPO) Chuan Zhou Zhuoyi Lv Jinrong Zhang (vivo) Hongtao Wang Yao-Jen Chang Vadim Seregin Marta Karczewicz (Qualcomm) | Tel: Email: | xuluhang@oppo.com chuan.zhou@vivo.com hongtaow@qti.qualcomm.com |
| Source: | OPPO, vivo Mobile Communication Co., Ltd, Qualcomm Incorporated |  |  |

_____________________________

- Abstract

This contribution reports test results of EE2-1.24, where combinations of MPM related modifications are further investigated. These combination tests were implemented on top of ECM-8.0 reference software. The summarized results are shown as follows.

EE2 Test 1.24a on combined EE2 Test 1.21c and 1.22.

AI: -0.19%/-0.20%/-0.19% Y/U/V, 100%/102% EncT/DecT,

RA:-0.11%/-0.10%/-0.04% Y/U/V, 100%/100% EncT/DecT.

EE2 Test 1.24b on combined EE2 Test 1.21 and 1.23.

AI: -0.12%/-0.11%/-0.09% Y/U/V, 102%/111% EncT/DecT,

RA:

EE2 Test 1.24c on combined EE2 Test 1.22 and 1.23.

AI: -0.08%/-0.10%/-0.03% Y/U/V, 101%/103% EncT/DecT,

RA:

EE2 Test 1.24d on combined EE2 Test 1.21c, 1.22 and 1.23.

AI: -0.19%/-0.17%/-0.13% Y/U/V, 101%/104% EncT/DecT,

RA:

Introduction

Description of EE2 Test 1.21c

In EE2 Test 1.21c, two aspects (EE2-1.21a and EE2-1.21b) are proposed to improve MPM and TMRL prediction.

Aspect-1. the construction of intra candidate list for MPM and TMRL is modified as follows,

- Non-adjacent positions are added as candidates in constructing the intra candidate list.

- If the blocks are coded by SGPM or GPM at the neighboring or non-adjacent positions, the intra modes of the blocks are replaced by partitioning angles.

Aspect-2. the precision of angular prediction is extended from 65 to 129.

Description of EE2 Test 1.22

In EE2 Test 1.22, the intra prediction mode candidates derived by the method in Test 1.21c are sorted in ascending order of SAD cost. Up to 5 modes with the smallest SAD cost are selected. The SAD cost is computed between the prediction and the reconstruction samples of the template.

For the signaling part, MPM list is equally divided into four groups and the group index is parsed first. Then, a mode index is further parsed to indicate which mode in the selected group is used.

Description of EE2 Test 1.23

In EE2 Test 1.23, MPM derivation is performed as follows:

- For spatial neighbors of the current block, if a neighbor block is coded by MIP/IntraTMP/Planar mode, a derived angular mode is used for the MPM generation. The derived modes are tagged as ‘reserved’ for the partial MPM reordering performed in ECM-8.0.

- Secondary MPM modes are derived by adding offset to all primary angular modes, this process is the same as in ECM-8.0 for TMRL modes, where the offset is added to DIMD modes first.

- If MPM list is incomplete, the same default modes are added as in ECM-8.0.

Proposed method

The following four combinations are investigated in this proposal.

| # | Test |
| --- | --- |
| 1.24a | Test 1.21c + Test 1.22 |
| 1.24b | Test 1.21c + Test 1.23 |
| 1.24c | Test 1.22c + Test 1.23 |
| 1.24d | Test 1.21c + Test 1.22 + Test 1.23 |

Experimental Results

The proposed method was implemented on ECM-8.0. The summarized simulation results are as follow,

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EE2-1.24a over ECM-8.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.01% | -0.10% | 100% | 101% | -0.02% | -0.01% | 0.02% | 100% | 101% |
| Class A2 | -0.17% | -0.18% | -0.11% | 100% | 101% | -0.13% | -0.03% | 0.11% | 100% | 101% |
| Class B | -0.22% | -0.23% | -0.16% | 100% | 102% | -0.12% | -0.13% | -0.10% | 100% | 100% |
| Class C | -0.27% | -0.25% | -0.27% | 99% | 104% | -0.17% | -0.20% | -0.14% | 100% | 100% |
| Class E | -0.16% | -0.30% | -0.28% | 100% | 103% |  |  |  |  |  |
| Overall | -0.19% | -0.20% | -0.19% | 100% | 102% | -0.11% | -0.10% | -0.04% | 100% | 100% |
| Class D | -0.20% | -0.21% | -0.11% | 100% | 105% | -0.03% | 0.07% | -0.14% | 100% | 100% |
| Class F | -0.17% | -0.26% | -0.20% | 102% | 103% | -0.10% | -0.50% | -0.35% | 100% | 99% |

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EE2-1.24b over ECM-8.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.08% | 0.05% | 101% | 101% |  |  |  |  |  |
| Class A2 | -0.13% | -0.06% | -0.06% | 102% | 104% |  |  |  |  |  |
| Class B | -0.18% | -0.15% | -0.16% | 101% | 104% | -0.07% | -0.06% | -0.19% | 101% | 100% |
| Class C | -0.18% | -0.09% | -0.21% | 101% | 104% | -0.10% | -0.16% | -0.06% | 101% | 100% |
| Class E | -0.04% | -0.13% | 0.00% | 100% | 103% |  |  |  |  |  |
| Overall | -0.12% | -0.11% | -0.09% | 101% | 103% |  |  |  |  |  |
| Class D | -0.07% | -0.26% | -0.04% | 101% | 105% | 0.03% | 0.10% | 0.14% | 101% | 100% |
| Class F | 0.00% | -0.21% | -0.16% | 100% | 102% | -0.05% | -0.53% | -0.08% | 100% | 100% |

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EE2-1.24c over ECM-8.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.04% | 0.05% | 101% | 101% |  |  |  |  |  |
| Class A2 | -0.06% | -0.01% | 0.04% | 99% | 102% |  |  |  |  |  |
| Class B | -0.05% | -0.07% | 0.06% | 101% | 103% | -0.04% | -0.18% | -0.11% | 100% | 100% |
| Class C | -0.09% | -0.19% | -0.15% | 101% | 105% | -0.06% | -0.24% | -0.11% | 100% | 101% |
| Class E | -0.15% | -0.19% | -0.19% | 101% | 104% |  |  |  |  |  |
| Overall | -0.08% | -0.10% | -0.03% | 101% | 103% |  |  |  |  |  |
| Class D | -0.15% | -0.22% | -0.16% | 101% | 106% | -0.04% | -0.13% | -0.53% | 100% | 100% |
| Class F | -0.08% | -0.18% | -0.13% | 100% | 103% | -0.01% | -0.34% | -0.18% | 100% | 100% |

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EE2-1.24d over ECM-8.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.09% | -0.01% | 101% | 103% |  |  |  |  |  |
| Class A2 | -0.18% | -0.03% | -0.11% | 101% | 103% |  |  |  |  |  |
| Class B | -0.23% | -0.16% | -0.14% | 101% | 104% | -0.10% | -0.10% | -0.13% | 100% | 100% |
| Class C | -0.26% | -0.29% | -0.23% | 101% | 106% | -0.11% | -0.23% | -0.21% | 100% | 101% |
| Class E | -0.18% | -0.28% | -0.10% | 100% | 105% |  |  |  |  |  |
| Overall | -0.19% | -0.17% | -0.13% | 101% | 104% |  |  |  |  |  |
| Class D | -0.20% | -0.27% | -0.26% | 100% | 109% | -0.08% | 0.04% | -0.08% | 100% | 100% |
| Class F | -0.14% | -0.14% | -0.05% | 100% | 104% | -0.09% | -0.34% | -0.42% | 100% | 100% |

Conclusions

References

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-8.0.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).