| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0086-v2 |
| --- | --- |

| Title: | EE2-1.20i/j: Combination of IntraTMP tests |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang, Lai Zhang, Yue Yu, Haoping Yu, Dong Wang (OPPO) Yanzhuo Ma, Hongli Zhang, Junyan Huo Fuzheng Yang (Xidian Univ.) Po-Han Lin, Jian-Liang Lin, Vadim Seregin, Marta Karczewicz (Qualcomm Incorporated) Xinwei Li, Ru-Ling Liao, Jie Chen, Yan Ye (Alibaba group) | Tel: Email: | wangfan6@oppo.com yzma@mail.xidian.edu.cn pohanlin@qti.qualcomm.com sid.lxw@alibaba-inc.com |
| Source: | OPPO, Xidian Univ., Qualcomm Incorporated, Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the test results of combination tests EE2-1.20i/j. EE2-1.20i is a combination test of Test 1.10 + Test 1.11 + Test 1.12 + Test 1.15 + Test 1.16 + Test 1.19 + Test 1.14a. EE2-1.20j is a combination test of Test 1.10 + Test 1.11 + Test 1.12 + Test 1.15 + Test 1.16 + Test 1.19 + Test 1.14c.

On top of ECM-8.0, simulation results are reported as follows:

EE2-1.20i

AI {-0.92% Y, -0.98% U, -0.95% V, xx% EncT, xx% DecT }

RA {xx% Y, xx% U, xx% V, xx% EncT, xx% DecT }

EE2-1.20j

AI {-1.02% Y, -1.00% U, -1.01% V, xx% EncT, xx% DecT }

RA {xx% Y, xx% U, xx% V, xx% EncT, xx% DecT }

Introduction

In EE2-1.10 [1], Multi-candidate IntraTMP is investigated. In the proposed multi-candidate IntraTMP method, a candidate list is constructed with the candidate BVs ranked in ascending order of their template matching costs, and the index of the selected candidate is signalled in the bitstream.

In EE2-1.11 [2], an IntraTMP fusion method is investigated. The proposed method derives multiple IntraTMP matched blocks and then fuses them to produce a better overall prediction.

In EE2-1.12 [3], sub-pel precision is supported in IntraTMP. The sub-pel positions around the integer-pel position obtained by template matching are further enabled to IntraTMP. The DCT-IF filter is used for sub-pel interpolation.

In EE2-1.15 [4], IntraTMP with model derived prediction block is studied. Parameters of a model are derived using a template of the current block and a corresponding matching template. The prediction block is obtained by applying the model to filter the reference block.

In EE2-1.16 [5], an IntraTMP fusion method is proposed that blends multiple reference blocks to derive the final prediction block, with Wiener-filter based weight derivation method. These reference blocks are obtained by Block Vectors (BVs) in template matching search process.

In EE2-1.19 [6], it is proposed to introduce 3 additional IntraTMP modes: left template, above template and L-shape fusion modes. The left and above template modes use only left side or above side to derive the template matching candidates. The fusion mode fuses the best 2 or the best 5 L-shape  candidates by a template matching cost based or MSE minimization based linear combination formula.

In EE2-1.14 [7], the search range of IntraTMP is investigated for the 2 aspects shown below:

- EE2-1.14a: Extended the adjacent search area of IntraTMP

- EE2-1.14b: Enlarge the search range for small blocks

Proposed method

The proposed syntax change is as follows:

…

intra_tmp_flag

if(intra_tmp_flag) {

intra_tmp_fusion_flag

if(intra_tmp_fusion_flag) {

intra_tmp_fusion_weight_type

intra_tmp_fusion_idx

} else{

intra_tmp_idx

intra_tmp_filter_flag

if(!intra_tmp_filter_flag) {

intra_tmp_sub_pel_precision_idx

if(intra_tmp_sub_pel_precision_idx != 0) {

intra_tmp_sub_pel_direction_idx

}

}

}

}

…

intra_tmp_flag indicates whether the intra prediction type for current block is IntraTMP or not.

intra_tmp_fusion_flag indicates whether fusion is used or not for current block.

intra_tmp_fusion_idx specifies the candidate set used for IntraTMP fusion. The range of intra_tmp_fusion_idx is 0 to 2, and intra_tmp_fusion_idx is used to indicate one of the three candidate sets {BV0 to BV4}, {BV5 to BV9}, {BV10 to BV14}.

intra_tmp_fusion_weight_type indicates whether the SAD based weight derivation method or the Wiener-filter based weight derivation method is used.

intra_tmp_idx specifies the index of BV in the candidate list used for current block. The range of intra_tmp_idx is 0 to 18. Candidates from L-shape template, top template and left template are included in the same candidate list.

intra_tmp_sub_pel_precision_idx specifies the precision index for current block. The range of intra_tmp_sub_pel_precision_idx is 0 to 3, used to indicate integer-pel precision, 1/2-pel precision, 1/4-pel precision and 3/4-pel precision, respectively.

intra_tmp_sub_pel_direction_idx specifies the sub-pel direction index for current block. The range of intra_tmp_sub_pel_phase_idx is 0 to 7.

The difference between 1.20i and 1.20j is that 1.20i uses 1.14a while 1.20j uses 1.14c.

Simulation results

EE2-1.20i/j were implemented on top of the ECM-8.0 [8] software and tested based on the common test condition as specified in the Exploration Experiments 2 document (JVET-AC2024 [9]). The test results are summarized below.

Table 1. EE2-1.20i on top of ECM-8.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.13% | -0.10% |  |  |
| Class A2 | -1.32% | -1.51% | -1.51% |  |  |
| Class B | -0.94% | -0.99% | -0.85% |  |  |
| Class C | -0.40% | -0.49% | -0.33% |  |  |
| Class E | -1.99% | -1.93% | -2.21% |  |  |
| Overall | -0.92% | -0.98% | -0.95% |  |  |
| Class D | -0.63% | -0.76% | -0.81% |  |  |
| Class F | -0.30% | -0.33% | -0.31% |  |  |
| Class TGM |  |  |  |  |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

Table 2. EE2-1.20j on top of ECM-8.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.11% | 0.04% |  |  |
| Class A2 | -1.37% | -1.57% | -1.60% |  |  |
| Class B | -1.03% | -0.98% | -1.01% |  |  |
| Class C | -0.46% | -0.29% | -0.44% |  |  |
| Class E | -2.31% | -2.27% | -2.27% |  |  |
| Overall | -1.02% | -1.00% | -1.01% |  |  |
| Class D | -0.69% | -0.91% | -0.71% |  |  |
| Class F | -0.45% | -0.57% | -0.64% |  |  |
| Class TGM |  |  |  |  |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

Conclusions

It is recommended to adopt 1.20j in ECM.

Reference

- F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang, “EE2-1.10 multi-candidate IntraTMP”, JVET-AD0073, April 2023.

- L. Zhang, F. Wang, Y. Yu, H. Yu, D. Wang, “EE2-1.11: Intra template matching prediction fusion”, JVET-AD0072, April 2023.

- X. Li, R.-L. Liao, J. Chen, Y. Ye, “EE2-1.12: Intra TMP with sub-pel precision”, JVET-AD0125, April 2023.

- J.-Y. Huo, W.-H. Qiao, X. Hao, Z.-Y. Zhang, H.-Q. Du, Y.-Z. Ma, F.-Z. Yang, “EE2-1.15a: Intra template matching (Intra TMP) based on linear filter model”, JVET-AD0112, April 2023.

- J.-Y. Huo, H.-Q. Du, H.-L. Zhang, W.-H. Qiao, Y.-Z. Ma, F.-Z. Yang, “EE2-1.16: A Fusion method of Intra Template Matching Prediction(Intra TMP)”, JVET-AD0116, April 2023.

- P.-H. Lin, J.-L- Lin, V. Seregin, M. Karczewicz, “EE2-1.19: IntraTMP with multiple modes”, JVET-AD0194, April 2023.

- Y. Ma, J. Huo, H. Du, H. Zhang, Z. Zhang, W. Qiao, F. Yang (Xidian), F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO), “EE2-1.14c combination test of EE2-1.14a and EE2-1.14b”, JVET-AD0075, April 2023.

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AC2024, January 2023

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).