| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0101-v1 |
| --- | --- |

| Title: | Additional conformance tests of scaling window functionality for multilayer coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Email: | iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution proposes additional conformance test of scaling window functionality for multilayer coding. In 28th JVET meeting, it was decided to issue additional conformance bitstreams for VVC multilayer configurations. Content layering is one of the practical multilayer coding use cases reported in JVET-AB0087, where main content is coded as base layer and main content with overlaying sub-content is coded as enhancement layer. When the main content is scaled down and the L-shaped content is placed in the margin as the input for the enhancement layer, scaling window functionality might work quite efficiently. The proposed conformance bitstreams are attached in the contribution and the merge request for the software package for supporting scaling window will be submitted during meeting.

Introduction

In 28th JVET meeting, it was decided to issue additional conformance bitstreams for VVC multilayer configurations. So far, the test for multiple profiles applying to different OLSs, and test for profiles applying to multiple independent layers are considered as new test bitstreams.

In JVET-AB0087, some use cases using multilayer coding for broadcast and streaming applications were reported. One example is the case where main content is coded as base layer and main content with overlaying sub-content is coded as enhancement layer, which is later named as “Content layering”. The variation of the sub-content in content layering application is L-shaped sub-content, where the main content is scaled down and L-shaped sub-content is placed in the margin space as shown in Figure 1. For coding of L-shaped sub-contents, multilayer coding with scaling window functionality might be quite efficient.

Figure 1. Example of L- shape sub-content

However, this use case is missing in the current draft of additional conformance bitstreams for VVC multilayer configuration, so that this contribution proposes addition of the conformance bitstreams using scaling window functionality as described in the following section.

Proposed conformance bitstream

Test bitstream ScalingWindowA_NHK

Specification: The bitstream is coded with two layers, where base layer is coded with scaling window disabled and enhancement layer is coded with scaling window enabled.

Functional stage: High-level syntax.

Purpose: Check that the decoder supporting scaling window.

Test bitstream ScalingWindowB_NHK

Specification: The bitstream is coded with two layers, where base layer is coded with scaling window enabled and enhancement layer is coded with scaling window disabled.

Functional stage: High-level syntax.

Purpose: Check that the decoder supporting scaling window.

Test bitstream ScalingWindowC_NHK

Specification: The bitstream is coded with two layers, where both base and enhancement layers are coded with scaling window enabled. The sizes of scaling windows are different.

Functional stage: High-level syntax.

Purpose: Check that the decoder supporting scaling window.

The proposed conformance bitstreams are generated by VTM-18.0 with the modification for supporting scaling window and are attached in the contribution. The merge request for the software package for supporting scaling window is planned to be submitted after the meeting.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).