| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0105-v1 |
| --- | --- |

| Title: | Modifications of common test condition and ECM software |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Tomohiro Ikai | Tel: Email: | yasugi.yukinobu@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

The encoding time required for the ECM experiments increases with each version, and the time required for QP=22 is particularly long and slowing down the development. On the other hand, the decoding time of the ECM software reaches several times longer compared to that of VTM, but it is observed that unnecessary processing takes a non-trivial part. This contribution proposes two modifications to speed up the encoder and decoder: the first is to modify CTC so that the sign prediction parameter (NumSignPred) is set to a smaller value for experiments with QP=22. The second is to change the ECM software to avoid unnecessary initializations of large buffers in release builds. These changes are reported to show the following experimental results under the All-Intra condition:

Test1 (CTC change): BD-rate(Y,U,V)= 0.01%,0.01%,0.00%  ET,DT=98%,98%

Test2 (ECM modification): BD-rate(Y,U,V)= 0.00%,0.00%,0.00%  ET,DT=99%,94%

Test1+2(combination): BD-rate(Y,U,V)= 0.01%,0.01%,0.00%  ET,DT=97%,92%

Introduction

The time required for experiments using the ECM software increases with each version. Figure 1 shows encoding times of ECM-8.0 under the All-Intra condition in our computer cluster. The DaylightRoad2 sequence with QP 22 is the most time-consuming experiment of all and it took more than 229 hours (9.5 days). There are often situations where we are waiting for this experiment to be completed, and improving this would accelerate the development cycle.

Figure 1: Encoding time in ECM-8.0 (All Intra)

While the decoding time is important criteria for coding algorithm development, the current ECM’s decoding time may not particularly accurate due to unnecessary initialization overhead.

Proposal

This contribution proposes modifications of the CTC and ECM software from two aspects.

Aspect 1: CTC modification

The encoding parameter NumSignPred given to sign prediction is fixed to 8 under the CTC. Since the smaller the value of NumSignPred, the shorter the processing time, we suggest a smaller value for NumSignPred in QP22 experiments. According to our experiments, setting NumSignPred to 4 can significantly reduce the runtime with QP22 while maintaining almost the same coding efficiency.

Aspect 2: ECM software modification

The ECM software uses a number of memset functions to initialize buffers. We have found that some of them are done just to be sure. If the buffer size is in kilobytes, a lot of CPU time is required for initialization. Logically, some of those initializations are unnecessary because if the software is working correctly, each element in these buffers should only be accessed after the values have been derived and stored.

However, these careful initializations are sometimes useful for debugging. Therefore, it is preferable to modify such initializations to be used only in a debug build. Specifically, it is recommended to enclose them with the _DEBUG macro. It was found that this significantly reduce decoder runtime with no impact on coding efficiency. This would give a more accurate measure of the complexity of the ECM software.

Experimental results

Test 1 (Aspect 1)

Only the experiments for QP22 were performed; results for the other QPs are identical to the anchors.

Table 1: Experimental results of Test 1 (NumSignPred=4 for QP22)

Table 1 shows the experimental results of test 1. Note that the data except for QP22 are identical to the anchor, so that EncT and DecT, which are averages over al QPs, are smaller than the actual effect at QP22.

Table 2: Encoding and decoding times for QP22 over ECM-8.0

Table 2 shows the ratio of encoding and decoding times only for QP22. For encoding, the run time reduction is 6-11% for AI and 2-3% for RA. For decoding, the effect is 3-10% for AI and 1-3% for RA.

Test 2 (Aspect 2)

Table 3: Experimental results of Test 2 (memset reduction)

Table 3 shows the experimental results of test 2. The reduction of memset functions did not affect coding efficiency at all, while reducing the run time, especially decoding under the All-Intra condition.

Test 1+2 (Combination of aspect1 and 2)

Table 4: Experimental results of Test 1+2 (NumSignPred=4 for QP22 and memset reduction)

Table 4 shows the experimental results of test 1+2. Note that the values of BD-rate are identical to test 1.

Table 5: Encoding and decoding times for QP22 over ECM-8.0

Table 5 shows the ratio of encoding and decoding times for QP22 in test1+2. According to the results of Test 1+2, on average, the reduction in run time is additive. The encoding time of DaylightRoad2 QP22 was 206 hours, which is almost a day (23 hours) less than the anchor.

Conclusion

We suggest the followings:

- Modify the CTC to use NumSignPred=4 for QP22 experiments.

- Make changes to the ECM software to disable unnecessary memset functions in release builds.

These changes are expected to reduce the encoder run time by an average of 9% in QP22 cases, resulting in an accelerated development cycle. In addition, the reduction in decoder run time would lead to more accurate assessment of complexity.

Acknowledgements

These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).