| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0107-v1 |
| --- | --- |

| Title: | EE1-related: Simplified parameter selection for filter set #1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Kai Zhang, Li Zhang | Tel: Email: | yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a simplified parameter selection method for filter set #1 to reduce its encoder complexity while maintaining the performance. In particular, the number of parameters for selection is reduced from 3 to 2, and the derivation of the second parameter is dependent on temporal layer id.

On top of NNNC-4.0 with NN tools enabled (abbreviated as NNVC-4.0-NnIntra-NnFilterSet1) and NNVC-4.0 with NN tools disabled (abbreviated as NNVC-4.0-Anchor), the proposed method shows BD-rate and complexity changes reported as below:

Compared with NNVC-4.0-NnIntra-NnFilterset1 (test is NN-based Intra + NN-based filter set #1 + proposed simplification):

RA: 0.01%, 0.29%, 0.28%, EncT 87%, DecT 98%

LDB: 0.01%, 0.10%, 0.02%, EncT 84%, DecT 97%

Compared with NNVC-4.0-Anchor (test is NN-based Intra + NN-based filter set #1 + proposed simplification):

RA: -11.01%, -21.62%, -22.28%, EncT 167%, DecT 31375%

LDB: -8.59%, -20.06%, -21.10%, EncT 172%, DecT 28738%

Introduction

NNVC common software includes two sets of deep in-loop filter, known as filter set #1 and filter set #0 respectively [1]. In filter set #1, each picture or block could determine whether to apply the NN-based in-loop filter or not. When the NN-based filter is determined to be applied to a picture or a block, the QP parameter is selected from a candidate list, which contains three QP parameters by default.

Proposed method

To lower encoder complexity and maintain performance, this contribution proposes to use two parameter candidates by default and derive the second parameter based on the temporal layer id.

Denote the sequence level QP as q, the candidate list includes QP parameters {Param_1, Param_2}. For low temporal layers, Param_1 = q, Param_2 = q-5. For high temporal layers, Param_1 = q, Param_2 = q+5. Note that other designs of the proposed method such as NN filter model, residual scaling, combination with deblocking, etc. remain the same as the existing NN-based filter set #1.

Experimental results

The proposed simplified parameter selection method is integrated into NNVC-4.0 (filter set #1) [2] and tested according to the common test conditions defined in [3].

BD-rate changes of {Y, Cb, Cr} compared with NNVC-4.0-NnIntra-NnFilterSet1 and NNVC-4.0-Anchor are summarized in Tables below. Note test is NN-based Intra + NN-based filter set #1 + proposed simplification for all cases.

Compared with NNVC-4.0-NnIntra-NnFilterSet1, the proposed method brings on average {0.01%, 0.29%, 0.28%} and {0.01%, 0.10%, 0.02%} BD-rate changes for {Y, Cb, Cr} under RA, LB. The encoding time reductions for RA and LB are 13% and 16% respectively.

Compared with NNVC-4.0-Anchor, the proposed method brings on average {-11.01%, -21.62%, -22.28%}, and {-8.59%, -20.06%, -21.10%} BD-rate changes for {Y, Cb, Cr} under RA and LB configurations.

Anchored on NNVC-4.0-NnIntra-NnFilterSet1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0-NnIntra-NnFilterSet1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.01% | 0.21% | 0.32% | 86% | 98% |
| Class A2 | 0.02% | 0.22% | 0.36% | 87% | 99% |
| Class B | 0.00% | 0.35% | 0.21% | 86% | 98% |
| Class C | 0.01% | 0.31% | 0.29% | 88% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.29% | 0.28% | 87% | 98% |
| Class D | -0.01% | 0.57% | 0.68% | 89% | 98% |
| Class F | -0.04% | 0.11% | 0.18% | 82% | 91% |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over NNVC-4.0-NnIntra-NnFilterSet1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.07% | 0.20% | 86% | 98% |
| Class C | 0.06% | 0.30% | 0.52% | 86% | 97% |
| Class E | 0.03% | -0.13% | -0.97% | 78% | 95% |
| Overall | 0.01% | 0.10% | 0.02% | 84% | 97% |
| Class D | -0.07% | 0.01% | 0.67% | 88% | 100% |
| Class F | 0.02% | 0.33% | 0.55% | 82% | 94% |

Anchored on NNVC-4.0-Anchor

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0-Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -10.52% | -17.54% | -21.94% | 176% | 33497% |
| Class A2 | -11.00% | -20.62% | -16.65% | 170% | 32371% |
| Class B | -10.54% | -22.75% | -23.55% | 172% | 33202% |
| Class C | -11.98% | -24.03% | -25.19% | 152% | 27186% |
| Class E |  |  |  |  |  |
| Overall | -11.01% | -21.62% | -22.28% | 167% | 31375% |
| Class D | -13.89% | -24.65% | -27.91% | 141% | 24446% |
| Class F | -6.45% | -14.96% | -13.13% | 209% | 13204% |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over NNVC-4.0-Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.83% | -20.08% | -20.96% | 160% | 33183% |
| Class C | -9.47% | -21.78% | -21.86% | 143% | 29175% |
| Class E | -8.66% | -17.72% | -20.33% | 245% | 22163% |
| Overall | -8.59% | -20.06% | -21.10% | 172% | 28738% |
| Class D | -11.21% | -24.97% | -27.35% | 131% | 24278% |
| Class F | -5.57% | -13.80% | -10.82% | 202% | 15626% |

Conclusions

This contribution presents a simplified parameter selection method for filter set #1. The proposed filtering method shows a favorable trade-off in terms of the coding performance and complexity. It is suggested to adopt the proposed method into NNVC common software.

References

- Y. Li, H. Wang, L. Wang, F. Galpin, J. Ström, Description of algorithms and software in neural network-based video coding (NNVC) version 2, JVET-AC2019, by teleconference, January 2023.

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NNVC-4.0

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AC2016, by teleconference, January 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).