| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0118-v3 |
| --- | --- |

| Title: | EE2-1.20h: Combination of EE2-1.14a, EE2-1.15a and EE2-1.16 for Intra TMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Nengfu Qiu, Hongli Zhang, Yanzhuo Ma, Fuzheng Yang, Hui Yuan | Email: | jyhuo@mail.xidian.edu.cn nfqiu@stu.xidian.edu.cn hlz@stu.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn huiyuan@sdu.edu.cn |
| Source: | Xidian University, Shandong University. |  |  |

_____________________________

- Abstract

This contribution reports on experiment EE2-1.20h on Combination of EE2 tests: Test 1.14a, Test 1.15a and Test 1.16.

The experimental results on top of ECM-8.0 are as below:

AI(Y/U/V): -0.46%/-0.54%/-0.52%, EncT 100%, DecT 103%

RA(Y/U/V): -0.16%/-0.10%/-0.16%, EncT xx%, DecT xx%

Introduction

In ECM8.0[1], IntraTMP is enabled for both camera-captured videos and screen contents. For a predefined search range, the encoder searches for the most similar template to the current template in a reconstructed part of the current frame and uses the corresponding block as a prediction block.

In EE2-1.14a[2], it is proposed to extend R3 and R4 regions due to the vertical upper side and horizontal left side of the current block have also been reconstrcted. It is assumed that the adjacent regions of the coding block has a higher spatial correlation with the coding block. Fig. 1 depicts the extended regions based on IntraTMP.

Fig.1 The extended regions based on IntraTMP

In EE2-1.15a[3], it is proposed to apply a linear filter model based Intra TMP (Intra TMP-FLM) to improve effecience. The proposed 6-tap linear filter to improve Intra TMP prediction. The filter input consists of 5 spatial luma samples in the reference block and a bias term. Filter coefficients are derived for each block separately using regression based MSE minimization on samples between reference template and current template. Template size and shapes are same as in intraTMP, the template size used for training is 4 lines above and to the left of the current block depending on their availability.

In EE2-1.16[4], a IntraTMP Fusion method is proposed. Fig. 2 depicts the procedure of IntraTMP Fusion.

Fig.2 The procedure of IntraTMP Fusion

First, the BVs corresponding to N matching templates are searched with step size 2; then, the N candidate reference blocks corresponding to the N candidate matching templates are obtained through BVs; last, the candidate reference blocks are fused according to weights as the prediction of the current block. It is worth noting that the final candidate quantity N is determined by the number of valid candidates actually found, in which N is greater than or equal to 1 as well as less than or equal to 3.

The fusion method is proposed as follows:

predSamples=n=0N-1wn*refBlockn+wN*midValue

where predSamples represent the final predicted block, refBlockn is one of the N candidate reference blocks. For 10-bit content, midValue is 512. The weights wn are derived from candidate matching templates and current template using gaussian solver method used in CCCM.

In ECM8.0, usage of the IntraTMP is signalled with a CU level flag. Specifically, the new IntraTMP Fusion method is considered as a sub-mode of IntraTMP and is signalled with IntraTMP_Fusion_Flag.

Proposed Method

In this contribution,It proposes a combination of EE2-1.14a, EE2-1.15a and EE2-1.16, that is,

- It is proposed to apply a linear filter model based Intra TMP (Intra TMP-FLM).

- It proposes a IntraTMP Fusion method that blends reference blocks obtained by Block Vectors(BVs) in template matching search process.

- Usage of the Intra TMP-FLM and Intra TMP Fusion are signalled coded CU level flag. When they come to signalling, Intra TMP-FLM and Inra TMP Fusion are considered as sub-mode of Intra TMP.

- It is proposed to extend R3 and R4 regions of Intra TMP.

Experimental results

The proposed method was implemented based on ECM-8.0 under common test conditions [5].

The results of All Intra and Random Access configuration are summarized in the Table 1. More detail results can be found in the accompanying Excel tables.

Table 1. Simulation results of the proposed method

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.16% | -0.09% | 99% | 101% |
| Class A2 | -0.77% | -0.93% | -0.88% | 99% | 104% |
| Class B | -0.49% | -0.55% | -0.49% | 101% | 104% |
| Class C | -0.20% | -0.23% | -0.27% | 101% | 101% |
| Class E | -0.85% | -0.92% | -0.96% | 101% | 103% |
| Overall | -0.46% | -0.54% | -0.52% | 100% | 103% |
| Class D | -0.41% | -0.65% | -0.49% | 101% | 100% |
| Class F | -0.04% | -0.28% | -0.07% | 101% | 102% |
| Class TGM |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.07% | -0.08% | #NUM! | #NUM! |
| Class A2 | -0.39% | -0.28% | -0.21% | #NUM! | #NUM! |
| Class B | -0.20% | -0.22% | -0.13% | #NUM! | #NUM! |
| Class C | -0.05% | 0.07% | -0.23% | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.10% | -0.16% | #NUM! | #NUM! |
| Class D | -0.04% | 0.04% | -0.02% | #NUM! | #NUM! |
| Class F | -0.11% | -0.48% | -0.23% | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusions

This contribution proposed a combination of EE2-1.14a, EE2-1.15a and EE2-1.16 for Intra TMP. Compared to ECM-8.0, the contribution achieves BD-rate saving of {-0.46% (Y), -0.54% (U), -0.52% (V)} for the AI configuration. It is recommended to include the proposed method into the next version of ECM.

- 5 References

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- J.-Y. Huo, H.-Q. Du, H.-L. Zhang, Z.-Y. Zhang, W.-H. Qiao, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), “EE2-1.14a: Extend search area in Intra Template Matching Prediction(IntraTMP)”, JVET-AD0115, April 2023

- J.-Y. Huo, W.-H. Qiao, X. Hao, Z.-Y. Zhang, H.-Q. Du, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), “EE2-1.15a: Intra template matching (Intra TMP) based on linear filter model”, JVET-AD0112, April 2023

- J.-Y. Huo, H.-Q. Du, H.-L. Zhang, W.-H. Qiao, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), “EE2-1.16: A Fusion method of Intra Template Matching Prediction(Intra TMP)”, JVET-AD0116, April 2023

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

- V. Seregin, J. Chen etc, “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AC2024, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 29th Meeting: by teleconference, January 2023.

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).